H5 Site condition report consultation number 24056

Environmental permitting: H5 Site condition report – Proposals for updated guidance

Date: November 2025

We are the Environment Agency. We protect and improve the environment.

We help people and wildlife adapt to climate change and reduce its impacts, including flooding, drought, sea level rise and coastal erosion.

We improve the quality of our water, land and air by tackling pollution. We work with businesses to help them comply with environmental regulations. A healthy and diverse environment enhances people's lives and contributes to economic growth.

We can’t do this alone. We work as part of the Defra group (Department for Environment, Food & Rural Affairs), with the rest of government, local councils, businesses, civil society groups and local communities to create a better place for people and wildlife.

Published by:

Environment Agency
Horizon House, Deanery Road,
Bristol BS1 5AH

www.gov.uk/environment-agency

© Environment Agency 2025

All rights reserved. This document may be reproduced with prior permission of the Environment Agency.

Further copies of this report are available from our publications catalogue: www.gov.uk/government/publications or our National Customer Contact Centre: 03708 506 506

Email: enquiries@environment-agency.gov.uk

Contents

1 Introduction	5

2 About this consultation	5

2.1 What we’re consulting on	5

2.2 What this consultation means to you	6

3 Site condition report process	6

4 Overview of updates	8

4.1 The updated guidance	8

4.2 How we will use your information	8

4.3 Privacy notice	8

5 Your details	9

5.1 Consultation questions	11

Produce an application site condition report	11

Maintain your site condition report	16

Produce a surrender site condition report	18

General guidance feedback	19

Site condition report template	20

Stage 1 to 3 assessment completed examples	20

Charges	21

Business impacts	21

Further comments	22

6. Responding to this consultation	22

6.1 Important dates	22

6.2 How to respond	22

6.3 Publishing our consultation response	23

6.4 Consultation principles	23

Would you like to find out more about us or your environment?	24

incident hotline	24

floodline	24

Environment first	24

## 1 Introduction

We are responsible for regulating facilities that could harm the Environment, as set out in the Environmental Permitting (England and Wales) Regulations 2016 (EPR).

We require a site condition report when an applicant applies for a:

- bespoke or standard rules permit for an A1 installation carrying out any of the activities listed in Part 2 of Schedule 1 to EPR

- bespoke or standard rules waste operation permit carrying out any waste disposal or recovery activity not listed in Part 2 of schedule 1 to EPR

- bespoke permit for a mining waste operation as specified in Part 2 of Schedule 20 of EPR

- waste handling or treatment areas associated with a permanent deposit of waste (landfill) and deposit for recovery activities

We have updated the 2013 H5 Site Condition Report guidance, and this is what we are consulting on.

## 2 About this consultation

This document describes:

- What we are consulting on

- An overview of the site condition report process

- Overview of updates

- How you can respond to the consultation

- How we will gain your consent and use your information

- The principles we follow when running this consultation

This is to help you understand our proposals and provide you with sufficient information to enable you to comment on them. We will consider your answers and comments when finalising our site condition report guidance.

The consultation will run for a period of 6 weeks from 24 November 2025 until midnight on 4 January 2026. We will publish our response, together with the updated site condition report guidance, on GOV.UK.

## 2.1 What we’re consulting on

This consultation is to seek your views on the proposed updated guidance

Once this guidance is published, the 2013 site condition report guidance and Regulatory Guidance Note 9 (RGN9) – Surrender will be withdrawn. It is also our intention to withdraw the 2013 H5 Site Condition Report Template and replace it with a revised template subject to responses received as part of this consultation.

This consultation is to seek your views on the proposed updated guidance including:

- Environmental permitting: produce an application site condition report

- Environmental permitting: maintain your site condition report

- Environmental permitting: produce a surrender site condition report

- Environmental permitting: stage 1 to 3 assessment completed examples

- Environmental permitting: site condition report template

- Withdrawal of the 2013 H5 Site Condition Report and Template PDFs

- Withdrawal of the 2013 RGN9 – Surrender guidance

- the requirement for standard rules waste operations permits to produce a site condition report at application

## 2.2 What this consultation means to you

This consultation will be of particular interest to:

- operators, trade associations and businesses: this is your opportunity to make sure that the updated guidance works for you and your industry whilst also providing the necessary protection to the environment and human health

- other regulators, consultants, the public, community groups and non-governmental organisations with an interest in environmental issues: this is your opportunity to make sure that the revised guidance provides the necessary protection to the environment and human health, whilst still being useful to industry

## 3 Site condition report process

The Environmental Permitting (England and Wales) Regulations 2016 (EPR) use a ‘no deterioration’ approach to permitted activities.  This means the site must be returned to the original condition it was in when the permit was issued.

A site condition report describes and records the condition of the soil and groundwater for a regulated site at points in time. Where required, the permit holder must:

- produce an application site condition report

- maintain the site condition report

- produce a surrender site condition report

This will enable the permit holder to meet their legal requirements to prove that:

- they have protected soil and groundwater from their permitted activities

- their site is in a satisfactory state when they apply to surrender all or part of their permit

For the application site condition report, the condition of the soil and groundwater must be described by setting a point of reference, for judging whether there has been any additional contamination of the site during operation of the regulated facility.

The industrial emissions directive places additional requirements on A1 installations - facilities which carry out industrial processes like refineries, manufacturing sites, factories, process industries and certain waste activities, to:

- complete a baseline report where the activity involves the use, production or release of relevant hazardous substances with the potential to pollute soil and groundwater

- undertake periodic soil and groundwater monitoring where required

Once a permit is issued, the permitted activities must not cause pollution, and the site condition report must be maintained.

The permit holder must:

- keep records in their written management system

- notify us about proposed changes to permitted activities such as changes to the site boundary, substances used and pollution prevention measures

- inspect and maintain their pollution prevention measures

- manage any pollution incidents promptly and effectively. It is not acceptable to wait until surrender of the permit before removing any pollution

- carry out soil and groundwater monitoring, if applicable

When the permit holder wants to apply to surrender all or part of their permit, they will need to produce a surrender site condition report and provide:

- evidence that they meet the criteria for a low-risk surrender, if applicable

- a summary of how they have protected soil and groundwater

- details of how they have decommissioned the site

- surrender reference data and a quantified comparison, if applicable

- details of any remediation that was required

- a statement that the site is in a satisfactory state

A1 installations with a risk from relevant hazardous substances must carry out an intrusive investigation to collect soil and groundwater samples to use as surrender reference data to enable a quantitative comparison to be made against baseline conditions.

We will accept a surrender application if we are satisfied that the permit holder has taken the necessary measures to:

- avoid any pollution risks from the permitted activities

- return the site to a satisfactory state, taking into account the state of the site before the permitted activities started

## 4 Overview of updates

## 4.1 The updated guidance

- requires holders of standard rules waste operations permits to produce a site condition report (this was previously a recommendation only)

- clarifies the information required in a site condition report at application, maintain and surrender stages

- clarifies when baseline reporting and periodic monitoring of soil and groundwater is required for sites to meet Industrial Emissions Directive (IED) requirements

- provides a structure for undertaking the Stage 1 to 3 assessment for hazardous substances

- provides completed examples of the Stage 1 to 3 assessment

- provides an updated site condition report template

The site condition report guidance, report template and Stage 1 to 3 completed example can be accessed in full via the ‘related’ section of the consultation page on Citizen Space (at the bottom of the overview page).

## 4.2 How we will use your information

The Environment Agency will make all responses publicly available during and after the consultation, unless you have specifically requested that we keep your response confidential. This includes comments received online and by email.

We will not publish names of individuals who respond or personal data. But we will publish the name of the organisation for those responses made on behalf of organisations

We will not respond individually to responses. After the consultation has closed, we will publish a consultation response document on GOV.UK and contact you to let you know when this is available.

In accordance with the Freedom of Information Act 2000, we may be required to publish your response to this consultation but will not include any personal information. If you have requested your response to be kept confidential, we may still be required to provide a summary of it.

## 4.3 Privacy notice

The Environment Agency would like to keep you informed about the outcomes of the consultation. If you would like to receive an email acknowledging your response and telling you when we’ve published the consultation response document, please provide your email address with your response.

By giving us your email address, you consent for us to email you about the consultation. We will keep your details until we have notified you of the response document publication.

We will not share your details with any other third party without your clear and full consent, unless required to by law.

You can withdraw your consent to receive these emails at any time by contacting us at conland@environment-agency.gov.uk

The Environment Agency is the data controller for the personal data you provide. For more information on how we deal with your personal data please see our personal information charter on GOV.UK.

You can email our Data Protection team: dataprotection@environment-agency.gov.uk.

## 5 Your details

We welcome your views on the updated site condition report guidance.

What is your email address?

By giving us your email address you consent for us to email you about the consultation. We will keep your details until we have notified you about publication of the response document.

We will not share your details with any other third party without your clear and full consent, unless required to by law.

You can withdraw your consent to receive these emails at any time by contacting us at: consultation.enquiries@environment-agency.gov.uk.

Email: ________________________________________________

Can we publish your response? We will not include personal information.

This is a required question, please select one of the following:

£ Yes

£ No

If you answered no, please tell us why below as we will need to understand this when responding to any Freedom of Information request.

When we come to analyse the results of this consultation, it would help us to know if you are responding as an individual or on behalf of an organisation or group.

Please select from the following options:

£  Responding as an operator

£  Responding on behalf of an organisation or group

£  Other

If you're responding on behalf of an organisation or group, please tell us who you are responding on behalf of and include its type e.g. business, environmental group.

If you have selected ‘Other’ please specify.

What is your main area of business?

£  A1 Installation

£  Waste operation

£  Mining waste operation

£  Consultancy

£  Other

If you selected other, please specify.

____________________________________

## 5.1 Consultation questions ## Produce an application site condition report

The updated guidance details when a site condition report is required: You must produce a site condition report when you apply for a: - bespoke or standard rules permit for an A1 installation carrying out any of the activities listed in Part 2 of Schedule 1 to EPR – these include low impact and intensive farm installations - bespoke or standard rules waste operation permit carrying out any waste disposal or recovery activity not listed in Part 2 of Schedule 1 to EPR - bespoke permit for a mining waste operation as specified in Part 2 of Schedule 20 to EPR Permanent deposits of waste (landfill) You must provide a site condition report for the waste handling or treatment areas associated with a permanent deposit of waste (landfill) and deposit for recovery activities. For the permanent deposits of waste, you must follow: - Landfill operators: environmental permits when you apply for a permit - Landfill and deposit for recovery: aftercare and permit surrender when you apply to surrender your permit When you do not need to produce a site condition report You do not need to produce a site condition report for a: - exempt activity - mobile plant permit - stand-alone water discharge, groundwater or flood risk activity - medium combustion plant and specified generators - non-nuclear radioactive substances facility and mobile apparatus - radioactive substances activity at a nuclear licensed site Question 1: Is the guidance clear when a site condition report is required? Please tick the relevant box

£ Yes

£ No

£ Don’t know £ No opinion

Please explain your answer.

The updated guidance details the information required to produce an application site condition report under a series of headings: - Provide details about the site - Describe the environmental setting - Provide details of any pollution history - Do a site walkover - Review existing infrastructure - Stage 1 to 3 assessment for A1 installations - Stage 1 to 3 assessment for other sites - Develop a conceptual site model - Set a point of reference - Plan ahead for any soil and groundwater monitoring Question 2: Is it clear what you have to do to produce an application site condition report? Please tick the relevant box

£ Yes

£ No

£ Don’t know £ No opinion

Please explain your answer.

The updated guidance provides a Stage 1 to 3 assessment that A1 installations must follow to identify relevant hazardous substances with the potential to pollute soil and groundwater. The 3 stages are:

- Stage 1: Identify hazardous substances

- Stage 2: Identify relevant hazardous substances

- Stage 3: Assess the potential pollution risk

A worked example has been provided.

Question 3: Is the stage 1 to 3 assessment clear and easy to follow? Please tick the relevant box. £ Yes. £ No. £ Don’t know £ Not applicable/No opinion. Please explain your answer.. Question 4: Is the guidance clear on how to identify hazardous substances? Please tick the relevant box

£ Yes

£ No

£ Don’t know £ No opinion

Please explain your answer. The updated guidance recommends that all other sites should complete a stage 1 to 3 assessment to identify if there is a risk of pollution to soil or groundwater from hazardous substances or any other potentially polluting substances the site is proposing to use, produce or release. The information can also be used to help develop the sites management system. Applicants for A1 installations who do not identify hazardous substances in their stage 1 assessment can continue the assessment for any other potentially polluting substances. If a risk to soil and/or groundwater is identified from substances that will be used, produced or released, they can be referred to as relevant substances. If a risk from relevant substances has been established, establishing baseline reference data should be considered. Question 5. Do you think other potentially polluting substances should be included in the stage 1 to 3 assessment? Please tick the relevant box. £ Yes. £ No. £ Don’t know £ Not applicable/no opinion. Please explain your answer.. If you disagree, do you have any suggestions how these should be assessed? We require a point of reference for judging whether there has been any additional contamination of the site during operation of the regulated facility. A point of reference can be: - existing relevant soil and groundwater data - published background concentrations - collection of new soil and groundwater data The updated guidance provides further information on setting the point of reference. Applicants for A1 installations with a risk from relevant hazardous substances must establish baseline reference data. All sites should establish baseline reference data if these apply: - there is evidence of historic land contamination - proposed activities could give rise to emissions of the same substances that may already be present in the soil and groundwater - there is a potential pathway for the same substances to be released to soil and groundwater We will use the baseline reference data to assess if the activities have caused pollution at permit surrender. Establishing baseline reference data helps prove the operator is not responsible for historic land contamination under the permit. Question 6. Is it clear what is meant by set a point of reference? Please tick the relevant box. £ Yes. £ No. £ Don’t know £ No opinion Please explain your answer.. Question 7. Is the updated guidance clear when baseline reference data must be established for A1 installations Please tick the relevant box. £ Yes. £ No. £ Don’t know £ No opinion. Please explain your answer. Question 8. Does the guidance address the benefits of establishing baseline reference data for all other sites and A1 installations without relevant hazardous substances? Please tick the relevant box. £ Yes. £ No. £ Don’t know. £ No opinion. Please explain your answer.. ## Maintain your site condition report

The site condition report must be kept up to date as long as the permit is held. This will help prove that soil and groundwater are being protected. To achieve this, the site must: - keep records - notify the Environment Agency about proposed changes to an activity that is not currently covered by the permit such as changes to the site boundary, changes to substances. - inspect and maintain pollution prevention measures - manage any new pollution incidents promptly and effectively - do soil and groundwater monitoring, if applicable The updated guidance provides details of the information required in the maintain site condition report. Question 9: Is it clear in the guidance when the site condition report should be updated? Please tick the relevant box. £ Yes. £ No. £ Don’t know. £ No opinion. Please explain your answer..

Question 10: Is it clear in the guidance when you must carry out soil and groundwater monitoring? Please tick the relevant box. £ Yes. £ No. £ Don’t know. £ No opinion. Please explain your answer..

Question 11: Is it clear in the guidance that if you have a pollution incident you must return the site back to its original condition? Please tick the relevant box. £ Yes. £ No. £ Don’t know. £ No opinion. Please explain your answer. ## Produce a surrender site condition report We must accept a surrender application if we are satisfied that the necessary measures have been taken to: - avoid any pollution risks from the permitted activities - return the site to a satisfactory state, taking into account the state of the site before the permitted activities started This is the legal test for surrender. A surrender site condition report must be produced which provides evidence that the permitted activities and decommissioning works have not caused pollution and that the site is in a satisfactory state. The updated guidance provides details of the information required in the surrender site condition report and when an intrusive site investigation must be undertaken to collect surrender reference data which are soil and groundwater samples. The updated guidance also details when a low-risk surrender can be applied for. Question 12: Does the guidance explain when you can apply for a low-risk surrender? Please tick the relevant box. £ Yes. £ No. £ Don’t know. £ No opinion. Please explain your answer...

Question 13: How clear is the guidance when you need to carry out an intrusive site investigation and collect surrender reference data? Please tick the relevant box

£ Yes, it is clear

£ No, it is not clear

£ Don’t know whether it is clear or not

£ No opinion

Please explain your answer.

## General guidance feedback

Question 14: What improvements, if any, would you suggest for the guidance? Please explain your answer.

Question 15: What is your view on whether holders of standard rules permits for waste operations should produce a site condition report? Please tick the relevant box  Strongly agree  Agree  Neither agree nor disagree  Disagree  Strongly disagree  Not applicable Please explain your answer. If you disagree, how should holders of standard rules permits for waste operations demonstrate their site is in a satisfactory state when they apply to surrender all or part of their permit? Please explain your answer. ## Site condition report template Question 16: In what ways does the site condition report template meet or not meet your needs?. Please explain your answer.. ## Stage 1 to 3 assessment completed examples Question 17: How helpful did you find the stage 1 to 3 examples? Please tick the relevant box. £ Very helpful. £ Helpful. £ Somewhat helpful £ Not helpful £ Not applicable.. Please explain your answer.. ## Charges We will need to make charges to cover the regulatory costs we incur in processing and determining applications and in carrying out compliance work associated with these updates to the guidance. We are currently developing our charges and we will be consulted on these in the next wider charges consultation. ## Business impacts. You only need to complete this section if your business activities could be covered by the site condition report guidance. The Growth Duty requires us to have regard to the desirability of promoting economic growth, alongside our other statutory duties. As part of this duty, we are carrying out an assessment of the economic impacts of the proposed revisions to site condition reports.. Question 18: Are there any potential economic impacts, either positive or negative, that the amendments to the guidance could have on your business? Please tick the relevant box. £ Yes. £ No. £ Don’t know. £ No opinion. Please explain your answer. ## Further comments Please provide any further comments or observations that you would like us to consider as part of this consultation.

## 6. Responding to this consultation

## 6.1 Important dates

The consultation will run for a period of 6 weeks from 24 November 2025 until midnight on 4 January 2026.

## 6.2 How to respond

You can view the consultation documents and questions online on the Environment Agency’s Citizen Space consultation website.

### Respond online

Please submit your response on our Citizen Space consultation website, as it helps us to:

- gather all responses in one place

- summarise responses quickly and accurately

- reduce the costs of the consultations

### Respond by email

If you prefer, you can submit your response by email by using the response form, which you will find under the ‘Related’ section of the consultation on Citizen Space. Please email your completed response form with the subject header of “Environmental permitting: H5 Site condition report – Proposals for updated guidance” to: conland@environment-agency.gov.uk

### Ask for a printed version

Please contact us if you need a printed version of the document to be posted to you. You can do this by contacting our:

National Customer Contact Centre
Telephone: 03708 506 506

Minicom for the hard of hearing: 03702 422 549
Monday to Friday, 8am to 6pm

## 6.3 Publishing our consultation response

We will publish our response on GOV.UK within 12 weeks of this consultation closing and before we implement the updated H5 Site Condition Report Guidance. It will include a summary of the comments and queries we receive. We will add a link to this document on the consultation page on Citizen Space.

## 6.4 Consultation principles

We are running this consultation in accordance with the guidance set out in the government's Consultation Principles.

If you believe the consultation has not been run in accordance with the principles, please email: consultation.enquiries@environment-agency.gov.uk.

Otherwise, for all other queries relating to this consultation please email conland@environment-agency.gov.uk

# Would you like to find out more about us or your environment?

Then call us on

03708 506 506 (Monday to Friday, 8am to 6pm)

Email: enquiries@environment-agency.gov.uk

Or visit our website

www.gov.uk/environment-agency

## incident hotline

0800 807060 (24 hours)

## floodline

0345 988 1188 (24 hours)

Find out about call charges (https://www.gov.uk/call-charges)

## Environment first

Are you viewing this onscreen? Please consider the environment and only print if absolutely necessary. If you are reading a paper copy, please don’t forget to reuse and recycle.