# DRAFT RULES

Statutory Guidance

# SR2025 No 8:

# Vehicle storage, depollution and dismantling (authorised treatment) facility

The Environmental Permitting (England & Wales) Regulations 2016

## Introductory note

This introductory note does not form a part of these standard rules.

Operators will need to check the generic risk assessment and the rules in detail including waste motor vehicle standards version 1 dated 17 December 2025 to make sure they can comply with them.

Any standard permit issued for these rules by the Environment Agency will:

- name a specific operator who can use the permit

- include a site plan showing where the facility is and its extent

- require the operator to comply with these rules

The permit allows the named operator to operate a vehicle storage, depollution and dismantling facility and allows the recovery of all waste motor vehicles at a specified location.

The total quantity of waste that can be accepted at a site under these rules must be less than 25,000 tonnes a year.

Burning of any wastes, either in the open, inside buildings or in any form of incinerator is not permitted.

These rules do not allow any point source emission into surface waters or groundwater except

under the emissions of substances not controlled by emission limits rule:

- liquids may be discharged into a sewer subject to a consent issued by the local water company

- liquids may be taken off-site in a tanker for disposal or recovery

- clean surface water from roofs, or from areas of the site that are not being used in connection with storing and treating waste, may be discharged directly to surface waters, or to groundwater by seepage through the soil through a soakaway

- clean surface water from storage areas for fully depolluted vehicles, uncontaminated plastic, glass and ferrous and non- ferrous metal wastes arising from the treatment of end-of-life vehicles may discharge to a sealed drainage system or to surface waters or groundwater through drainage systems designed and constructed so water discharged does not adversely impact the water quality of receiving water bodies

The facility, except for activities carried out with a building, must be a minimum distance from certain types of sensitive site. It cannot be within:

- 200 metres of a European site, Ramsar, Site of Special Scientific Interest or Marine Conservation Zone

- 50 metres of a National Nature Reserve, Local Nature Reserve or Local Wildlife Site

- 50 metres of a protected habitat or a protected species that the Environment Agency considers is at risk from this activity

- a groundwater source protection zone 1 or 2, or where a source protection zone has not been defined then not within 50 metres of any well, spring or borehole used for the supply of water for human consumption. This includes private water supplies

- 10 metres of an unculverted watercourse

End of introductory note

# Rules

Management

- 1.1 	General management

- There will be a rule containing the management system requirements, an indication of potential drafting alternatives is below;

- Either

1.1.1 The operator shall manage and operate the following activities:

(a) in accordance with a written management system that identifies and minimises risks of pollution, so far as is reasonably practicable, including those arising from operations, maintenance, accidents, incidents, non-conformances, closure and those drawn to the attention of the operator as a result of complaints

(b) using sufficient competent persons and resources

1.1.2 Records demonstrating compliance with rule 1.1.1 shall be maintained.

1.1.3 Any person having duties that are or may be affected by the matters set out in these standard rules shall have convenient access to a copy of these rules, the permit and the management system.

1.1.4 The operator shall comply with the requirements of an approved competence scheme.

Or

    - The operator shall have and maintain a written management system that:

- Identifies the risks of pollution from the activities including those arising from operations, maintenance, accidents, incidents, non-conformances, closure and those drawn to the attention of the operator as a result of complaints; and

- Specifies control measures that prevent, or where that is not practicable, minimise the risks of pollution and will ensure that the rules of this permit are complied with; and

- Includes plans for training, maintenance, process control monitoring, tracking conformance with the written management system, dealing with accidents, incidents and complaints.

1.1.2 	The operator shall manage and operate the activities:

- in accordance with the written management system specified in rule 1.1.1; and

- using sufficient competent persons and resources.

1.1.3    The operator shall:

- review the written management system at least annually to ensure continued compliance with rule 1.1.1; and

- keep a record of any reviews and changes made to the management system.

1.1.4     The operator shall comply with the requirements of an approved competence scheme.

- 1.2   Avoidance, recovery and disposal of wastes produced by the activities

- 1.2.1	The operator shall take appropriate measures to ensure that:

the waste hierarchy referred to in Article 4 of the Waste Framework Directive is applied to the generation of waste by the activities; and

any waste generated by the activities is treated in accordance with the waste hierarchy referred to in Article 4 of the Waste Framework Directive; and

where disposal is necessary, this is undertaken in a manner which minimises its impact on the environment.

- 1.2.2	The operator shall review and record at least every four years whether changes to those measures should be made and take any further appropriate measures identified by a review.

Operations

- 2.1   Permitted activities

2.1.1 The operator is only authorised to carry out activities with the following descriptions:

(a) waste activity codes for recovery:

R3: Recycling/reclamation of organic substances (not solvents)

R4: Recycling or reclamation of metals and metal compounds

R5: Recycling or reclamation of other inorganic materials

R13: Storage of wastes pending any of the operations numbered R1 to R12 (excluding temporary storage, pending collection, on the site where it is produced)

2.1.2 	Treatment is limited to:

(a) depollution of waste motor vehicles

(b) dismantling including sorting, separation or cutting (using hand-held equipment only) and compacting including baling activities

2.1.3 	Wastes shall be stored for no longer than 1 year prior to disposal and 3 years prior to recovery.

2.1.4  The maximum quantity of hazardous waste stored at the site at any one time shall not exceed 10 tonnes.

### 2.2	The site

2.2.1 	The activities shall not extend beyond the site, being the land shown edged in green on the site plan attached to the permit.

2.2.2 	The activities, except for vehicle storage, depollution and dismantling activities carried out within a building, shall not be carried out within:

(a) 200 metres of a European Site, Ramsar, SSSI or Marine Conservation Zone

(b) 50 metres of a National Nature Reserve, Local Nature Reserve or Local Wildlife Site

(c) 50 metres of a protected habitat or a protected species that the Environment Agency considers is at risk from this activity

(d) a groundwater source protection zone 1 or 2, or where a source protection zone has not been defined then not within 50 metres of any well, spring or borehole used for the supply of water for human consumption (including private water supplies)

(e) 10 metres of an unculverted watercourse

### 2.3 Waste acceptance

2.3.1 Waste shall only be accepted at the site if all the following apply:

(a) it does not consist solely or mainly of dust, powder or loose fibres

(b) it is not in a form which is either sludge or liquid

(c) it conforms to the description in the transfer documentation supplied by the producer and holder

(d) it falls within the types, quantities, waste codes and descriptions given in Schedule 1 of these rules

2.3.2 Any waste that does not comply with rule 2.3.1 shall be rejected and moved to a designated quarantine area pending removal to a facility able to accept that waste.

2.3.3 Records demonstrating compliance with rule 2.3.1 and rule 2.3.2 shall be maintained and kept for at least 2 years.

2.3   Operating techniques

2.3.1	The activities shall, subject to the rules of this permit, be operated using the standards detailed in the waste motor vehicles standards document version 1 dated 17 December 2025 published by the Environment Agency.

    - Pre-operational conditions

    - The Environment Agency shall be notified of the proposed date of commencement of operations at least 14 days prior to the activities being brought into operation.

- 3.	Emissions and monitoring

- 3.1   	Emissions to air, land and water

- 3.1.1 There shall be no point source emissions to water, air or land.

- 3.2   Emissions of substances not controlled by emission limits

- 3.2.1	Emissions of substances not controlled by emission limits (excluding odour) shall not cause pollution. The operator shall not be taken to have breached this condition if appropriate measures have been taken to prevent or where that is not practicable, to minimise, those emissions.

- 3.3   Odour

- 3.3.1	Emissions from the activities shall be free from odour at levels likely to cause pollution outside the site, as perceived by an authorised officer of the Environment Agency, unless the operator has used appropriate measures to prevent or where that is not practicable to minimise the odour.

- 3.4   Noise and Vibration

- 3.4.1  Emissions from the activities shall be free from noise and vibration at levels likely to  cause pollution outside the site, as perceived by an authorised officer of the Environment Agency, unless the operator has used appropriate measures, to prevent or where that is not practicable to minimise the noise and vibration.

### 3.5 	Fire prevention

- 3.5.1	The operator shall take all appropriate measures to prevent fires and accidents on site and minimise the risk of pollution from them.

- Information

### 4.1 Records

4.1.1 All records required to be made by these standard rules shall:

(a) be legible

(b) be made as soon as reasonably practicable

(c) if amended, be amended in such a way that the original and any subsequent amendments remain legible or are capable of retrieval

(d) be retained, unless otherwise agreed by the Environment Agency, for at least 6 years from the date when the records were made, or in the case of the following records until licence surrender:

(i) off-site environmental effects

(ii) matters which affect the condition of the land and groundwater

4.1.2 	The operator shall maintain convenient access, in either electronic or hard copy, to the records, plans and management system required to be maintained by these standard rules

#### 4.2 	Reporting and notifications

4.2.1 	All reports and notifications required by these standard rules shall be made in writing, using the contact details supplied by the Environment Agency. Where reports and notifications must be made immediately, they may be provided verbally by telephone to the Environment Agency Incident Hotline 0800 80 70 60.

4.2.2 	Within one month of the end of each year, the operator shall submit to the Environment Agency using the form made available for the purpose, the information specified on the form relating to the site and the waste accepted and removed from it during the previous year.

4.2.3 	In the event:

(a) that the operation of the activities gives rise to an incident or accident which significantly affects or may significantly affect the environment, the operator must immediately do all of the following:

(i) inform the Environment Agency, using the Environment Agency Incident Hotline 0800 80 70 60 if the report is being made verbally

(ii) take the measures necessary to limit the environmental consequences of such an incident or accident

(iii) take the measures necessary to prevent further possible incidents or accidents

(b) of a breach of any rule the operator must immediately do both of the following:

(i) inform the Environment Agency, using the Environment Agency Incident Hotline 0800 80 70 60 if the report is being made verbally

(ii) take the measures necessary to ensure that compliance is restored within the shortest possible time

(c) of a breach of permit condition which poses an immediate danger to human health or threatens to cause an immediate significant adverse effect on the environment, the operator must immediately suspend the operation of the activities or the relevant part of it until compliance with the permit conditions has been restored

4.2.4 	Written confirmation of actual or potential pollution incidents and breaches of rules shall be submitted to the Environment Agency within 24 hours.

4.2.5 	Where the Environment Agency has requested in writing that it shall be notified when the operator is to undertake monitoring or spot sampling, the operator shall inform the Environment Agency when the relevant monitoring or spot sampling is to take place. The operator shall provide this information to the Environment Agency at least 14 days before the date the monitoring is to be undertaken.

4.2.6	The Environment Agency shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules:

(a) where the operator is a registered company, both of the following apply:

(i) any change in the operator’s trading name, registered name or registered office address

(ii) any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up

(b) where the operator is a corporate body other than a registered company, both of the following apply:

(i) any change in the operator’s name or address

(ii) any steps taken with a view to the dissolution of the operator

(c) in any other case, all the following apply:

(i) the death of any of the named operators (where the operator consists of more than one named individual)

(ii) any change in the operator’s name or address

(iii) any steps taken with a view to the operator, or any one of them, going into bankruptcy, entering into a composition or arrangement with creditors, or, in the case of them being in a partnership, dissolving the partnership

(iv) any closure of the site longer than 15 days

### 4.3 Interpretation

4.3.1	In this set of standard rules the expressions listed in this rule (4.3) shall have the meaning given.

“accident” means an accident that may result in pollution.

“authorised officer” means any person authorised by the Environment Agency under section 108(1) of The Environment Act 1995 to exercise, in accordance with the terms of any such authorisation, any power specified in Section 108(4) of that Act.

“cutting using hand-held equipment” means cutting typically utilising either an oxy-acetylene gas cutting torch or abrasive disc cutter tool to cut or resize large pieces of scrap metal into more manageable sizes; powder torches and plasma torches may be used to cut heat-resistant scrap (for example, pig iron, copper, bronze).

“D” means a disposal operation provided for in Annex I to Directive 2008/98/EC of the European Parliament and of the Council of 19 November 2008 on Waste.

“depollution” means the minimum technical requirements for the treatment of end-of-life vehicles as set out in Annex I (3) of the End-of-Life Vehicles Directive (2000/53/EC), namely:

- removal of batteries and liquefied gas tanks

- removal or neutralisation of potential explosive components (air bags for example)

- removal and separate collection and storage of fuel, motor oil, transmission oil, gearbox oil, hydraulic oil, cooling liquids, antifreeze, brake fluids, air conditioning system fluids or gases and any other fluid contained in the end-of-life vehicle unless they are necessary for the re-use of the parts concerned

- removal, as far as feasible, of all components identified as containing mercury

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the activities, either from emission points specified in these standard rules or from other localised or diffuse sources, which are not controlled by an emission limit.

“European Site” means Special Area of Conservation or candidate Special Area of Conservation or Special Protection Area or proposed Special Protection Area in England and Wales, within the meaning of Council Directives 79/409/EEC on the conservation of wild birds and 92/43/EEC on the conservation of natural habitats and of wild flora and fauna and the Conservation (Natural Habitats &c) Regulations 1994. Internationally designated Ramsar sites are dealt with in the same way as European sites as a matter of government policy and for the purpose of these rules will be considered as a European Site.

“groundwater” means all water, which is below the surface of the ground in the saturation zone and in direct contact with the ground or subsoil.

“hazardous waste” has the meaning given in the Hazardous Waste (England and Wales) Regulations 2005 .

“quarter” means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.

“R” means a recovery operation provided for in Annex II to Directive 2008/98/EC of the European Parliament and of the Council of 19 November 2008 on Waste.

“Ramsar site” means a wetland of international importance, designated under the Ramsar Convention (an international agreement signed in Ramsar, Iran, in 1971). It is government policy to treat Ramsar sites the same as European sites.

“separation” means separating wastes into different material types, components and grades.

“sorting” means sorting that may be undertaken by hand or handheld machinery. Sorting enables materials to be processed or recycled appropriately. It may involve separation of different waste types or the separation of different metal types including:

- different ferrous metals

- non-ferrous metals

- non-metallic materials (plastic for example)

“SSSI” means Site of Special Scientific Interest within the meaning of the Wildlife and Countryside Act 1981 (as amended by the Countryside and Rights of Way Act 2000).

“waste code” means the 6 digit code referable to a type of waste in accordance with the List of Wastes and in relation to hazardous waste, includes the asterisk.

“waste motor vehicle” means a wheeled vehicle for use on land and that does not operate on rails that is waste within the meaning of Article 3(1) of the Waste Framework Directive.

“year” means calendar year commencing on 1st January.

Schedule 1

### Waste types and quantities

#### Maximum quantities

The total quantity of waste accepted at the site shall be less than 25,000 tonnes a year.

#### Exclusions

Wastes derived from the depollution or dismantling of waste motor vehicles elsewhere shall not be accepted at the site.

Permitted waste types

Waste code	Description

16 		Wastes not otherwise specified in the list

16 01	End-of-life vehicles from different means of transport [including off-road machinery] and wastes from dismantling of end-of-life vehicles and vehicle maintenance (except 13,14, 16 06 and 16 08)

16 01 04*	End-of-life vehicles

16 01 06	End-of-life vehicles (containing neither liquids nor other hazardous components)

End of standard rules