Site:		Henbury Quarry 											Number: 	RA24

Activity: 	 Environmental Aspect and impact Assessment.

Date of Assessment:	January 25							Risk Assessment undertaken by: Lee Milford, Tony Hinton

| Aspect | Impact | Sev | Existing Control Measures | Lky | Risk Rating | Additional Comments (where RR is H or M) | Lky | Final Risk Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dust Crushing, screening, Traffic. | Detriment of neighbour’s health and wellbeing. Cloaking of vegetation and wildlife habitat. | M | Wet process on wash plant. Dust suppression used else were. | M | M | Dust suppression unit used on haul roads | L | L |
| Noise Crushing, screening, Traffic. | Detriment of neighbour’s health and wellbeing. | L | Acoustic bunds in place on perimeters in production area. Latest technologies employed to reduce Noise. Trained operators | L | L |  |  |  |
| Ground water Abstraction and consumption | Lowering water table stress on resource | L | Abstraction under licence. Minimize use. Avoid leaks | L | L |  |  |  |
| Ground water Contamination | Silt, oil, other contaminants getting into ground water. Directly and indirectly. SPZ2 across majority of waste site. SPZ1 in crushing area and adjacent pond. | H | Abstraction points at high level to avoid direct impact. No Contaminated waste accepted on the site. Water stored in clay lined ponds. Oil spills on ground cleaned up using spill kits. Disposed of as hazardous waste. Natural geology provides at least 4 layers of impermeable clay. | L | M | Staff Training. Emergency drills. Daily inspections North pond in spz1 pumped to primary pond when full via underground pipeline. | L | L |
| Wastewater Aggregate washing | Water with silt and fines | L | Water recycling system employed, using flocculant to clean the water before it is returned to the pond. Run off controlled and recycled. Stored in clay lined ponds. Visual inspections carried out on water quality. Any sign of oil on the water, floating booms with be deployed across the flow to remove the contamination. | L | L |  |  |  |
| clay soil | Visual impact on site | L | Stored in perimeter bunds to be used in site restoration. Nutrient rich soil, Grass and wildflower seeded to encourage biodiversity. | L | L | Work towards declassification as a waste. |  |  |
| Exhaust Emission Mobile plant | Damage to atmosphere. Carbon Emissions | L | Latest technologies employed. Machines are modern and serviced. reduce the use of Fossil fuels whereever possible. | M | L |  |  |  |
| Visual Impact Plant Lighting | Detriment to neighbours’ wellbeing | L | Plant lighting off when not in use. Visual bunds around production areas. Tree planting | L | L |  |  |  |
| Visual Impact Plant structures, stockpiles | Detriment of neighbour’s health and wellbeing. | L | Sit screened buy bunds and tree planting. All structures and stockpiles are below screening. Grass seeded | L | L |  |  |  |
| Ground contamination Oil spills | Contamination of ground. | M | Oils spill kits and training provided. Oils stored in bunded containers. Disposed of by licenced carriers. | L | L |  |  |  |
| Ground contamination litter | Damage to environment. | L | Litter bins provided Disposed of by licenced carriers. | L | L |  |  |  |
| Trespass | Danger to trespasser and to property | L | Site fenced and gates locked. CCTV and alarm system. Machines lock and keys secured. Natural barriers used were possible. Bramble rhododendron gorse. | L | L |  |  |  |

| Mandatory PPE | Head Protection |  | Orange High Viz |  | Foot Protection (inc mid sole) |  | Hand Protection |  | Eye Protection |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Review Record

This risk assessment must be reviewed annually, any change to the site environment or in the event of an incident or accident.

| NAME (Please print clearly) | SIGNATURE | DATE OF REVIEW |
| --- | --- | --- |
| Lee Milford |  | 22/01/25 |
| Lee Milford |  | 24/11/25 |

The signatures confirm that this risk assessment has been briefed and the control measures in place to deal with the hazards and risks.

| NAME (Please print clearly) | SIGNATURE | DATE OF BRIEFING |
| --- | --- | --- |