APPENDIX A

NON-TECHNICAL SUMMARY

SUMMARY OF TREATMENT ACTIVITIES

Activity

Liquid waste will comprise two discrete activities at site.

Firstly, the storage only of non-hazardous and hazardous liquid wastes as separate activities.

Secondly, the simple treatment of oily water, a process to separate water and solids from waste oil to produce a secondary fuel oil.

Process

Under its ISO14001 accreditation the site has an Environmental Aspect and Impact Plan for the site that addresses the associated activities and risks associated with the activities at site.

Delivery vehicles will be directed to the tank which is fully enclosed with the exception of an entry point and a vent. The vent uses a pressure valve to automatically open and shut the valve which avoids breathing emissions.

The tank hose from the vehicle will be connected to the tank and the contents discharged into the tank.

Liquid waste – oily water

Waste oil received at the facility will be processed by the storage of oily water feedstock in enclosed tanks. There will be no thermal or other treatment of the waste at site other than the simple separation [gravity] of the oil and water content prior to the filtration and removal of any solids destined for further treatment off-site or final disposal to landfill.

There will be a single storage tank provided with a theoretical maximum capacity of approx. 21,000 litres and will be used for the reception of and the treatment of oil water separation process.

The first stage of the treatment process allows the oil and water to separate using simple gravitational separation.

Filtration; separation of solids by filtration prior to discharge of contaminated water from the oil water separation stage into interceptors prior to discharge off-site to foul water sewer under the existing Trade Effluent Discharge Consent.

The resultant oil in the tank will be pumped into a tanker to be transferred to a site for secondary fuel oil production.

Liquid waste – sewage/septic tanks

There will be a single storage tank provided with a theoretical maximum capacity of approx. 21,000 litres and will be used for the reception and storage only of sewage/septic tank wastes.

Liquid waste – other

There will be a single storage tank provided with a theoretical maximum capacity of approx. 21,000 litres and will be used for the reception and storage only of other liquid wastes.

| Source | Pathway | Receptor | Harm | Likelihood | Consequence | Magnitude of Risk | Justification of Magnitude | Risk Management | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Point source during transfer from tanker to tank | Air | Staff | Inhalation of VOCs Release of odour | Low | Medium | Low – Medium | Site staff to have full PPE. Unloading process will be enclosed. | ISO14001. Full PPE provided. Short duration associated with connecting tanker. Maintenance of all tanks and pipe work. Training for all staff. | Low |
| Point source during transfer from tanker to tank | Air | Local residents | Inhalation of VOCs Release of odour | Low | Medium | Low – Medium | No nearby residents | ISO14001. Closest residents c650m distant Short duration associated with connecting tanker. Likelihood of off-site emissions is very low. Maintenance of all tanks and pipe work. Training for all staff. | Very low |
| Transfer between tank and tanker | Air | Local residents | Inhalation of VOCs Release of odour | Low | Medium | Low – Medium | No nearby residents | ISO14001. Closest residents c650m distant Short duration associated with connecting tanker. Likelihood of off-site emissions is very low. | Very low |
| Transfer between tank and tanker | Air | Local residents | Inhalation of VOCs Release of odour | Low | Medium | Low – Medium | No nearby residents | ISO14001. Short duration associated with connecting tanker. Likelihood of off-site emissions is very low. | Very low |
| Transfer of water to foul sewer | Air/Water | Local residents | Inhalation of VOCs Release of odour | Low | Low | Low | Water to enter foul sewer via oil interceptor to remove any residual oil | ISO14001. Full maintenance of drainage system, gullies, pipe-work and interceptor. | Very low |