Non-Technical Summary

Manor Farm Poultry Unit is applying for a permit for 48500 bird places for the production of free range eggs. The installation lies to the east of Heapham at NGR SK 90088 88728. Existing house one (Aviary system with belt removal) houses 32250 birds and the proposed house two (Aviary System with belt removal) will house 16250 birds.

Birds will be housed at point of lay and depopulated at the end of the birds egg laying cycle, this will be done on an all-out all in basis. There will be approximately one cycle per annum. Birds are allowed to range by means of pop holes at the base of sides of the houses.

Before bird arrival the house floor will be covered to a minimum depth of 2 cm of bulk shavings. Temperature and humidity will be closely monitored on a daily basis to achieve bird comfort and a relative humidity of 55-60%, this should achieve litter with a high dry matter content, which is important to minimising emissions. Ventilation is controlled by high velocity extraction fans, these are roof mounted on houses 1 & 2, (7m/s efflux and 5.5m release height). The birds themselves generate sufficient heat to negate the need for any additional heating.

Water is via a nipple drinking system fitted with cups to reduce leakage and spills leading to drier litter.

Birds are fed a minimum of three diets during their cycle, with gradually reducing levels of protein and phosphorous as bird age increases.

Feed is delivered from a UKAS accredited feed mill and blown into bulk feed bins situated adjacent to the houses, from the feed bins the feed is augered into the houses and distributed to the birds via a pan feeding system.

Fallen stock will be recorded daily and securely stored in vermin proof containers awaiting regular collection by a licensed renderer.

Manure belts are operated twice weekly removing litter from the houses.

At depletion, any remaining litter will be removed from the poultry houses and stored in the manure store, which has a maximum capacity of 400t, for use as fertiliser on operator controlled ground. The site will then be pressure washed, disinfected and dried out prior to the cycle beginning again. All wash waters will be contained in sealed underground dirty water tanks.

Clean roof and yard drainage are directed to underground stone soakaways, excepting periods of washdown. No discharges to watercourse.

The above measures are designed to reduce emissions, trees and hedges will trap dust particles reducing odour. Ammonia emissions will be reduced by reduced protein feed, maintaining good litter conditions with high dry matter. Containment of wash waters will prevent pollutants being released to the environment.

Records of tonnages of litter and wash water removal are recorded, wash water will be exported off site.