WRM

wrm-ltd.co.uk

01943 468138

Environmental Risk Assessment

v2.0

Environmental and sustainability solutions provided to

Advanced Demolition Limited

WRM-LTD.CO.UK

This report was prepared by Walker Resource Management Ltd (WRM’) within the terms of its engagement and in direct response to a scope of services. This report is strictly limited to the purpose and the facts and matters stated in it and does not apply directly or indirectly and must not be used for any other application, purpose, use or matter. In preparing the report, WRM may have relied upon information provided to it at the time by other parties. WRM accepts no responsibility as to the accuracy or completeness of information provided by those parties at the time of preparing the report. The report does not take into account any changes in information that may have occurred since the publication of the report. If the information relied upon is subsequently determined to be false, inaccurate, or incomplete then it is possible that the observations and conclusions expressed in the report may have changed. WRM does not warrant the contents of this report and shall not assume any responsibility or liability for loss whatsoever to any third party caused by, related to, or arising out of any use or reliance on the report howsoever. No part of this report, its attachments or appendices may be reproduced by any process without the written consent of WRM. All enquiries should be directed to WRM.

| Document Title | Environmental Risk Assessment |  |
| --- | --- | --- |
| Client | Advanced Demolition Limited |  |
| Revision | v2.0 |  |
| Date | 08/01/2026 |  |
| Document Reference | EPR-B02 |  |
| Project Reference | 1504/J05 |  |
| Author: Bryanna Heaton |  | Reviewer: Graeme Kennett |

Copyright ©

All material on these pages, including without limitation text, logos, icons and photographs, is copyright material of Walker Resource Management Ltd (WRM). Use of this material may only be made with the express, prior, written permission of WRM. This document was produced solely for use by the named client to whom the document refers. The methodology (if any) contained in this report is provided to you in confidence and must not be disclosed or copied to third parties without the prior written agreement of WRM. Disclosure of that information may constitute an actionable breach of confidence or may otherwise prejudice our commercial interests.

| Revision | Details | Date |
| --- | --- | --- |
| 0.1 | Initial Draft | 23/06/2025 |
| 1.0 | First Issue | 27/06/2025 |
| 2.0 | Second issue | 08/01/2026 |

Revision Log

CONTENTS

Contents

1.0	Introduction	1

1.1	Assessment Process	1

2.0	Environmental Risk Assessment	2

# Introduction

Walker Resource Management Limited (WRM) are acting as consultants for Advanced Demolition Limited (hereon referred to as ‘ADL’), who have commissioned WRM to produce an Environmental Risk Assessment in line with operational activities associated with a waste transfer station facility.

ADL are seeking a bespoke environmental permit to operate as a non-hazardous waste transfer station as detailed in the Supporting Information (Document Reference: EPR-A01). The table in Section 2 presents the identified risks on site, the potential linkages from source, pathway and receptor, and provides an assessment of the residual risk following the proposed risk management strategy.

## Assessment Process

The guidance Risk Assessments for your Environmental Permit produced by the Environment Agency and DEFRA gives a five-step process for assessing the site activity and the risk to local amenity to successful produce an Environmental Risk Assessment:

- Identify and consider risks for your site, and the sources of the risks.

- Identify the receptors (people, animals, property and anything else that could be affected by the hazard) at risk from your site.

- Identify the possible pathways from the sources of the risks to the receptors.

- Assess risks relevant to your specific activity and check they’re acceptable and can be screened out.

- State what you’ll do to control risks if they’re too high.

This risk assessment will identify people or parts of the environment that could be harmed by the activity, carries out risk assessments for these potential sources and provides an assessment of the residual risk after the implementation of the proposed risk management strategy.

# Environmental Risk Assessment

| Pollutant Model |  |  | Judgement |  |  |  | Action |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | Pathway | Receptor | P | C | M | Justification of Magnitude | Risk Management | Residual Risk |
| Fugitive emissions of dust, fibres and particulates. | Deposition from air and inhalation. | Local human population. | Med | High | Med | Permitted waste types do not include dusts, powders or loose fibres. Movement of waste on site has the potential to produce dust and so a medium magnitude risk is estimated. | - The site will be kept clean and dust suppression will be used on the site when required. - Solid wastes are stored in bays to prevent fugitive emissions. - Daily site inspections. - Site is situated on an impermeable surface with sealed drainage. | Low |
| Litter | Air transport, then deposition. | Local human population. | Low | Med | Med | Nuisance, loss of amenity. | - On arrival all wastes will be deposited in the waste reception area. - Wastes are inspected on arrival and are rejected if the EWC code does not meet the allowable inputs as detailed in the Waste Acceptance Procedure. - Daily inspection of the site will be carried out and remove any litter which has accumulated. - In the event that litter does escape from the site, it shall be retrieved as soon as is practicable, and no later than one hour after the end of the working day. | Low |
| Waste, litter and mud on local roads. | Vehicles entering and leaving site. | Local human population. | Low | Med | Med | Waste could be coming from dirty sites but all waste shall be stored within separated bays. | - Daily inspection of the site will be carried out and site clean-up of mud or debris when required. - All vehicles leaving the site shall, before leaving the site, be checked for mud and cleaned as necessary. - Vehicles leaving the site shall be checked to ensure that they are clear of loose waste and that any waste being transported off site is appropriately secured. - The nature of the site, on impermeable concrete surfacing reduces the risk of mud being transported onto the roads. - In the event that waste, litter or mud does escape from the site, it shall be retrieved as soon as is practicable, and no later than one hour after the end of the working day. | Low |
| Odour. | Air transport then inhalation. | Local human population. | Low | Med | Low | Odour is not considered to present a significant risk as the types of materials accepted are generally not odorous. | - Odour from waste is mitigated through the Waste Acceptance Procedure and inspections upon arrival. | Very low |
| Noise and vibration. | Noise through the air and vibration through the ground. | Local human population. | High | Med | High | Residents or workers at nearby residential / commercial / industrial properties often sensitive to noise and vibration. Conducting of “noisy” operations e.g. baling, shearing. | - Equipment undergoes regular maintenance and servicing. - No shredding or crushing carried out on site. | Low |
| Scavenging animals and scavenging birds. | Air transport and over land. | Local human population. Local environment. | Low | Med | Low | Permitted wastes are not likely to attract scavenging animals and birds. Building areas have the potential may become nesting/breeding sites. | - Waste stored in designated areas on site. - General waste and food waste from staff welfare facilities stored in bins. - Pest control contract. | Very low |
| Pests. | Air transport and over land. | Local human population. Local environment. | Low | Med | Low | Waste with potential to attract pests (e.g. organic materials) is prioritised for quick transfer and is stored in covered or enclosed containers. Building areas have the potential may become nesting/breeding sites. | - On detection or notification of pest infestations, or evidence of such, immediate action shall be taken to secure the attendance of a professional pest control contractor, to eliminate the pest infestation. | Very low |
| Flooding of site. | Flood waters. | Local human population. Local environment. | Low | High | Med | Permitted waste types are non-hazardous and hazardous and therefore waste washed off site could pollute waters. | - The site is not located within a Flood Zone 2 or 3 and therefore has a very low risk of flooding. - The site shall store waste in designated storage areas. - Impermeable concrete surface with falls towards drains prevents run-off from flowing off site. - All surface waters generated on site falls on an impermeable surface towards the direction of a gully. - The site has a sealed drainage system. | Low |
| All on-site hazards: wastes; machinery and vehicles. | Direct physical contact. | Local human population. Local environment. | Med | High | High | Delivery vehicles and mobile plant actively moving around the site. | - No public access to the site. - CCTV surrounding the site. - Palisade fencing around the perimeter of the site. - Daily lock up at end of each working day. - Signs onsite outlining risks. - Wastes accepted under the supervision of a technically competent manager. - Wastes stored within designated waste storage area. - Traffic management flows. - Reversing sounds on vehicles and mobile equipment. | Low |
| Liquid generated from fuel tank/bund failure or direct runoff of contaminated surface water. | Permeate/ flow through soil. | Ground Water. | High | High | High | The facility is located inside a Source Protection Zone 1. Fuel is dispensed from a bunded fuel tank located on the impermeable concrete surface. | - Impermeable concrete surface with falls towards drains prevents run-off from flowing off site. - All surface waters generated on site falls on an impermeable surface towards the direction of a gully. - Tanks are bunded. - The site has a sealed drainage system. - Fuel tank is bunded and situated on impermeable concrete. - Spill kits and drain cover mats present on site for use if a spill is detected. - Routine inspection and maintenance of fuel tank and site surface. - The site drainage system is detailed in the Drainage Management Plan. - Emergency procedures are outlined in the Accident Management Plan. - Staff training on spillages. - Batteries are stored in a lidded box on an impermeable surface. | Low |
| Spillage of liquids. | Direct run-off from site across ground surface, via surface water drains, ditches etc. | All surface waters close to and downstream of site. Acute effects: oxygen depletion, fish kill and algal blooms. | Med | High | High | There is no liquid waste processed on site. However, spillage of liquids such as fuel or oil could cause long-term harm. All surface water on site will be discharged to sewer via an interceptor under discharge consent. | - Impermeable concrete surface with falls towards drains prevents run-off from flowing off site. - All surface waters generated on site falls on an impermeable surface towards the direction of a gully. - The site has a sealed drainage system. - Fuel tank is bunded and situated on impermeable concrete. - Spill kits and drain cover mats present on site for use if a spill is detected. - Routine inspection and maintenance of fuel tank and site surface. - The site drainage system is detailed in the Drainage Management Plan. - Emergency procedures are outlined in the Accident Management Plan. - Staff training on spillages. - COSHH risk assessments have been carried out on all hazardous substances stored onsite, which will be stored in a locked, bunded and controlled chemical storage unit. | Low |
| Spillage of liquids. | All surface waters close to and downstream of site. | All surface waters close to and downstream of site. Chronic effects: deterioration of water quality. | Low | High | Med | Spillage of liquids such as fuel could cause long-term harm. All surface water on site will be discharged to sewer via an interceptor under discharge consent. | - Impermeable concrete surface with falls towards drains prevents run-off from flowing off site. - All surface waters generated on site falls on an impermeable surface towards the direction of a gully. - The site has a sealed drainage system. - Fuel tank is bunded and situated on impermeable concrete. - Spill kits and drain cover mats present on site for use if a spill is detected. - Routine inspection and maintenance of fuel tank and site surface. - The site drainage system is detailed in the Drainage Management Plan. - Emergency procedures are outlined in the Accident Management Plan. - Staff training on spillages. - COSHH risk assessments have been carried out on all hazardous substances stored onsite, which will be stored in a bunded, locked and controlled chemical storage. | Low |
| Spillage of liquids. | Direct run-off from site across ground surface, via surface water drains, ditches etc. then abstraction. | Acute effects, closure of abstraction intakes. | Low | High | Med | Spillage of liquids such as fuel could cause long-term harm. All surface water on site will be discharged to sewer via an interceptor under discharge consent. Watercourse must have medium / high flow for abstraction to be permitted, which will dilute contaminated run-off. | - Impermeable concrete surface with falls towards drains prevents run-off from flowing off site. - All surface waters generated on site falls on an impermeable surface towards the direction of a gully. - The site has a sealed drainage system. - Fuel tank is bunded and situated on impermeable concrete. - Spill kits and drain cover mats present on site for use if a spill is detected. - Routine inspection and maintenance of fuel tank and site surface. - The site drainage system is detailed in the Drainage Management Plan. - Emergency procedures are outlined in the Accident Management Plan. - Staff training on spillages. - COSHH risk assessments have been carried out on all hazardous substances stored onsite, which will be stored in a bunded, locked and controlled chemical storage. | Low |
| Accidental fire causing the release of polluting materials to air (smoke or fumes), water or land. | Air transport of smoke. Spillages and contaminated firewater by run off or via drainage system. | Human population and environment. | Med | High | High | Extremely rare – appropriately stored waste, fuel and limited sources of ignition. Waste accepted onto site has low flammability. Site also has Fire Prevention Plan in place. However, the proximity of sensitive receptors means the magnitude is high. Waste fires are not common but approximately 300 fires per year are linked to waste activities. Impact on health and amenity can be significant for many days or weeks. | - Authorised activities do not permit the burning of waste. - Fire Prevention Plan. - Accident Management Plan. - Daily site inspection and housekeeping measures. - Waste is stored in designated storage areas. - Monitoring of stockpiles. | Low |
| Arson and / or vandalism causing the release of polluting materials to air (smoke or fumes), water or land. | Air transport of smoke. Spillages and contaminated firewater by run-off to or via drainage system. | Human population and local environment | Med | High | High | Proximity of sensitive receptors. | - No public access to the site. - CCTV surrounding the site. - Fire Prevention Plan. - Palisade fencing around the perimeter of the site. - Daily lock up at end of each working day. - Accident Management Plan. | Low |
| Serious fire. | Direct run-off of fire water across site to surface waters. | All surface waters close to and downstream of site. | Low | High | Med | Extremely rare – low flammability, appropriately stored waste, fuel and limited sources of ignition. FPP in place. Waste fires are not common but approximately 300 fires per year are linked to waste activities. In event of fire, sufficient fire water can be sourced. Contaminated firewater run-off can kill fish and aquatic life. | - Authorised activities do not authorise the burning of waste. - Fire Prevention Plan. - Accident Management Plan. - Complaints procedure and investigation. - Daily site inspection and housekeeping measures. - Waste is storage in designated storage areas. | Low |
| Any | Any | Rye Meads SSSI (within 1000m of the site) | Med | High | High | Waste operations may cause harm to and deterioration of nature conservation sites. | - Rye Meads meadows are the last substantial remnants of ancient flood-meadows on the rich alluvial soils of the Lea Valley. - Site is 906m to the southwest and unlikely to affect the SSSI due to the low dust generation measures employed on site and large number of industrial buildings that will affect any dispersion. | Very Low |
| P = Possibility C = Consequence M = Magnitude |  |  |  |  |  |  |  |  |