WRM

wrm-ltd.co.uk

01943 468138

Dust and Emissions Management Plan

v1.0

Environmental and sustainability solutions provided to

Advanced Demolition Ltd

WRM-LTD.CO.UK

This report was prepared by Walker Resource Management Ltd (WRM’) within the terms of its engagement and in direct response to a scope of services. This report is strictly limited to the purpose and the facts and matters stated in it and does not apply directly or indirectly and must not be used for any other application, purpose, use or matter. In preparing the report, WRM may have relied upon information provided to it at the time by other parties. WRM accepts no responsibility as to the accuracy or completeness of information provided by those parties at the time of preparing the report. The report does not take into account any changes in information that may have occurred since the publication of the report. If the information relied upon is subsequently determined to be false, inaccurate, or incomplete then it is possible that the observations and conclusions expressed in the report may have changed. WRM does not warrant the contents of this report and shall not assume any responsibility or liability for loss whatsoever to any third party caused by, related to, or arising out of any use or reliance on the report howsoever. No part of this report, its attachments or appendices may be reproduced by any process without the written consent of WRM. All enquiries should be directed to WRM.

| Document Title | Dust and Emissions Management Plan |  |
| --- | --- | --- |
| Client | Advanced Demolition Ltd |  |
| Revision | v1.0 |  |
| Date | 09/01/2026 |  |
| Document Reference | EPR-A01 |  |
| Project Reference | 1496/J05 |  |
| Author: Bryanna Heaton |  | Reviewer: Graeme Kennett |

Copyright ©

All material on these pages, including without limitation text, logos, icons and photographs, is copyright material of Walker Resource Management Ltd (WRM). Use of this material may only be made with the express, prior, written permission of WRM. This document was produced solely for use by the named client to whom the document refers. The methodology (if any) contained in this report is provided to you in confidence and must not be disclosed or copied to third parties without the prior written agreement of WRM. Disclosure of that information may constitute an actionable breach of confidence or may otherwise prejudice our commercial interests.

| Revision | Details | Date |
| --- | --- | --- |
| 0.1 | Initial draft | 24/06/2025 |
| 1.0 | Final version for submission | 08/01/2026 |

Revision Log

CONTENTS

Contents

1.0	Introduction	3

1.1	Receptors	3

2.0	Site operations	11

2.1	Delivery and unloading	11

2.2	Sorting and storage inside the recycling building	11

2.3	Waste records	12

2.4	Types of materials processed and treatment undertaken	12

2.5	Storage methods and stockpile configuration	13

2.6	Site surfacing and impermeable areas	13

2.7	Vehicle movements and routing	13

2.8	Wheel wash and vehicle cleanliness	13

2.9	Drainage and water discharge	13

2.10	Buildings: construction, enclosure, and ventilation	14

2.11	Waste storage arrangements and heights	14

2.12	Dust suppression systems and locations	14

3.0	Dust and particulate matter (PM10) management	15

3.1	Responsibility for DEMP implementation and training	15

4.0	Dust characteristics, risk context, and monitoring approach	36

4.1	Definitions used within this DEMP	36

4.2	Activities with potential to generate dust	37

4.3	Dust monitoring methodology	37

4.4	Monitoring locations and inspection regime	38

4.5	Escalation and review of monitoring requirements	38

4.6	Records, assurance, and management review	38

4.7	Out-of-hours considerations	38

5.0	Abnormal Events	40

5.1	Summer daily maximum temperature	40

5.2	Winter daily temperatures	42

5.3	Daily extreme rainfall	43

5.4	There is potential for drainage systems and interceptors to be overwhelmed.	43

5.5	Average winter rainfall	43

5.6	Drier summers	44

5.7	River flow	45

5.8	Storms	45

5.9	Estimating magnitude of risk	47

5.10	Ceasing of operations	47

5.11	Actions when alarm is triggered	49

6.0	Reporting and Complaints Response	50

6.1	Community engagement	50

6.2	Complaints management and reporting	50

6.3	Record keeping:	51

6.4	Dust pollution complaint response	52

6.5	Alignment with Appropriate Measures	52

7.0	Dust Complaint Form	55

List of tables

Table 1-1: Premises details	1

Table 1-2: Version and revision history.	1

Table 1-3: Checklist	2

Table 1-1: Sensitive receptor table	3

Table 1-2: Other sources of dust and/or other emissions	9

Table 2-1: Mobile plant and equipment	14

Table 3-1: Sources and control of fugitive dust / particulate emissions	1

Table 3-2: Measures that will be used on site to control dust/particulates (PM10) and other emissions.	4

Table 3-3: Appropriate measures used on site	12

Table 3-4: Contingency measures	16

Table 5-1: Estimating the magnitude of risk	5

Table 5-2: Risk matrix for the ceasing of operations	5

Table 5-3: Action required per risk magnitude rating	6

Table 6-1: Dust complaints response flow	1

Table 1-1: Premises details

| Premises Particulars |  |
| --- | --- |
| Premises name: Advanced Demolition Limited Address: Advanced Demolition Limited Pindar Road, Hoddesdon, EN11 0BZ Tel no: TBC Site opening times: Monday to Friday 06:00 – 17:00 Saturday 06:00-12:00. | Activities undertaken: The permit authorises the operation of a waste facility transferring and treating a range of non-hazardous wastes. |
|  | Owner/employer/person in control of the workplace: Site Manager – Billy Marchant |

| Document owner |  |
| --- | --- |
| Document author: |  |
| Version number: |  |

Table 1-2: Version and revision history.

| Version | Date | Originator | Checked by | Approved by | Amendments |
| --- | --- | --- | --- | --- | --- |
| 1 | 01/26 | WRM Ltd |  |  | Original |

Table 1-3: Checklist

|  | Operator response |
| --- | --- |
| Have you provided receptor information required in Section 1 below, including a site map showing receptors and receptor table? | Yes |
| Have you provided a detailed description of the site covering everything required in the Section 2 section below? | Yes |
| Have you provided information required in Section 3 below about the DEMP, the sources of dust and the appropriate measures that you have committed to for managing dust and emissions on site? | Yes |
| Have you provided all the information required in Section 4 below about particulate monitoring, types of analysers, data management, location of equipment etc? | Yes |
| Have you included all abnormal events and how these will be managed as required in Section 5 below? | Yes |

# Introduction

Advanced Demolition Limited (ADL), ‘the operator’ operate an Environmental Management System (EMS) to ensure that their operations meet high environmental standards and comply with the legislative requirements applicable to the site.  The DEMP is adopted into the site EMS to ensure that operations do not impact significantly upon the environment.

The operation has the potential to cause air quality impacts because of fugitive dust emissions associated with the operation of the facility, as well as road traffic exhaust emissions from vehicles travelling to and from the site. The potential effects and control measures are contained in this DEMP and the site-specific risk assessment.

This DEMP has been produced in accordance with EA Dust & Particulate Emission Management Plan Guidance ‘Control and Monitor emissions for your environmental permit’ (published 1st February 2016)  and version 12 (August 2025) of the Dust & Emission Management Plan (DEMP) template, and relates to waste materials accepted, stored and treated at the site which have the potential to produce fugitive emissions.

Receptors

Table 1-1: Sensitive receptor table

| Receptor | Land Use | Approximate distance from the site (m) | Grid Reference |
| --- | --- | --- | --- |
| SR01 – Industrial Property within Industrial Estate | Industrial | 60m | 538062, 209010 |
| SR02 – Industrial Property within Industrial Estate | Industrial | 66m | 538052, 209117 |
| SR03 – Industrial Property within Industrial Estate | Industrial | 21m | 538135, 209112 |
| SR04 - Industrial Property within Industrial Estate | Industrial | 31m | 538110, 209126 |
| SR05 - Industrial Property within Industrial Estate | Industrial | 50m | 538156, 209136 |
| SR06 - Industrial Property within Industrial Estate | Industrial | 60m | 538131, 209151 |
| SR07 - Industrial Property within Industrial Estate | Industrial | 70m | 538082, 209168 |
| SR08 - Industrial Property within Industrial Estate | Industrial | 83m | 538172, 209165 |
| SR09 - Industrial Property within Industrial Estate | Industrial | 110m | 538162, 209192 |
| SR10 - Industrial Property within Industrial Estate | Industrial | 126m | 538121, 209217 |
| SR11 - Industrial Property within Industrial Estate | Industrial | 154m | 538141, 209249 |
| SR12 – Industrial Property within Industrial Estate | Industrial | 140m | 538202, 209211 |
| SR13 – Industrial Property within Industrial Estate | Industrial | 180m | 538194, 209265 |
| SR14 - Industrial Property within Industrial Estate | Industrial | 210m | 538215, 209283 |
| SR15 - Industrial Property within Industrial Estate | Industrial | 240m | 538225, 209324 |
| SR16 - Industrial Property within Industrial Estate | Industrial | 276m | 538232, 209346 |
| SR17 - Industrial Property within Industrial Estate | Industrial | 264m | 538274, 209310 |
| SR18 - Industrial Property within Industrial Estate | Industrial | 291m | 538268, 209343 |
| SR19 - Industrial Property within Industrial Estate | Industrial | 309m | 538283, 209358 |
| SR20 - Industrial Property within Industrial Estate | Industrial | 340m | 538306, 209383 |
| SR21 - Industrial Property within Industrial Estate | Industrial | 50m | 538221, 209093 |
| SR22 - Industrial Property within Industrial Estate | Industrial | 125m | 538236, 208967 |
| SR23 - Industrial Property within Industrial Estate | Industrial | 140m | 538294, 209053 |
| SR24 - Industrial Property within Industrial Estate | Industrial | 161m | 538312, 209091 |
| SR25 - Industrial Property within Industrial Estate | Industrial | 174m | 538322, 209116 |
| SR26 - Industrial Property within Industrial Estate | Industrial | 195m | 538340, 209130 |
| SR27 - Industrial Property within Industrial Estate | Industrial | 218m | 538352, 209155 |
| SR28 - Industrial Property within Industrial Estate | Industrial | 235m | 538363, 209180 |
| SR29 - Industrial Property within Industrial Estate | Industrial | 260m | 538377, 209197 |
| SR30 - Industrial Property within Industrial Estate | Industrial | 275m | 538384, 209221 |
| SR31 - Industrial Property within Industrial Estate | Industrial | 303m | 538396, 209245 |
| SR32 - Industrial Property within Industrial Estate | Industrial | 320m | 538398, 209277 |
| SR33 - Industrial Property within Industrial Estate | Industrial | 270m | 538350, 209278 |
| SR34 - Industrial Property within Industrial Estate | Industrial | 215m | 537940, 208927 |
| SR35 - Industrial Property within Industrial Estate | Industrial | 289m | 537915, 208838 |
| SR36 - Industrial Property within Industrial Estate | Industrial | 371m | 537897, 208767 |
| SR37 - Industrial Property within Industrial Estate | Industrial | 170m | 538025, 208933 |
| SR38 - Industrial Property within Industrial Estate | Industrial | 202m | 538086, 208919 |
| SR39 - Industrial Property within Industrial Estate | Industrial | 265m | 538006, 208867 |
| SR40 - Industrial Property within Industrial Estate | Industrial | 310m | 537959, 208812 |
| SR41 - Industrial Property within Industrial Estate | Industrial | 327m | 537973, 208778 |
| SR42 - Industrial Property within Industrial Estate | Industrial | 350m | 537981, 208725 |
| SR43 - Industrial Property within Industrial Estate | Industrial | 148 | 538081, 208923 |
| SR44 - Industrial Property within Industrial Estate | Industrial | 160 | 538115, 208865 |
| SR45 - Industrial Property within Industrial Estate | Industrial | 339 | 538076, 208719 |
| SR46 - Industrial Property within Industrial Estate | Industrial | 425 | 537939, 208669 |
| SR47 - Industrial Property within Industrial Estate | Industrial | 414 | 537994, 208672 |
| SR48 - Industrial Property within Industrial Estate | Industrial | 490 | 537966, 208595 |
| SR49 - Industrial Property within Industrial Estate | Industrial | 550 | 537972, 208521 |
| SR50 - Industrial Property within Industrial Estate | Industrial | 460 | 537900, 208662 |
| SR51 - Industrial Property within Industrial Estate | Industrial | 507 | 537898, 208609 |
| SR52 - Industrial Property within Industrial Estate | Industrial | 545 | 537893, 208548 |
| SR53 - Industrial Property within Industrial Estate | Industrial | 551 | 538031, 208491 |
| SR54 - Industrial Property within Industrial Estate | Industrial | 392 | 538116, 208605 |
| SR55 - Industrial Property within Industrial Estate | Industrial | 382 | 538180, 208686 |
| SR56 - Industrial Property within Industrial Estate | Industrial | 322 | 538219, 208744 |
| SR57 - Industrial Property within Industrial Estate | Industrial | 225 | 538216, 208834 |
| SR58 - Industrial Property within Industrial Estate | Industrial | 281 | 538277, 208813 |
| SR59 - Industrial Property within Industrial Estate | Industrial | 201 | 538286, 208920 |
| SR60 - Industrial Property within Industrial Estate | Industrial | 247 | 538326, 208888 |
| SR61 - Industrial Property within Industrial Estate | Industrial | 215 | 538368, 208968 |
| SR62 - Industrial Property within Industrial Estate | Industrial | 226 | 538378, 209000 |
| SR63 - Industrial Property within Industrial Estate | Industrial | 243 | 538387, 209069 |
| SR64 - Industrial Property within Industrial Estate | Industrial | 256 | 538400, 209093 |
| SR65 - Industrial Property within Industrial Estate | Industrial | 294 | 538442, 209061 |
| SR66 - Industrial Property within Industrial Estate | Industrial | 322 | 538470, 209036 |
| SR67 - Industrial Property within Industrial Estate | Industrial | 306 | 538446, 209019 |
| SR68 - Industrial Property within Industrial Estate | Industrial | 289 | 538461, 209119 |
| SR69 - Industrial Property within Industrial Estate | Industrial | 336 | 538475, 209197 |
| SR70 - Industrial Property within Industrial Estate | Industrial | 370 | 538480, 209250 |
| SR71 - Industrial Property within Industrial Estate | Industrial | 390 | 538458, 209311 |
| SR72 - Industrial Property within Industrial Estate | Industrial | 395 | 538454, 209364 |
| SR73 - Industrial Property within Industrial Estate | Industrial | 387 | 538453, 209458 |
| SR74 - Industrial Property within Industrial Estate | Industrial | 450 | 538453, 209515 |
| SR75 - Industrial Property within Industrial Estate | Industrial | 414 | 538392, 209465 |
| SR76 - Industrial Property within Industrial Estate | Industrial | 515 | 538463, 209537 |
| SR77 - Residential Properties on New River Close | Residential | 181 | 537928, 209114 |
| SR78 - Residential Properties on New River Close | Residential | 205 | 537942, 209142 |
| SR79 - Residential Properties on Marsh Close | Residential | 215 | 537908, 209080 |
| SR80 - Residential Properties on Stortford Road | Residential | 265 | 537864, 209118 |
| SR81 - Residential Properties on Stortford Road | Residential | 305 | 537825, 209151 |
| SR82 - Residential Properties on Stortford Road | Residential | 193 | 537998, 209225 |
| SR83 - Residential Properties on Stortford Road | Residential | 217 | 538065, 209289 |
| SR84 - Residential Properties on Stortford Road | Residential | 245 | 538108, 209333 |
| SR 85 - Residential Properties on Stortford Road | Residential | 350 | 538211, 209432 |
| SR86 – Residential Properties on Gladstone Road | Residential | 260 | 537900, 209346 |
| SR87 – Residential Properties on River Avenue | Residential | 372 | 537812, 209315 |
| SR88 - Residential Properties on Cranborne Road | Residential | 290 | 537993, 209398 |
| SR89 – Residential Properties on Aylsham Road | Residential | 370 | 538092, 209451 |
| SR90 – Residential Properties on Ivy Terrace | Residential | 395 | 538162, 209494 |
| SR91 – Residential Properties on Essex Road | Residential | 500 | 537644, 209168 |
| SR92 – Recreational Ground off Rye Road | Residential | 490 | 537640, 209366 |
| SR93 – Residential Properties on Rye Road | Residential | 505 | 537737, 209517 |
| SR94 – Residential Properties on Lilac Road | Residential | 669 | 537712, 209621 |
| SR95 – Residential Properties on Hawthorn Road | Residential | 761 | 537718, 209730 |
| SR96 – Residential Properties on Whitley Road | Residential | 806 | 537874, 209842 |
| SR97 – Residential Properties on Crossfield Road | Residential | 600 | 537886, 209658 |
| SR98 – Residential Properties on Walton Road | Residential | 594 | 538029, 209684 |
| SR99 – Residential Properties on Haward Road | Residential | 660 | 538105, 209750 |
| SR100 – Residential Properties on Rumbold Road | Residential | 555 | 538133, 209689 |
| SR101 – Industrial Property off Ratty’s Lane | Industrial | 375 | 538506, 208868 |
| SR102 – Power Station off Ratty’s Lane | Industrial | 600 | 538748, 209046 |

Table 1-2: Other sources of dust and/or other emissions

| Company | Address | Business type | Distance from boundary |
| --- | --- | --- | --- |

Figure 1-1: Wind rose (Meteoblue)

# Site operations

Waste will be delivered to the site by;

- 32t LGV tippers

- Hi Ab LGV (grab lorry)

- Skip wagons (hook and chain and ro-ro)

- Articulated large goods vehicles (LGVs)

- Small vans/tippers

All LGVs operated by the operator will have a Euro 6 emission rating. It is likely that 3rd party hauliers will be at least Euro 5 emission rating.

All 8-wheel tipper vehicles will be fitted with a fly sheet and 44t articulated LGVs fitted with an easy sheet.

Skip wagons/grab lorries will also be sheeted.

Delivery and unloading

- All waste will be delivered to the site in covered skips through the existing site entrance.

- Once on the site and after being weighed, skips will be unloaded under cover of the recycling building. The waste materials will then be sorted and stored in the internal storage bays pending transfer.

- Soil and rubble skips will be unloaded directly into the designated storage bays.

Sorting and storage inside the recycling building

Once deposited inside the recycling building the waste will be sorted with the aid of mechanical mobile plant, including a wheeled loading shovel and 360 excavator with grab.

Excluding soils and rubble, sorted materials will be positioned inside the recycling building in designated storage bays pending external transfer.

The storage bays will be constructed using concrete panels and will allow for:

- green waste,

- wood,

- metal,

- plasterboard and

- residual waste that cannot be recycled.

Waste records

Records of all materials entering and the leaving the site shall be recorded, as soon as reasonably practicable and retained securely for a minimum of three years. Records will be clear, legible and available for viewing (on site). For commercial and industrial waste, the following information will be acquired:

- Details of the waste producer including their name, address, and contact details

- A description of the waste

- European Waste Classification (EWC) code

- The source of the waste (the producer’s business and the specific process that has created the waste)

- Information on the nature and variability of the waste production process

- Information about the history of the producer site if it may be relevant to the classification of the wate (e.g., soils and other construction and demolition arisings from a site contaminated by previous industrial uses)

- The waste’s physical form

- The waste’s composition (based on representative samples if necessary)

- A description of the wastes odour and whether it is likely to be odorous

- An estimate of the quantity expected

- For mirror entry LoW codes, assessment evidence will be retained

Types of materials processed and treatment undertaken

The site operates solely as a non-hazardous waste transfer facility. Wastes received are not subject to crushing, screening, shredding or other mechanical size-reduction activities at Hoddesdon. Typical materials include mixed commercial and industrial wastes, residual wastes, and source-segregated recyclables. Waste handling is limited to inspection, bulking, consolidation, and transfer for onward recovery or disposal. These activities are inherently low dust-generating when compared with processing operations.

Storage methods and stockpile configuration

Waste is stored either within designated concrete bays or in enclosed containers/skips. There are no free-standing stockpiles on site. Bay walls are constructed of reinforced concrete and are 4m in height. Waste is always stored 1m below the top of bay walls which provides effective containment and wind shielding, minimising the potential for dust liberation.

Site surfacing and impermeable areas

All operational areas of the site, including waste reception, storage, and vehicle circulation routes, are surfaced with impermeable reinforced concrete. These surfaces are designed to withstand vehicle movements and facilitate effective cleaning and dust suppression. No unmade or permeable surfaces are present within operational areas.

Vehicle movements and routing

Vehicles enter the site via the main access gate and proceed directly to the designated reception and tipping area. Following unloading, vehicles exit via the same internal route. Vehicle movements are controlled and low-speed, with no cross-site haul roads or unnecessary circulation. The limited extent of vehicle movements and the hard-surfaced nature of the site significantly reduce the potential for dust generation.

Wheel wash and vehicle cleanliness

A dedicated wheel wash is not required due to the fully concrete-surfaced nature of the site and the absence of dusty processing activities. However, a pressure washer is available on site and used as required to clean vehicle wheels, bodywork, and hardstanding areas, particularly during dry conditions or if visible dust tracking is observed.

Drainage and water discharge

Surface water from operational and storage areas drains via the site’s sealed drainage system to on-site interceptors prior to discharge to the foul sewer under an appropriate trade effluent consent. There is no discharge to groundwater. Any water used for dust suppression remains within the impermeable drainage network, ensuring environmental protection and regulatory compliance.

Buildings: construction, enclosure, and ventilation

Waste handling takes place within the purpose-built industrial building. The structure comprises steel portal framing with profiled metal cladding and a sealed concrete floor. All wall and roof junctions are sealed to prevent fugitive emissions. The building is enclosed on three sides, with access via roller shutter doors which remain closed when not in active use. Access and egress points are limited and controlled. Given the low dust-generating nature of the activities, no dedicated air extraction or filtration system is required; natural ventilation via controlled door openings is sufficient.

Waste storage arrangements and heights

Waste is stored in concrete bays and enclosed containers. Bay walls are 4m high, with waste stored no higher than 3m. Containerised wastes are fully enclosed. There are no freestanding stockpiles. Storage heights do not materially vary by waste type, as all wastes handled are managed within the same controlled infrastructure.

Dust suppression systems and locations

Dust suppression is primarily preventative, based on enclosure, good housekeeping, and operational controls. Suppression measures include:

- Regular inspection and cleaning of hardstanding areas.

- Use of a mobile hose or pressure washer for dampening surfaces during dry or windy conditions.

- Immediate clean-up of spillages.

There are no fixed suppression systems, as they are not necessary given the enclosed operations and absence of dust-intensive processing. Mobile suppression equipment is available and can be deployed anywhere within operational areas as required.

Table 2-1: Mobile plant and equipment

| Description | Make | TAN (Type Approval Number) | Emission Rating |  | Hired / owned? | ULSD used? |
| --- | --- | --- | --- | --- | --- | --- |

# Dust and particulate matter (PM10) management

Responsibility for DEMP implementation and training

Overall aims and objectives of the DEMP

The aim of this Dust and Emissions Management Plan (DEMP) is to ensure that dust emissions arising from site activities are prevented where possible and otherwise minimised through effective operational controls, staff competence, and ongoing review. The DEMP supports compliance with the Environmental Permit, protects local amenity and sensitive receptors, and integrates with the site’s wider Environmental Management System (EMS).

Responsibility for the DEMP

Overall responsibility for implementation, maintenance, and effectiveness of the DEMP rests with the Site Manager. The Site Manager is responsible for ensuring that all site activities are undertaken in accordance with the requirements of the DEMP and that appropriate controls are in place and functioning effectively at all times.

Deputy responsibility

In the absence of the Site Manager, responsibility for the DEMP is delegated to the Assistant Site Manager / Supervisor, who is fully briefed on the requirements of the plan and has the authority to implement dust control measures and halt activities if necessary.

Location and availability of the plan

The current version of the DEMP is stored electronically within the site’s management system and as a hard copy in the site office. The plan is readily accessible to all relevant staff and made available to the Environment Agency upon request.

Staff competence to implement and review the DEMP

All operational staff receive site-specific induction training which includes dust awareness, identification of potential dust sources, and implementation of mitigation measures set out in the DEMP. Competence is maintained through a combination of formal training, toolbox talks, and on-the-job supervision. The Site Manager is responsible for reviewing the effectiveness of the DEMP as part of routine site inspections and management reviews.

Review of the plan

The DEMP is formally reviewed at least annually, and additionally:

- Following any significant change to site activities or waste types accepted

- After a substantiated dust complaint

- Following any incident or near miss relating to dust emissions

- If required by the Environment Agency

Reviews ensure the plan remains proportionate, current, and effective.

Responsibility for staff training

The Site Manager is responsible for ensuring that staff receive appropriate training relevant to dust management and implementation of the DEMP. Training provided includes site induction, dust risk awareness, good housekeeping practices, and the correct use of suppression equipment.

Training frequency and delivery

Training is delivered through a combination of internal instruction, toolbox talks, and external training providers where appropriate. Dust management training is provided at induction and reinforced through periodic toolbox talks, typically on an annual basis or more frequently if required by operational or regulatory change.

Refresher training

Refresher training is provided at least annually and whenever there is a change to the DEMP, site layout, operational practices, or regulatory requirements. Additional refresher training may be provided following any dust-related incident or complaint.

Integration with other management documents

The DEMP forms part of the site’s wider Environmental Management System and operates in conjunction with other key documents, including the Fire Prevention Plan, Accident Management Plan, and site operating procedures. These documents are cross-referenced to ensure consistent controls, shared responsibilities, and a coordinated approach to environmental risk management.

Other relevant information

Dust management performance is monitored through routine visual inspections of site surfaces, storage areas, and vehicle movements. Any issues identified are recorded and corrective actions implemented promptly. This proactive management approach ensures that dust risks remain low and that the DEMP continues to function effectively as a live operational document.

Table 3-1: Sources and control of fugitive dust / particulate emissions

| Source | Pathway | Receptor | Impact | Control |
| --- | --- | --- | --- | --- |
| Vehicles entering or leaving site with debris on wheels | Tracking dust on wheels and vehicles, then mud dropping off wheels/vehicles when dry | Local highways | Visual soiling, also consequent resuspension as airborne particulates | Vehicles unlikely to have come in to contact with mud prior to accessing the site. Concrete surface ensures debris is not deposited on the public highway. |
| Vehicles and plant moving around the site kicking up dust | Atmospheric dispersion | Neighbouring businesses | Airborne particles | Regular dampening down of all vehicle routes will be undertaken. Implementation of a 5mph speed limit Implementation of a ‘no-idling’ policy Limiting the double-handling of material Dampening surfaces to limit resuspension of dust. Regular cleaning of site/spills in accordance with EMS. |
| Material falling from vehicles | Material falling from vehicle bodies | Local highways | Visual soiling, also consequent resuspension as airborne particulates | Sides/lips of tipping bodies swept before departure to remove any accumulated materials. Vehicles delivering waste are sheeted. Skips that are not immediately sorted will be sheeted to contain any light fractions and dust which could be blown out. Where debris is identified as an ongoing issue a road sweeper is deployed. All areas are subject to regular housekeeping. |
| Site surfaces | Contamination of trafficked areas. | Neighbouring businesses | Visual soiling, also consequent resuspension as airborne particulates | Site surfaces used by traffic will be regularly cleaned along with the measures described above. Regular dampening down of all vehicle routes will be undertaken. |
| Tipping of wastes in the open | Atmospheric dispersion | Neighbouring businesses | Airborne particles | Accepted waste materials are generally not inherently dusty due to moisture content. Use of the minimum drop heights. Storing waste in bays with a minimum of 0.5m freeboard to prevent ‘wind whip. Storage of plasterboard in covered container. Use of mist sprays on known dust-generating activities as standard Increase use of water sprays on stored waste |
| Tipping, storage and sorting of waste inside the process building | Escape from buildings and subsequent atmospheric dispersion | Neighbouring businesses | Airborne particles | Direct doors away from most sensitive receptors. Minimise source strength by misting/water/barrier techniques. |
| Particulate emissions from machinery exhausts | Atmospheric dispersion | Neighbouring businesses | Airborne particles | Anti-idling policy in place. |
| Stockpiling materials | Atmospheric dispersion | Neighbouring businesses | Airborne particles | The finer processed materials have a limited potential to generate airborne particles during loading. Loading shovel dump heights will be kept as low as possible. Material will be tipped on to stockpiles in a controlled manner to reduce the potential for particulates to be propelled from the trailer. Shovel buckets will be kept as low as possible to the ground to reduce spillage. Dampening down of stockpiles will be undertaken during loading periods, if required. |
| Wind-whip from stockpiles | Atmospheric dispersion | Neighbouring businesses | Airborne particles | Dampening down of stockpiles will be undertaken if required. |

Table 3-2: Measures that will be used on site to control dust/particulates (PM10) and other emissions.

| Abatement measure | Description/effect | Overall consideration and implementation | Trigger for implementation |
| --- | --- | --- | --- |
| Enclosure within the process building | Creating a solid barrier between the source of dust and particulates and receptors is likely to be the most effective method of control, provided that the building entrances and exits are well managed. | Procedures in place to manage the building and its integrity. Inputs are processed rapidly. Building is open fronted but orientated away from sensitive receptors to prevent dust migration from the operation. | Creating a solid barrier between the source of dust and particulates and receptors is likely to be the most effective method of control, provided that the building entrances and exits are well managed. |
| Site layout in relation to receptors | Crushing and screening activities that emit dust particles are located at the maximum potential distance from down-wind receptors without impacting upwind receptors. Adjacent forestry belt offers protection from the wind. 3m and 2.5m fencing to the south side of the site. | Dust control has been considered by the operator through good process and site design, as well as identification of good housekeeping procedures. The control methods are based on: Good operating and management practices to avoid emissions arising from activities; Good process design to minimise emissions; Abatement or control to reduce dust emissions; and, Disrupting the emission pathway to sensitive receptors | All processing equipment is fitted with dust suppression equipment to prevent emissions. Standard Operating Procedure (SOP) and training of staff with respect to correct operation of the equipment. This lessens the likelihood of dust from the operations causing a nuisance to others. However, the objective is to reduce dust at source and not to let dust leave the site boundary. |
| Site speed limit, ‘no idling’ policy and minimisation of vehicle movements on site | Reducing vehicle movements and idling will reduce emissions from vehicles. Procurement policy to only purchase clean burn road vehicles and non-road going mobile machinery. Enforcement of the mandatory 10mph speed limit will limit re-suspension of particulates by vehicle wheels. | Straightforward to implement as part of good practice. Measures are identified clearly in the site management system. 5 mph speed limit is identified clearly in the site management system and on site and implemented as appropriate measures. | If significant volumes of dust are noted during routine visual monitoring the following actions will be taken: Observations undertaken to ensure that vehicles are obeying speed limits; and, Additional road sweeping. Staff training. Site signage. |
| Minimising drop heights for waste. | Minimising the height at which waste is handled will reduce the distance over which debris, dust and particulates could be blown and dispersed by winds. | Measure in place and form part of toolbox talks. Steps are identified clearly in the site management system and implemented as appropriate measures. Plant operatives lowering the grabs, shovels, conveyors on equipment being used to move or treat potentially dusty materials. | Control measure always in place during site operations and will form a part of toolbox talks on an ongoing basis. |
| Ceasing operation during high winds and/or prevailing wind direction | Mobilisation of dust and particulates is likely to be greater during periods of strong winds and hence ceasing operation at these times may reduce peak pollution events. | SM will monitor weather forecasts and ensure the necessary on-site precautionary measures are in place to prevent emissions. All personnel employed on site will undertake visual monitoring for dust throughout the working day. Any observed problems will be reported to the SM who will investigate the cause and implement any necessary remedial action. | If significant volumes of dust are noted during routine visual monitoring the following action will be taken: All dust suppression equipment is operating correctly Action taken to ensure that vehicles are obeying speed limits; and, Additional dampening down of all vehicle routes. Dust netting (of at least 1.5m high) fitted around the top of the fencing along the western and southern boundaries. |
| Easy to clean concrete impermeable surfaces | Creating an easy to clean impermeable surface, using concrete as opposed to hard core, reduces the amount of dust and particulate generated at ground level by vehicles and site activities. Enforcement of the mandatory 10mph speed limit will limit re-suspension of particulates by vehicle wheels. | Entire site has a concrete surface; Site has a 5mph speed limit. | Roadways in normal use and any other area where there is regular movement of vehicles have a concrete surface capable of being cleaned and are kept clean to prevent or minimise dust emissions and kept in good repair. |
| Minimisation of waste storage heights and volumes on site | Minimising the height at which waste is handled should reduce the distance over which debris, dust and particulates could be blown and dispersed by winds. Reducing storage volumes should reduce the surface area over which particulates can be mobilised. | This abatement measure is not likely to have a significant impact on dust emission levels. | At all times when waste is being stored. |
| Reduction in operations (waste throughput, vehicle size, operational hours) | Reducing the amount of activity on site, including no tipping or screening of high-risk loads during windy weather as well as associated traffic movements, should result in reduced emissions and re-suspension of dust and particulates from a site. | The site has procedures in place to reduce activity on site if required through complaints or known issues, or adverse weather conditions. This includes a weather station to monitor weather so that working procedures can be modified if required. | Screening equipment is fitted with suppression equipment to reduce dust emission potential. Material with inherent moisture greater than 3%, e.g., sand or gravel, would not be expected to give rise to emissions of dust. |
| Good housekeeping | A consistent, regular housekeeping regime, supported by management, is in place that ensures the site is regularly checked and issues remedied to prevent and remove dust and particulate build up. | Currently in place and requires minimal equipment. Encourages a sense of pride and satisfaction amongst the staff which promotes vigilance and a positive culture. Staff target the areas not caught by the road sweeper and other cleaning apparatus. | Completion of inspection checklist and Housekeeping procedure. |
| Remedial measures |  |  |  |
| On-site sweeping | Sweeping can be effective in managing larger debris, dust and particulates but may also cause the mobilisation of smaller particles. Road sweeping vehicles damp down dust and particulates whilst brushing and collecting dust and particulates from the road surface, particularly at the kerbside. This may generate dust and particulate movement that may become a Health and Safety issue if the filters and spray bars on the sweepers are not maintained. | Easy to apply but less effective than other measures. Covered in the management system procedures and implemented thoroughly. Operation covered by regular toolbox talks along with the triggers for operation of the sweeper. Sweeper maintained to ensure that its operation is effective. Manufacturer maintenance schedules are adhered to detailing when consumable items on road sweepers are replaced (filters, brushes etc). | Roadways in normal use and any other area where there is regular movement of vehicles have a surface capable of being cleaned. Working surfaces kept clean to prevent or minimise dust emissions and kept in good repair. |
| Water suppression with hoses & water jets | Damping down of site areas can reduce dust and particulate re-suspension and may assist in the cleaning of the site if combined with sweeping. | Can be water intensive, so only to be used in adverse weather conditions. | Mobile dust control suppression units will be utilised across the site when necessarry. The units spray a fine mist at the source of dust emissions and can be manoeuvred to accommodate wind direction; |
| Water suppression with mist sprays | Installation of mist sprays at point source emissions can assist in the damping down of dust and particulates, therefore, reducing emissions from site. | Very effective at controlling point source emissions of dust and particulates. Can be installed to conveyors and areas where waste is dropped. ‘Halo’ rings can be fitted to conveyor drops on screeners to minimise dispersion. | Mobile dust control suppression units will be utilised across the site when necessarry. The units spray a fine mist at the source of dust emissions and can be manoeuvred to accommodate wind direction; |
| Material storage heights | Staff training. | Implemented via Waste Storage and Handling Procedure in EMS. | A mark showing the required freeboard height, will be added to all storage areas to allow the easy identification by Site Operatives. Waste storage areas will be checked regularly as part of the Inspection Checklist. |

Table 3-3: Appropriate measures used on site

| Abatement measure | Description/effect | Overall consideration and implementation | Implementation control measure |
| --- | --- | --- | --- |
| 5 mph site speed limit for all vehicles | Minimisation of fugitive emissions from site surfacing/ vehicle wheels/ loads by keeping vehicle speed low | All drivers delivering waste are reminded of the speed limit Operator’s primary control is the site induction and toolbox talks. | If non-compliance is observed, a strike will be given, which when tallied up to 3 strikes for repeat offenders, the haulier will be contacted, and driver banned from site. |
| Vehicles are sheeted upon arrival and exit | Vehicles are sheeted upon arrival and departure. Vehicles uncover and unload at waste reception area and sheet up once loaded when removing waste or recovered material from site. | All drivers delivering waste and collecting waste/recovered materials are reminded to keep vehicles sheeted. Driver’s under the Operator’s primary control will be subject to a site induction and toolbox talks. | If non-compliance is observed, a strike will be given, which when tallied up to 3 strikes for repeat offenders, the haulier will be contacted, and driver banned from site. |
| Visual monitoring inspection & checklist | Daily visual monitoring is required to be completed daily by the Site Supervisor or nominated site operative, where wind direction, airborne dust, dust soiling and weather conditions is to be recorded at external and internal locations. This will inform the need to use any additional preventative measures. | The number of visual inspections is increased in accordance with the weather conditions and following an emissions incident or complaint. The inspections are undertaken during normal operating conditions and not during breaks. The inspection will include a check of surface condition, waste acceptance, tipping/loading activities, waste and material storage, use and effectiveness of dust suppression systems and condition of process structures. | A minimum of 1 visual inspection will be undertaken per day. During dry / windy conditions, four inspections will be undertaken per day. |
| Air emissions awareness training | All staff receive internal air emissions awareness training at site induction and via regular toolbox talks to foster awareness on emissions reduction. | All staff receive internal air emissions awareness training at site induction and through regular toolbox talks. | All staff receive internal air emissions awareness training at site induction and through regular toolbox talks. Training/ enforcement as required following investigation of an excessive dust emission event. |
| Plant complies with current emission standards | New and replacement items of plant to be specified to meet the relevant emission standards. | New and replacement items of plant to be specified to meet the relevant emission standards. In the event of breakdown, replacement plant will be to Tier 4 standards wherever possible. If this is not possible, plant meeting Tier 2 or 3 may be used for short periods whilst alternatives are sourced. | Procurement specification. |
| Routine servicing of plant and equipment. | All items of plant and equipment are serviced in line with manufacturers’ guidance to reduce NO2 and particulate emissions. A preventative maintenance regime is in place to make sure that equipment is maintained in good condition. | Good maintenance ensures plant is working satisfactorily and is efficient in fuel burn and emission control. Scheduled maintenance of abatement plant such as dust suppression systems and sprays on process equipment reduces the risk of failure and potential release of emissions. | Frequency of servicing is undertaken in line with manufacturer’s guidance, or as faults or excessive emissions are identified. |
| Plant and equipment switched off when not in use. | No idling policy in place. | The importance of this measure is reinforced during the regularly held toolbox talks, site inductions and during site walkovers by the SM and the site operative nominated for visual dust monitoring. Good management control will minimise NO2 and PM10 emissions from exhaust fumes. | During site walkovers by the SM and/or supervisors, operatives and drivers will be reminded to switch off their engines if idling is observed. Repeat incidences will be recorded in the Site Diary. |
| On-site sweeping | Sweeping can be effective in managing larger debris, dust and particulates but may also cause the mobilisation of smaller particles. Road sweeping vehicles damp down dust and particulates whilst brushing and collecting dust and particulates from the road surface, particularly at the kerbside. This may generate dust and particulate movement that may become a Health and Safety issue if the filters and spray bars on the sweepers are not maintained. | Easy to apply but less effective than other measures. Covered in the management system procedures and implemented thoroughly. Operation covered by regular toolbox talks along with the triggers for operation of the sweeper. Sweeper maintained to ensure that its operation is effective. Manufacturer maintenance schedules are adhered to detailing when consumable items on road sweepers are replaced (filters, brushes etc). | Roadways in normal use and any other area where there is regular movement of vehicles have a surface capable of being cleaned. They are kept clean to prevent or minimise dust emissions and are kept in good repair. |
| Water suppression with hoses & water jets | Damping down of site areas can reduce dust and particulate re-suspension and may assist in the cleaning of the site if combined with sweeping. | Can be water intensive, so only to be used in adverse weather conditions. | Mobile dust control suppression units will be utilised across the site. The units spray a fine mist at the source of dust emissions and can be manoeuvred to accommodate wind direction; |
| Water suppression with mist sprays | Installation of mist sprays at point source emissions like conveyors, crusher etc. It can also assist in the damping down of dust and particulates, therefore, reducing emissions from site. | Very effective at controlling point source emissions of dust and particulates. Can be installed to conveyors and areas where waste is dropped. ‘Halo’ rings can be fitted to conveyor drops on concrete crushers and screeners to minimise dispersion. | Mobile dust control suppression units will be utilised across the site. The units spray a fine mist at the source of dust emissions and can be manoeuvred to accommodate wind direction; |

Table 3-4: Contingency measures

| Potential emission source | Risk | Typical actions to reduce emissions | Trigger for contingency measure | Specific contingency measure (Backstop) | Monitoring trigger that will indicate a return to normal operations |
| --- | --- | --- | --- | --- | --- |
| Dusty feedstock received at the processing site | Crushed waste accepted at the site produces excessive dust | The mobile dust control suppression units will dampen down areas of dust generation. | Dust detected at the point of discharge. [Dust detected at boundary] | Increase wetting measures. [Remove from site] | Dust not detected at site boundary. |
| Stockpiled unprocessed feedstock becomes too dry in hot weather. | Waste stockpiles are held for excessive periods of time without processing taking place. | Stockpiles on site are processed quickly. Material deposited first is processed first. The mobile dust control suppression units will dampen down areas of dust generation. | Long periods of hot, dry weather reducing the moisture content of the material. [Dust detected at boundary] | Ensure all material is processed as soon as possible and placed in stockpiles. [Remove from site] | Dust not detected at site boundary. |
| Sorting of materials | Waste materials are sorted when the wind direction is towards areas accessed, or inhabited, by sensitive receptors. | Mobile dust control suppression units will be utilised across the site. The units spray a fine mist at the source of dust emissions and can be manoeuvred to accommodate wind direction. | Weather forecast is used to ascertain the effect of the wind speed/direction on the operation. [Dust detected at boundary] | Operations that may release excessive dust are programmed to take place only during favourable weather conditions. Careful monitoring of the conditions will ensure that the ‘window’ for processing is broad enough to allow flexibility to wait for suitable weather conditions. [Cease processing and remove if excessively dusty] | Dust not detected at site boundary. |
| Facility maintenance/equipment cleaning | Accumulations of waste material are allowed to degrade and become dry and produce dust. | Regular clean downs of machinery will prevent dust from accumulating. Areas will be swept/wetted when empty to prevent dust blowing. Regular dampening down of all vehicle routes will be undertaken; | When a stockpile is planned for removal, the area will be swept/tidied as the pile is reduced/removed. | Once a stockpile has been moved, it’s footprint will be scraped with the loader bucket to prevent a build – up of material. | Dust not detected at site boundary. |
| Vehicles leaving site | Spilt accumulations of material are allowed to dry out become desiccated and produce emissions. | Brushing down of trailer sides and sheets in place before leaving will prevent material dropping from trailers and prevent dust being generated. | Long periods of hot, dry weather reducing the moisture content of the material. | All vehicles are checked and prevented from leaving site if not sheeted or clean. | Dust not detected at site boundary. |

# Dust characteristics, risk context, and monitoring approach

The site is located on an established industrial estate at Pindar Road, Hoddesdon. The nearest sensitive residential receptor is located approximately 180 metres from the site boundary. Surrounding land uses are predominantly industrial and commercial in nature.

The site operates solely as a non-hazardous waste transfer station and does not undertake crushing, screening, shredding, or other mechanically intensive processing activities. In accordance with Non-hazardous and inert waste: appropriate measures for permitted facilities - Guidance - GOV.UK, dust risk is therefore considered inherently low and capable of being effectively managed through prevention, enclosure, and good operational control rather than continuous quantitative monitoring.

Dust is a generic term for particulate matter (PM) and typically comprises airborne particles within the size range of approximately 1 to 75 micrometres (µm). Particles below 10 µm (PM₁₀ and smaller) are primarily associated with potential health effects, whereas larger particles are associated with amenity impacts such as visible deposition.

Dust that may arise from site activities is expected to consist predominantly of larger, settleable particles, generated during vehicle movements and the handling of dry waste materials. These particles are short-lived, settle rapidly, and are most effectively managed through containment, housekeeping, and visual monitoring, consistent with EA guidance.

Definitions used within this DEMP

Within this Dust and Emissions Management Plan:

Excessively dusty material refers to waste containing a high proportion of dry fine particles such that handling would give rise to visible dust clouds. Wastes consisting primarily of powders, dusts, or loose fibrous materials are not accepted at the site.

Excessive dust generation refers to dust emissions that are visible beyond the site boundary, have the potential to cause amenity impacts at nearby receptors, or result in noticeable loss of local visibility.

Not effective, in relation to mitigation measures, describes circumstances where existing controls are insufficient to prevent dust emissions from leaving the site and additional or revised measures are required.

Activities with potential to generate dust

Activities with a limited potential to generate dust include:

- Vehicle movements on impermeable concrete hardstanding during dry conditions;

- The unloading, handling, and bulking of certain dry waste materials within concrete bays or enclosed areas.

No crushing, screening, or inert processing activities are undertaken at the site.

Dust monitoring methodology

In line with the EA Appropriate Measures, dust monitoring at the site is qualitative and risk-based, relying primarily on routine visual inspections rather than fixed particulate monitoring equipment. This approach is considered proportionate given:

- The absence of dust-intensive processing;

- The industrial estate location;

- The separation distance to sensitive receptors; and

- The enclosed and hard-surfaced nature of operations.

Site staff supervising waste handling activities maintain continuous awareness for visible dust emissions and surface deposition during operations. Where dust is observed with the potential to migrate beyond the site boundary, immediate corrective action is taken, which may include:

- Pausing the activity;

- Damping down surfaces or materials;

- Reviewing handling methods or storage practices.

Any dust-related incidents and the actions taken are recorded in the site diary.

Monitoring locations and inspection regime

Visual dust monitoring forms part of routine daily site inspections and task-based operational oversight. Inspections are not restricted to a single fixed location but follow a flexible route around the site perimeter, informed by:

- Prevailing wind direction;

- Weather conditions; and

- The nature and location of active operations.

Particular attention is given to downwind site boundaries. This approach ensures that inspections remain representative and effective, in accordance with EA guidance.

Escalation and review of monitoring requirements

The risk assessment has concluded that dust emissions can be screened out as insignificant under normal operating conditions. However, should persistent dust emissions occur, or should substantiated complaints be received, the operator will review:

- The effectiveness of existing mitigation measures; and

- The suitability of the monitoring regime.

Where necessary, the scope, location, and frequency of any particulate monitoring will be agreed with a competent monitoring specialist to ensure that monitoring is targeted, proportionate, and meaningful.

Records, assurance, and management review

All visual monitoring observations, weather conditions, wind direction, inspection dates, and the name of the inspecting staff member are recorded in the site diary and retained on site in accordance with the Environmental Management System (EMS).

Senior site management reviews inspection records periodically to identify trends, confirm ongoing compliance with Appropriate Measures, and ensure continuous improvement in dust control.

Out-of-hours considerations

No waste handling activities take place outside operational hours, and therefore no dust monitoring is required during these periods. Waste storage arrangements, enclosure, and the distance to sensitive receptors provide effective control of dust risks when the site is not operational. Any complaints received relating to non-operational periods will trigger a review of mitigation measures as appropriate.

# Abnormal Events

Climate projections show that over the coming decades the UK will face an increased risk of climate change impacts, including:

- extreme rainfall, leading to more frequent and severe floods

- heat waves

- drought

- rise in sea levels and tidal surges

- storms

- wildfires

These impacts have been considered within the following risk assessment and how they could have an impact on the business directly, in the supply chains and consumers.

Considerations behind this CCRA will help ADL to:

- Remain in compliance with the environmental permit, other obligations and regulations.

- Minimise our impact on the environment during an extreme weather event and the event’s impact on operations, either through accidental release or abnormal operation.

- Improve our resilience and business continuity by avoiding unplanned start-ups, shutdowns and other business interruptions.

Summer daily maximum temperature

Summer daily maximum temperature may be around 7°C higher compared to average summer temperatures now, with the potential to reach extreme temperatures as high as over 40°C with increasing frequency based on today’s values.

- Potential for increased waste reactions or fires involving heat sensitive or combustible waste.

Mitigation measures include:

- The site deals with inert materials suitable for soil-forming, none of which possess properties that are likely to lead to self-combustion.

- Potential for fire if the temperature exceeds the heat rating of components in electrical equipment or components are subjected to intense and direct sunlight.

Mitigation measures include:

- Plant and equipment used is designed to be operated in quarries and is constructed to enable continued operation in extremely harsh environments.

- Shading electrical equipment subject to direct sunlight for prolonged periods of time.

- Potential increase in high temperature expansion and stress of plant, pipework and fittings. UV degradation of plastic pipes and hoses causing them to fail.

Mitigation measures include:

- Plant and equipment used is designed to be operated in quarries and is constructed to enable continued operation in extremely harsh environments.

- Shading electrical equipment subject to direct sunlight for prolonged periods of time.

- Regular inspection and preventative maintenance of site, plant and equipment as part of the programmed preventative maintenance schedule.

- Potential increased dust emissions from processing areas, stockpiled material and site roads. Reduced availability of water for dust suppression.

Mitigation measures include:

- Equipment is fitted with dust suppression equipment.

- Wheel wash on-site to prevent mud being trafficked off-site and generating dust.

- Site is regularly ‘dry-brushed’ to reduce dust generation from access roads.

- Rainfall capturing, collecting and storing uncontaminated rainwater from roofs and yard areas during high rainfall periods and stored for use in dust suppression systems.

- Long periods of hot and dry weather could lead to a drought and may have an impact on water supplies for:

- emergency water usage

- cooling systems

- fire fighting

- processes that require water as input for example aggregate and soil washing plants

Mitigation measures include:

- Water level usage will be reviewed on a regular basis. This includes assessing those systems and processes that have a critical need for water and what the baseline needs are.

- Water recirculation is employed where possible to reduce wastage.

- Where possible, methods will be used to reduce evaporation.

- Shading electrical equipment subject to direct sunlight for prolonged periods of time.

- Regular inspection and preventative maintenance of site, plant and equipment as part of the programmed preventative maintenance schedule.

- All surface water run-off will be directed into drainage ditches adjacent to the main haul roads and within the sand and gravel extraction areas where sumps/ soakaways will collect the water.

- Potential increased risk of pests and scavengers from stockpiled waste such as food and drink containers, food contaminated wastes and ‘black bag’ type wastes.

Mitigation measures include:

- Waste materials that are attractive to pests and scavengers will be prohibited from entering the site.

- Potential increased risk of wildfires impacting the site.

Mitigation measures include:

- Site is located within an industrial estate and is protected from the risk from wildfires due to the absence of combustible materials around the site.

Winter daily temperatures

Winter daily temperatures could be 4°C more than the current average with the potential for more extreme temperatures, both warmer and colder than present.

- Slightly higher winter maximums could generate regular odour complaints and pest infestations.

Mitigation measures include:

- Waste materials that are attractive to pests and scavengers will be prohibited from entering the site.

- Lower winter temperatures could result in an increased risk of pipes (or similar) freezing.

Mitigation measures include:

- Plant and equipment used is designed to be operated in quarries and is constructed to enable continued operation in extremely harsh environments.

- Regular inspection and preventative maintenance of site, plant and equipment as part of the programmed preventative maintenance schedule.

Daily extreme rainfall

Daily rainfall intensity could increase by up to 20% on today’s values.

- Potential for increased site surface water and flooding.

Mitigation measures include:

- The highest risk of flooding at this location is from surface water.

- Surface water flooding, also known as flash flooding, happens when rainwater cannot drain away through normal drainage systems.

- The yearly chance of flooding at the site is rated as low.

- Gullies and channels will be kept clear of debris.

- Inspection of detention/retention features after storms to ensure they drain as designed

There is potential for drainage systems and interceptors to be overwhelmed.

Mitigation measures include:

- The overall direction of the movement of water will be maintained within the existing drainage systems and the conveyance routes (flow paths) will be checked for blockages or obstructions.

Average winter rainfall

Average winter rainfall may increase by over 40% on today’s averages.

- Potential for increased site surface water and flooding.

Mitigation measures include:

- The overall direction of the movement of water will be maintained within the existing drainage systems and the conveyance routes (flow paths) will be checked for blockages or obstructions.

- There is potential for drainage systems and interceptors to be overwhelmed.

Mitigation measures include:

- Due to the nature of the operation, all waste storage/treatment activities must be carried out on an impermeable surface to protect the aquifer, as such mitigations are restricted to flow restrictions.

- The overall direction of the movement of water will be maintained within the existing drainage channels and the conveyance routes (flow paths) will be checked for blockages or obstructions.

Drier summers

Summers could potentially see up to 40% less rain than now.

- Long periods of hot and dry weather could lead to a drought and may have an impact on water supplies for:

- emergency water usage

- cooling systems

- fire fighting

- dust suppression

- processes that require water as input for example aggregate and soil washing plants

Mitigation measures include:

- Water level usage will be reviewed on a regular basis. This includes assessing those systems and processes that have a critical need for water and what the baseline needs are.

- Water recirculation is employed where possible to reduce wastage.

- Where possible, methods will be used to reduce evaporation.

- Shading electrical equipment subject to direct sunlight for prolonged periods of time.

- Regular inspection and preventative maintenance of site, plant and equipment as part of the programmed preventative maintenance schedule.

- There is potential increased impact of discharge to watercourse from on-site drainage systems where connected to water courses.

Mitigation measures include:

- The overall direction of the movement of water will be maintained within the existing drainage channels and the conveyance routes (flow paths) will be checked for blockages or obstructions.

River flow

The flow within the watercourses could be 50% greater than it is currently when at its peak, and 80%

less than now when at its lowest.

Mitigation measures include:

- Site is located 75m from the New River.

- The site drainage system does not rely on connection to surface water, however it is acknowledged that the discharge adds to the load at the waste water treatment works.

- The overall direction of the movement of water will be maintained within the existing drainage channels and the conveyance routes (flow paths) will be checked for blockages or obstructions.

Storms

Storms could see a change in frequency and intensity. The unique combination of increased wind

speeds, increased rainfall, and lightning during these events provides the potential for more extreme

storm impacts.

- Potential for high winds to damage buildings and infrastructure and blow waste from the site.

Mitigation measures include:

- Site is located within an industrial estate so wind speeds are likely to be reduced in severity due to the nature of the surroundings.

- Waste types on site are likely to contain litter are netted when high winds are forecast..

- Housekeeping and cleaning measures in place to ensure particulates on external surfaces are minimised.

- Control systems do not rely on electricity to control emissions, during extreme stormy weather operations at the can be paused.

- Stockpiles are orientated to reduce exposure to the prevailing wind.

- Potential for high winds to cause problems with stability of above ground storage tanks on jacks. This poses a risk to staff, plant infrastructure and the potential to release the contents of the storage tank.

Mitigation measures include:

- Fuel station is ground based.

- Potential for lightning strikes to damage buildings and infrastructure.

Mitigation measures include:

- Assessing the potential for and impact of lightning strikes on buildings, equipment and plant.

- Assessing the need to install lightning conductors.

The facility will have drainage infrastructure in place at the site so that potentially all site drainage is captured and directed via a sealed system, consisting of impermeable concrete pavement which falls towards the drain channels that captures all liquids and directs it to a sealed tank. It is sized to ensure it can cope with the extra demands of climate change with regards to excessive rainfall.

Operational procedures at the site will monitor and manage amenity and accident risks from the proposed activities and includes provision for the monitoring of odour, noise, and fugitive emissions.

As the management measures detailed in the abnormal events will be in place from commencement of operations, the conclusion has been reached that the proposed waste materials and treatment activities, are unlikely to be significantly affected.

The following section details the assessment process to be taken when determining if and which activities on Site should stop to prevent excessive dust emissions.

Estimating magnitude of risk

Table 6.1 provides a matrix for estimating the magnitude of risk from a potential hazard, considering both the probability and consequences of the hazard occurring.

The magnitude of risk determines the level of management required to reduce the probability of the hazard occurring.

In this DEMP, the hazard is the excessive emission of dust from the Site.

Table 5-1: Estimating the magnitude of risk

| Probability | Magnitude of risk | Consequence |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | High | Medium | Low | Negligible |
|  | High | Very high | High | Medium/low | Very low |
|  | Medium | High | Medium | Low | Very low |
|  | Low | High/medium | Medium/low | Low | Very low |
|  | Negligible | Low/medium/high | Medium/low | Low | Negligible |

Ceasing of operations

Table 5-2: Risk matrix for the ceasing of operations

| Conditions | Probability | Consequence | Risk magnitude |
| --- | --- | --- | --- |
| Risk from waste treatment activities |  |  |  |
| Dry, cool weather (<20deg) | M | L | L |
| Dry, hot weather (>20deg) | M | M | M |
| Dry, hot (>20deg), windy (>4 Beaufort) | M | M | M/H |
| Dry, windy weather (>4 Beaufort) | M | M | L/M |
| Wet, windy weather (>4 Beaufort) | H | L | L |
| Dry, little wind (<3 Beaufort) | L | L | L |
| Gale-force winds (>8 Beaufort) | M | H | M/H |
| Plant failure or breakdown (spray bars or other essential water suppression or water supply) | M | H | M/H |
| Preventative and remedial mitigation measures not effective | L | H | H |
| Risk from waste storage |  |  |  |
| Dry, cool weather (<20deg) | M | L | L |
| Dry, hot weather (>20deg) | M | M | M |
| Dry, hot (>20deg), windy (>4 Beaufort) | M | M | M/H |
| Dry, windy weather (>4 Beaufort) | M | M | M/L |
| Wet, windy weather (>4 Beaufort) | H | N | L |
| Dry, little wind (<3 Beaufort) | L | M | L |
| Gale-force winds (>8 Beaufort) | M | M | M/L |
| Plant failure or breakdown (spray bars or other essential water suppression or water supply) | M | H | M/L |
| Preventative and remedial mitigation measures not effective | L | H | H |
| Risk from other site activities |  |  |  |
| Mud tracked onto highway in dry conditions | L | M | M |
| Mud tracked onto highway in wet conditions | M | L | L |
| Debris on site surface in dry conditions and still conditions (<4 Beaufort) | M | L | M/L |
| Debris on site surface in dry conditions and windy conditions (>4 Beaufort) | M | M | M |
| Debris on site surface in wet conditions | M | L | L |

Table 5-3: Action required per risk magnitude rating

| Risk magnitude | Action |
| --- | --- |
| Low | Continued implementation of preventative measures |
| Medium | Dust emissions are likely. Relevant activities* stop until additional remedial mitigation is implemented. Relevant waste activity may temporarily cease - can resume upon implementation of additional mitigation if measure is effective. Relevant waste activity must stop if excessive dust emissions are observed. Relevant waste activity can resume when the conditions no longer apply/ additional remedial mitigation is implemented and there are no significant dust emissions |
| High | Relevant waste activity will stop. In the case of waste storage this will mean either removal or covering of the offending waste within 1 day if remedial mitigation is not effective. Relevant waste activity can resume when the conditions no longer apply/ additional remedial mitigation is effectively implemented and there are no excessive dust emissions. |

Actions when alarm is triggered

When the ‘action level’ is triggered, the following actions are taken:

- The Site Manager (SM) assesses the activities underway and/or the nature of the waste materials being delivered immediately prior to the complaint being received, to work out what has caused the complaint.

- If the source cannot be established confidently, the likely dust/particulate generating activities will be suspended, i.e., shredding, loading, screening.

- Where the source originates from the site the SM will take appropriate action in terms of dust/particulate abatement, to ensure that the complaint is followed through. This may take the form of the following;

- Investigating the source of the dust/particulates to prevent a re-occurrence.

- Suspending operations which are not being conducted using best-practice controls.

- Additional use of the dust abatement measures.

- Logging findings of a – c in the site diary, and in the reporting template within the relevant appendix of the Environmental Permit.

In all cases, any new lessons learnt from the investigations are considered by company management and implemented into the dust & particulate emission management plan (if not already included), to prevent a re-occurrence of the complaint.

The complaint is not the sole indicator of a dust event at the site; the continuous visual monitoring of potential dust sources and activities safeguard play a vital part in managing dust and particulates.

# Reporting and Complaints Response

Community engagement

The site is committed to maintaining positive relationships with neighbouring businesses and local residents. Being a good neighbour is an integral part of the site’s Environmental Management System (EMS) and Appropriate Measures approach.

Key actions include:

- Providing a single, clearly advertised point of contact for environmental concerns, including dust complaints (contact details displayed at the site entrance and on the company website).

- Keeping complainants informed of the steps being taken to investigate and address issues.

- Sharing information on dust mitigation measures when requested, to demonstrate proactive management.

- Liaising with local businesses and residents when operational changes may increase dust potential, such as handling unusually dry waste materials.

- Engagement is primarily through direct communication via phone, email, or site visits where appropriate, ensuring transparency and responsiveness in line with EA guidance.

Complaints management and reporting

All dust complaints received by the site are treated as a priority and are managed in accordance with EA Appropriate Measures, as well as the conditions of the Environmental Permit.

Process for handling complaints:

- Receipt of complaint: Complaints may be received via telephone, email, or in writing. Details of the complaint, including the time, date, and nature of the issue, are recorded immediately in the Site Complaints Log.

- Initial assessment: The Site Manager, or their deputy in their absence, reviews the complaint within 2 working days, in line with standard permit conditions.

- Investigation: The responsible site staff carry out an investigation, including:

- Visual inspection of operations at the relevant time;

- Assessment of meteorological conditions (wind direction, speed, and dryness of surfaces);

- Review of operational controls in place at the time; and

- Confirmation that dust mitigation measures were implemented correctly.

- Corrective action: If dust is identified leaving the site, immediate corrective action is taken, which may include pausing operations, damping down materials, or reviewing handling practices. The outcome is documented in the Site Complaints Log.

- Response to complainant: The complainant is informed of the findings and any measures taken to prevent recurrence. This may be via phone, email, or written correspondence.

- Review: All complaints and remedial actions are reviewed by senior management during routine environmental inspections to identify trends and implement continuous improvement measures.

Record keeping:

The Site Complaints Log is retained on-site and includes:

- Complainant details;

- Date and time of complaint;

- Details of operations at the time;

- Actions taken to investigate and remediate;

- Outcome and communication with the complainant; and

- Staff responsible for actions taken.

Records are retained in accordance with the EMS and are available for review by the Environment Agency.

Dust pollution complaint response

In the event that the operator receives a dust complaint:

- The Site Manager is immediately notified and assumes responsibility for investigating the matter.

- The investigation is undertaken within 48 hours, in line with permit expectations and EA guidance.

- The cause of the emissions is assessed, mitigation measures are reviewed, and corrective actions are implemented without delay.

- The results of the investigation, including any adjustments to operational practices or dust suppression measures, are documented and reported back to the EA within the required timeframe.

All dust complaint investigations and outcomes are cross-referenced with ongoing complaints monitoring to ensure consistency, trend identification, and continuous improvement.

Alignment with Appropriate Measures

This complaints management framework is fully aligned with EA Appropriate Measures for Waste Treatment and Transfer, which emphasises:

- Prevention over monitoring: Mitigation measures (enclosure, hard surfacing, operational control) minimise dust emissions before they leave the site;

- Task-based inspection and response: Visual monitoring and rapid response to complaints are considered more effective than routine quantitative PM monitoring for low-risk operations;

- Proportionate record-keeping: All complaints, visual inspections, and remedial actions are documented to demonstrate proactive control and continuous improvement;

- Community engagement: Keeping local businesses and residents informed supports transparency and maintains good neighbour relations.

This approach ensures that dust complaints are handled efficiently, effectively, and in a manner proportionate to the low inherent dust risk at this non-hazardous waste transfer station.

Table 6-1: Dust complaints response flow

| Step | Action | Responsible Person | Timing / Notes |
| --- | --- | --- | --- |
| 1 | Receipt of complaint – via phone, email, or in writing | Site Manager / Site Office Staff | Recorded immediately in Site Complaints Log |
| 2 | Initial review & prioritisation – assess potential dust source, wind, and operational context | Site Manager (or deputy) | Within 2 working days of receipt |
| 3 | Investigation – visual inspection of site operations, weather conditions, dust mitigation measures | Operational staff supervised by Site Manager | Immediately following initial review |
| 4 | Corrective action – dampen materials, pause operations, review handling practices, or adjust mitigation | Operational staff / Site Manager | Immediate, as required |
| 5 | Record findings & actions – document investigation, actions taken, and responsible staff | Site Manager / Site Office | Enter into Site Complaints Log; retain with EMS records |
| 6 | Complainant feedback – inform complainant of findings and measures taken | Site Manager | Within 2–5 working days (or sooner if urgent) |
| 7 | Management review & trend analysis – identify repeated sources, adjust operations, review DEMP if necessary | Senior Management | Periodically (at least quarterly) or after any recurring complaints |
| 8 | Dust complaint response (if issued by EA) – full investigation, corrective measures, report back to EA | Site Manager | Investigation completed within 48 hours; report submitted within permit timescale |

# Dust Complaint Form

| Complaint Details |  |  |  |
| --- | --- | --- | --- |
| Complainant name |  |  |  |
| Address Postcode |  |  |  |
| Contact details |  |  |  |
| Tel |  |  |  |
| Email |  |  |  |
| Date |  |  |  |
| Complaint ref number |  |  |  |
| Complaint details |  |  |  |
| Investigation Details |  |  |  |
| Investigation carried out by |  |  |  |
| Position |  |  |  |
| Date & time investigation carried out |  |  |  |
| Weather conditions |  |  |  |
| Wind direction and speed |  |  |  |
| Investigation findings |  |  |  |
| Feedback given to EA and/or LA |  |  |  |
| Date feedback given |  |  |  |
| Feedback given to public |  |  |  |
| Date feedback given |  |  |  |
| Review and Improve |  |  |  |
| Improvements needed to prevent a reoccurrence |  |  |  |
| Proposed date for completion of the improvements |  |  |  |
| Actual date for completion |  |  |  |
| If different insert reason for delay |  |  |  |
| Does the DMP need to be updated |  |  |  |
| Date that the DMP updated |  |  |  |
| Closure |  |  |  |
| Site manager review date |  |  |  |
| Site manager signature to confirm no further action required |  |  |  |