WRM

wrm-ltd.co.uk

01943 468138

Supporting Information

v2.0

Environmental and sustainability solutions provided to

Stowmarket Skips Ltd

WRM-LTD.CO.UK

This report was prepared by Walker Resource Management Ltd (WRM’) within the terms of its engagement and in direct response to a scope of services. This report is strictly limited to the purpose and the facts and matters stated in it and does not apply directly or indirectly and must not be used for any other application, purpose, use or matter. In preparing the report, WRM may have relied upon information provided to it at the time by other parties. WRM accepts no responsibility as to the accuracy or completeness of information provided by those parties at the time of preparing the report. The report does not take into account any changes in information that may have occurred since the publication of the report. If the information relied upon is subsequently determined to be false, inaccurate, or incomplete then it is possible that the observations and conclusions expressed in the report may have changed. WRM does not warrant the contents of this report and shall not assume any responsibility or liability for loss whatsoever to any third party caused by, related to, or arising out of any use or reliance on the report howsoever. No part of this report, its attachments or appendices may be reproduced by any process without the written consent of WRM. All enquiries should be directed to WRM.

| Document Title | Supporting Information |  |
| --- | --- | --- |
| Client | Stowmarket Skips Ltd |  |
| Revision | v2.0 |  |
| Date | 18/12/2025 |  |
| Document Reference | EPR-A01 |  |
| Project Reference | 1496/J05 |  |
| Author: Bryanna Heaton |  | Reviewer: Graeme Kennett |

Copyright ©

All material on these pages, including without limitation text, logos, icons and photographs, is copyright material of Walker Resource Management Ltd (WRM). Use of this material may only be made with the express, prior, written permission of WRM. This document was produced solely for use by the named client to whom the document refers. The methodology (if any) contained in this report is provided to you in confidence and must not be disclosed or copied to third parties without the prior written agreement of WRM. Disclosure of that information may constitute an actionable breach of confidence or may otherwise prejudice our commercial interests.

| Revision | Details | Date |
| --- | --- | --- |
| 0.1 | Initial draft | 24/06/2025 |
| 1.0 | Final version for submission | 27/06/2025 |
| 2.0 | Amended version for submission | 08/01/2026 |

Revision Log

CONTENTS

Contents

1.0	SITE DETAILS	1

1.1	Location	1

1.2	Site Grid Reference	1

1.3	Permit reference	1

2.0	Proposed changes	2

2.1	Regulatory overview	2

2.2	Site permit position	2

2.3	Permit variation	2

2.4	Waste streams	3

2.5	Site boundary	4

2.6	Permitted activities	4

2.7	Site throughput	4

3.0	Non-technical summary	5

3.1	Environmental receptors	5

3.2	Site redevelopment	6

4.0	Site procedures	7

4.1	Operating techniques	7

4.2	Delivery and unloading	8

4.3	Sorting and storage inside the recycling building	8

5.0	Impact on the Environment	9

5.1	Odour	9

5.2	Dust	9

5.3	Drainage system	10

6.0	Site Management	11

7.0	Site Condition Report	12

8.0	Business Information	13

8.1	Company Name	13

8.2	Company number	13

8.3	Companies House Link	13

8.4	Date accessed	13

9.0	Additional Documents	14

List of tables

Table 2-1: Accepted waste streams	3

Table 2-2: Non-accepted waste streams	4

Table 8-1: Director details	13

# SITE DETAILS

The site, previously operated by Advanced Demolition Limited (ADL), is a non-hazardous waste treatment and transfer facility located in an industrial area of Hoddesdon and was permitted to carry out waste activities under a Standard Rules SR2015_No8 permit. The permit is to be transferred to Stowmarket Skips Ltd.

The Site comprises a parcel of land located approximately 800m east of Hoddesdon town centre and is accessed off Pindar Road at the south-eastern corner of the Site. The surrounding area is generally comprised of residential, commercial and various industrial developments and properties, including warehouse units and vehicle services. The Site is bound to the north by a factory namely IMPEX Foods Ltd, to the east by Pindar Road, to the south by New England Estate and to the west by New England Estate and industrial properties.

## Location

The Geddings

Pindar Road

Hoddesdon

Hertfordshire

EN11 0BZ

## Site Grid Reference

TL3811309063

## Permit reference

EPR/FB3801SA

# Proposed changes

## Regulatory overview

Waste Permitting in England has recently undergone significant changes, in that the Environment Agency (EA) has introduced a series of new Standard Rules (SR) permits designed to enhance environmental protections and streamline regulatory processes. This change has resulted in the EA issuing 8 new permits after the consolidation of 28 SR permits that will be withdrawn after the transitional period has ended on 27th June 2025.

Although these changes present new opportunities for operators, those whose operations do not fit within the new Standard Rules permits are now required to submit a bespoke permit variation by 27th June 2025 to continue to operate.

The EA has issued Extension to comply with new standard rules permits: RPS 331 - GOV.UK due to delays in notification of the changes and that they will not enforce compliance with the revised standard rules permits until 27th June 2025.

## Site permit position

The site is currently permitted to operate under a Standard Rules SR2015_No8 permit. However, after submitting a pre-app request to the Environment Agency (EA) the screening results indicate that the site is not eligible to apply for a SR2022 No 5: non-hazardous waste transfer with asbestos, hazardous batteries, cable and WEEE storage - GOV.UK permit.

The reason for illegibility is that the site lies within a Source Protection Zone (SPZ) 1.

## Permit variation

With the exception of the specified wastes in table 1-1, the site will, in practical terms, operate in accordance with the conditions stated in the SR2022 No 5: non-hazardous waste transfer with asbestos, hazardous batteries, cable and WEEE storage - GOV.UK permit.

This includes the stated sections of Non-hazardous and inert waste: appropriate measures for permitted facilities - Guidance - GOV.UK.

## Waste streams

Table 2-1: Accepted waste streams

| Lower-risk non-hazardous wastes (outdoor storage permitted) |  |  |  |
| --- | --- | --- | --- |
| 17 | 17 01 01 | Concrete | Outdoor permitted |
| 17 | 17 01 02 | Bricks | Outdoor permitted |
| 17 | 17 01 03 | Tiles and ceramics | Outdoor permitted |
| 17 | 17 01 07 | Mixtures of concrete, bricks, tiles and ceramics | Outdoor permitted |
| 17 | 17 02 01 | Wood | Outdoor permitted |
| 17 | 17 02 02 | Glass | Outdoor permitted |
| 17 | 17 03 02 | Bituminous mixtures (non-coal tar) | Outdoor permitted |
| 17 | 17 05 04 | Soil and stones (non-hazardous) | Outdoor permitted |
| 17 | 17 05 08 | Track ballast (non-hazardous) | Outdoor permitted |
| 19 | 19 12 05 | Glass | Outdoor permitted |
| 19 | 19 12 09 | Minerals (sand, stones) | Outdoor permitted |
| 20 | 20 02 02 | Soil and stones | Outdoor permitted |
| Wastes requiring storage within a building |  |  |  |
| 15 | 15 01 01 | Paper and cardboard packaging | Inside building only |
| 15 | 15 01 02 | Plastic packaging | Inside building only |
| 15 | 15 01 03 | Wooden packaging | Inside building only |
| 15 | 15 01 04 | Metallic packaging | Inside building only |
| 15 | 15 01 06 | Mixed packaging | Inside building only |
| 16 | 16 01 03 | End-of-life tyres | Inside building only |
| 16 | 16 02 14 | Discarded equipment (non-hazardous) | Inside building only |
| 16 | 16 02 16 | Components removed from discarded equipment | Inside building only |
| 17 | 17 04 01 | Copper, bronze, brass | Inside building only |
| 17 | 17 04 02 | Aluminium | Inside building only |
| 17 | 17 04 03 | Lead | Inside building only |
| 17 | 17 04 04 | Zinc | Inside building only |
| 17 | 17 04 05 | Iron and steel | Inside building only |
| 17 | 17 04 07 | Mixed metals | Inside building only |
| 19 | 19 12 01 | Paper and cardboard | Inside building only |
| 19 | 19 12 02 | Ferrous metal | Inside building only |
| 19 | 19 12 03 | Non-ferrous metal | Inside building only |
| 19 | 19 12 04 | Plastic and rubber | Inside building only |
| 19 | 19 12 07 | Wood | Inside building only |
| 20 | 20 01 01 | Paper and cardboard | Inside building only |
| 20 | 20 01 02 | Glass | Inside building only |
| 20 | 20 01 10 | Clothes | Inside building only |
| 20 | 20 01 11 | Textiles | Inside building only |
| 20 | 20 03 01 | Mixed municipal waste | Inside building only |

## Site boundary

No change.

## Permitted activities

Permitted activities on site are limited to:

- Transfer

- Treatment (further limited to sorting, compaction and baling)

- Storage

## Site throughput

No change

# Non-technical summary

## Environmental receptors

The site is not located within;

- 500 metres of a European site, Ramsar, SSSI or Marine Conservation Zone (MCZ);

- 50 metres of a National Nature Reserve, Local Nature Reserve, Local Wildlife Site, Ancient woodland or Scheduled Ancient Monument;

- 50 metres of a site that has species or habitats protected under the Biodiversity Action Plan that the Environment Agency considers at risk to this activity;

- 250 metres within the presence of Great Crested Newts where it is linked to the breeding ponds of the newts by good habitat;

- 10 metres of a watercourse.

It is however, located within a groundwater Source Protection Zone (SPZ) 1.

Figure 3-1: Site location in relation to SPZ1 (in red)

## Site redevelopment

The site is to be upgraded to better protect the environment and operator efficiency and civil works will involve the following;

- Improved site drainage, including interceptor and penstock valve;

- Improved site layout

- Waste sortation building

- Concrete bay storage

# Site procedures

## Operating techniques

The site will be operated in accordance with the requirements of Non-hazardous and inert waste: appropriate measures for permitted facilities - Guidance - GOV.UK , including, but not limited to, the following:

- Site management.

- Site operations.

- Emissions and monitoring; and

- Records and information.

Operational management procedures ensure that:

- The risks that the activities pose to the environment are identified.

- The measures that are required to minimise the risks are identified.

- The activities are managed in accordance with the management system and operating techniques document.

- Performance against the management system is audited at regular intervals; and

- Compliance with the environmental permit.

The risk management and mitigation measures employed at the site and identified in the environmental risk assessment are detailed in the sites operating techniques document.

In summary, the rules and operating procedures employed at the site will ensure the following with respect to the specified waste management activities:

- Unless stated otherwise, all waste will be stored and sorted on an impermeable surface with sealed drainage system.

- Concrete surfacing falls towards drainage channels ensuring that potentially contaminated runoff is contained on site and directed to the sewer via a Trade Effluent Consent.

- Strict waste acceptance procedures are adhered to ensure only permitted wastes are accepted on site.

- The Site Manager (SM) will continue to ensure that regular inspections are made of the site. If necessary, remedial measures will be arranged as soon as possible.

## Delivery and unloading

- All waste will be delivered to the site through the existing site entrance.

- Once on the site and after being weighed, vehicles will be unloaded under cover of the recycling building. The waste materials will then be sorted and stored in the storage bays pending removal.

- Soil and rubble skips will be unloaded directly into the designated storage bays on the southern level.

## Sorting and storage inside the recycling building

- Once deposited inside the recycling building the waste will be sorted with the aid of mechanical mobile plant, including a wheeled loading shovel, and/or manually, and 360o excavator with grab.

- Excluding soils and rubble, sorted materials will be stored inside the designated storage bays pending removal from site for reuse, recycling, composting or disposal.

- The storage bays will be constructed using concrete panels and will allow for:

- green waste,

- wood,

- metal,

- plasterboard and

- residual waste that cannot be recycled.

# Impact on the Environment

A site-specific risk assessment (SSRA) has been carried out to assess the environmental risks posed by the activity.

There are no point source emissions to land, air, surface or groundwater from the upgraded facility. The existing discharge to sewer will remain.

Operational procedures at the site will monitor and manage amenity and accident risks from the proposed activities and includes provision for the monitoring of odour, noise, and fugitive emissions.

As the management measures detailed in the risk assessment will be in place at permit issue, the conclusion has been reached that the proposed waste materials and treatment activities, are unlikely to result in a significant accident risk or risk to the local environment, including from odour and noise, or pollution of surface or ground waters.

## Odour

An odour assessment was undertaken to evaluate potential impacts and specify the requirement for any mitigation.

The risk of potential effects because of odour emissions from the facility during the operational phase was assessed and included consideration of the source emission potential, pathway effectiveness and sensitivity of relevant receptors in the vicinity of the site.

The results of the assessment indicated the overall effects because of the development were predicted to be not significant.

## Dust

A Dust Assessment was undertaken to evaluate potential impacts and specify the requirement for any mitigation.

The risk of potential effects because of fugitive dust emissions from the facility during the operational phase was assessed and included consideration of the source emission potential, pathway effectiveness and sensitivity of relevant receptors in the vicinity of the site. The results of the assessment indicated the overall effects because of the development were predicted to be not significant.

The upgraded measures to reduce dust are;

- All operations are currently carried out in the open air. The building will enable waste sorting to take place under cover. The building will reduce the incidence of fugitive dust emissions, as it will have a roof and will be enclosed on three sides.

- Storage of recycled inert materials in replacement 4m high external storage bays. The material within the bays will be stored below the height of the bay walls to minimise wind whip.

## Drainage system

Drainage infrastructure is in place so that all potentially contaminated run-off is captured.

The surface water runoff from the impermeable area will discharge into geocellular crates in the centre of the site. From here the water will outfall into the existing surface water system to the east of the site via a three-stage interceptor. An above ground rainwater harvesting tank will be located near the recycling building to provide additional water reserves.

# Site Management

Site management will comprise of the following staff members:

- A Technically Competent Manager (TCM); who will manage the operation and regularly attend site in compliance with the defined attendance requirement.

- A site supervisor; who will be responsible for the ongoing operation who may also undertake office and plant operation duties; and

- Other trained plant operators as required.

# Site Condition Report

The Site Condition Report (SCR) has been updated for this application.

The facility will operate with due regard to the conditions of the environmental permit and all relevant environmental legislation to ensure that land and groundwater is protected during the lifetime of the site and that the land is in a satisfactory state when the permit is eventually surrendered.

The possibility of any significant releases to the ground occurring during the lifetime of the permit is limited. Minor spillages, if they occur, are dealt with immediately by trained staff using appropriate spill response procedure and spill kits located around the site.

# Business Information

## Company Name

Stowmarket Skips Limited

## Company number

12531436

## Companies House Link

STOWMARKET SKIPS LTD overview - Find and update company information - GOV.UK

## Date accessed

24th December 2025

Table 8-1: Director details

| Name | Date of Birth |
| --- | --- |
| Noah Aaron BULKIN | _ _ May 1977 |
| Michael Damian DARRAGH | _ _ February 1971 |
| Belinda SPACIE |  |
| Mathew Alexander STEWART | 24 July 1969 |
| Quentin Richard STEWART | 2 February 1968 |

# Additional Documents

| Reference Code | Document Title |
| --- | --- |
| EPR-A01 | Supporting Information |
| EPR-A02 | Appropriate Measures Assessment |
| EPR-B01 | EMS Manual |
| EPR-B02 | Environmental Risk Assessment |
| EPR-B03 | Fire Prevention Plan |
| EPR-B04 | Site Condition Report |
| EPR-OP01 | Waste Acceptance Procedure |