Site Condition Report

| 1.0 SITE DETAILS |  |
| --- | --- |
| Applicants Name | Higher Meadow Poultry Ltd |
| Activity Address | Higher Hall Farm Holewater Hill High Bray Devon EX32 7QN |
| National Grid Reference | 269574, 134485 |
| Document Reference for Site Condition Report at permit application and surrender | 26th July 2026 |
|  | Location Plan |
|  | Layout/Drainage Plan |

| 2.0 Condition of the Land at Permit issue |  |
| --- | --- |
| Environmental setting including: Geology Hydrogeology Surface Waters | The poultry unit lies on level ground to the northeast of High Bray. Predominant land use is arable farming. Field pattern is semi large scale with a strong pattern of field boundaries. There are sensitive receptors within 400m of site. Topography and Drainage The poultry houses are between 210 and 220 metres above sea level. Soil bunding and tree planting will help to minimise the visual intrusion normally associated with poultry units. Site clean water drainage all goes to French drains & swales to an infiltration basin. Geology and Hydrogeology According to the BGS Extract British Geological Survey map shows that the Bedrock geology comprises Baggy Sandstone Formation – Sandstone, Siltstone & Mudstone - of the Devonian age. Searches indicate the site is underlain with a Bedrock Aquifer – Secondary A. Superficial drift aquifer – None The Site is situated in a Groundwater Vulnerability Zone – High. The site is situated within a Surface Nitrate Vulnerable Zone. The site is not situated in a Groundwater/Source water protection zone. The site is situated in a drinking water surface protection zone. Hydrology General drainage is in a southerly direction. Average rainfall for this area is 1100 mm. |
| Pollution history including: Pollution incidents that may have affected land Historical land uses and associated contaminants Any visual/olfactory evidence of existing contamination Evidence of damage to pollution prevention measures | None noted Polluting substances – None noted Previous use prior to 2026 – Free range broiler production. None noted None noted |
| Evidence of historical contamination | N/A |
| 3.0 Permitted Activities | Four poultry houses for Broiler rearing. This will give a total of 176,000 places. Heating will be provided by LPG heaters. The working area where vehicles operate is laid to concrete. Feed is delivered in covered lorries and stored on site in vermin proof steel galvanised bins. Immediately following depopulation, litter is removed off site and sold to an AD plant. The houses are then washed and disinfected prior to the cycle beginning again. Underground storage tanks have been installed to catch all wash waters. Wash water will be collected by a licensed disposal company. Dead birds are removed from the houses and stored in sealed containers in a temperature controlled store, awaiting collection. Diesel fuel storage is in a bunded integral tank in the standby generator. |
| Non permitted activities undertaken | N/A |
| Document References | Location Plan Layout/Drainage Plan H1 Assessment |
| 4.0 Changes to Activity | N/A |
| 5.0 Measures taken to protect Land | Site will be operated in compliance with “how to comply” routine maintenance schedules are followed and recorded and with any abnormal operations recorded. |
| 6.0 Pollution Incidents | N/A |
| 7.0 Soil, gas and water quality monitoring | No monitoring will be undertaken at the installation. |