Suite 2717

37 Westminster Buildings

Theatre Square

Nottingham

NG1 6LG

Tel No: 01773 603331

E-mail: nvc.acoustics@btinternet.com

Noise Impact Assessment

For Environmental Permit

For the Proposed Hillhouse IBA Facility

At

Hillhouse Business Park, Thornton-Cleveleys, Lancashire

For

Fortis IBA Limited

Consultant: D.R. Kettlewell MSc MIOA MAE I.Eng

# Report No.: R24.0703/DRK

# Noise & Vibration Consultants Ltd

Member of Institute of Acoustics

Member of Association of Noise Consultants

Member of Academy of Experts

Report prepared by: D R Kettlewell MSc MIOA MAE I.Eng – Principal Consultant: Date: 16th July 2024

CONTENTS

Section					  		Page Number

1		Introduction						1

2		Location, Receptors & Baseline Levels		3

3		Review of Noise Guidance & Standards 		7

4		Noise Predictions					11

5		Conclusions						16

References						18

Figure 1:         Site location, Baseline Monitoring and Noise Sensitive Receptors

Figure 2:	Plan of Site Showing Layout

Figure 3:  	Side Elevation Drawings of Proposed IBA Processing Building

Figure 4:  	IBA Storage Building (including Processed IBA)

Figure 5:	Metal Storage Building

APPENDICES

Appendix 1  	Basic Acoustic Terminology

Appendix 2 	Baseline Sound Survey Details

Appendix 3	Baseline Sound Survey Results

Appendix 4	Site Operational Noise Levels & Mitigation Measures

Appendix 5	Noise Model Settings & Mapping Results

Appendix 6 	Consultant’s Experience & Qualifications

1.0	Introduction

1.1	Noise & Vibration Consultants Ltd (“NVC”) have been instructed by Axis acting on behalf of Fortis IBA Limited (the `Applicant’) to review the site layout, construction and plant locations and advise on predicted noise levels with the assumed noise mitigation measures for the proposed Incinerated Bottom Ash (“IBA”) Facility at Hillhouse Business Park, Thornton-Cleveleys, Lancashire.

1.2	We have been asked to carry out a noise impact assessment using empirical data taken from measurements made at other Fortis IBA plant operating in the UK, to determine the resultant noise contribution at identified noise sensitive receptors (“NSR”).

1.3	The main areas of plant and buildings relevant to the assessment include the following (as shown on Figure 2 of the attached):

Incinerated Bottom Ash (IBA) Facility:

The key elements of the Installation are as follows:-

i)     Processing plant, housed within a Processing Building comprising:

a.    Feed hopper;

b.    Modular plant including conveyors, magnets, eddy current separators and screens;

ii)    Processing Building (a clad, steel portal-framed building measuring circa 17m in height and with a footprint of 2,200 m2);

iii)   Metal Storage Bay Building, a 3-sided, steel framed cladded building;

iv)   IBA and IBAA storage areas within a Storage Building circa 15m in height. This building would be a 3-sided, steel framed cladded building, with the north-eastern façade open to provide sufficient access for site traffic and mobile plant. The building measures 230 metres in length by 92 metres wide (at the widest section), occupying a footprint of 1.68 hectares;

v)    Demolition of the existing office building within the south-east of the Site;

vi)   Modular office, laboratory and welfare units;

vii)  3 x standard 28ft containers, measuring 6 metres long by 3 metres wide for storage of site equipment;

viii) Elevated conveyors to transport material between the Storage Building and the Processing Building;

ix)   Retention of the pedestrian walkway spanning Royles Brook, connecting the two parts of the Site;

x)    Retaining wall (3 m high at the Storage Building and Metal Storage Bay Building area; 2.1 m high at the Processing Building area);

xi)   A purpose built, impermeable, sealed surface, to be provided in accordance with the requirements of the Environmental Permit being sought alongside the planning application;

xii)  Boundary treatments, a gated site access, weighbridges and wheelwash; and

xiii) Parking for staff and visitors.

1.4	The IBA would operate 24 hours a day and 7 days a week (except during the night-time period on a Saturday into a Sunday morning). Deliveries to the facility would occur during daytime periods with waste brought to the facility between the hours of 06.00 and 19.00 Monday to Saturday excluding Sundays and Bank Holidays.

1.5	Information used in this review has been obtained from the following sources:

Ordnance Survey maps of the local area;

Layout and elevation drawings of the Installation (Fortis IBA and TRS);

National Planning Policy Framework – December 2023;

Planning Practice Guidance – June 2021;

Noise Policy Statement for England (NPSE) – March 2010;

Noise and vibration management: environmental permits (January 2022);

BS4142: 2014+A1:2019 ‘Methods for rating and assessing industrial and commercial sound’

BS8233: 2014 ‘Guidance on sound insulation and noise reduction for buildings’

World Health Organisation: ‘Guidelines for Community Noise’ - April 1999;

World Health Organisation ‘Night Noise Guidelines for Europe’ – 2009;

Department of Transport ‘Calculation of Road Traffic Noise': 1988;

- ISO 9613-2: 1996 Acoustics – Attenuation of Sound During Propagation Outdoors;

EA – Guidance: Noise and Vibration Management: Environmental Permits (January 2022); and

- Chapter 7 ‘Noise & Vibration’ of the Environmental Statement (ES) and associated Appendices.

2.0	Location, receptors & baseline levels

2.1	Chapter 7 ‘Noise and Vibration’ of the ES and the associated Appendices for review include the following details:

- Location of the nearest noise sensitive receptors (NSR);

- Baseline daytime and night-time noise levels at NSR; and

- Predicted noise levels from proposed plant at the Site.

Location

2.2	The Site is located within the Hillhouse Enterprise Zone, Thornton-Cleveleys, located to the south-east of the town of Fleetwood and on the western banks of the River Wyre Estuary. The location of the Site is shown on Drawings 3566-01-01 and 3566-01-02.

2.3	The Site comprises an area of previously developed ’brownfield’ land. There are two main parts/sections to the Site with the first being a larger area of land set between the River Wyre (to the northeast) and Royles Brook (to the south west), and the second being a smaller parcel set to the south west of Royles Brook, with these pieces of land linked by a bridge over Royles Brook.

2.4	The Site is bound:

i)         to the north-east by a public right of way (‘PRoW’), with the River Wyre beyond;

ii)        to the south and south-east by an area of woodland, beyond which is Flint’s Caravan Park and the Stanah Substation; and

iii)        to the west and north-west by the remainder of Hillhouse International Enterprise Zone, comprising various employment and industrial uses, and vacant pieces of brownfield land.

2.5	The Site is accessed by a private road (South Road), which connects to other private estate roads within the Hillhouse International Enterprise Zone. Access to the Hillhouse International Enterprise Zone is provided via Bourne Road (a Gatehouse and weighbridge are located at the entrance to the Enterprise Zone), which connects to the nearest adopted highway (Fleetwood Road North) to the north-west of the Enterprise Zone.

Receptors

2.6	The ‘Noise & Vibration’ Chapter of the ES submitted in August 2024 details the NSR, with the following residential receptors identified in the area around the Site (see Figure 2.1 for ease of reference). The baseline sound monitoring positions are described in the ES as follows:

2.7	Position A (Receptor R1: Rear of Roscoe Avenue) – Southwest of Site (grid reference: 334664 443015)

This monitoring position is representative of the closest existing receptor southwest of the Installation and is located just north and adjacent to Roscoe Avenue. Noise levels at this location are dominated by local road traffic noise and industrial noise from the Business Park. The noise meter was positioned to the rear of the nearest dwelling and representative of dwellings local to the Business Park.

2.8	Position B (Receptor R2: Rear of Caravan Site) – South of Site (grid reference: 335020 443212)

Position B was chosen as a suitable monitoring position to represent typical baseline levels in the vicinity of the caravan park properties south of the Installation. Noise levels at this location are generally formed by noise from the Business Park, distant road traffic noise and birdsong.

2.9	Position C (Receptor R3: Wyre Way) – East of Site (grid reference: 335300 443302)

This monitoring position is representative of typical baseline levels in proximity to the southeastern corner of the Business Park and was located adjacent to the River Wyre saltmarsh and mudflats. Noise levels at this location are generally formed by the Business Park activities and birdsong.

2.10	The above monitoring positions represent the NSR relative to the IBA Facility and as noise levels reduce relative to increasing distance (under normal environmental and assessment conditions) then impacts at other more distant NSR will experience a similar or lower impact.

Figure 2.1: Location of baseline monitoring and NSR positions

2.11	Receptors off Roscoe Avenue and Bentley Green (Receptor R1) are located southwest of the Site boundary at circa 300m to 370m.

2.12	Receptors off (Receptor R2) are located to the south to southeast of the Site, circa 40m to 130m at the closest approach to the Site boundary. There is an earth mound screen on intervening land between the caravans and the site boundary which is circa 5m above the caravan park height datum.

2.13	The nearest ecological receptors to the Site are located to the north and east and include the River Wyre saltmarsh and mudflats where bird species use the area for passage and wintering birds, referred to as Receptor E1 at circa 50m to 110m distance from the Installation boundary. The landform to the east of the Site includes elevated landform immediately adjacent to the Installation boundary which forms the sea wall and PROW for Wyre Way. The land then drops down to a lower height datum towards the saltmarsh and mudflats (marked as NSR E1 on Figure 2.1).

Baseline Noise Levels

2.14	The Noise & Vibration Chapter of the ES provides details of baseline sound monitoring undertaken at the NSR during daytime and night-time periods. Tables 2.1 and 2.2 provides detail of the baseline residual and representative background sound levels.

2.15	A baseline sound survey was undertaken over a 4-day period including a weekend from Friday 19th through to Monday 22nd April 2024 at three fixed locations under suitable monitoring weather conditions and is considered to provide representative baseline sound levels during the quietest period of the week.

2.16	The nearest sensitive receptor (R2: caravan park) is deemed to be the most sensitive receptor (located southeast of the Installation), due to the separation distance.

2.17	The results of baseline monitoring are provided in Table 2.1 and 2.2.

Table 2.1: Existing Background Sound Levels at Monitoring Positions during Daytime

| Monitoring Position (Refer to Figure 2.1) | Time Period (0700-2300) | LAeq dB | LA10 dB | LA90 dB | LAmax dB | Representative1 LA90 dB |
| --- | --- | --- | --- | --- | --- | --- |
| Position A: Rear of Roscoe Av. | Daytime | 52 | 54 | 45 | 44-82 | 42 |
| Position B: Rear of Caravan Site | Daytime | 51 | 52 | 46 | 45-93 | 46 |
| Position C: Wyre Way | Daytime | 52 | 54 | 46 | 41-89 | 45 |

1 Note: Takes into account the median, mean and most commonplace LA90 based on statistical analysis, whichever is lowest.

Table 2.2: Existing Background Sound Levels at Monitoring Positions during Night-time

| Monitoring Position (Refer to Figure 2.1) | Time Period (2300-0700) | LAeq dB | LA10 dB | LA90 dB | LAmax dB | Representative1 LA90 dB |
| --- | --- | --- | --- | --- | --- | --- |
| Position A: Rear of Roscoe Av. | Night-time | 49 | 51 | 42 | 37-82 | 40 |
| Position B: Rear of Caravan Site | Night-time | 51 | 52 | 45 | 49-78 | 44 |
| Position C: Wyre Way | Night-time | 52 | 54 | 47 | 41-80 | 45 |

1Note: Takes into account the median, mean and most commonplace LA90 based on statistical analysis, whichever is lowest.

  - The results of existing background sound measurements taken at the residential fixed monitoring positions indicate that representative background sound levels during the daytime period (0700-2300 hours) vary between 42dB and 46dB LA90 and during the night-time period (i.e. between 2300-0700 hours) between 40dB and 44dB LA90.

  - In context, the residual sound levels at the monitoring positions during the daytime period (0700-2300 hours) vary between 51dB and 52dB LAeq and during the night-time period (i.e. between 2300-0700 hours) between 49dB and 51dB LAeq.

3.0	REVIEW OF NOISE GUIDANCE AND STANDARDS

3.1	Introduction

3.1.1	The ambient environmental noise at any location will vary according to the activities occurring around the location. In the vicinity of a busy motorway, for example, the noise level will remain fairly constant due to the relatively steady noise input from road traffic, whereas the noise level close to a source of high noise over short periods, such as an airport, will vary over a much wider range. It is therefore necessary to consider how to quantify the existing noise levels in an area in order to accurately assess the acceptability of the introduction of a new noise source.

3.1.2	The background sound level, defined as the LA90 parameter, represents the sound level exceeded for 90% of a measurement period, or the ninety- percentile level. It generally reflects the quieter sound level between peak events and generally ignores the effects of short-term higher sound level events. Another way of describing the LA90 level is that it represents the `troughs’ of the sound climate and the lower 10% of the fluctuating ambient sound.

3.1.3	The ten-percentile level LA10, represents the level that is exceeded for 10% of the measurement period and is therefore an indication of the higher levels of sound. This is commonly used to describe and quantify noise from road traffic as it reflects the higher levels of noise and the upper 10% of the fluctuating ambient noise.

3.1.4	The equivalent continuous sound pressure level or LAeq parameter, is a measure of the average sound energy over a given time period. It will include noise from all contributing sources. This is commonly used to describe and quantify noise from specific industrial noise sources.

Noise Guidance and Standards

3.1.5	The noise and vibration chapter of the ES sets out the relevant guidance and standards for noise for industrial development. This provides detail of the most relevant noise criteria upon which the Site should be assessed, this includes:

- BS4142: 2014+A1:2019 `Methods for rating and assessing industrial and commercial sound’ (updated in June 2019);

- BS8233: 2014 `Guidance on sound insulation and noise reduction for buildings’;

- World Health Organisation (WHO) Night Noise Guidelines for Europe: 2009; and

- Environment Agency (EA) Guidance (Noise and vibration management: environmental permits January 2022).

BS4142:2014+A1:2019 `Methods for rating and assessing industrial and commercial sound’

3.1.6	In terms of defining a suitable noise limit, achieving a low impact in accordance with BS4142:2014+A1:2019 would provide suitable protection for the amenity of NSR. The Standard is based on the measurement of background sound using LA90 noise measurements, compared to source noise levels measured in LAeq units.  Once any corrections have been applied for source noise tonality, distinct impulses and intermittency etc., the difference between these two measurements (i.e. known as the `rating’ level) determines the impact magnitude.

Typically, the greater the difference, the greater the magnitude of the impact.

A difference of around +10 dB or more is likely to be an indication of a significant adverse impact (although this can be dependent on the context).

A difference of around +5 dB is likely to be an indication of an adverse impact, depending on the context.

The lower the rating level is, relative to the measured background sound level, the less likely it is that the specific sound source will have an adverse impact or a significant adverse impact. Where the rating level does not exceed the background sound level, this is an indication of the specific sound source having a low impact (although this can be dependent on the context).

3.1.7	In defining noise limits and the significance of noise impacts from the Installation, BS4142 indicates that a low impact occurs where the `rating’ level from site (which includes a noise character) does not exceed the representative baseline sound level and an adverse impact where the `rating’ level is around +5dB above baseline sound levels (depending on the context i.e. if residual sound levels significantly increase).

3.1.8	Based on the requirements set out in BS4142: 2014+A1:2019 to achieve a low impact, the following noise limits would apply:

Table 3.1: Noise Limits Daytime (relative to BS4142 low impact)

| NSR | Location |  | Day (0700-2300 hours) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Grid Reference X Y | Representative Background Sound Level LA90 dB | Residual Sound Level LAeq dB | Rating Level for low impact LAeq1hr dB |
| R1 | R1: Roscoe Avenue & Bentley Green (south- west of Site). | 334668 443000 & 334866 442980 | 42 | 52 | 42 |
| R2 | R2: Caravan Park (southeast and east of Site). | 335085 443214 & 335222 443224 | 46 | 51 | 46 |

Table 3.2: Noise Limits Night-time

| NSR | Location |  | Night (2300-0700 hours) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Grid Reference X Y | Representative Background Sound Level LA90 dB | Residual Sound Level LAeq dB | Rating Level for low impact LAeq1hr dB |
| R1 | R1: Roscoe Avenue & Bentley Green (south- west of Site). | 334668 443000 & 334866 442980 | 40 | 49 | 40 |
| R2 | R2: Caravan Park (southeast and east of Site). | 335085 443214 & 335222 443224 | 44 | 51 | 44 |

Latest Advice Set Out in Environment Agency (EA) Guidance (Noise and vibration management: environmental permits January 2022)

3.1.9	The latest guidance from the EA (Noise and vibration management: environmental permits – January 2022) states:

“For industrial noise impacts where the sound is neither impulsive nor tonal, but you can readily distinguish it against the usual residual acoustic environment, the environment agencies will expect you to apply a minimum character correction of +3 decibels (dB) ‘other’. This is unless you can robustly justify that you do not need such a correction.”

3.1.10	The proposed mitigation measures have been designed to eliminate noise character at NSRs and the application of BAT techniques in order to comply with the EA guidance in eliminating perceptible noise character by including the following specific measures:

- Plant located within buildings.

- Mobile plant fitted with non-tonal reversing alarms.

- HGVs would drive into the IBA Storage building rather than having to reverse.

- IBA Storage (input side) would include absorbent lining to the roof to reduce reverberant noise levels. Concrete to lower section of walls.

- Dividing wall introduced between the IBA input storage and the processed IBA area to minimise noise at the southern end of the building, which is closest to the caravan park.

- Insulated cladding included on the sensitive wall areas of the IBA Storage and the IBA Processing Building walls and roof.

- Metal Storage building fitted with insulated cladding to walls and roof.

- Door into the IBA Processing Building formed by insulated electric roller shutter door and would be closed except for temporary vehicle access, maintenance or emergency.

- Conveyors enclosed.

- No transfer of material during night-time periods between the Processing Building and the Metal Store.

Other Relevant Guidance and Standards

BS8233: 2014 `Guidance on sound insulation and noise reduction for buildings’

3.1.11		The British Standard BS8233 provides additional guidance on noise levels within buildings. These are based on the WHO recommendations and the criteria given in BS8233 for unoccupied spaces within residential properties.

3.1.12	The guidance provided in section 7.7 of BS8233 provides recommended internal ambient noise levels for resting, dining and sleeping within residential dwellings. Table 3.3 provides detail of the levels given in the standard.

Table 3.3: BS8233: 2014 Indoor ambient noise levels for dwellings

| Activity | Location | 07:00 to 23:00 | 23:00 to 07:00 |
| --- | --- | --- | --- |
| Resting Dining Sleeping (daytime resting) Study & work requiring concentration | Living Room Dining room/area Bedroom Office | 35 dB LAeq,16hours 40 dB LAeq,16hours 35 dB LAeq,16hours 35-45dB LAeq,16hours | - - 30 dB LAeq,8hours - |

3.1.13	For a partially open window the standard refers to a reduction of approximately 15dB. This would therefore indicate a noise level outside the window of approximately 50-55dB LAeq,16hours for living rooms during daytime and 45dB LAeq,8 hours during night-time outside bedrooms. For office environments the external noise level would be in the region of 50-60dB LAeq,16hours on the assumption that the office window is open.

World Health Organisation (WHO) Night Noise Guidelines for Europe: 2009

3.1.14	In 2009, the WHO published `Night Noise Guidelines for Europe’, which it describes as an extension to the WHO `Guidelines for community noise’ (1999). It concludes that “Considering the scientific evidence on the thresholds of night noise exposure indicated by Lnight,outside as defined in the Environmental Noise Directive (2002/48/EC), an Lnight,outside of 40dB should be the target of the night noise guideline (NNG) to protect the public, including the most vulnerable groups such as children, the chronically ill and the elderly. Lnight,outside value of 55dB is recommended as an interim target for those countries where the NNG cannot be achieved in the short term for various reasons, and where policy-makers choose to adopt a stepwise approach.” The target of 40dB(A) outside would relate to an internal noise level within bedrooms of 25-30dB(A) Leq8hours.

4.0	NOISE PREDICTIONS

  - Mitigation Strategy

    - The predicted noise levels from the Installation have been calculated using the noise levels provided within Appendix 2. The noise levels are based on plant noise data provided by Fortis IBA on other similar projects in the UK and predicted internal reverberant noise levels with the proposed mitigation measures. The noise mitigation strategy includes the following mitigation strategy:

Design Layout

- The design layout has taken into account the most significant noise sources and positioned these to have minimum impact on the NSR. This includes:

- Having the open side of the IBA Storage building facing a less sensitive direction.

- Keeping the IBA internal loading area towards the northern end of the building and creating a dividing wall between the IBA input area to the northern end of the building and processed material to the south.

- Avoiding the need for vehicles to reverse into buildings.

Noise Control Measures

- Sound power levels or reverberant sound pressure levels of plant as detailed in Appendix 2.

- IBA Storage contained within a building.

- IBA Storage building to have an internal dividing wall between the IBA Input Storage Area and the processed IBA storage area via single skin cladding to Rw=24dB or greater.

- The walls and roof of the IBA processed storage area (i.e. the two sections of the IBA storage building closest to the site entrance) formed by single skin cladding to Rw=24dB or greater.

- IBA input Storage Area rear facade (southwest) façade constructed from double skin insulated cladding to a minimum of Rw=38dB and lower concrete walls (nominally 3m in height) where material storage and bulking required.

- Northwest façade of IBA Storage Building wall and roof formed by single skin cladding to minimum Rw24dB. Lower wall formed by concrete (nominally 3m in height) where material storage and bulking required.

- Roof of the IBA Input Storage Area to be formed by cladding having an absorbent acoustic lining (i.e. perforated facing to the mineral wool infill with minimum 30% open area) to Rw=34dB). No skylights to be fitted into the roof design. The absorbent roof lining provides a reduction in reverberant noise build-up in the building to minimise noise `break-out’ via the open side of the building.

- Processing plant building constructed from double skin insulated cladding to a minimum of Rw=38dB.

- Door into the Processing Building closed except for access to vehicles for offloading and collection unless for maintenance or emergency formed by an electric insulated roller shutter door (Rw typically 18dB).

- Conveyors between the Processing Building and the IBA Storage Building to be enclosed with single skin cladding or similar to Rw=24dB.

- Any openings around conveyors into the Processing Building walls to be acoustically treated to minimise noise `break-out’ via closure plates or cladding.

- Metal storage contained within a building with partial open front.

- Metal Storage building walls and roof constructed from double skin insulated cladding to a minimum of Rw=38dB and lower concrete walls (nominally 3m in height) where material storage and bulking required. Opening required at the SW wall to be centralised to the opening and the sidewalls to the opening closed off using acoustic cladding to Rw=38dB.

- Design to ensure no noise character is perceptible at NSR in accordance with BS4142: 2014+A1:2019.

- Mobile plant and site-controlled vehicles fitted with non-tonal reversing alarms (i.e. broadband noise, `white noise’ or SMART type reversing alarms).

4.1.2	For the purpose of the noise model we have referred to the layout plans and elevations of the IBA Facility provided by Fortis IBA and TRS.

    - In terms of assumed sound reduction index for associated cladding and doors relative to the IBA buildings we have assumed the following acoustic performance, which is detailed below in Table 4.1.

Table 4.1: Assumed SRI values for the cladding, louvres and doors into the buildings

4.1.4	The information assumed for noise emission levels for the plant provides us with noise data for input into the noise model and is provided in Appendix 2 and below for ease of reference.

Estimated grid references for the main buildings include the following:

- Northern corner of IBA Storage building: 334913 443535

- Southern corner of IBA Storage building: 334969 443326

- Northwestern corner of IBA Processing building: 334890 443366

- Southern corner of IBA Processing building: 334979 443294

Table 4.2: Noise Source Input Data

    - For daytime periods, the Site is assumed to have HGVs moving to and from the Installation and the door into the Processing Building would be closed. For calculation purposes the number of HGVs over the daytime period on an hourly basis is assumed to be 9 (i.e. 18 movements per hour) as a worst case scenario.

  - Noise Prediction Modelling

4.2.1	Noise prediction modelling of the Installation is based on the information detailed in Table 4.1 and 4.2 using CadnaA noise prediction modelling software. The results of the noise mapping are shown below and Appendix 3 (noise maps 1 to 2).

4.2.2	The model utilises ISO9613-2 as the method of calculation for propagation from the building to the receptor. Settings for the noise model include the following, which is based on industry accepted standards:

Ground effect (Agr) = 1 (hard ground within the site boundary) & 0 (soft ground absorption due to intervening topography between site boundary and NSR)

Temperature = 10oC

Humidity = 70%

Height above ground for receiver = 4m (night-time) 1.5m (daytime)

Maximum order of reflection = 1

4.2.3	Drawings of the layout of the Installation and building elevations have been provided and the main building assumed heights within the noise model would indicate the following:

- IBA Processing Building: 15m to 17m

- IBA Storage Building: 15m to 17m

- Metal Store Building: 8m to 9m

4.2.4	The results of the noise predictions are provided below in Table 4.3, which include the assumed building cladding and insulation specification for doors and louvres provided in Table 4.1.

Table 4.3: Noise Prediction Results at Residential Receptors with noise mitigation

| Noise Receptor | Predicted Rating1 Noise Level LAeq dB with mitigation | Representative2 Background Sound Level LA90 dB | Level Difference [predicted with LA90] dB |
| --- | --- | --- | --- |
| Daytime (0700-2300 hours) LAeq dB1 hour |  |  |  |
| R1: Roscoe Avenue & Bentley Green (south- west of Site). | 36-38 | 42 | -6 to -4 |
| R2: Caravan Park (southeast and east of Site). | 41-45 | 46 | -5 to -1 |
| Night-time (2300-0700 hours) LAeq dB15mins |  |  |  |
| R1: Roscoe Avenue & Bentley Green (south- west of Site). | 36-37 | 40 | -4 to -3 |
| R2: Caravan Park (southeast and east of Site). | 38-41 | 44 | -6 to -3 |

Note 1: Perceptible noise characteristics at NSR locations is not expected due to proposed mitigation measures. The daytime period includes for HGV movement on Site.

Note 2: Based on a 4-day period of baseline sound monitoring including a weekend at NSR.

4.2.5	The above table shows the impact relative to the likely noise condition and shows that the noise contribution is between 1dB and 6dB lower than the representative daytime background sound levels.

4.2.6	The night-time table of results shows that the Site noise rating noise level is predicted to be between 3dB and 6dB lower than the representative background sound levels.

4.2.7	With the proposed mitigation strategy, the Site is not expected to produce any noise character at NSR.

4.2.8	The results show that the daytime and night-time predicted noise levels (according to BS4142: 2014+A1:2019) are likely to show a low impact magnitude.

Ecological Receptors

    - Ecological receptors included in the ES Chapter include the following:

- River Wyre saltmarsh and mudflats.

- 4.2.10	Predicted noise levels at the above ecological receptors are provided below in Table 4.4.

Table 4.4: Noise Prediction Results at Non-residential Receptors with noise mitigation

| Noise Receptor | Predicted Noise Level LAeq1hr dB | Residual Sound Level LAeq dB | Noise Limits LAeq1hr dB | Excess over limit LAeq dB |
| --- | --- | --- | --- | --- |
| Daytime |  |  |  |  |
| River Wyre saltmarsh & mudflats | 44-55 | 52 | 55 | 0 |
| Night-time |  |  |  |  |
| Rover Wyre saltmarsh & mudflats | 43-55 | 52 | 55 | 0 |

4.2.11	The above resultant noise levels at the ecological receptors shows no significant impact.

Noise Limits at Residential Receptors

4.2.12	For noise limits, we would expect that given the residual and background sound levels in the area, the design of the Installation should aim to achieve a noise contribution that is no higher than the background sound level during daytime and night-time periods.

LAmax Levels

4.2.13	LAmax levels predicted for the operation of the IBA Facility at residential NSR are provided below in Table 4.5. Results from measurements at other sites processing IBA in the UK indicate that in near field positions the LAmax levels are circa 7dB to 14dB higher than the LAeq levels.

Table 4.5: Noise Prediction Results LAmax (night-time)

| Noise Receptor | Night-time Predicted Operational Noise Level LAmax dB | WHO night noise guidelines external limits (bedroom window) LAmax dB (free field) |
| --- | --- | --- |
| R1: Roscoe Avenue & Bentley Green (south- west of Site). | 43-52 | 57 |
| R2: Caravan Park (southeast and east of Site). | 45-54 | 57 |

4.2.14	The LAmax levels are predicted to be below the WHO guidelines for sleep disturbance.

5.0	Conclusions

Noise Limits

5.1	In terms of noise limits at NSRs the indicative background noise levels enable us to establish reasonable noise limits based on satisfying appropriate and relevant standards and guidance.

5.2	By applying BS4142 methodology, we have assumed that reasonable noise limits at each NSR for daytime and night-time operational periods would achieve a `rating’ level that does not exceed background sound levels and night-time operating periods would also comply with the LAmax sleep disturbance criteria as a contribution at NSR.

5.3	The night-time levels are predicted to be between 3dB and 6dB lower than the representative background sound levels at NSR.

Noise Predictions

5.4	A noise prediction model has been developed based on information from the Applicant from similar facilities and information concerning the Installation. The prediction model used includes the use of ISO9613-2 which is a nationally recognised calculation method to provide good accuracy.

5.5	Within BS4142: 2014, section 10.3 deals with `uncertainty in calculation’ and states:

“Uncertainty in calculating sound levels can arise from:

    - uncertainty in any measured sound levels used in the calculations;

    - uncertainty in the operation or sound emission characteristics of the specific sound source and any assumed sound power levels;

    - uncertainty in the calculation method;

    - simplifying the real situation to “fit” the model (user influence on modelling); and

    - error in the calculation process.

Where the sound power level is used for calculating sound pressure levels, it ought to be representative of the source and the conditions under which the source is expected to operate.

Where possible, use recognized standards to establish the sound power level and the uncertainty (e.g. BS EN ISO 3740 and BS EN ISO 3747). Where it is not possible to use appropriate standards, describe the method of establishing the sound power level, report the uncertainty and state the reasons for using this method.

Use a validated method of calculating sound levels, e.g. ISO 9613-2 or similar. If an alternative calculation method is used, fully describe the method and state the reasons for using this method.

Check the implementation of the calculation method for errors.

For simple cases, e.g. where the level of variability in sound propagation resulting from changes in meteorological conditions is likely to be small, simple calculation methods might be sufficient.”

5.6	In terms of the prediction calculations undertaken, the following points are noted:

- A recognised standard for calculation has been used with appropriate settings to give an accurate prediction.

- Input data for the IBA Facility has been based on typical plant noise levels from measurements at other Fortis IBA sites. Empirical noise data for mobile plant has been referenced from the NVC library.

- Detailed layout of the Installation and elevations for the proposed IBA storage and processing buildings have been used to inform the noise model.

- Typical manufacturers’ data on building cladding for wall and roof areas, and library data on roller shutter doors and other proposed wall materials has been referenced for input data into the noise model.

- Baseline levels have been recorded by NVC over a weekday and weekend period in the vicinity of the NSR positions.

5.7	The only potential variation in predicted noise levels is likely to be as a result of sound propagation resulting from changes in meteorological conditions. This is difficult to predict, and in the situation, where there is a positive wind vector in the direction of nearest sensitive receptors the actual background noise level could, in any case, be higher than when measured under ideal conditions. We therefore would not consider this to be a significant factor due to the fact that when assessing the Site for compliance this would be carried out in suitable meteorological conditions. It should be noted that the predominant national wind direction is southwest to west during the year, which would mean that the wind direction under these conditions would blow from the sensitive receptor towards the Site and therefore provide benefit of wind directivity and assist in reducing noise levels further.

Assessment Results

5.8	The results show no exceedance above relevant noise limits determined from standards and guidance for daytime and night-time periods at NSR locations.

5.9	Predicted noise levels using typical plant operating noise levels with appropriate noise mitigation would produce levels which would be well within sleep disturbance criteria, guidance levels within BS8233: 2014, WHO guidelines for community noise and amenity, and produce a low impact magnitude in accordance with BS4142: 2014+A1:2019.

REFERENCES

- ISO 9613-2: 1996 Acoustics – ‘Attenuation of Sound During Propagation Outdoors – Part 2: General Method of Calculation’

- British Standards BS4142: 2014+A1:2019 `Methods for rating and assessing industrial and commercial sound’

- BS8233: 2014 `Guidance on sound insulation and noise reduction for buildings’

- Guidelines for Community Noise – World Health Organisation: April 1999

- World Health Organisation ‘Night Noise Guidelines for Europe’ – 2009

- BS7445: 2003 - Description and measurement of environmental noise

- National Planning Policy Framework – December 2023

- Planning Practice Guidance – June 2021

- Noise Policy Statement for England (NPSE) – March 2010

- Noise and vibration management: environmental permits (January 2022)

- Chapter 7 ‘Noise & Vibration' of the Environmental Statement

- Environmental Statement Chapter 7 Appendices

FIGURES

Figure 1: Site location, Baseline Monitoring and Noise Sensitive Receptors

Baseline Noise Monitoring Positions

Residential Receptor Positions

Nearest Sensitive Ecological Receptor Positions

Figure 2: Plan of Site Showing Layout

Figure 3:  Side Elevation Drawings of Proposed IBA Processing Building

East Elevation 						Front Elevation

West Elevation						Rear Elevation

Figure 4: IBA Storage Building (including Processed IBA)

Figure 5: Metal Storage Building

APPENDIX 1

Basic Acoustic Terminology

Sound is produced by mechanical vibration of a surface, which sets up rapid pressure fluctuations in the surrounding air.

Sound Pressure Level is a measurement of the size of these pressure fluctuations.  It is expressed in decibels (dB) on a logarithmic scale.  Each 3 dB increase in sound pressure level represents a doubling of the sound energy.  An increase of around 10dB is said to subjectively double the sound level. The threshold of hearing is approximately 0 dB.

The rate at which the pressure fluctuations occur determines the pitch or frequency of the sound.  The frequency is expressed in Hertz (Hz), that is, cycles per second.  The human ear is sensitive to sounds from about 20 Hz to 20,000 Hz.  Although sound can be of one discrete frequency - a 'pure tone' - most noises are made up of many different frequencies.

The human ear is more sensitive to some frequencies than others, and modern instruments can measure sound in the same 'subjective' way.  This is the basis of the A-weighted sound level dB(A), normally used to assess the effect of noise on people.  The dB(A) weighting emphasises or reduces the importance of certain frequencies within the audible range.

Noise Measurement

The measurement of sound pressure level is only really meaningful where the level of noise is constant.  In the typical industrial environment noise levels can vary widely and sometimes short duration high levels of noise are interspersed with periods of relative quiet.  The most widely used means of 'averaging' the noise over a period of time is the Equivalent Continuous Sound Level.  Normally written as LAeq  this value takes into account both the level of noise and the length of time over which it occurs.  There are many meters available which are capable of measuring LAeq by electronic integration over the measurement period.

The LAeq or A-weighted equivalent continuous noise level is a measure of the total noise energy over a stated time period and includes all the varying noise levels and re-expresses as an 'average', allowing for the length of time for which each noise level was presented.

The LAn parameters are defined as the noise levels which are exceeded for n% of the monitoring period, thus, for example, the LA90 parameter is the noise level exceeded for 90% of the 15-minute period, i.e. 13.5 minutes.  The LA50 parameter is the noise level exceeded for 50% of the hourly period, i.e. 30 minutes, etc.  The Lmax parameter is the maximum RMS A-weighted noise level occurring during the measurement period.

The definition in layman’s terms is given below for terminology used in the measurement and results obtained during the survey work.

A-weighting: Normal hearing covers the frequency (pitch) range from about 20Hz to 20,000 Hz but sensitivity of the ear is greatest between about 500Hz and 5000Hz.  The "A-weighting" is an electrical circuit built into noise meters to mimic this characteristic of the human ear.

Ambient noise:  The totally encompassing sound in a given situation at a given time usually composed of sound from many sources near and far.

Attenuation:   Noise reduction

Background noise:  The general quiet periods of ambient noise when the noise source under investigation is not there.

Decibel (dB):  The unit of measurement for sound based on a logarithmic scale.  0dB is the threshold of normal hearing; 140dB is the threshold of pain.  A change of 1dB is only detectable under controlled laboratory conditions.

dB(A) [decibel A weighted]:  Decibels measured on a sound level meter incorporating a frequency weighting (A weighting) serves to distinguish sounds of different frequency (or pitch) in a similar way to how the human ear responds.  Measurements in dB(A) broadly agrees with an individual's assessment of loudness. A change of 3dB(A) is the minimum perceptible under normal everyday conditions, and a change of 10dB(A) corresponds roughly to doubling or halving the loudness of sound.

dB(C): [decibel C weighted]: Frequency weighting which does not alter low frequency octave band levels by very much compared to `A'  weighting.  Similar to linear reading (i.e. linear does not alter frequency spectra at all)

Frequency (Hz):  The number of sound waves to pass a point in one second.

LAeq:  This is a noise index used to describe the "average" level of a noise that varies with time (T).  It allows for the different sensitivities of the human ear to different frequencies (pitch), and averages fluctuating noise levels in a manner, which correlates well with human perceptions of loudness.

LA10,T:  This noise index gives an indication of the upper limit or peak levels of the fluctuating noise.  It is the "A weighted" noise level exceeded for 10 per cent of the specified measurement period (T). e.g. If the measurement period was over 10 hours and the LA10 reading was say 60dB, then this means that for 1 hour out of 10 the level went above 60dB.

LA90,T:  This noise index gives an indication of the lower limit or levels of the fluctuating noise.  It is the "A weighted" noise level exceeded for 90 per cent of the specified measurement period (T). e.g. If the measurement period was over 10 hours and the LA90 reading was say 50dB, then this means that for 9 hours out of 10 the level went above 50dB.

LAmax:  This is the highest `A’ weighted noise level recorded during a noise measurement period.

Residual noise:  The ambient noise remaining at a given position in a given situation when the noise source under investigation is not there.

Specific noise: The noise source under investigation for assessing the likelihood of complaints

Examples of typical noise levels

| Source/Activity | Indicative noise level [dB(A)] |
| --- | --- |
| Threshold of hearing | 0 |
| Rural night-time background | 20-40 |
| Quiet bedroom | 35 |
| Wind farm at 350m | 35-45 |
| Busy road at 5km | 35-45 |
| Car at 65km/h at 100m | 55 |
| Busy general office | 60 |
| Conversation | 60 |
| Truck at 50km/h at 100m | 65 |
| City Traffic at 5m | 75-85 |
| Pneumatic drill at 7m | 95 |
| Jet aircraft at 250m | 105 |
| Threshold of pain | 140 |

APPENDIX 2

baseline sound survey details

BASELINE SURVEY DETAILS

NOISE INSTRUMENTATION, METHODOLOGY & SURVEY DETAILS

Survey Methodology

Instrumentation and Fieldwork Details

The background sound measurements used in the assessment were undertaken in April 2024. The measurements were undertaken in proximity to accessible nearest sensitive receptors to identify typical baseline sound levels. The monitoring of residual and background sound was carried out during a weekday and weekend period such that the lowest likely representative background sound levels could be included for the assessment.

The following instrumentation was used for all noise measurements:

November 2023

| Manufacturer | Description | Type | Calibration Due date | Serial No. |
| --- | --- | --- | --- | --- |
| Cirrus | Real Time Analyser | CR:171A | July 2024 | G061253 |
| Cirrus | Real Time Analyser | CR:171B | April 2025 | G056142 |
| Cirrus | Real Time Analyser | CR:1710 | March 2025 | G304789 |
| Cirrus | Acoustic Calibrator | CR: 531A | May 2024 | 031523 |

The following set-up parameters were used on the sound level meters during noise measurement:

Static Noise Monitoring:

Time Weighting:	Fast

Frequency Weighting:	‘A’

Measurement Period:	15minute intervals

Calibration

Calibration setting:	94dB

The noise meters were calibrated with the electronic calibrator prior to commencement and on completion of the Survey.  No significant drift in calibration was observed.

Survey Dates and Personnel

Baseline Survey – Thursday 18th to Monday 22nd April 2024

Static noise measurement positions in April 2024 were chosen following liaison with the Business Park to carry out monitoring at or in proximity to NSR to the Site to establish typical baseline sound level data (see Appendix C for detailed information). Mr. D.R. Kettlewell of Noise & Vibration Consultants Limited set up and retrieved the monitoring equipment.

Measurements were recorded at three fixed monitoring positions during daytime and night-time periods (i.e. A to C as shown on Figure 1). Data logging of LAeq, LA10, LA90 and LAmax were recorded at 15-minute intervals for information on the variation of typical baseline sound levels.

The noise meters were mounted on a tripod at a height of circa 1.5 metres above ground level and fitted with a wind and rain shield.

Meteorological Conditions

Weather details were recorded by the NVC Consultant using a Davis Vantage Vue weather station (model 6250UK, product no.#6351UK) during the period of the Survey. Any unsuitable monitoring periods were removed from the data set (i.e. high winds or rain or temperatures below zero) as detailed below.

Photographs of Monitoring Positions

Position A:  Land Adjacent to Nearest Property off Roscoe Avenue

Weather Station adjacent to Roscoe Avenue Receptors

Position B:  Adjacent to Caravan Park Boundary

Weather Conditions During Baseline Sound Survey

APPENDIX 3

Baseline Sound Survey Results

Summary of LA90 levels at Baseline Monitoring Positions

APPENDIX 4

site operational noise levels & MITIGATION MEASURES

Assumed Noise Levels for Site Plant & Cladding Performance

(including noise mitigation measures)

| Plant Type or Area | Treatment (Cladding Performance Rw) dB | Sound Power (SWL) Sound Pressure Level (SPL) at roof/walls | Assumed % Operating Time | Period of Operation |
| --- | --- | --- | --- | --- |
| IBA Storage building (IBA input store area) | Cladding Rw 38dB (SW wall) Concrete lower wall (3m) | 80 (Rev SPL) | 100 | Daytime/Night-time |
| IBA Storage building (IBA input store area) | Cladding to Roof Rw 34dB (perforated inner skin with rockwool & solid outer skin) | 80 (Rev SPL) | 100 | Daytime/Night-time |
| IBA Storage building (IBA input store area) | Cladding Rw 24dB (NW wall) Concrete lower wall (3m) | 80 (Rev SPL) | 100 | Daytime/Night-time |
| IBA Store (processed IBA) | Cladding (single skin) to walls & roof Rw 24dB | 67 (Rev SPL) | 100 | Daytime/Night-time |
| IBA Store (processed IBA) Dividing wall | Cladding (single skin) Rw 24 | 67 (Rev SPL) | 100 | Daytime/Night-time |
| Processing Building | Roof & Walls Rw 38dB | 90 (Rev SPL) | 100 | Daytime/Night-time |
| Conveyor between Processing building & IBA Store | Enclosed Rw 24dB cladding | 80 (SWL) | 100 | Daytime/Night-time |
| Metal Storage building | Cladding Rw 38dB (walls/roof). Concrete lower wall (3m) | 89 (Rev SPL) | 100 | Daytime |
| HGV | - | 72 @ 10m (SPL) | 9 per hour (day) each way | Daytime |
| Processing Building Door | Insulated electric roller shutter door Rw 18dB | 90 (Rev SPL) | 100 | Daytime/Night-time |
| Mobile Plant | Fitted with non-tonal reversing alarms | - | Variable | Daytime/Night-time |
| Noise Character (i.e. tonal, impulsivity and intermittency) | Design of plant to ensure no perceptible noise character at NSR | - | 100 | Daytime/Night-time |
| Tractor/Trailer | - | 106 (SWL) | 6 per hour | Daytime |
| Telehandler | - | 103 (SWL) | Variable (10 per hour) | Daytime/Night-time |

APPENDIX 5

NOISE MODEL SETTINGS & MAPPING RESULTS

INPUT DATA FOR ISO 9613 CALCS

Noise Prediction Model

There are a number of empirical or semi-empirical sound propagation models in common use. One of these is ISO9613-2 which is the International Standard used to predict noise propagation.

The noise levels produced by the IBA plant at each of the nearest sensitive receptors has been calculated using a computer model, which is based on ISO 9613, Acoustics – Attenuation of Sound During Propagation Outdoors [1996]. The propagation model described in Part 2 of the standard provides a method for predicting sound pressure levels.

The computer model utilises octave band frequency data of the noise source to assess and predict the noise contribution with the site in full operation.

The ISO propagation model provides a method for calculating the sound pressure level at a specific position by taking the sound power level radiating from the building facades in frequency bands and subtracting a number of attenuation factors according to the following:

Predicted sound pressure level =

Lw + D – Ageo – Agr – Abar - Amisc

The prediction modelling uses octave band frequency sound power level data calculated in different wall and roof areas of the IBA plant and corrects the level for the following additional propagation factors and attenuation:

Octave band frequency spectra:

Based on empirical noise measurements recorded at a similar site in the UK when under load conditions. The noise levels at specific face positions are provided below that have been used for the noise model.

D – Directivity Factor

The Directivity Index will depend on the radiating surface and whether it is located in free space, at junction of two surfaces or more and the correction factor changes accordingly. Directivity factor is generally = 2.

Ageo - Geometrical Divergence

The geometrical divergence of sound waves accounts for the spherical spreading in the free field from a point source resulting in attenuation depending on distance, which relates to the following correction:

Ageo = 20 x log (d) + 11 [where d = distance from the noise source]

Receiver height assumed = Daytime = 1.5m, Night-time = 4.0m above ground level (except for caravan site and ecological receptors where the height assumed is 1.5m during night-time).

Aatm  - Atmospheric Absorption

When sound energy propagates through the atmosphere it is attenuated as a result of the conversion of the sound energy into heat. The attenuation is dependent upon the relative humidity and the temperature of the air through which the sound energy is travelling. The attenuation is also dependent upon the frequency content of the sound energy with higher levels of attenuation towards higher frequencies.

The attenuation therefore depends upon the distance from the sound source and according to ISO9613 is calculated according to the following formula:

Aatm = d x α [Where d = distance from the source

a = atmospheric absorption coefficient in dB/m]

From ISO9613 Part 1 [1996] I have used values of `a’ corresponding to a temperature of 10oC and a relative humidity of 70%. This will give an indication of the lowest likely atmospheric attenuation as examples worked at 20deg C and -5deg C indicate a reduction of around -0.5dB(A) on those values calculated. The values for each one-third octave band are given below in table 1.

Table 1: Atmospheric absorption attenuation based on temperature of 10oC and a relative humidity of 70%

| Third Octave Band Centre Frequency (Hz) | 50 | 63 | 80 | 100 | 125 | 160 | 200 | 250 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Atmospheric Absorption Coefficient (dB/km) | 0.0785 | 0.122 | 0.186 | 0.28 | 0.411 | 0.584 | 0.797 | 1.04 |
| Third Octave Band Centre Frequency (Hz) | 315 | 400 | 500 | 630 | 800 | 1k | 1.25k | 1.6k |
| Atmospheric Absorption Coefficient (dB/km) | 1.31 | 1.6 | 1.93 | 2.33 | 2.87 | 3.66 | 4.86 | 6.73 |
| Third Octave Band Centre Frequency (Hz) | 2k | 2.5k | 3.15k | 4k | 5k | 6.3k | 8k | 10k |
| Atmospheric Absorption Coefficient (dB/km) | 9.66 | 14.3 | 21.5 | 32.8 | 50.2 | 76.9 | 117 | 175 |

Agr – Ground Effect

Ground Effect for Calcs

G = 0 to 1 (soft to hard ground absorption)

The ground effect is a result of the interference of sound reflected by the ground which interferes with the direct sound propagating from the noise source to the receiver. The prediction of the ground effects is relatively complex and is dependent upon a number of factors including ground conditions, source height, receiver height and the propagation height between the source and receiver. The ground conditions are described according to a variable `G’ which varies between 0 for `hard’ ground and 1 for `soft’ ground. Hard ground refers to paving, concrete and any sites with low porosity. Soft ground refers to grassland, trees or other vegetation. I have assumed a ground factor of G = 1 (hard ground) for areas within the Site boundary and G = 0 to represent a soft ground absorption to NSR as intervening ground is generally formed mainly by a mixture of vacant undeveloped land and woodland/grassed areas and is therefore appropriate. I have taken the source height as being the height of the relevant section of building and a receiver height of 1.5 metres for daytime and 4 metres for night-time operations for residential receptors (except caravans and ecological receptors at 1.5m for night-time).

Abar – Barrier Attenuation

When there is a solid barrier between any noise source and the receiver position the noise level will be reduced. The level of attenuation resulting will depend upon the barrier position, barrier size, receiver position and frequency content relative to the noise source. For the purpose of these calculations, we have not included any local screening from existing industrial buildings adjacent to the Site or local garden screening. We have included any earth mound screening on intervening land between the Site and the caravan park and to the eastern boundary relative to the saltmarsh and mudflat areas.

Amisc – Miscellaneous Other Effects

This additional attenuation effect described in ISO9613 allows for the effects of propagation through foliage. I have not taken account of any such effects and in my expert opinion they are unlikely to significantly reduce noise levels below those predicted.

Mobile Plant Noise Sources

The noise model allows for HGV movement within the site boundary (based on a line source). These are accounted for during the daytime operating period based on 9 HGV movements in and out per hour for the Proposed IBA facility. Additional mobile plant movement between the Processing and Storage buildings is allowed based on 6 movements per hour and 10 movements per hour between the skip area at the Processing building and the skip covered storage area.

Noise Mapping Results

Noise Map 1: Daytime Noise Contours (with Additional Mitigation Strategy)

Noise Map 2: Night-time Noise Contours (with Additional Mitigation Strategy)

APPENDIX 6

CONSULTANT’S EXPERIENCE & QUALIFICATIONS

Principal Consultant: Dean Robert Kettlewell - MSc MIOA MAE I.Eng

(Director & Principal Acoustic Consultant)

# Précis

As Director and Principle Acoustic Consultant with Noise & Vibration Consultants Ltd, Dean has over 40 years background experience in a wide range of issues relating to environmental, industrial and commercial noise and vibration assessment. He currently manages corporate and unit specific contracts for:

- Environmental Noise Impact Assessments

- Industrial Noise Assessment and Control

- Planning Issues for Residential and Commercial Development

- Noise at Work Regulations Assessments

- Building Acoustics and Sound Insulation Tests

- Expert Witness representation for Planning Appeals

- Integrated Pollution Prevention and Control (IPPC) Applications

- Wind Farm Noise Impact Assessments

- Entertainment Noise Assessment and Control

- Architectural Acoustics

- Specialist knowledge in the Design of Noise Control Systems

- Ground borne vibration measurement and assessment

- Assessment of Environmental & Industrial Noise Nuisance

- Project Management of Noise Control Systems

## Relevant Work Experience

Director & Principal Consultant - Noise & Vibration Consultants Ltd 	 	2001- to date

#### Senior Acoustic Consultant - Vibrock Limited 	1998 - 2001

Associate & Principal Acoustic Consultant - John Savidge & Associates	1994 - 1998

Technical Manager – LBJ Limited (Noise Control Division)			1990 - 1994

Technical Engineer/ Technical Manager (1988) - Vibac (Noise Control) Ltd	1982 - 1990

## Qualifications and Education

M.Sc. Applied Acoustics (Derby University – Distinction)

##### HNC Electrical & Electronic Engineering

IOA Diploma in Acoustics & Noise Control

IOA Certificate in Law and Administration

Certificate of Competence in Workplace Noise Assessment

Certificate of Competence in Ground Vibration Monitoring

Affiliations:		Member of Institute of Acoustics (MIOA)

Member of Academy of Experts (MAE)

Member of Association of Noise Consultants (ANC)

Incorporated Engineer (I.Eng)

NVC

Noise &

Vibration

Consultants Ltd