# Technical Standards

Farm name: Cherry Rock Farm, Old Oak Farm, Newberries Farm	Operator: G & S Phelps	Permit number: EPR/NP3239UE

Review Date: 18th May 2026

| Schedule 1 activity or directly associated activity (DAA) description | Relevant technical guidance note |
| --- | --- |
| Feed storage and preparation | - Selection and use of feed are in accordance with Sector Guidance Note (SGN) EPR6.09 ‘How to comply with your environmental permit for intensive farming’ - Feed is stored in purpose-built, covered, feed silos and tanks located next to broiler sheds. Feed is delivered to the farm by lorry from feed suppliers. Feed is blown, augured or pumped directly from the lorry into the relevant storage silos. Feed is piped from the silos to the sheds, minimising dust emissions. t - Areas around buildings are kept free from build-up of manure and spilt feed - Selection and use of feed are in accordance with SGN EPR6.09 ‘How to comply with your environmental permit for intensive farming’ - Rations are matched to the animals’ needs. A nutritionist is employed to regularly review and reformulate diets, to optimise production and minimise excretion of nutrients. |
| Dirty Water & Manure storage | - All manure is kept within broiler houses until houses are destocked. - Manure is removed from buildings and exported directly to a third party. - Dirty water pits are present at all sites. Diverter valves are used when water entering the pits is required to be stored. This occurs at wash down after destocking. - All dirty water is exported off sites to a third party. |

| Dirty water and manure management | - All dirty water and manure is exported from the site. Records are kept of the arrangements in place when dirty water and manure is exported from the site. We have written confirmation that the recipient will spread dirty water and manures to land in accordance with the Defra Code of Good Agricultural Practice and that the spreading will be in accordance with a manure management plan for receiving land. - There are contingency arrangements in place, should the land become unavailable |
| --- | --- |
| Fuel, oils and chemical storage | - Fuel oil, oils, pesticides and veterinary medicines are all stored in bunded areas capable of retaining any spillage - Fuel oil for the standby generators is stored in bunded tanks that meets the requirements of SSAFO. The bund has a capacity of 110% of the oil tank. The bund base and walls are impermeable to oil and water and designed to catch leaks from tank fittings (including the tanker connection point, site gauge and shut-off valve). The tank is not within 10 m of a watercourse. There are no yard drains, ditches or land drains within 10 m. The tank tap through which fuel oil can be discharged is within the bund. The tap is locked shut when not in use - The hose and tap are locked inside the bund when not in use |
| Housing | - Housing design and management is in accordance with SGN EPR6.09 ‘How to comply with your environmental permit for intensive farming’ - Litter housing system is in use at the farm. - Maintenance and upgrades take place regularly. The housing and drainage have been assessed as BAT. |

|  | - The animal housing is of insulated construction. The housing is well insulated, where appropriate, and the sheds have a damp-proof course, which helps to reduce heat loss and condensation - All buildings and structures on site are maintained in good repair, in accordance with the management system. There is a programme of inspection and planned preventative maintenance for the housing and drainage. Floors and walls are kept clean. Any cracks and damaged areas of yards and walls are repaired - Litter is removed when houses are destocked. - Drinkers have been designed to prevent leakage. - Service checks are carried out on the ventilation system monthly. |
| --- | --- |
| Drainage | - There are no direct releases to ground water. - Diverter valves are in place for when the yards are dirty in order to stop dirty water entering reed beds, swales and ditches. - Copies of Drainage Plans are kept with the all compliance documentation. - The clean water drainage systems are not contaminated. - Yard areas are kept visibly clean, drainage channels are kept clear and spilt feed and dust are cleaned up - Drainage from the animal housing and water from cleaning out is treated as dirty and directed to the dirty water tanks for storage and removal. - Disinfectant footbaths are designed not to overflow. Used disinfectant is added to the dirty water storage. |
| Livestock numbers and movements | - A system is in place to record the number of animals on the farm at any one time. Animal movements on and off the farm are also recorded; these records will be available for inspection. |
| Carcase incinerator | - Fallen stock is disposed of in accordance with the current Animal By-Products Regulations. - All dead stock is disposed of via deadstock collection, as per the site plan to avoid contamination in lockable bins. |
| Pollution prevention measures | - All operations are assessed annually for opportunities to reduce pollution risk and implementation schedules developed as appropriate - All staff are trained in pollution risk identification, minimisation and emergency procedures for general site activity and activity relating to their work duties - There is an accident management plan in place, with a procedure to review incidents |
| Veterinary medicines and pest | - Pesticides and veterinary medicines are kept in a store capable of retaining spillage, resistant to fire, and are kept |

| control | dry, frost-free and secure. Vermin control chemicals are brought on site by a registered contractor for use as needed. Chemicals to control flies and other insect pests will be stored within bunded and locked store. |
| --- | --- |
| Hazardous waste | - Veterinary waste is removed by the vet for safe disposal. Other hazardous waste, such as fluorescent light bulbs, waste oil, aerosols, etc. are removed by a licensed contractor with an adequate audit trail, meeting the requirements of the Environmental Permitting Regulations |

# Buildings inventory

For location of buildings, refer to the Site Plans

| Building name and ref on plan | No of places | Type of ventilation | Floor type | Manure management | Feed | Other |
| --- | --- | --- | --- | --- | --- | --- |
| Cherry Rock Farm – Houses 1-4 | 150,000 | Side extraction | Solid | Litter. Removed when houses are destocked. | Dry feed | Insulated building |
| Old Oak Farm – Houses 1-6 | 240,000 | High Velocity Roof Fans >5.5m | Solid | Litter. Removed when houses are destocked. | Dry Feed | Insulated building |
| Newberries Farm – Houses 1-4 | 115,000 | High Velocity Roof Fans >5.5m | Solid | Litter. Removed when houses are destocked | Dry Feed | Insulated building |

# Emissions

Table of emission points

| Emission point reference | Emission point description and location | Source |
| --- | --- | --- |
| Air |  |  |
| Cherry Rock Farm | Side extraction ventilation | Houses 1-4 |
| Cherry Rock Farm | Biomass boiler (NGR – SO 78151 26424) | 2 boilers with a thermal input of 0.221 MWth each |
| Old Oak Farm | High Velocity Roof Fans | Houses 1-6 |
| Old Oak Farm | Biomass Boiler (NGR – SO 77459 26539) | 1 boiler with a thermal input of 1.094 MWth |
| Newberries Farm | High Velocity Roof Fans | Houses 1-4 |
| Newberries Farm | Biomass Boiler (NGR – SO 78880 26017) | 2 boilers with a thermal input of 0.221 MWth each |
| Land |  |  |
| Cherry Rock Farm | Soakaway to ditch via reed bed at South of the site | Discharge of clean roof and yard water via French drains from reed bed to ditch via soakaway. |
| Old Oak Farm | Soakway to ditch via swale at Southwest of the site | Discharge of clean roof and yard water via French drains from swale to ditch via soakaway. |
| Newberries Farm | Soakaway to ditch via diverter valve in tank at North of site | Discharge of clean roof and yard water via French drains from diverter valve in underground tank to ditch |
| Water |  |  |
| Cherry Rock Farm | Discharge to ditch via reed bed | Roof water from buildings and clean yard water via French drains |
| Old Oak Farm | Discharge to ditch via swale | Roof Water from buildings and clean yard water via French drains |
| Newberries Farm | Discharge to ditch via diverter valve in underground tank | Roof water from buildings and clean yard water via French drains |

Flies

The farm manager undertakes regular inspections of the site. Appropriate actions are in place to prevent and control flies, should a nuisance arise. See Pest Management Plan (page 13).

# Site operations and pollution prevention measures

| 1.Site | 2. Substance | 3. Relevant | 4. Possible failure | 5. History/records or visual | 6. Do pollution | 7. Provide details of pollution | 8.Testing and |
| --- | --- | --- | --- | --- | --- | --- | --- |
| operations |  | activity | mechanism and potential | evidence of leaks of potentially | prevention | prevention measures | inspection of |
| (storage and |  |  | for pollution | polluting substances to land | measures exist |  | pollution |
| use) |  |  |  | associated with the activities | for relevant |  | prevention |
|  |  |  |  | that could result in ongoing | activity? |  | measures |
|  |  |  |  | emissions to land, e.g. cracking |  |  |  |
|  |  |  |  | in hard standing, leaking tank |  | To include: primary, e.g. tanks or | Note: If you are not |
|  |  |  |  | or bund |  | pipework; secondary, e.g. bund or | able to supply all of |
|  |  |  |  | Detail any incidents of pollution or | Yes/No | hard standing and, where | this information at |
|  |  |  |  | spills from the relevant activity. |  | present, tertiary, e.g. oil | present, you may |
|  |  |  |  | This can be based on visual |  | interceptor | submit the details |
|  |  |  |  | assessment during site walk over |  |  | with your Accident |
|  |  |  |  | or other records and data sources |  |  | Management Plan |
| Vehicle and | Fuel oil | Storage | Failure of tank leading to | None identified | Yes | Concrete base and bund | Tank, fittings and |
| machine fuel |  |  | spillage to land |  |  | containing tank and fill point | bund visually |
|  |  |  |  |  |  | Store locked when not | inspected monthly |
|  |  |  |  |  |  | in use | and following any |
|  |  |  |  |  |  |  | notified spill |
|  |  |  |  |  |  | Complies with SSAFO |  |
|  |  |  |  |  |  | Regulations |  |
|  |  | Delivery by | Spillage from road tanker on | None | Yes | Delivery by supplier’s vehicle. | Concrete hard |
|  |  | road tanker | installation yards entering |  |  |  | standing area |
|  |  | tanker off- | Spillage from road tanker or |  |  | Tank and fixed pipework within | Bunded area and |
|  |  | loading | delivery pipework to yard |  |  | bunded area | tank visually |
|  |  |  |  |  |  | Concrete hard standing | inspected before |
|  |  |  |  |  |  | Materials available to soak up | each delivery |
|  |  |  |  |  |  | minor spills |  |
|  |  | Fueling | Spillage on yard, overflowing | None | Yes | As above. Automatic closing | As above |
|  |  | vehicles | tanks |  |  | trigger, locks on valves stored in |  |
|  |  |  |  |  |  | bund when not in use |  |
|  |  |  |  |  |  | Record kept of fuel use, |  |
|  |  |  |  |  |  | regularly reviewed |  |

| 1.Site | 2. Substance | 3. Relevant | 4. Possible failure | 5. History/records or visual | 6. Do pollution | 7. Provide details of pollution | 8.Testing and |
| --- | --- | --- | --- | --- | --- | --- | --- |
| operations |  | activity | mechanism and potential | evidence of leaks of potentially | prevention | prevention measures | inspection of |
| (storage and |  |  | for pollution | polluting substances to land | measures exist |  | pollution |
| use) |  |  |  | associated with the activities | for relevant |  | prevention |
|  |  |  |  | that could result in ongoing | activity? |  | measures |
|  |  |  |  | emissions to land, e.g. cracking |  |  |  |
|  |  |  |  | in hard standing, leaking tank |  | To include: primary, e.g. tanks or | Note: If you are not |
|  |  |  |  | or bund |  | pipework; secondary, e.g. bund or | able to supply all of |
|  |  |  |  | Detail any incidents of pollution or | Yes/No | hard standing and, where | this information at |
|  |  |  |  | spills from the relevant activity. |  | present, tertiary, e.g. oil | present, you may |
|  |  |  |  | This can be based on visual |  | interceptor | submit the details |
|  |  |  |  | assessment during site walk over |  |  | with your Accident |
|  |  |  |  | or other records and data sources |  |  | Management Plan |
| Feed | Nutrients: Phosphorus and nitrogen Dust | Delivery to storage areas: dry bulk | Spillage, split or failed pipework, dust, failure of bins | None | Yes | Purpose-made dedicated stores | Pipework and bins inspected monthly to assess condition |
|  |  | Delivery to | Spillage, split bags | None | Yes | Purpose-made dedicated stores storesstores | Regular inspection |
|  |  | storage |  |  |  |  | of facilities and |
|  |  | areas: dry |  |  |  |  | equipment |
|  |  | bagged |  |  |  |  |  |
|  |  | Distribution: all | Broken augers | None | Yes | Auger runs only when necessary. Mostly within buildings | Regular inspection of facilities and equipment |
| Dirty | Nutrients, | Storage | Structural failure | None | Yes | Dedicated purpose-built | Regular inspection |
| water/wash | pesticides, |  |  | Pollution prevention |  | and minimise contamination |  |
| waters | biocides |  |  | measures in good condition |  | Regular monitoring of tanks and |  |
|  |  |  |  |  |  | store contents. Diverter valves in place to prevent storage of clean water. |  |

| 1.Site | 2. Substance | 3. Relevant | 4. Possible failure | 5. History/records or visual | 6. Do pollution | 7. Provide details of pollution | 8.Testing and |
| --- | --- | --- | --- | --- | --- | --- | --- |
| operations |  | activity | mechanism and potential | evidence of leaks of potentially | prevention | prevention measures | inspection of |
| (storage and |  |  | for pollution | polluting substances to land | measures exist |  | pollution |
| use) |  |  |  | associated with the activities | for relevant |  | prevention |
|  |  |  |  | that could result in ongoing | activity? |  | measures |
|  |  |  |  | emissions to land, e.g. cracking |  |  |  |
|  |  |  |  | in hard standing, leaking tank |  | To include: primary, e.g. tanks or | Note: If you are not |
|  |  |  |  | or bund |  | pipework; secondary, e.g. bund or | able to supply all of |
|  |  |  |  | Detail any incidents of pollution or | Yes/No | hard standing and, where | this information at |
|  |  |  |  | spills from the relevant activity. |  | present, tertiary, e.g. oil | present, you may |
|  |  |  |  | This can be based on visual |  | interceptor | submit the details |
|  |  |  |  | assessment during site walk over |  |  | with your Accident |
|  |  |  |  | or other records and data sources |  |  | Management Plan |
|  |  | Transfer from storage to tanker | Pipework failure Leaking tanker | None | Yes | All equipment is regularly serviced. All equipment used for removal serviced and maintained by third party removing dirty water. | Regular inspection of facilities and equipment |
|  |  | Road transport to field | Tanker failure, road accident | None | Yes | Fully trained operators All tanker work completed by external contractors with their own equipment | Regular inspection of facilities and equipment |
|  |  | Field | Surface run-off, drain | None | Yes | Spreading in accordance with | Regular soil nutrient |
|  |  | spreading | contamination |  |  | Manure Management Plan and | testing |
|  |  |  |  |  |  | advice from qualified person |  |
|  |  |  | Over-application of plant |  |  |  |  |
|  |  |  | nutrients |  |  |  |  |

| 1.Site | 2. Substance | 3. Relevant | 4. Possible failure | 5. History/records or visual | 6. Do pollution | 7. Provide details of pollution | 8.Testing and |
| --- | --- | --- | --- | --- | --- | --- | --- |
| operations |  | activity | mechanism and potential | evidence of leaks of potentially | prevention | prevention measures | inspection of |
| (storage and |  |  | for pollution | polluting substances to land | measures exist |  | pollution |
| use) |  |  |  | associated with the activities | for relevant |  | prevention |
|  |  |  |  | that could result in ongoing | activity? |  | measures |
|  |  |  |  | emissions to land, e.g. cracking |  |  |  |
|  |  |  |  | in hard standing, leaking tank |  | To include: primary, e.g. tanks or | Note: If you are not |
|  |  |  |  | or bund |  | pipework; secondary, e.g. bund or | able to supply all of |
|  |  |  |  | Detail any incidents of pollution or | Yes/No | hard standing and, where | this information at |
|  |  |  |  | spills from the relevant activity. |  | present, tertiary, e.g. oil | present, you may |
|  |  |  |  | This can be based on visual |  | interceptor | submit the details |
|  |  |  |  | assessment during site walk over |  |  | with your Accident |
|  |  |  |  | or other records and data sources |  |  | Management Plan |
| Pesticides | List | Delivery and | Spillage, leaks, overflowing, | None | Yes | Transfer directly from delivery | Deliveries monitored |
| and biocides | substances | transfer from | contamination of clean |  |  | vehicle to dedicated store | Regular inspection |
|  | used | vehicle to | drains |  |  | Damaged or suspect packaging | of facilities and |
|  |  | on-site |  |  |  | rejected at time of delivery | equipment |
|  |  | storage |  |  |  | Dedicated contained store to | Full application |
|  |  | Storage of |  |  |  | current specification | records |
|  |  | pesticides |  |  |  | Records kept | Regular inspection |
|  |  | Mixing of |  |  |  | Dedicated mixing area, | of storage area |
|  |  | pesticides |  |  |  | impermeable base, drains to | Records kept |
|  |  | Application |  |  |  | Dirty water store. |  |
|  |  | foot dip and |  |  |  | Trained staff with appropriate |  |
|  |  | wheel wash |  |  |  | qualifications |  |
|  |  | use |  |  |  | Relevant Codes of Practice |  |
|  |  | Transfer of |  |  |  | followed |  |
|  |  | pesticide |  |  |  | Foot dips on good concrete, |  |
|  |  | and biocide |  |  |  | drains to dirty |  |
|  |  | Disposal of |  |  |  | water system |  |
|  |  | waste |  |  |  | Foot dips located where |  |
|  |  | packaging |  |  |  | overflowing gutters will not dilute foot dips |  |
|  |  |  |  |  |  | Wheel wash constructed from |  |
|  |  |  |  |  |  | reinforced concrete with sealed |  |
|  |  |  |  |  |  | joints |  |
|  |  |  |  |  |  | Dedicated container, |  |
|  |  |  |  |  |  | impermeable hard standing |  |
|  |  |  |  |  |  | within bund |  |
|  |  |  |  |  |  | Removed from site by third party. |  |
|  |  |  |  |  |  | Dedicated storage area |  |
|  |  |  |  |  |  | Removal by licensed contractor. |  |

| 1.Site | 2. Substance | 3. Relevant | 4. Possible failure | 5. History/records or visual | 6. Do pollution | 7. Provide details of pollution | 8.Testing and |
| --- | --- | --- | --- | --- | --- | --- | --- |
| operations |  | activity | mechanism and potential | evidence of leaks of potentially | prevention | prevention measures | inspection of |
| (storage and |  |  | for pollution | polluting substances to land | measures exist |  | pollution |
| use) |  |  |  | associated with the activities | for relevant |  | prevention |
|  |  |  |  | that could result in ongoing | activity? |  | measures |
|  |  |  |  | emissions to land, e.g. cracking |  |  |  |
|  |  |  |  | in hard standing, leaking tank |  | To include primary, e.g. tanks or | Note: If you are not |
|  |  |  |  | or bund |  | pipework; secondary, e.g. bund or | able to supply all of |
|  |  |  |  | Detail any incidents of pollution or | Yes/No | hard standing and, where | this information at |
|  |  |  |  | spills from the relevant activity. |  | present, tertiary, e.g. oil | present, you may |
|  |  |  |  | This can be based on visual |  | interceptor | submit the details |
|  |  |  |  | assessment during site walk over |  |  | with your Accident |
|  |  |  |  | or other records and data sources |  |  | Management Plan |
| Lightly | Ammonia, | Surface | See dirty water | None | Yes | No areas of the yard are | Hard standing |
| Contaminated water. | nitrates, | water |  |  |  | Lightly contaminated | inspected monthly, |

Pest Management Plan

On-site and Pest Management Plan (PMP) assessment (options not mandatory): Template checklist for poultry farms

| Source | Method | On site check | PMP check | Comment |
| --- | --- | --- | --- | --- |
| PMP | Manage site activities in accordance with the PMP |  |  |  |
| Fly monitoring | Follow routine monitoring for flies using: resting counts; adhesive paper fly catches, fly larval counts, other |  |  | Regular checks of building by Farm Partner |
|  | Fly species identified |  |  |  |
|  | Trigger levels followed for the relevant monitoring method/s to initiate insecticidal control |  |  |  |
| Manure management | Daily check of water lines and drinkers for defects and/or spillages |  |  | All checks completed daily by farm staff |
|  | Buildings are watertight, with no water ingress from outside |  |  | All checked daily by farm staff |
|  | Manure holding areas well ventilated |  |  | N/A |
|  | Liquid feed stores are appropriately sealed and external sources and surrounding areas are kept as clean as practically possible. Try to organise vents so that flies cannot pass through these |  |  | N/A |
|  | Slurry removed frequently in accordance to NVZ regulations and FRfW |  |  | N/A |
| Infrastructure | Buildings are in good condition and kept well maintained |  |  | Maintenance Schedule kept |
| Carcases | Fallen stock is removed frequently |  |  | Regular checks by Farm Partner |
|  | Fallen stock bins are covered and locked |  |  | Regular checks by Farm Partner |
| Housekeeping | Spillages are cleaned up as soon as possible |  |  | Regular checks by Farm Partner |
|  | Rubbish bins are emptied regularly |  |  | Regular checks by Farm Partner |

| Insecticide control options | Insecticide labels are complied with and records kept of all treatments |  |  | When required |
| --- | --- | --- | --- | --- |
|  | Fly baits used |  |  | When required |