G & S PHELPS

CHERRY ROCK FARM POULTRY UNIT

BLACKWELLS END

HARTBURY

GLOUCESTER

GL19 3DB

NEWBERRIES FARM POULTRY UNIT

BLACKWELLS END

HARTPURY

GLOUCESTERSHIRE

GL19 3DB

OLD OAK FARM POULTRY UNIT

FORGE END

UPLEADON

NEWENT

GLOUCESTERSHIRE

GL18 1EG

PERMIT NUMBER EPR/NP3239UE

This plan will be reviewed annually or immediately if there is a substantiated complaint.

Potential Odour Sources

The following sources have been identified as contributing to a potential medium - high risk odour source.

Odour emissions from compound feed selection

Odour emissions from feed delivery and storage

Odour emissions from ventilation techniques

Odour emissions from litter conditions and management

Odour emissions from carcass storage and disposal

Odour emissions from fluctuations in bird stocking densities (growth curves)

Odour emissions from drinking water systems

Odour emissions from de-stocking (thinning and final depletion)

Odour emissions from cleanout (litter removal)

Odour emissions from dirty water generation and storage (washout)

Odour emissions from litter/ manure

Odour emissions from carcass storage

Odour emissions from dust build up

Pathways and Receptors

The pathway for all of the above sources would be via the atmosphere, with the most sensitive receptors being inhabitants of nearby residential dwellings the wind direction will significantly influence how receptors are affected.

Receptors

See plan on next page

List of potentially affected properties from airborne odour issues

| Receptor Description | Grid Reference | Distance from permit boundary (metres) | Direction |
| --- | --- | --- | --- |
| Leadon Stables | SO 78260 06660 | 208 | North |
| Cherry Rock Farm | SO 78260 28420 | 60 | East |
| Cox’s Farm | SO 78260 37130 | 200 | West |
| Prestberries Farm | SO 78260 50190 | 300 | West |
| Corse Court Farm | SO 78260 79450 | 360 | North |
| Newberries Farm | SO 78260 57120 | 286 | West |
| 1 Prestbury Villa | SO 78250 46820 | 400 | West |
| 2 Prestbury Villa | SO 78250 46830 | 400 | West |

Person Responsible

It is the responsibility of the duty manager to ensure that this plan is adhered to and followed at all times.

It is also the responsibility of the manager to investigate any and all complaints diligently, to take any remedial action and to review this plan. The results must be reported both to the Partnership and to the complainant.

A complaint form is attached at the end of this plan.

## Introduction

This plan has been prepared as part of the EPR permit variation as there are sensitive receptors within 400 metres of the installation. The following table sets out:-

- the likely sources of odour arising from the poultry unit;

- the procedures planned at Cherry Rock Farm in order to prevent or minimise odour levels.

Typical Odour Sources and Actions Taken to Minimise Odours

| Odour Related Issue | Potential sources of Risks and Problems | Actions taken to minimise odour and odour risks at Cherry Rock Farm, Old Oak Farm & Newberries Farm | Completion date |
| --- | --- | --- | --- |
| Manufacture and selection of feed | - Milling and mixing of compound feeds. - The use of poor quality and odorous ingredients. - Feeds which are ‘unbalanced’ in nutrients, leading to increased excretion and litter moisture and emissions of ammonia and other odorous compounds to air. | - Reduce protein content of feed in accordance with H2C - No milling or mixing will take place on site - Feed specifications are prepared by the feed compounder’s nutrition specialist - Feed is supplied only from UKASTA accredited feed mills, so that only approved raw materials are used - Avoid fine grinding of feed - Follow good housekeeping and clean up all spills - Feed deliveries monitored to avoid dust or spills - Enclosed handling, storage and on-site transport - When moving feed, avoid drops and open chutes - Relocation of any odorous activities, storage or extraction points from buildings - Avoidance of and immediate treatment of any pests e.g. mealworms | In place |
| Feed delivery and storage | - Spillage of feed during delivery and storage. - Creation of dust during feed delivery. | - Feed delivery systems are sealed to minimise atmospheric dust. - Any spillage of feed around the bin is immediately swept up. - The condition of feed bins is checked every 7 weeks so that any damage or leaks can be identified and repaired. The manager will ensure that any faulty bin is repaired ASAP - Feed deliveries are monitored to avoid dust and spills. | In place |
| Ventilation | - Inadequate air movement in the house, | - The ventilation system is regularly adjusted according to the age and requirements of the flock - The ventilation system is designed to efficiently remove moisture from the house. - Twice daily checks of all houses are made. - Minimal ventilation will be used during destocking consistant with bird welfare. | In place |
| Litter management | - Odours arising from wet litter (see above). - The use of insufficient or poor quality litter. - pillage of water from drinking systems. - Disease outbreaks, leading to wet litter. | - Controls on feed and ventilation (see above) help to maintain litter quality. Additional controls include:- - Use of nipple drinkers to minimise spillage. - Daily checks of drinkers are undertaken to avoid capping. - Insulated walls and ceilings to prevent condensation. - Concrete floors to prevent water ingress. - Stocking density at optimal levels to prevent overcrowding. - Addition of more clean, dry bedding material, inspection carried out daily. Any signs of capping or greasiness will result in action being taken - Use of a health plan, with specialist veterinary input used as necessary. | In place |
| Carcass disposal | - Inadequate storage of carcasses on site. | - Weekly collections - Storage containers kept cool / shaded - Storage containers kept covered and locked - Leaks prevented from storage containers, daily inspection of area. Should a leak be detected then the bin would be emptied and washed out immediately. It should then be replaced ASAP - Storage location away from sensitive receptors - Use of odour suppressant near container - Collection made under the National Fallen Stock Scheme - There is no stagnant water as site has sloping concrete surfaces, and if any occurred it would be washed away immediately | In place |
| House Clean Out | - Creation of dust associated with litter removal from houses. - Use of odorous products to clean houses. | - Clean out contained to avoid odours - Buildings sealed during and/or after cleanout - Separate collection system for wash water - Litter is carefully placed into trailers positioned at the entrance to each house. When full, the trailer is covered and leaves site ASAP - Litter is removed as soon as possible after destocking without causing excessive odour (2 days max to remove all litter per site). - Whole site wash down completed in 6 days per site - Minimal ventilation will be used. - Only Defra approved detergents and disinfectants are used - Wash water tanks hold one complete wash down (Old Oak Farm) - See attached protocol for wash down procedures at Cherry Rock and Newberries farms, there have been no incidents at either property. - Wash water tanks are emptied to avoid overflowing as soon as practical after wash down complete. - Clean out takes place as soon as possible after destocking. | In place |
| Used litter | - Storage of used litter on site. - Transport of litter and applications to land. | - There is no storage of used litter on site at any time. - Litter is transported in covered trailers. - There is no double handling - All litter land-spread is under the control of a separate farming business and subject to a written agreement and will be spread in line with the Code of Good Agricultural Practise | In place |
| Dirty water management | - ‘Standing’ dirty water during the production cycle or at clean out. - Applications of dirty water to land. | - Some areas around the house are concreted, others are grassed over and remain clean during the production cycle. - At clean-out, dirty water is directed to underground tanks for storage. - The dirty water tank is inspected at least twice per day during wash down to ensure that it does not overflow. - Dirty water spread to land owned by the same business will be spread in line with the Code of Good Agricultural Practise. The business has a litter Management Plan - Dirty water drains are inspected annually by the duty manager. Any fault is immediately repaired | In place |
| Complaints | - Odour - Noise - Other | - The duty manager is responsible for taking details of the complaint as per the Complaint Form - The complaint will be investigated immediately - Remedial action will be taken immediately - This plan will be reviewed - The findings of the investigation, the action taken and any changes to this plan will be reported to both the complainant and to the partnership |  |
|  | - Remedial/contingency measures | - Identify cause - If due to damp/wet litter add more fresh dry litter and/or/both increase ventilation rate depending on the severity. - If due to carcass storage issues have bins emptied ASAP - Use odour suppressants if appropriate |  |

Old Oak and Newberries Farms use high velocity ridge fans for all minimum ventilation purposes which will achieve the greatest dispersion of any air/odour that is vented out of the sheds, these fans are located in the ridge to achieve this objective. Cherry Rock uses low velocity side wall extraction fans for minimum ventilation; the low velocity discourages the air from moving far from the sheds. In all the sheds extra summer ventilation is by low velocity end of house fans, because they are low velocity then the air does not travel very far. At Old Oak and Newberries farms air is drawn in using side vents, due to the design of the roof space and the positioning of the fans this creates a circular movement of air within the shed helping to keep the litter dry and prevent wet litter and hence any odour issues. At Cherry Rock air is drawn in through the ridge, again through the design of these air inlets and the positioning of the fans a circular movement of air is created within the shed helping to keep the litter dry and prevent wet litter and hence any odour issues

There have been no odour complaints at any of the sites. The greatest risk of odour creation and therefore the risk of an odour complaint is in the latter stages of the crop. Therefore, monitoring will take place weekly up until day 25 when it will become daily. Monitoring will take place around the permit boundary for all sites, checking for high levels of odour. Visual and nasal inspections will take place. It will be done first job in the morning before the staff enter the sheds and become desensitized. The result of this walk is to be recorded (see below) and results available for inspection. If there is a verifiable complaint, then the frequency of the monitoring will increase, and remedial action will take place.

As part of the ‘good housekeeping’ dust from the end of house fans is inspected daily and regularly swept up during the summer period when these fans may be in use. These fans are only used when the outside ambient temperature approaches the same temperature as that within the sheds so they might only be used for short periods each day.

The assessment of the moisture content within the litter is undertaken using basic techniques such as observation. For example, when the operator squeezes a handful of litter, if should stay together for a few seconds and then fall through the fingers. If the litter falls through too quickly the litter is too dry, if it clumps then it is too wet.

During catching / destocking catching curtains are used where possible and a minimum ventilation rate consistent with bird welfare. At times of high ambient temperature or when high humidity poses a threat to the birds, catching, loading and transportation create particular risks of heat stress. Destocking is undertaken at night to assist this process. The destocking process is designed to take the minimum time possible which is consistent with ensuring bird welfare and will typically take 2 hours per load, in between loads the doors are closed.

The site operators are committed to maintaining site cleanliness. Any spillages are dealt with promptly and correctly. All wash water is adequately contained; Terminal hygiene plan is followed at all times; Suitable chemical products will be selected, and the correct dilution rates are adhered to; Limit washing operations at weekends and bank holidays where possible. Washing operations not to take place during inappropriate weather conditions Dirty Water is directed to sealed underground tank for storage. It is then spread onto land as weather conditions permit.

All staff have training in Environmental Management which includes site and house maintenance, this includes the management of the litter covering inspection and remedial action.

Birds will not stay on the farm longer than has been agreed unless it is beyond the control of the company e.g. Avian Influenza.

All three sites have emergency generators which start automatically in the event of a power outage. The alarm system warns the duty manager of this event. The site will have 2 members of staff with one always on call. This ensures that the house is always ventilated preventing the buildup of odour.

The sheds are continuously monitored for humidity and temperature, the highs and lows are recorded each day, this monitoring also causes the computer system to increase ventilation and or more heating helping to prevent odour issues.

In the event of any complaints being received, the operator will carry out a full inspection as per the complaint and also notify the Environment Agency.

A copy of the complaint form has previously been inspected by the local Environment Officer.

Contingency Measures

The table below identifies what happens if the day-to-day management of the business has failed to control odour. This occurs when odour complaints have been substantiated or the daily odour monitoring carried out by the staff has identified odour.

| Odour Contingency |  |  |  |
| --- | --- | --- | --- |
| Source | Potential Cause | Primary Mitigation Measure | Secondary Mitigation Measure |
| Feed Delivery and Storage | Failure of pipe coupling to feed silo creating a leak or spillage | Trigger point Unable to clear spill quickly and at risk of rain soaking and /or contaminating watercourse. Mitigation Farm workers to build a temporary bund around the spill with straw or soil Farm manager to contact specialist removal contractor immediately Cessation of Action No spillage remains and any waste generated by the spill has been removed from site by specialist contractor. | Trigger point Implemented primary mitigation measures and spill remains and unable to clear up, specialist removal firm cannot come on site for over 12 hours. Mitigation Farm workers spread soil or straw over the top of the spill to reduce odours. Bagged up feed to be deposited into covered skips. Cessation of Action No spillage remains and any waste generated by the spill has been removed from site by specialist contractor. . |
| Feed Delivery and Storage | Failure feed bins | Trigger Point Spillage occurs Mitigation Any spillages with be cleared up immediately by farm staff and if fit for consumption will be placed in spare plastic feed bags and fed back to the pigs. Any spillages unfit for consumption will be cleared into skips immediately and removed from site by licensed waste contractors within 24hrs Spillages over 500 kg – farm staff will notify the feed delivery company immediately who will be required to send a vehicle out to clear and remove all split feeds within 8 hours. Farm partner to order replacement feed bin immediately. Cessation of Action No spillage remains and any waste generated by the spill has been removed from site by specialist contractor. Feed bin has been replaced and in working order. . | Trigger point Unable to clear spill quickly and at risk of rain soaking and /or contaminating watercourse. Replacement feed bin not currently on site to allow normal feeding protocol to continue. Mitigation Farm workers build a temporary bund around the spill with straw and cover spill with straw/soil. Farm manager to contact specialist removal contractor immediately Farm partner to reduce normal feed order and order more often to enable the farm to operate with one bin less. Cessation of Action No spillage remains and any waste generated by the spill has been removed from site by specialist contractor. Feed bin on site and installed by qualified contractors. . |
| Carcase storage and disposal | Carcass bins leaking | Trigger Point Damaged or worn bins Mitigation Once identified, farm partner to immediately order new bins or repair if this is practical. Do not use the damaged bin Cessation of Action New sealed bins delivered within 48 hours | Trigger Point Damaged or worn bins cannot be replaced within 48 hours. Mitigation Farm partner to contact National Fallen Stock to arrange for immediate collection of deadstock. Deadstock collection to continue Cessation of Action New sealed bins delivered and in use. |
| Dirty Water Handling | Damaged or blocked channels or pipes | Trigger Point Blockage and overspill or difficulty experienced when sucking out dirty water. Mitigation Cease removal immediately until blockage can be cleared. Immediately capture any dirty water escape with a straw/soil bund. Call qualified contractor to repair it immediately Cessation of Action Dirty water system repaired and working within 24 hours | Trigger Point Blockage and overspill or difficulty experienced when sucking out dirty water that has not been rectified within 24 hours. Mitigation Contact a local contractor to remove as soon as possible. Cessation of Action Dirty water system repaired and working. |
| Dirty water Handling | Unable to export dirty water due to unsuitable spreading conditions | Trigger point Unsuitable spreading conditions due to weather. Mitigation Operator to confirm to farm partner and alternative location found to spread dirty water. Cessation of Action Dirty water removed from site. | Trigger point No suitable alternative land found. Mitigation Export dirty water to other farmers who have spare capacity within stores. No formal agreements are currently in place. If this is not possible then employ road tankers to export to suitable land or place in store Cessation of action Dirty water removed from site. |
| Litter Handling | Unable to export litter after destocking. | Trigger Point Third party unable to remove litter after destocking has been undertaken. Mitigation Source an alternative contractor to remove litter from houses. Cessation of Action All litter removed from house. House has been washed and disinfected. | Trigger Point Alternative contractors cannot be sourced to remove litter. Mitigation Source equipment (loader and trailers). Farm staff to undertake removal of litter. Farm partner to source a suitable location for litter to be exported to. Cessation of Action All litter removed from house. House has been washed and disinfected. |
| Deadstock Handling | Significant disease situation leading to excess of deadstock. | Trigger Point Inspection has identified a serious disease situation with high mortality. Mitigation Techniques & Speed of Response If the quantity of deadstock is beyond the available bin storage capacity, then an area in an alternative area is used to store deadstock. Deadstock are covered and left in the shed awaiting fallen stock by approved contractor as soon as possible. Farm partner to contact licenced fallen stock scheme and request immediate removal and additional deadstock containers ordered immediately. Collection by fallen stock scheme is arranged and completed within 24 hours and then ongoing at daily intervals if required (and permitted) by disease situation. Cessation of Action Stock health is returned to normal status source of disease has been identified and eradicated | Trigger Point Disease situation is not responding to treatment and mortalities are increasing significantly over 48-hour period. Emergency Measure & Speed of Response Situation would invariably require immediate involvement of veterinarian. Arrange for removal of stock from house for slaughter (subject to veterinary guidance). Action is within 72 hours Do not re-use house until fully cleaned, disinfected and microbiologically tested Cessation of Action House is cleared for re-stocking and source of disease has been identified and eradicated. |
| Housing and ventilation | Failure of water pipe or drinker creating spillage of water and leading to poor quality wet and odorous bedding and feed in building. | Trigger Point Inspection has identified a water leak inside the house. Mitigation Undertake the following actions in proportion to the size and duration of the leak. Immediately isolate the pipe or drinker to prevent any further leakage of water. Immediately segregate the area of spillage from the stock with barriers. Depending on extent of leak, farm partner either calls engineer to repair immediately or instructs farm staff to repair immediately. Cessation of Action Drinker system is fully repaired, within 24 hrs. | Trigger Point Water leak cannot be stopped or is excessive and not been repaired within 24 hours. Mitigation Arrange for removal of stock from the affected part of the house and transfer to alternative house, farm or slaughter. Shut off supply to house and introduce temporary manual drinking troughs if welfare needs dictate. Cessation of Action Drinker system is repaired and functioning. |
| Housing and ventilation | Overstocking in house due to non-collection leading to greater numbers | Trigger Point Stocking density has exceeded guidelines due to operational failing in movement programme. Mitigation Site operator to give consent to move excess stock to other sites owned by the operator immediately. Cessation of Action Stock numbers and stocking density are back down below industry guidelines. | Trigger Point As for primary mitigation but exporting to other sites is not available. Mitigation Sell stock to other farmers. Cessation of Action Stock numbers and stocking density are back down below industry guidelines. |
| Housing and ventilation | Abnormal climatic conditions impacting on sensitive receptors | Trigger Point Weather condition (e.g. unusual wind direction, very still air, heavy humidity). Mitigation Techniques & Speed of Response Implement additional perimeter / sniff testing to monitor for potential problems. Adjust fan velocities at selected points to address specific conditions in relation to sensitive receptors. Manual over-ride in response to conditions. Take action as above if forecasts indicate unusually long periods of very abnormal weather (e.g. > 2 days). Cessation of Action Prevailing / normal weather patterns return | No additional emergency actions are identified in this instance. Weather conditions and forecasts are monitored. If extended period of abnormal weather is anticipated contact should be made with potentially impacted receptor sites. |
| Housing and ventilation | Failure of the Ventilation System | Trigger Point Alarm and/or inspection has identified a major ventilation system failure which is not related to power supply (i.e. not resolved by back-up power generation) Mitigation Maximise natural air-flow through house by manually opening doors, side vents and fan baffles – subject to age and condition of stock. Cessation of Action Ventilation system is restored | Trigger Point Unable to restore ventilation to house Mitigation Arrange for removal of stock from house and transfer to alternative house, farm or slaughter. Action is within 24 hours (stock health as well as odour factors are paramount) Cessation of Action House is empty, litter removed and cleaning completed. Ventilation system is repaired and functioning. |
| Housing and ventilation | Unable to complete wash out and disinfectant of sheds due to washer breakdown. | Trigger Point Pressure washers fail Mitigation Contractors to source alternative pressure washers Cessation of action Pressure washer has been and repaired the pressure washer within 48hrs. | Trigger Point Pressure washer fails and repairs contractor cannot repair within 48hours Mitigation Farm partner to contact alternative washing contractors to complete shed washing and disinfecting. Cessation of action All shed washing completed. |
| Feed Quality | Poor quality / condition of feed at delivery (e.g. excessive fines/dust or damp) creating blockages that could result in feed spoiling and creating odour from silos or feeders. | Trigger Point Blocked feed system is identified during or after delivery Mitigation Techniques & Speed of Response Immediately isolate the system and notify feed mill to make immediate collection and replacement from silo if appropriate. Temporarily use bagged feed transferred from another silo if this is needed for stock welfare. Immediately dismantle and clear any part of the feed system at risk of block. Bag any feed to be disposed of in sealed plastic bags. Cessation of Action Satisfactory feed delivered into cleared system and confirmed by visual inspection of bulk and sample. | Trigger Point Unable to clear blocked silo/system Emergency Measure & Speed of Response Introduce a temporary mobile silo (trailer) to replace blocked system. Act within 2 days if problem has persisted and bagged feed is being used. Cessation of Action Permanent feed system is back available for use. |

| Odour Report Form for Cherry Rock Limited |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Location: |  |  |  |  |  |  |  |
| Date : |  |  |  |  |  |  |  |
| Time of test |  |  |  |  |  |  |  |
| Location of test |  |  |  |  |  |  |  |
| Weather conditions (dry, rain, fog, snow etc.) |  |  |  |  |  |  |  |
| Temperature (very warm, warm, mild, cold or degrees if known) |  |  |  |  |  |  |  |
| Wind strength (none, light, steady, strong, gusting) Wind direction |  |  |  |  |  |  |  |
| Intensity (see below) |  |  |  |  |  |  |  |
| Duration (of test) |  |  |  |  |  |  |  |
| Constant or intermittent in this period or persistence |  |  |  |  |  |  |  |
| What does it smell like? |  |  |  |  |  |  |  |
| Receptor sensitivity (see below) |  |  |  |  |  |  |  |
| Is the source evident? |  |  |  |  |  |  |  |
| Any other comments or observations |  |  |  |  |  |  |  |