Odour Management Plan

The following plan has been prepared as part of the EPR permit application.

The following tables highlight the likely sources of odour arising from poultry broiler production at Stack Field Farm

Actions and measures are listed that will prevent where possible or minimise odour emissions at Stack Field Farm

Site plan shows all material storage areas and potential odour emission sources.

Plan to be reviewed every year from permit issue date, prior to any major changes to operations ( to ensure effectiveness ) or following any complaint, any changes to OMP or other management plans to be documented dated and signed and Area Officer notified.

Actions and preventative measures in OMP referenced from Odour Assessment Document and Fugitive Emissions Assessment in line with the H1 Risk Assessment, to be implemented in conjunction with the following key documents;

Emergency Plan

Technical Standards

Routine Maintenance Schedule

Key responsibility for the OMP and the referenced plans are the Operator or deputies who have been briefed on the requirements.

Example Odour Complaint form attached.

The table below lists all sensitive receptors with 400m.

| Receptor Name | Description | Distance | Orientation | National Grid Reference |
| --- | --- | --- | --- | --- |
| Nursery | Nursery | 267m | South South-west | 466765,335982 |
| Garage | Commercial | 280m | South South-west | 466740,335983 |
| Residence | Residential | 270m | South South-west | 466781,335958 |

| Odour Related Issue | Potential Risks and Problems | Actions taken to minimise odour and odour risks at Stack Field Farm | Completion date |
| --- | --- | --- | --- |
| Manufacture and selection of feed | Milling and mixing of compound feeds. The use of poor quality and odorous ingredients. Feeds which are ‘unbalanced’ in nutrients, leading to increased excretion and litter moisture and emissions of ammonia and other odorous compounds to air. | No on-site milling and mixing. Feed specifications are prepared by the feed compounder’s nutrition specialist. Feed is supplied only from UKASTA accredited feed mills, so that only approved raw materials are used. Protein is reduced in accordance with SGN EPR6.09 ‘How to comply with your environmental permit for intensive farming’ ‘How to comply with your environmental permit for intensive farming’. | In place |
| Broiler production | Odour levels and monitoring | Daily olfactory checks with any abnormalities recorded and investigated, this will entail recording any detection of odour and severity, this will increase in frequency during the final two weeks of crop cycle (Twice daily) and during littering out. Daily olfactory checks of any abnormal housekeeping odours. Wind sock on site for prevailing wind direction detection | In Place In place |
| Feed delivery and storage | Spillage of feed during delivery and storage. Creation of dust during feed delivery. | Feed delivery systems are sealed to minimise atmospheric dust. Any spillage of feed around the bin is immediately swept up. The condition of feed bins is checked frequently so that any damage or leaks can be identified. Feed deliveries are monitored to avoid dust and spills. | In place |
| Ventilation and heating Systems/Dust | Inadequate air movement in the house, leading to high humidity and wet litter Inadequate system design, causing poor dispersal of odours. Extraction fans located close to sensitive receptors. Dust | Use of roof extraction fans to aid dispersion, checked prior to cycle commencement by qualified electrician who will provide 24hr breakdown cover The ventilation and heating system is regularly adjusted to match the age and requirements of the flock. The ventilation system is designed to efficiently remove moisture from the house. Indirect heating system giving lower humidity levels. Humidity recorded daily and maintained in the range of 55 – 65% keeping a balance of dry litter and avoiding dust production. Stock inspections carried out by trained staff to avoid panicking birds creating dust. | In place |
| Litter management | Odours arising from wet litter (see above). | Controls on feed and ventilation (see above) help to maintain litter quality. Additional controls include:- Use of nipple drinkers with drip cups to minimise spillage. Daily checks of drinker height and pressures to avoid capping. Insulated walls and ceilings to prevent condensation. Concrete floors to prevent ingress of water. Stocking levels at optimum to prevent overcrowding. Use of veterinarian bespoke health plan. | In place |
| Carcase disposal | Inadequate storage of carcasses on site | Carcasses placed into plastic sealed bags, stored in sealed, shaded and vermin proof containers away from sensitive receptors. Frequent (3/5 times per week) Collection of carcasses. | In place |
| House clean out | Creation of dust associated with litter removal from houses Use of odorous products during cleaning. | Litter carefully placed into trailers positioned close to doors. Trailers sheeted before leaving fill position. Only DEFRA approved and suitable products used. Chemical containers triple washed at point of use. Wash water tank levels monitored during washing and emptied as required to prevent overfill. Litter out carried out within 24 hours following destocking per house (72 hours total for site) | In place In place |
| Used litter | Storage of used litter on site. Transport of litter and land spreading. | No storage on site at any time. All trailers sheeted before leaving fill position. Avoidance of double handling. Litter removed from site and sold to third parties. | In place |
| Washing operations including vehicles | Loss of dirty water to land or watercourse | Use of specialist contractors for washing operations. Bespoke terminal hygiene program followed, detailing quantities of water and chemical dilution rates. Manager responsible for diverter valve operation. No washing operations actioned unless key staff on site Key staff monitoring washing operations ensuring effective drainage to dirty water tanks. Dirty water tanks monitored during wash down to maintain freeboard. Washing operations completed within three days. Vehicle washing at designated wash point. All sediment traps and drains cleaned both before and after washing operations | In place |
| Fugitive emissions | Leaks to doors, bin pipes, feed bins, fuel and chemical storage | Checks to feed storage and fill pipes as per routine maintenance schedule. Fuel oil in approved bunded storage tank. | In place |
| Dirty water management | Standing dirty water during the production cycle or at clean out. Application of dirty water to land. | Working areas around houses are concreted and kept clean during production cycle. At clean out dirty water from houses together with lightly contaminated yard wash is directed to the underground storage tanks, before being removed off site by a third party with any surplus litter. | In place |
| Abnormal operations | Water leak/pipe failure Bird health/sickness | Water consumption monitored daily ensuring early detection, wet area blanket covered with top up bedding material to prevent increased odour. Veterinarian contacted (24hour cover) Litter covered with fresh top up bedding to minimise increase in odour until bird health recovered. Abnormal events documented, dated and signed, appropriate plans reviewed and updated to prevent reoccurrence ie. Routine maintenance schedule, Technical standards | In place |
| Waste production/storage | Odour from production or storage areas | No storage or production of odorous waste on site. Waste management plan in force detailing types and quantities produced along with disposal routes. Records kept on site. | In place |
| Materials/storage | Potential odour source | Feed delivered into sealed vermin proof silos. Sealed delivery system into poultry houses with no milling or mixing on site. Remaining feed at end of cycle stored in sealed silo and used on subsequent cycle. 3 month shelf life of feed negating the need for removal. Chemicals in secure bunded shed free from frost and unauthorised entry together with any veterinarian products/medicine | In place |

Monitoring procedure

No formal monitoring currently undertaken, If substantiated complaints are founded then monitoring to take place on site to identify source of issue immediate action to be taken to minimise / rectify Source.

Complaints Procedure

In the event of odour complaints being received, as well as notifying Environment Agency Area Officer and recording the complaint, operator will visit area from where complaint has been received from.

In the event of an odour complaint being received the following steps will be followed with the information being recorded on the Complaint recording form, this will be the responsibility of Operator/Site Manager. Farm sign will be displayed at site entrance or nearest point of public access with 24 hour contact details.

1.	Name and address of complainant along with contact details

2.	Nature of the complaint

3.	Time and date of occurrence

4.	Weather conditions at that time (wind direction, temperature and humidity)

5.	Operational Data (eg. During production cycle, age of birds, litter conditions, de-littering, wash-down, disinfection)

6.	Actions taken following investigation

7.	All complaints will be logged in Complaints Record file held on site for inspection.

| # Odour Contingency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| # Source | # Potential Cause | # Trigger Factor with timed action | # Mitigation Measures to be implemented and remain operative until cessation trigger verified | # Additional Mitigation | # Cessation Trigger |
| # Feed delivery and storage | # Pipe or bin failure causing leak | # Daily inspection # 2hrs | # Repair to pipe work or feed bin with immediate effect, use other bins, spills cleaned up immediately. Integrity of pipe work and bin checking frequency reviewed and updated in routine maintenance and inspection document, with changes recorded and dated. | # Bin/pipework replaced | # Visual inspection |
| # Carcase storage and disposal | # Container failure/damage | # Daily Inspection # 1hr | # Carcases removed from damaged container into additional container, damaged container replaced/repaired immediately. | # N/A # Alternative collection implemented | # Visual Inspection |
| # Variations in stocking density/bird growth | # Rapid bird growth or poor growth due to illness. | # Deviation in predicted growth # 24hrs | # Bird growth monitored Daily # Ventilation and heating controls advanced to account for additional live-weight within house. Veterinarian advice sought immediately for bird illness with additional bedding added to prevent/minimise odour release. Document and record abnormalities. Ensure stocking density complies with BAT standards and bird permit places. | # Immediate veterinarian advice sought | # Growth rates normal |
| # Ventilation System | # Fan/system failure | # System fully alarmed # 1hr | # Alternative ventilation fan used as houses have spare capacity installed, electrician call out | # N/A | # Repairs effected and documented |
| # Drinker systems | # Leaky systems/pipe failure | # Deviation in expected water consumption # 2hrs | # Any leaks isolated and repaired immediately. Wet areas covered with additional bedding to minimise odour. Arrange system integrity testing at cycle end, findings to be documented and recorded, pipe work/system parts to be replaced as per report. | # N/A | # Normal consumption |
| # Bird depletion | # Fugitive odour release | # OMP monitoring # 2hrs | # Minimum ventilation rate to prevent fugitive release of odour, review OMP with any changes documented and recorded and submitted to Environment Agency Area Officer for approval. | # N/A | # OMP monitoring recording reduced low levels |
| # Litter Removal | # Fugitive odour release from poultry houses | # Raised odour levels during OMP monitoring # 2hrs | # Minimum ventilation rate to prevent fugitive release of odour, review OMP with any changes documented and recorded and submitted to Environment Agency Area Officer for approval. | # Review of littering out procedures | # OMP monitoring levels returned to normal |
| # Washing operations/dirty water | # Odour release from drainage/storage # Delay in dirty water removal # Blocked drains | # Raised odour levels during OMP monitoring # 2hrs # Washing procedure monitoring # 4hrs # 1hr | # Arrange drainage integrity testing and drain cleaning, record and document findings. Dirty water tanks filled with clean water and agitated prior to removal to remove any possible sediment/stagnation. # Washing operations suspended, agreement with neighbouring farms for dirty water removal # Blockage cleared | # Ventilation rates decreased # Licensed waste disposal contractor used preventing extended storage requirement # Specialist drainage contractor called out | # OMP monitoring levels normal # Normal washing resumed after visual inspection |
| # Litter/manure | # Wet litter | # Raised odour levels during OMP monitoring # 2hrs | # Additional bedding applied to maintain dry friable litter. # Initiate olfactory checks to ensure effectiveness. | # Additional ventilation implemented to dry litter | # OMP monitoring levels normal |

# Odour Complaint Form

| Installation Name |  | Date Recorded |  |  | Reference Number |
| --- | --- | --- | --- | --- | --- |
| Name and Address of caller: |  |  |  |  |  |
| Tel. No. of caller |  |  |  |  |  |
| Location of caller in relation to Installation |  |  |  |  |  |
| Time and Date of complaint |  |  |  |  |  |
| Date, Time and duration of Offending odour |  |  |  |  |  |
| Callers description of odour |  |  |  |  |  |
| Has the caller any other Comments about the odour? |  |  |  |  |  |
| Weather conditions |  |  |  |  |  |
| Wind strength and direction |  |  |  |  |  |
| Any previous complaints Relating to this odour? |  |  |  |  |  |
| Any other relevant information |  |  |  |  |  |
| Potential odour sources that could give rise to the complaint |  |  |  |  |  |
| Operating conditions at the time offending odour occurred |  |  |  |  |  |
| Follow up Date and time caller contacted |  |  |  |  |  |
| Action taken |  |  |  |  |  |
| Amendment requirement to Odour Management Plan |  |  |  |  |  |
| Form completed by |  |  | Signed |  |  |

Version 3 April 2026