Site Condition Report

| 1.0 SITE DETAILS |  |
| --- | --- |
| Applicants Name | High Slade Properties Ltd |
| Activity Address | Stack Field Farm Fosse Way Cropwell Bishop Nottinghamshire NG12 2JU |
| National Grid Reference | 466991,336295 |
| Document Reference for Site Condition Report at permit application and surrender | 8th August 2017 |
|  | Location Plan |
|  | Layout/Drainage Plan |

| 2.0 Condition of the Land at Permit issue |  |
| --- | --- |
| Environmental setting including: Geology Hydrogeology Surface Waters | The poultry unit lies on level ground to the West of Cropwell Bishop, Predominant land use is arable farming. Field pattern is semi large scale with a strong pattern of field boundaries. There are sensitive receptors within 400m of site. Topography and Drainage The poultry houses are 40 metres above sea level. Existing mature hedges and some woodland help to minimise the visual intrusion normally associated with poultry units. Site clean water drainage all goes to Attenuation pond. Geology and Hydrogeology According to the BGS Extract Map shows the site geology to be of Triassic period of Rock Type – Branscombe Mudstone Formation. Searches indicate the site is underlain with a Bedrock Aquifer of Secondary B. The Site is underlain with major aquifer low, situated in a Groundwater Vulnerability Zone. The site is situated within a Surface water Nitrate Vulnerable Zone. The site is not situated in a Groundwater/Source protection zone. Hydrology General drainage is in a Westerly direction. Average rainfall for this area is 1100 mm. There are no nearby watercourses listed. |
| Pollution history including: Pollution incidents that may have affected land Historical land uses and associated contaminants Any visual/olfactory evidence of existing contamination Evidence of damage to pollution prevention measures | None noted Polluting substances – None noted Previous use prior to 2017 – Arable Farming. None noted None noted |
| Evidence of historical contamination | N/A |
| 3.0 Permitted Activities | The site will consist of Four poultry houses for Broiler chicken. This will give a total of 220,000 places. Biomass boiler using biomass fuel provides Hot water heaters with back up LPG heaters for heating the poultry houses. The working area where vehicles operate is laid to concrete. Feed is delivered in covered lorries and stored on site in vermin proof steel galvanised bins. Immediately following depopulation, litter is removed from sheds and sold. The houses are then washed and disinfected prior to the cycle beginning again. Underground storage tanks will have been installed to catch all wash waters. Dead birds are removed from the houses and stored in sealed containers awaiting collection from a licensed renderer Diesel fuel storage is in a bunded tank/storage area. |
| Non permitted activities undertaken | N/A |
| Document References | Location Plan Layout/Drainage Plan H1 Assessment |
| 4.0 Changes to Activity | Operator is proposing to add an additional 4 poultry houses and increase broiler places to 360,000. Additional ground to be included in the installation boundary. |
| 5.0 Measures taken to protect Land | Site will be operated in compliance with “how to comply” routine maintenance schedules are followed and recorded and with any abnormal operations recorded. |
| 6.0 Pollution Incidents | N/A |
| 7.0 Soil, gas and water quality monitoring | No monitoring will be undertaken at the installation. |