Site Condition Report

| 1.0 SITE DETAILS |  |
| --- | --- |
| Applicants Name | IHP Ltd |
| Activity Address | Firs Field Farm Poultry Unit Bungay Road Hempnall Norfolk NR15 2NG |
| National Grid Reference | 625515,293930 |
| Document Reference for Site Condition Report at permit application and surrender | 07/01/2026 |
|  | Location Plan |
|  | Layout/Drainage Plan |

| 2.0 Condition of the Land at Permit issue |  |
| --- | --- |
| Environmental setting including: Geology Hydrogeology Surface Waters | The poultry unit lies on level ground to the East of Hempnall, Predominant land use is arable farming. Field pattern is semi large scale with a strong pattern of field boundaries. There are sensitive receptors within 400m of site. Topography and Drainage The poultry houses are 35-40 metres above sea level. Existing mature hedges and some woodland help to minimise the visual intrusion normally associated with poultry units. Site clean water drainage all goes to off-site ditch. Geology and Hydrogeology According to the BGS Extract Map shows the site geology to be of Quaternary period of Rock Type – Norwich Crag Formation. Searches indicate the site is underlain with a Principal Bedrock Aquifer, and a Superficial Aquifer – Secondary (undifferentiated). The Site is situated in a Groundwater Vulnerability Zone of medium classification. The site is situated within a Surface water Nitrate Vulnerable Zone. The site is not situated in a Groundwater/Source protection zone. No drinking water protection or safeguard zones. Hydrology General drainage is in a Southerly direction. Average rainfall for this area is 1100 mm. |
| Pollution history including: Pollution incidents that may have affected land Historical land uses and associated contaminants Any visual/olfactory evidence of existing contamination Evidence of damage to pollution prevention measures | None noted Polluting substances – None noted Previous use prior to 2025 – Arable Farming. None noted None noted |
| Evidence of historical contamination | N/A |
| 3.0 Permitted Activities | The site will consist of Three poultry houses for Broiler chicken. This will give a total of 250,000 places. Direct LPG heaters for heating the poultry houses. The working area where vehicles operate is laid to concrete. Feed is delivered in covered lorries and stored on site in vermin proof steel galvanised bins. Immediately following depopulation, litter is removed from sheds and sold. The houses are then washed and disinfected prior to the cycle beginning again. Underground storage tanks will have been installed to catch all wash waters. Dead birds are removed from the houses and stored in sealed containers awaiting collection from a licensed renderer Diesel fuel storage is in a bunded tank/storage area. |
| Non permitted activities undertaken | N/A |
| Document References | Location Plan Layout/Drainage Plan H1 Assessment |
| 4.0 Changes to Activity | Existing permit for Duck rearing, changing to Broilers. |
| 5.0 Measures taken to protect Land | Site will be operated in compliance with “how to comply” routine maintenance schedules are followed and recorded and with any abnormal operations recorded. |
| 6.0 Pollution Incidents | N/A |
| 7.0 Soil, gas and water quality monitoring | No monitoring will be undertaken at the installation. |