- Purpose of this Statement

This BAT Applicability and Screening Statement has been prepared to demonstrate how the Best Available Techniques (BAT) conclusions relevant to the installation have been identified, screened for applicability, and implemented in accordance with:

- Commission Implementing Decision (EU) 2019/2031
BAT conclusions for the Food, Drink and Milk industries

- The Environmental Permitting (England and Wales) Regulations 2016

- Environment Agency guidance on BAT and permit determination

This document supports the Environmental Permit application and forms part of the site’s Environmental Management System (EMS).

- Installation Scope and Activities

The Inspired Global Cuisine Limited Oldham installation undertakes:

- Cooking and preparation of soups, sauces and ready meals

- Chilling and freezing of food products

- Assembly, packaging and cold storage

- Ancillary hygiene and cleaning operations (CIP)

- Dispatch and logistics activities

The installation does not undertake slaughtering, rendering, dairy processing, brewing, fermentation, animal feed manufacture, sugar/starch production, or oil/fat refining.

The activity therefore falls within IED activity 6.4(b) (treatment and processing of animal and vegetable raw materials for food production).

- BAT Applicability Approach

BAT conclusions have been reviewed and screened as follows:

- BAT 1–15 (General BAT conclusions)
These BAT conclusions are applicable to all installations within the scope of the Food, Drink and Milk sector and are therefore applicable to this installation.

- Sector-specific BAT conclusions (BAT 16 onwards)
These have been screened against the activities undertaken at the installation.
Only BAT conclusions relevant to the site’s operations have been identified as applicable. BAT conclusions relating to activities not undertaken have been formally screened out and justified as not applicable.

Applicability decisions and implementation status are recorded in the BAT Gap Analysis Register (EHS IMS 9.1.3.2).

- Applicable BAT Conclusions

4.1 General BAT Conclusions (BAT 1-15)

BAT 1–15 are applicable to the installation and are implemented through the site’s Environmental Management System and supporting operational controls, including:

- Environmental policy, objectives and governance

- Identification of environmental aspects and risks

- Resource and emissions inventories

- Energy and water efficiency measures

- Waste minimisation and segregation

- Control of chemicals and hazardous substances

- Noise and odour management where relevant

- Emergency preparedness and response arrangements

Where the installation is not yet operational, applicable BAT measures will be fully implemented prior to commencement of operations.

4.2 Energy Use and Utilities (BAT 6)

Energy use at the installation is BAT-aligned and is summarised as follows:

- Steam is generated exclusively for process heating and hygiene purposes, including cooking operations and cleaning/sanitation (CIP).

- Steam is not used for electricity generation and the installation does not operate combined heat and power (CHP).

- Process steam is supplied by high-efficiency natural gas-fired steam boilers, designed to comply with the Medium Combustion Plant Directive (MCPD) emission limit values.

- Electrical demand is partially offset by on-site solar photovoltaic (PV) generation.

This energy configuration represents Best Available Techniques for the Food and Drink sector, where reliable steam generation is essential for food safety and hygiene and where energy efficiency and emissions control are prioritised.

- Non-Applicable BAT Conclusion

BAT conclusions relating to the following activities are not applicable to the installation, as these activities are not undertaken on site:

- Slaughtering or rendering of animals or animal by-products

- Dairy and milk processing

- Brewing, distilling or beverage fermentation

- Sugar, starch or yeast manufacture

- Ethanol production

- Animal feed manufacture

- Oilseed processing or vegetable oil refining

These BAT conclusions have been screened out and are recorded as Not Applicable within the BAT Gap Analysis Register, with justification provided.

- Demonstration of BAT Compliance

Compliance with applicable BAT conclusions is demonstrated through the following documents and assessments:

- BAT Gap Analysis Register (EHS IMS 9.1.3.2)

- Environmental Risk Assessment (ERA) (EHS IMS 9.1.4)

- H1 Air and H1 Water Risk Assessments

- Part B6 Water Discharge Information

- Part B2.5 Medium Combustion Plant Information

- Site Condition Report (SRC)

- Supporting management plans (Odour, Noise, Drainage, Incident Response, Ammonia Emergency)

The application of BAT ensures emissions are prevented or minimised at source and that environmental risks, including risks to designated habitats, are reduced to as low as reasonably practicable (ALARP).

- Conclusion

The applicable BAT conclusions for the Food, Drink and Milk sector have been correctly identified, screened and applied. The installation is considered compliant with BAT, and BAT requirements are embedded within the site Environmental Management System and operational controls.