Schedule 1 Activity Classification Statement

The Inspired Global Cuisine Limited Oldham installation undertakes the cooking, cooling, assembly and packing of frozen ready meals, soups and sauces.

The maximum theoretical capacity of the installation is in excess of 75 tonnes of food product per day, based on continuous 24-hour operation of all production lines at full design capacity.

Approximately 86% of the installation’s throughput comprises food products containing animal by-products (Category 3). The remaining 14% of production comprises non-animal by-product (non-ABP) materials.

As the installation processes both animal and non-animal raw materials, the activity is correctly classified under Schedule 1, Section 6.8(d)(iii)(bb) – Food and Drink Sector (Part A(1)) of the Environmental Permitting (England and Wales) Regulations 2016.

The installation also includes associated Medium Combustion Plant activities, which are addressed separately within the permit application.

Name of the Installation: Inspired Global Cuisine Ltd (IGC)

Annual Capacity: Processing Capacity 66000 Tonnes  / Storage Capacity 31418 for Raw materials and Packaging

Maximum Daily Production Capacity: 210 tonnes per day

Schedule 1 Activity: Section 6.8 (d)(iii)(aa) – Food and Drink Sector + Medium Combustion Plant (MCP)

Raw Materials Table (Some Values TBC After Commissioning)

| Description of raw material & composition | Maximum amount on site at any one time (tonnes) | Annual throughput (tonnes/year) | Description of use & main hazards |
| --- | --- | --- | --- |
| Natural Gas (piped supply) | Not stored on site | TBC after first year of operation | Fuel for MCP boilers. Flammable gas — controlled under DSEAR. |
| Water (process, boiler feed, cleaning) | TBC after commissioning | TBC after first year of operation | Used for food processing, steam generation, hygiene and cooling towers. Low hazard. |
| Ammonia (R717) – Azanefreezer charge | <650 kg total charge | Fixed system charge | Refrigeration for cold store. Toxic gas if released. Fully external with BMS leak detection. |
| CO₂ Refrigerant | TBC (fixed system charge) after commissioning | TBC | Used for chillers and factory cooling. Asphyxiant hazard in confined spaces. |
| Glycol (MPG/MEG mix) | TBC after commissioning | TBC after first year of operation | Heat transfer fluid. Environmental hazard if released. |
| Cooking oils (vegetable oils) | 1 Ton | TBC after first year of operation | Used in production, canteen. Slip hazard / combustible. |
| Animal By‑Products (Cat 3) | 31418 Tonnes (this is based off the storage capacity of the Cold stores/Chillers) | TBC | Food ingredients. Biological hazard if not temperature‑controlled. |
| Vegetable raw materials | 31418 Tonnes (this is based off the storage capacity of the Cold stores/Chillers) | TBC | Food ingredients. Low hazard. |
| Sauces, dairy, liquid stocks | 31418 Tonnes (this is based off the storage capacity of the Cold stores/Chillers) | TBC | Production inputs. Potential effluent contaminant if spilled. |
| Packaging (trays, film, sleeves, cardboard) | 31418 (this is based off the storage capacity of the Distribution Center Storage) | TBC | Used in packing finished products. Low hazard. |
| Cleaning chemicals (detergents, disinfectants) | 50 Tonnes (this is based off the storage capacity of the Chemical storage areas) | TBC | Hygiene operations. Corrosive/irritant. SDS available. |
| Engineering chemicals (oils, lubricants, greases) | 10 Tonnes based off the Storage areas in Engineering | TBC | Maintenance activities. Environmental pollutant if spilled. |
| Boiler chemicals (oxygen scavenger, treatment chemicals) | 10 Tonnes based off the storage area in the boiler house | TBC | Water treatment. Corrosive/irritant. |
| F‑Gas refrigerants (HVAC systems) | <500kg in the system | Closed system | Office HVAC. Low GWP blends. |
| Waste raw materials (food waste, ABP Cat 3 residues) | 60 Tonnes | 2080 Tonnes, waste will increase/decrease depending on throughput tonnage, we have 3% scrap rate on manufacturing processes | Temporary storage prior to disposal. Odour risk if not contained. |