DUST/BIOAEROSOL MANAGEMENT PLAN

One sensitive receptor is within 100m of the boundary.

The main possible dust/bioaerosol sources are listed below:

- Feed delivery

- Feeding systems

- Bedding material

- Litter management

- Ventilation systems

- Stock inspections/management

- Cleaning operations

- Bird numbers

The flowing table describes how these sources are controlled

| Possible dust source | Process | Reduction technique | Completion date |
| --- | --- | --- | --- |
| Feed | Feed delivery to silos Feeding systems Feed spillage Process | All bulk deliveries in sealed and covered trailers. Discharged directly into silos through sealed pipe work. Silo vent pipes covered with containment bags. Feed delivered to sheds via sealed auger system and dropped directly to feed pans inside. Any feed spillages cleared up and disposed of immediately. | In place In place In place Completion date |
| Feed | Feed type | All feed is pelleted and coated to prevent break up and dust production Some rations include a percentage of whole wheat.. | In place |
| Bedding | Bedding type Bedding application | Initial bedding is of dust extracted woodchip or wood shavings Initial bedding spread by mini tractor and scraper, further bedding added by hand, using plastic wrapped bales. Bedding depths maintained to absorb moisture from birds. | In place In place |
| Litter | Litter management | Relative humidity and temperature controlled to maintain best litter conditions. | In place |
| Ventilation | Air exchange within rearing sheds | Ridge inlet and side extraction to ground from all sheds. Constantly monitored and adjusted to maintain best conditions. Any accumulation of dust beneath outlets cleaned up and disposed of appropriately | In place |
| Shed cleaning | Litter removal | All litter removed by mechanical handlers, placed directly onto trailers which are sheeted as soon as loaded Side doors remain closed during cleanout, all litter removed via end doors. | In place |
| Stock welfare | Husbandry practices | Husbandry work within sheds carried out in a quiet steady manner to reduce bird disturbance. | In place. |

This plan will be reviewed every four years unless a problem occurs in the meantime.

Should dust emissions become an issue, further mitigation measures, such as screening will be investigated/implemented.