Overview:

- When a change to operation, plant or machinery or infrastructure is planned, an assessment of the likely impacts should be performed to include possible additional impacts upon the environment. - This is a legal requirement for EPR permitted sites. Changes must be notified to the Environment Agency under Regulation108 (4) of the EP Regulations 2010 at least 14 days before the change is made.

| Project Title: Permit Number: Site Name: Operator name: | Changes to Site Infrastructure EP3032YV Freiston Poultry Farm Hook 2 Sisters |
| --- | --- |
| Date of planned change: | Retrospective submission; works already completed |
| Why are changes needed? | Changes to the ventilation: To support chicken placement. Changes to drainage: To provide increased storage and a more reliable system. Additional diesel storage: To run the standby generator. This has been added as a permanent fixture as a contingency in the event of power failure. |
| Description of planned changes: | Changes to the ventilation: Each shed has had 8 x gable end fans [Airmaster VC130C (-30Pa 40,700m3/h)] installed at the rear of each shed which are used infrequently for temperature control in hot weather. Daily adjustments are made to the temperature and ventilation programme to ensure optimum conditions for the poultry. Changes to drainage: Each growing unit has a sump of approximately 1m3, situated at the lowest point to collect and contain wash water. There are 2 above ground holding tanks; wash water from units 1 – 4 and 10 is piped into the 80,000L tank via a pumping chamber and wash water from units 5 - 9 is piped into the 30,000L holding tank. Containment of wash water is ensured by the use of 2 diverter systems to the 80,000L tank. Surface water run-off from roofs and concrete hardstanding is piped to a pond within the site boundary. The underground storage tanks previously used for the storage of wash down slurry were filled in once the new drainage system was bought on line. Additional diesel storage: The site has had a 3,000L diesel tank installed which serves the permanent standby generator. |
| Updated site plans / drainage plans required? | See attached updated plan |

| Emission | Will the change result in…? | Overview | Preventative Measures proposed to reduce risk |
| --- | --- | --- | --- |
| Listed Activity | An increase in the number of pigs or poultry reared on site at any one time | No |  |
| Permit | Contravention of Permit Condition | No |  |
| Energy | Change in energy use (gas/electricity, biomass) | No |  |
| Surface Water | Change in volume or substances in surface water released from site | No |  |
| Air | Change of emissions to air (ammonia / combustion gases) | No |  |
| Raw materials | Change in raw material use (includes cleaning chemicals) | No |  |
| Waste | Change in waste volume production (recovery or disposal) | No |  |
| Noise / Vibration | Increase in emissions of noise or vibration | No |  |
| Odour | Increase in emissions of odour | No |  |
| Accidents | Increased potential for accidents | No | Eg: Does the accident management plan need updating? |
| Fire | Any increased risk of fire | No | . |

| Record of Environment Officer’s decision (Confirm whether a variation is needed or not) Confirm whether further information is needed and by when (e.g. site plans) |  |  |  |
| --- | --- | --- | --- |
| Environment Officer Signature |  |  |  |
| Date: |  |  |  |
| Date Received: | Date Assessed: | Date Operator notified: | Date Uploaded to EDRM: |