Site Condition Report

| 1.0 SITE DETAILS |  |
| --- | --- |
| Applicants Name | Hawkes Ltd |
| Activity Address | North Keal Poultry Farm East Keal East Lindsey Lincolnshire PE22 9RL |
| National Grid Reference | 537115,362210 |
| Document Reference for Site Condition Report at permit application and surrender | May 2026 |
|  | Location Plan |
|  | Layout/Drainage Plan |

| 2.0 Condition of the Land at Permit issue |  |
| --- | --- |
| Environmental setting including: Geology Hydrogeology Surface Waters | The poultry unit lies on level ground to the southwest of East Keal. Predominant land use is arable farming. Field pattern is semi large scale with a strong pattern of field boundaries. There are sensitive receptors within 400m of site. Nearest being 221m to the Northwest. Topography and Drainage The poultry houses are between 5 and 10 metres above sea level. Soil bunding and tree planting will help to minimise the visual intrusion normally associated with poultry units. Site clean water drainage all goes to an unlined attenuation pond. Geology and Hydrogeology According to the BGS Extract British Geological Survey map shows that the Bedrock geology comprises Kimmeridge Clay formation – Mudstone, of the Jurassic period. Searches indicate the site is not underlain with a Bedrock Aquifer, with a superficial aquifer classified Secondary (Undifferentiated). The Site is not situated in a Groundwater Vulnerability Zone. The site is situated within a Surface Nitrate Vulnerable Zone. The site is not situated in a Groundwater/Source water protection zone. The site is not situated in a drinking water protection/safeguard zone Hydrology General drainage is in a southerly direction. Average rainfall for this area is 1100 mm. |
| Pollution history including: Pollution incidents that may have affected land Historical land uses and associated contaminants Any visual/olfactory evidence of existing contamination Evidence of damage to pollution prevention measures | None noted Polluting substances – None noted Previous use prior to 2026 – Arable farming. None noted None noted |
| Evidence of historical contamination | N/A |
| 3.0 Permitted Activities | Six poultry houses for Broiler chicken. This will give a total of 265,000 places. Heating will be provided by LPG heaters. The working area where vehicles operate is laid to concrete. Feed is delivered in covered lorries and stored on site in vermin proof steel galvanised bins. Immediately following depopulation, litter is removed off site and sold. The houses are then washed and disinfected prior to the cycle beginning again. Underground storage tanks have been installed to catch all wash waters. Dead birds are removed from the houses and stored in sealed containers awaiting collection. Diesel fuel storage is in a bunded integral tank in the standby generator. |
| Non permitted activities undertaken | N/A |
| Document References | Location Plan Layout/Drainage Plan H1 Assessment |
| 4.0 Changes to Activity | N/A |
| 5.0 Measures taken to protect Land | Site will be operated in compliance with “how to comply” routine maintenance schedules are followed and recorded and with any abnormal operations recorded. |
| 6.0 Pollution Incidents | N/A |
| 7.0 Soil, gas and water quality monitoring | No monitoring will be undertaken at the installation. |