SITE CONDITION REPORT

Overcote Poultry Unit

Hook2Sisters Ltd

Overcote Lane

Needlingworth

PE27 4TN

Permit Number:  EPR/SP3930UV

Grid Reference: TL34937180

December 2024

SITE CONDITION REPORT FOR OVERCOTE FARM

| 1.0 SITE DETAILS |  |  |
| --- | --- | --- |
| Name of the applicant | Hook 2 Sisters Ltd |  |
| Activity address | Overcote Poultry Unit, Needlingworth, St Ives, PE27 4TN |  |
| National Grid Reference | NGR (474504, 432932) |  |
| Document reference and dates for Site Condition Report at permit application and surrender | Site Condition Report – Application PSCAPPHOOK2049 (Dec 2024) |  |
| Document references for site plans (including location and boundaries) | Overcote Drainage Plan |  |

Note:

Appendix 1 shows the following:

- Site location, the area covered by the site condition report, and the location and nature of the activities and/or waste facilities on the site.

- Locations of receptors, sources of emissions/releases, and monitoring points.

- Site drainage.

- Site surfacing.

| 2.0 Condition of the land at permit issue |  |
| --- | --- |
| Environmental setting including: - geology - hydrogeology - surface waters | To the North of the site there is a sewage treatment works. To the West of the site is the village of Needlingworth. To the East of the site is a small holding and Dog day care centre. The site falls within the British Geological Society Sheet 187 Drift Edition The map shows the main geological unit underlying the site is Pleistocene and recent alluvium. The site is within 500m of a flood plain. There are no boreholes on site. Ground water vulnerability is medium to low. |

| Pollution history including: - pollution incidents that may have affected land - historical land-uses and associated contaminants - any visual/olfactory evidence of existing contamination - evidence of damage to pollution prevention measures |  | It is understood that the site has been a poultry farm since the 1960’s. Hook2Sisters are not aware of any pollution incidents at the farm. |
| --- | --- | --- |
| Evidence of historic contamination, for example, historical site investigation, assessment, remediation and verification reports (where available) |  | There is no evidence of existing contamination on the site at present. |
| Baseline soil and groundwater reference data |  |  |
| Supporting information |  |  |

| 3.0 Permitted activities |  |
| --- | --- |
| Permitted activities | Rearing of broiler chickens for meat production. Associated activities include the storage of diesel, LPG and feed. |
| Non-permitted activities undertaken | None |
| Document references for: - plan showing activity layout; and - environmental risk assessment. | Site drainage plan Environmental Risk Ass |

| 4.0 Changes to the activity |  |  |
| --- | --- | --- |
| Have there been any changes to the activity boundary? |  | Yes |
| Have there been any changes to the permitted activities? |  | No, The site will continue to be used for poultry production. |
| Have any ‘dangerous substances’ not identified in the Application Site Condition Report been used or produced as a result of the permitted activities? |  | No |
| Checklist of supporting information | Site plan – Drainage plan |  |

| 5.0 Measures taken to protect land |  |
| --- | --- |
| Whilst operating the site has historically been regularly audited by the Environmental Agency to ensure that it is compliant and adhering to the requirements of how to comply. |  |
| Checklist of supporting information | - Inspection records and summary of findings of inspections for all pollution prevention measures - Records of maintenance, repair and replacement of pollution prevention measures |

| 6.0 Pollution incidents that may have had an impact on land, and their remediation |  |
| --- | --- |
| There have been no historic pollution incidents at the site. |  |
| Checklist of supporting information | - Records of pollution incidents that may have impacted on land - Records of their investigation and remediation |

| 7.0 Soil gas and water quality monitoring (where undertaken) |  |
| --- | --- |
| N/A |  |
| Checklist of supporting information | - Description of soil gas and/or water monitoring undertaken - Monitoring results (including graphs) |