| Serial No: |  | FLT/EA/003 |  | Date of Entry: |  | 10/07/2023 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A |  | B |  | C |  | D |  |  |
| Probability of a significant Environment Impact |  | Potential Consequence |  | Resultant Impact |  | Legal Consequence |  |  |
| 1 - 6 | 3 | 1 - 6 | 5 | 1 - 6 | 4 | 1 - 6 |  | 5 |
| Environmental Risk Score Emergency Conditions A x B x C x D = |  |  |  |  | 300 |  |  |  |
| 1 – 6 | 3 | 1 – 6 | 2 | 1 – 6 | 3 | 1 – 6 | 5 |  |
| Environmental Risk Score Routine Conditions A x B x C x D = |  |  |  |  | 90 |  |  |  |
| Responsible Business Unit: |  |  |  | Engines Services |  |  |  |  |
| Line Manager/Process Supervisor |  |  |  | Engines Production Manager |  |  |  |  |
| Description of Activity, Process, Operation, Equipment |  |  |  | Engines Cleaning Plant emissions to air via one of three scrubbers. Fugitive emissions to air from evaporative losses from tanks and chemical decanting processes. |  |  |  |  |
| Description of Potential or Actual Environmental Effects: |  |  |  | Release of NOx (NO2), VOCs and Chromium to air. |  |  |  |  |
|  |  |  |  | Pollution of local habitats, ancient woodland, SSSI, Portsmouth Harbour and local air quality. |  |  |  |  |
| Statutory Licenses, Consents, Agreements and Exemption Certificates required and Applicable conditions |  |  |  | Environment Agency part A operating permit – EPR/NP3930KB (OBSOLETE) |  |  |  |  |
| Legislative Reference(s) applicable: |  |  |  | Pollution Prevention and Control Act 1999 |  |  |  |  |
|  |  |  |  | Environmental Permitting (England and Wales) Regulations 2016 |  |  |  |  |
|  |  |  |  | Health & Safety at Work Act 1974 |  |  |  |  |
|  |  |  |  | Environmental Protection Act 1990 |  |  |  |  |
|  |  |  |  | Environment Act 1995 |  |  |  |  |
|  |  |  |  | Control Of Substances Hazardous to Health Regulations 2002 |  |  |  |  |
|  |  |  |  | The Clean Air Act 1993 |  |  |  |  |
|  |  |  |  | DEFRA Process Guidance Note 4/01 (13) and EPR 2.07 March 2009 |  |  |  |  |
| Serial Numbers of Associated Register Entries |  |  |  | N/A |  |  |  |  |
| Additional Information / Notes (Control measures in place) ((Separate) Additional control measures) |  |  |  | Engine parts are submerged within chemical cleaning vats to either remove contamination (i.e. carbon, paint, oil, grease etc.) or provide a temporary protective corrosion resistant coating. Post treatment, parts are submerged within hot or cold water tanks to remove chemical residues and are then dried. Chemical vapour from the clean bay chemical tanks is removed from the workplace by LEV systems, the vapour is routed to one of three scrubbers at the rear of 110. The extracted air contains droplets of chemical residues from the chemical cleaning process, including NOx (NO2), VOCs and Chromium. This air is passed through the scrubber’s water spray in order to capture contaminants before discharge to air via the scrubber stack. The transport velocity in the stack is controlled to 9m per second with an efflux velocity of 15m per second. The stacks are tested annually by an MCERTS accredited technician against parameters identified within the permit, including NOx (NO2), VOCs and Chromium. Operator maintenance checks are carried out on the scrubbers periodically and there is a service contract in place for maintenance of the systems including cleaning. |  |  |  |  |

| Opportunities to reduce Environmental Impact |  |
| --- | --- |
| Scale of environmental impact | To review the maintenance and inspection requirements for pollution prevention and control equipment. Review BAT Assessment for Cleaning Bay EPR 2016 schedule 1, 2.3 activity |
| Population growth |  |
| Source of raw material |  |
| Energy supplies |  |
| Emissions of CO2 |  |
| Air pollution & damage to ozone |  |
| Water resources |  |
| Land issues |  |
| Loss of bio-diversity |  |
| Sustainability |  |