South Coast Waste Holdings Limited
Dust & Emissions Management Plan v1

South Coast Waste Holdings Limited
Dust & Emissions Management Plan

CONTENTS

11.
INTRODUCTION


1Roles and Responsibilities


1Site Description


2Site Plan


22.
SOURCES, PATHWAYS, RECEPTORS


2Source Materials - Inventory


3Pathways


3Receptors and Wind Direction


43.
PRIMARY Emission CONTROL MEASURES


4Releases


4Impacts


5Management of Releases


64.
Monitoring & Trigger Levels


6Introduction


7Communication


7Complaints procedure


85.
Incidents and Emergencies


9Appendix A  Site Plan 


10Appendix B Emission Survey Form


12Appendix  C Complaint Form




1. INTRODUCTION

1.1 This document provides the Emissions Management Plan prepared in accordance with Environment Agency Guidance
 in support of environmental permit application For a waste recycling facility at Ackworth Rd, Portsmouth PO3 5NS.
1.2 The overall responsibility for this procedure is the Environmental Manager.  The Environmental Manager will be responsible for ensuring the all staff are trained in this procedure.  

1.3 The Site Manager is responsible for ensuring that the day-to-day operations are carried out in accordance with this procedure. 

1.4 All staff are responsible for implementing this procedure and have a duty to carry out their roles to prevent emissions.
1.5 This plan aims to identify dust emission points as part of the South Coast process. These are identified in table 1 below. It must be noted that these are not continuous emissions but intermittent emissions of (dust) as the tipping/sorting is enclosed within a warehouse. The process line is covered and emissions are limited and maintained to ensure it's integrity. However on occasion dust emissions may eminate from this sources and it is for that reason this DEMP has been drafted.
Site Description 

The site is located in Ackworth Rd, Portsmouth PO3 5NS. The site is approximately 0.072 Ha and lies on an industrial estate approx. 2Km north east of Portsmouth Town Centre south of the A27. At the time of writing this FPP, the site currently consists of an enclosed warehouse unit where inert materials sorting & recycling is proposed to take place and a picking station with conveyor.

             The surrounding land use is a mix of light industrial & commercial.

Site operation includes the delivery, storage and treatment of household and commercial waste. The site has a picking station but the majority of waste is sorted manually. SC Waste Holdings Ltd wish to operate a waste treatment facility which predominantly sorts commercial & household non hazardous waste. The facility consists of a warehouse for tipping skip waste and a picking station for manual separation. Sorted waste is subsequently stored in concrete firebays externally. There is a trommel and picking station. No other treatment occurs at the site.
The site is bounded by 5m high walls and consists of a concrete impermeable surface. Sorting bays inside the warehouse are also concrete firebays. Treatment consisting only of sorting, & grading. 
Waste accepted per year will not exceed 75,000 tonnes.

Waste Codes Required on the permit.

20 01 01 
paper and cardboard 
20 01 02 
glass 
20 01 10 
clothes 
20 01 11 
textiles 
20 01 38 
wood other than that mentioned in 20 01 37 
20 01 39 
plastics 
20 01 40 
metals 
170407 
mixed metals  

191202 
ferrous metal 
191203 
non-ferrous metal 
15 01 06 
mixed packaging 
 

170407 
Mixed metals 
17 04 11 
Cables other than those mentioned in 17 04 10 
17 05 
soil 
17 05 04 
Soil and stones other than those mentioned in 17 05 03 
17 05 08 
Track ballast other than those mentioned in 17 05 07 
17 09 
Other construction and demolition wastes 
17 09 04 
Mixed construction and demolition wastes other than those mentioned in 17 09 01,17 09 02 and 17 09 03 
170101 
concrete 
170102 
bricks 
170103 
Tiles and ceramics 
17 01 07 
Mixtures of concrete,bricks,tilesand ceramics other than those mentioned in 17 01 06 
17 02 
Wood,glass,plastic
17 02 01 wood 
Site Plan

1.6 A basic site layout is provided in Appendix A and Appendix 1.  
1.7 The company Environmental Management System (EMS) will be updated and revised accordingly as a living document as environmental management develops
2. SOURCES, RELEASES, IMPACTS
2.1 This section sets out the protection sources of emissions, potential release points and receptors and abatement
2.2 Sorted waste is stored in concrete firebays as per FPP and appendix 1.
Source Materials - Inventory
2.3 Given the materials used in the process, it is considered that the which are most likely to give rise to emissions are those in Table 2.
	Emission source
	Activity Description
	Storage Technique
	Emissions & Abatement

	Picking/Process line
	Sorting of mixed waste
	Process under cover inside building
	Minimal run time and cover maintenance

	Vehicles loading and unloading
	Loading & unloading.
	Stored and sheeted 
	Vehicles enclosed and sheeted where required.

	Handling of materials on-site
	Movement of material on site
	stored in fore bays 
	Dust monitored and wind direction noted in site log. Damping down with water bowser and sprinklers if necessary


Table 2 Source Materials 
2.4 It is anticipated that the likelihood of dust being emitted from these materials is low on delivery or movement.  
2.5 It is anticipated that the likelihood of mud being emitted from these materials is medium on delivery or movement
2.6 It is anticipated that the likelihood of mud & dust being generated from on-site activities is low/medium.
Pathways
2.7 The pathways by which the emissions from the sources identified above may impact upon a receptor are primarily:
1. Air Movement of dust through air, not particularly relevant on a site which will store gypsum.

             Mud via vehicle wheels onto and off of site. There is only 1 way in and out of site, 

              Ackworth Rd. There is a 10mph speed limit and an anti-idling policy for all vehicles.
2. Direct exposure.  Particularly for staff, they will be exposed immediately to any  dusts from the handling on site.
Receptors and Wind Direction
2.8 The site is on an open industrial estate which means key receptors sensitive to dust are staff and visitors to the businesses which surround the site as well as environmental receptors. There is one SSSI within 1Km. The area is one of  industrial/commercial activity with no immediate residential neighbours.
2.9 There nearest residential properties are 300m north across the M27.
2.10 The key receptors are shown in Figure 1 below.
2.11 The prevailing wind direction in the area, where the site is located, is South-Westerly
 towards immediate neighbours.
2.12 The source Pathway Receptor Model can be seen in appendix D

2.13 [image: image1.jpg]Wind rose

Portsmouth meteoblue’
50.80°N, 1.09°W (6 m asl). —
Model: ERAST. =
N
NNW NNE
1000
NW NE
750
WNW 500 ENE
W »! E
WSW / ESE
Sw SE
SSw SSE
S
< 1 mph 1 -3 mph © 3-5mph ® 5-10 mph ® 10-15mph @ 15 - 20 mph
© 20-30mph @ > 30 mph




2.14 A wind rose is included for information. However all waste is stored in covered firebays where wind will have little to no impact.
3. [image: image2.jpg]7




	Priority habitat
	40m
	North

	Nearest Residential Chats worth Av
	250m
	North

	Langstone harbour SSSI
	500m 
	East

	Hilsea Lines LWS
	1Km
	East

	Solent Maritime SAC
	1Km
	South


Figure 1: Site and Key Receptors
4. PRIMARY EMISSION CONTROL MEASURES

4.1 The control measures set out in this DEMP are commensurate with the medium mud/dust potential for the wastes. 
4.2 Dust can only cause an impact when it is received at a receptor site. This DEMP has identified that the key opportunities for release of mud/dust are anticipated to be: 

1. When waste is delivered to site. 
2. When waste is moved on site. 
           3.   When wastes are handled on site. The site will minimise drop heights when transferring/loading and unloading materials.
4.   When product is loaded and removed from site
Dust Suppression
The site has access to fire hydrants located around the border of the site.
Suppression systems
There will be no excess water reach drains externally as all systems are operated indoors and do not produce excess water. All water retained by walls and floor which is impermeable concrete.

The system will have an emergency back up generator in case of power failure.

All sorting operations are indoors and so the weather conditions are not a factor. Deliveries are tipped internally and are covered during transit.

Housekeeping
The site has a roadsweeper that will be used to clean the site and access roads on a daily basis. The sweeper will be maintained to manufacturers standard and will be itself cleaned after use. The sweeper will focus on areas of high dust build up when completing its operation. The company has access to shovels (mechanical) and other cleaning equipment in the event of a spill on site.
Handling
Handling is minimised by careful loading, handling and unloading. This minimises dust emissions but also protects the machinery from adverse wear and tear and is an integral part of the process.

Impacts

4.3 The impacts of any emissions released as a result of South Coast’s activities will be linked to the receptors identified in Section 1, although dust is more likely to cause annoyance to neighbours rather than any lasting environmental impact due to it’s inert nature.
4.4 The receptors are more likely to be impacted upon by emissions in the following conditions:

· Prevailing wind direction is towards receptors; and 

· Local weather conditions. Warm weather will contribute to the increased dust levels, although wind would act as the main transport agent.
· Wet weather will damp down dust but contribute to increased levels of mud on surfaces

· Cumulative impacts. It is anticipated that cumulative impacts will be minimal.  The site is not in an area dominated by facilities which may cause additional mud/dust. 

· NOTE – All processing inside the building.

4.5 The impacts of emissions from the site are anticipated to be minimal, given the nature of the material being handled and the location within an industrial setting. However, this will be confirmed regularly with monitoring and communication with neighbours. Visual monitoring is considered sufficient given prevailing wind direction and distance from human sensitive receptors. The site daily log will record wind direction & strength and note any visible dust blows and act accordingly. The visual monitoring period will be undertaken for 30 minutes and monitoring will be conducted as part of the daily walk around by the on site environmental manager who is familiar with the site and operations. Any blows noted above “normal” during operations will be noted and acted upon as per section 5 below.
Management of Emissions
4.6 If emission monitoring or complaints indicate a problem, South Coast Solutions will respond appropriately.  Management measures to control releases will include:

· Reducing the handling time on site of materials.  As part of the site’s Management System, information on the materials on site will be recorded via a ticketing system, including arrival time on site.  The Site Manager is responsible for ensuring the residence time of materials site does not exceed that set out in the permit and therefore is able to identify how long each material is on site. Minimal handling will ensure that raw material & product are not moved too frequently and dust propagation is minimised
If the Site Manager deems that emissions are causing a disturbance, the Site Manager will log this as an incident, using the appropriate forms from the Management System and will take corrective action

The site management will liaise with the waste producers and transport contractors, with a view to minimising the storage and transport periods for the waste being delivered to the site;
· Unanticipated Emissions. Any unexpected emissions will be recorded as to composition, date and time noted at site and cause.
· Containment and abatement.  Given the nature of the material handled on site, it is not considered necessary to implement containment and abatement techniques, other than those mentioned above. However, this is a ‘live’ document and as such will adapt if emissions arise which are not managed by the actions in this DEMP. As standard damping down, wheel washes and sheeting will be employed in any case.
5. Monitoring & Trigger Levels
Introduction

5.1 To ensure that the emissions control measures set out in Section 3 are being effective, South Coast Solutions will ensure monitoring is in place and communication with potential receptors is maintained. 

5.2 The following monitoring activities are regularly undertaken to ensure continuous improvement: 

· Site inspections by the site manager (as outlined below)

· Site audits conducted by the company’s management;

· Site audits and inspections by the Environment Agency.

5.3 All site personnel will be responsible for reporting any dust/mud emission problems immediately to the site manager (or deputy).

Emissions monitoring

5.4 The Site Manager will ensure that regular inspections are made of the site and its perimeter in order to identify any sources of emissions and to establish whether any dust/mud is discernible at the perimeter and thus likely to impact upon receptors and neighbours or other road users. 
5.5 In the event that mud/dust is detected at the site boundary, additional monitoring will be undertaken at the sensitive receptors in accordance with Appendix B.  
5.6 An inspection will also take place in response to complaints.  The surveyor will undertake the survey at the location of the complaint and at potentially sensitive receptor locations in the vicinity downwind from the site.  At each location observations are made concerning the intensity of the dust/mud, its persistence and character (these details will be logged in the pro forma, see Appendix C).
5.7 The surveyor may be the site manager or alternatively a staff member from the office or external person who is not used to the emissions on the site. 
5.8 Trigger Levels. If emissions are detected at the assessment location and is judged to be a moderate or unacceptable, as defined in reference Table A - Appendix B, then the Site Manager and Management Team will be informed immediately, and corrective actions will be determined and implemented. 

5.9 Monitoring Frequency will be in accordance with Table 3
	Technique 
	Frequency

	Dust/mud Monitoring visible inspection
	Daily & Weekly at site perimeter.  
Positive Detection will lead to receptor monitoring.
Increase frequency in response to complaints

	Complaints system
	Continuous (24 hours) via telephone reporting system to Environment Agency

Direct complaints to site in operational hours


Table 3 Monitoring Frequency

Communication

5.10 Liaison with neighbours.  If emissions are anticipated on site or weather conditions predicted indicate dust/mud will increase, the Site Manager will liaise with neighbours and ensure they are aware of the situation, how long it is expected to last and how to make a complaint. 
5.11 Signboard. The site will have a legible signboard giving contact details for the Environment Agency and the operator.  These numbers can be used in order to make a complaint. 
Complaints procedure

5.12 All complaints, whether direct from people in the neighbourhood, or via the Environment Agency will be treated seriously by South Coast Waste Ltd and recorded in the Complaints Form in Appendix C.

Trigger Levels.  The operator recognise that persistent dust/mud can be a concern for neighbours and residential areas.  Every complaint is a trigger for management to take action to investigate the cause of a complaint, as set out in the complaints form.

5.13 Incidents and Emergencies
5.14 This section of the DEMP relates to potential incidents or emergencies which may impact on the ability of South Coast Ltd to control emissions from its site.

5.15 Potential incidents which may impact on mud/dust are outlined as follows:

	Potential Incident
	Actions

	Delivery of material which meets the requirements of the permit but which is unexpectedly awkward to handle and dusty
	The Site Manager will:

· Refuse to accept the material if it is deemed inappropriate for the site.

· If the material can be managed on site, an appropriate storage area will be used or a suitable area undercover will be used

· Record the incident in the site diary and using the appropriate management system forms and records.
· 

	High wind/still conditions
	If monitoring detects a problem, the site manager will:

· Ensure handling is kept to a minimum
· Record the incident in the site diary and using the appropriate management system forms and records.



Appendix-A  
A1
[image: image3.jpg]N
Address

SOUTH COAST WASTE LTD, ACKWORTH ROAD, PORTSMOUTH, CITY OF PORTSMOUTH,
Date Grid reference
03/09/2025 466498, 104129

Site

Picking
Entrance

Office \ EEny Station

e
Recycling Conveyor
Business

S60/grab Tipping/sorting
5 (o) CCTV  bays inside

building

4 8 12 16 20 24 28

32 36
Scale: 1:200 | Area: < 1ha | Paper size: A4




[image: image4.jpg]Location Plan Groundsure

Address i
SOUTH COAST WASTE LTD, ACKWORTH ROAD, PORTSMOUTH, CITY OF PORTSMOUTH,

Date Grid reference

03/09/2025 466466, 104094

20 40 60 80 100 120 140 160 180 200

Scale: 1:2500 | Area: 4ha | Paper size: A4

. ) ) ]
Ordnance  Mapping contents © Crown copyright and database rights 2025 Ordnance Survey
Q@sﬁ Survey 100035207 Groundsure

OCATION INTELUIGENCE




[image: image5.jpg]N
Address

SOUTH COAST WASTE LTD, ACKWORTH ROAD, PORTSMOUTH, CITY OF PORTSMOUTH,
Date Grid reference
03/09/2025 466498, 104129

___\ Impermeable concrete surface throughout
yard.

12 cm ramp at site entrance to prevent 4
egress of firewater

[]Begycung-ﬂ
=~ yBusiness

S

Conveyor

Gullies leading

\n 4= toblind sump

Perimeter of site kerbed/
ramped to prevent egress of

water \

I I I I I
4 8 12 16 20 24 28 32 36

Scale: 1:200 | Area: < 1ha | Paper size: A4




Appendix B Emissions Survey Form

	Emissions Survey Recording Form
	Reason for  Survey
	Mud/dust detected at boundary? Y/N

Complaint Y/N

Other

	Name of Surveyor
	
	Job Title
	

	Date
	
	Time of Survey Start/Finish
	

	Air Temp. oC 
	
	Wind Direction
	


Survey Results
	Location
	Mud/dust Intensity

See Reference Table A
	Mud/dust Extent

See Reference Table B
	Description of emission 
e.g. intermittent, ongoing?

	
	
	
	

	
	
	
	

	
	
	
	


Reference Table A: Emission Intensity 

	Intensity
	Description

	1
	No detectable emissions

	2
	Slight dust noticeable (barely detectable, need to stand still and look into wind) 

	3
	Moderate dust (easily detectable while walking, possibly offensive)

	4
	High dust levels (bearable, but intrusive and affects eyesight)

	5
	Very high dust/mud


Reference Table B: Emission Extent 

	 Extent
	Description

	1
	Local and not persistent (only detected during brief periods when wind blows)

	2
	Not persistent as above, but detected away from site boundary

	3
	Persistent but fairly localised

	4
	Persistent and pervasive up to 50m from site boundary

	5
	Persistent and widespread (detected >50m from site boundary)


Appendix  C Emissions Complaint Form
	Complainant Details

	Time and Date of Complaint
	

	Complainant Name & Address
	

	Complainant Phone Number/ Email address
	

	Emission Details

	Date noticed
	

	Time noticed
	

	Location of emission (if not address above)
	

	Wind Direction (e.g. From South West)
	

	Complainants Description of Emission:

	-What does it look like
	

	-Intensity (see reference Table A, Appendix B)
	

	Duration – how long was it detected for
	

	Constant/intermittent in this period
	

	Complainant comments/observations
	

	Additional Information

	Are there other complaints from this area?
	

	Is South Coast Ltd the likely source of the emission?
	

	What was happening on site when the emission occurred?
	

	Corrective Actions

	Actions Required
	

	Deadline for Actions 
	

	Person Responsible for implementing actions
	

	

	Form completed by
-Name and Role
	

	Date
	

	Signature
	


Appendix  D Source/Pathway/Receptor Model
	Source
	Pathway
	Receptor
	Type of impact
	Where relationship can be interrupted

	Mud 
	Tracking dust on wheels and vehicles, then mud dropping off wheels/vehicles when dry
	Roadways/ local environs / businesses
	Visual soiling, also consequent  resuspension as airborne particulates
	Remove mud before vehicles leave site by wheel washing. Need for a road sweeper to be on site every day

	Dust
	Blown off material
piles and from vehicle movements
	Roadways, protected habitats & species/businesses
	Visual dust deposition on vehicles/business housing
	Cover lorries before leaving site. Bowsers can damp down dust and sprinklers keep dust on the ground

	Tipping, storage and sorting of material in the open
	Atmospheric dispersion
	Businesses, protected habitats/species
	Visual dust deposition on vehicles/housing
	Minimise dust emissions by handling in low wind conditions and damping. Sheet vehicles and bays.

	Dust
	Atmospheric dispersion
	Businesses, protected habitats/species
	Wind direction predominantly towards residents

	Minimise dust emissions by handling in low wind conditions and damping. Sheet vehicles.

	Vehicle exhaust emissions
	Atmospheric dispersion
	Businesses, protected habitats/species
	Airborne particulates
	Regulatory controls and best-practice maintenance measures to minimise source strength

	Non-road going machinery exhaust emissions
	Atmospheric dispersion
	Businesses, protected habitats/species
	Airborne particulates
	Regulatory controls and best-practice maintenance measures to minimise  source strength


Appendix 1 Site Layout/Emission Points – red stars = potential emission points
[image: image6.jpg]Address

N
SOUTH COAST WASTE LTD, ACKWORTH ROAD, PORTSMOUTH, CITY OF PORTSMOUTH,
Date Grid reference
03/09/2025

466498, 104129

Site
Entrance

TV Picking
Potential dust Station
emission point

f Quarantine
Mobile plant parked
ohere when not in use B— aroa
Recycling

iness

tential dust
emission point

360 grab
(o) ccTv

Conveyor

Tipping/sorting
bays inside
building

I I I I
4 8 12 16 20 24 28

32 36

Scale: 1:200 | Area: < 1ha | Paper size: A4




Appendix 2 Complaints & Neighbourly Relations
EP7  Complaints & Neighbourly Relations

· Any abnormal emission to the environment evident, such as mud on roads or dust will be recorded in the site diary.

· Any complaints received by the site, concerning emissions to the environment, will be recorded in the site diary. Any complaint made by an external source will be documented and dealt with in accordance with this procedure. It will be the responsibility of management to arrange for the complaint to be investigated, take any remedial action necessary and respond to the complaint if appropriate. Any written documents concerning the complaint will be kept for at least 2 years.

· The complainant will be updated on investigations of the complaint within 5 working days

· If several complaints are made simultaneously then work will be suspended pending further investigation

· If it is visually evident that substances (mud, dust) are being tracked out of the site onto the public highway, steps will be immediately taken within the site to prevent further transfer of material and the public highway will be cleaned by the end of the working day. Any such event will be recorded in the site diary.

· The site boundary will be visually inspected weekly by management for dust and debris; this would be in conjunction with the weekly boundary security inspection. If any material has fallen over/ outside the boundary it will be cleared as soon as practically possible, but within a week of discovery and so prior to the next weekly inspection. Litter will be cleared on the day of discovery. The weekly checks will be recorded in the site diary.
�


� http://www.metoffice.gov.uk/climate/uk/regional-climates/so






4

January 2026

