# Noise Management Plan

Mr T Pickervance

New Hall Poultry Farm

The operations at New Hall Poultry Farm has the potential for noise production, prevailing wind is from the south west indicating the receptors located to the north east of the site would potentially be the highest at risk, the following plan has been prepared as part of the EPR permit application to prevent or minimise noise at New Hall Poultry Farm.

New Hall Poultry Farm has not been the subject of noise complaints, siting and location of noisy operations are sited as far as practical to be screened by woodland. Noise management plan to be reviewed annually or following a substantiated noise complaint.

Complaints Procedure

In the event of a substantiated noise complaint, the cause would be investigated and actions taken listed in the noise/contingency plans to cease the release. Area officer would be notified immediately, a review of the NMP conducted at the earliest opportunity with any changes communicated to Area officer for approval. A complaints report would be filled out and retained on site.

Example Noise Complaint form attached.

New Hall Poultry Farm operations have sensitive receptors within 400m of the site boundary. The site has no history of substantiated noise complaints.

The Table below gives location and direction of all sensitive receptors.

| Receptor Name | Description | Distance | Orientation | National Grid Reference |
| --- | --- | --- | --- | --- |
| Residence | Dwelling | 200m | East | 344554,435612 |
| Residence | Dwelling | 116m | North | 344336,435982 |
| Residence | Dwelling | 175m | North | 334349,436042 |
| Residence | Dwelling | 250m | Northwest | 344194,436113 |
| Group of Residences | Dwellings | 268m | East | 344613,435910 |
| Group of Residences | Dwellings | 268m | East | 344620,435859 |
| Group of Residences | Dwellings | 290m | East | 344650,435804 |
| Group of Residences | Dwellings | 334m | East | 344692,435800 |
| Group of Residences | Dwellings | 327m | East | 344682,435836 |
| Residence | Dwelling | 358m | East | 344716,435783 |
| Residence | Dwelling | 376m | East | 344735,435771 |
| Residence | Dwelling | 394m | East | 344745,435754 |

A walk around assessment will be conducted to establish possible sources of noise emissions, and consideration given to different operations occurring during the whole of the production cycle.

# Noise Management Plan

The main possible noise sources/operations are listed below:

- Ventilation Fans

- Feed Deliveries

- Feeding Systems

- Fuel Deliveries

- Alarms Systems

- Bird Catching

- Clean out Operations

- Maintenance + Repairs

- Set up and Placement

- Standby Generator testing

A table listing these sources with measures to control/reduce noise emissions is listed below.

In the event of a complaint a report would be filled in (example attached)

The measures given in Noise plan reference the Noise Assessment document completed as part of the H1 Assessment

Other reference documents are the Emergency plan, Technical Standards and Routine Maintenance Schedule.

# Noise Management Plan

| ## Potential Noise Problem | ### Minimisation Techniques | ### In Place Y/N | ### Completion ### Date |
| --- | --- | --- | --- |
| Ventilation Fans | Noise assessed during twice daily inspections (07.00-10.00 hrs and 16.00-18.00 hrs) Large capacity fans, reducing number of fans required Fans operated on an intermittent programme Regular end of cycle maintenance by qualified electrician. See routine maintenance schedule Any noisy fans isolated and electrician notified See Inspection and maintenance schedule/Technical standards See site plan | YES YES #### YES Yes Yes |  |
| Feed Deliveries | Delivery lorries fitted with silencers Large capacity lorries to reduce no. of deliveries Road/track maintenance Time restricted if required | #### YES #### YES YES If required | Continuous If required |
| Feeding Systems | Daily inspections of bin stocks to prevent augers running empty (07.00-10.00 hrs and 16.00-18.00 hrs) Feed bins located to reduce vehicle movements Internal feeders checked twice daily to ensure correct operation (07.00-10.00 hrs and 16.00-18.00 hrs) Regular end of cycle maintenance by qualified electrician. See Inspection and maintenance schedule/Technical standards | YES YES YES YES |  |
| Fuel Deliveries | Time restricted if required Large capacity vehicles used | If required Yes | If required |
| Vehicle movements | Speed restriction on site 10mph No idling engines or reversing warnings Vehicles regularly maintained Movements routed away from receptors as far as practical | Yes YES YES YES |  |
| Alarm Systems | Use of pagers or mobile phones No audible alarms on site | YES YES |  |
| Bird Catching | Catch teams fully trained and advised of need to keep noise to a minimum ie no shouting or playing of loud music. Crates to be placed handled carefully prior to house entry Lorries scheduled to minimise duration of catch Doors operated for entry and exit of forklift Lorries parked as close as possible to doors to reduce forklift travel Screen curtains fitted to lorries See Inspection and maintenance schedule/Technical standards/Key responsibilities | YES YES YES YES YES YES |  |
| Clean out operations | Litter removal during normal working hours (07.00-18.00 hrs) Trailers parked as close as possible to doors to reduce loader travel Large trailers used to reduce traffic Washing done during normal working hours 08.00hrs - 18.00hrs. See Inspection and maintenance schedule/Technical standards/Key responsibilities | YES YES YES YES |  |
| Maintenance/Repair | During normal working hours (07.00-1800 hrs excepting emergencies/breakdown Routine end of cycle servicing. See Inspection and maintenance schedule/Technical standards/Key responsibilities | YES YES |  |
| Set up/Placement | Normal working hours 08.00hrs - 18.00hrs. See Inspection and maintenance schedule/Technical standards/Key responsibilities | YES |  |
| Standby Generator | Test run weekly, typically 10.00am on Mondays. Housed in acoustic jacket. See Inspection and maintenance schedule/Technical standards/Key responsibilities See site plan | YES |  |
| Noisy operations | Sited as far as practical to be screened by woodland. | YES |  |

## Signed                                            Date                                               Review Date

Key Responsibilities

| ## Task | ## Staff position responsible |
| --- | --- |
| ## Noise assessments | ## Manager |
| ## Ventilation fans | ## Site Supervisor/Maintenance team |
| ## Roadway maintenance | ## Manager |
| ## Vehicle maintenance | ## Manager/Mill Transport Manager |
| ## Daily feed bin inspections | ## Site Supervisor/Stockman |
| ## Internal feeder checks | ## Site Supervisor/Stockman |
| ## Overseeing deliveries | ## Site Supervisor/Stockman |
| ## Overseeing bird catching | ## Site Supervisor/Stockman |
| ## Overseeing cleanout operations | ## Site Supervisor/Stockman/Manager |
| ## Alarm system/standby generator testing | ## Site Supervisor/Stockman |

Monitoring Procedure

No monitoring currently undertaken. If substantiated complaints are received then monitoring to take place on site to identify source of issue immediate action to be taken to minimise / rectify Source.

Noise Complaint Form

| Installation to which complaint relates |  |  |  | Date received |  | Reference number |
| --- | --- | --- | --- | --- | --- | --- |
| Name and Address of Caller |  |  |  |  |  |  |
| Telephone Number |  |  |  |  |  |  |
| Location of caller to Installation |  |  |  |  |  |  |
| Time and Date of complaint |  |  |  |  |  |  |
| Date/Time and Duration of Noise |  |  |  |  |  |  |
| Callers description of Noise |  |  |  |  |  |  |
| Other comments from caller |  |  |  |  |  |  |
| Weather conditions |  |  |  |  |  |  |
| Wind direction/Speed |  |  |  |  |  |  |
| Any previous complaints relating to this noise |  |  |  |  |  |  |
| Any other comments |  |  |  |  |  |  |
| Other information |  |  |  |  |  |  |
| Potential source of noise |  |  |  |  |  |  |
| Operations being carried out at time of complaint |  |  |  |  |  |  |
| Follow up. Date/Time caller Contacted |  |  |  |  |  |  |
| Action Taken: |  |  |  |  |  |  |
| Amendments required to plan |  |  |  |  |  |  |
| Completed By: |  |  | Signed: |  |  |  |

Noise management plan to be reviewed annually or following any changes to operations or a complaint.

Version 1 October 2024