# Environmental Risk Assessment
UHP Water Jetting Tyre Recycling

Document Reference: RA01

Company: Re-Tyre

Site Address: Unit 5 Sherrington Way, Lister Road Industrial Estate, Basingstoke, RG22 4DQ

Permit Type: Bespoke Environmental Permit – Tyre Recycling (UHP Water Jetting Process)

Version Control

| Version | Date | Prepared by | Reviewed by | Approved by |
| --- | --- | --- | --- | --- |
| 1.0 | 10/10/2025 | Re-Tyre Environmental Team | Charlotte Wingrove | Gavin Beedle |

This document is confidential and intended solely for the use of the Environment Agency and Re-Tyre in connection with the bespoke permit application.

# 1. Activity Description

Processing of waste tyres using ultra-high pressure (UHP) water jetting in a closed-loop recirculating water system. All activities occur indoors on an impermeable concrete-coated floor with no internal drainage. The foul sewer is outside the building and is physically separated from the process area. There is no discharge to surface water or foul sewer from the process; water is continuously filtered and reused. Process wastes comprise separated tyre material fractions (e.g., rubber, steel) stored in clearly labelled containers prior to transfer to authorised facilities.

Throughput and storage (for context): The facility typically processes up to [X tonnes/week] of tyres with a maximum on-site storage of [Y tonnes] at any time. Storage is organised to maintain clear separation distances in accordance with the site Fire Prevention Plan (FPP) and to prevent over-stacking.

# 2. Environmental Setting

The site is located on an established industrial estate with surrounding commercial and light industrial units. Operations take place within a fully enclosed building on impermeable surfacing. Drainage within the process area is sealed; no process water enters site drains. The nearest main surface water receptor is the River Loddon (Eastrop Park reach), approximately 2.58 km to the northeast. Underlying groundwater is present at depth; no on-site abstraction occurs. Flood risk: based on Environment Agency flood mapping, the site is understood to be in an area of low fluvial/tidal flood probability (Flood Zone 1) – this will be confirmed during determination.

# 3. Sensitive Receptors Within 500 Metres

A receptor screening has been completed for a 500 m radius around the site boundary. Given the sensitivity of certain receptors (healthcare and designated open space), selected receptors up to ~3.6 km are also noted for context.

Reference Point

- Site centroid (RG22 4DQ): 51.251212, -1.104207.

Residential

- Nearest established residential street: Wella Road (RG22 4AE) — ≈ 0.19 km (193 m). Coords 51.252557, -1.102461.

Education

- Chalk Ridge Primary School (RG22 4ER) — ≈ 0.52 km. Coords 51.246581, -1.105400.

- Cranbourne Secondary (RG21 3NP) — ≈ 0.72 km. Coords 51.256773, -1.098796.

- Brighton Hill Community School (RG22 4HS) — ≈ 0.84 km. Coords 51.247162, -1.114343.

Healthcare

- Camrose, Gillies & Hackwood GP (RG22 4EH) — ≈ 0.97 km. Coords 51.245046, -1.114040.

- Pemberley House Care Home (RG21 3HL) — ≈ 1.18 km. Coords 51.250595, -1.087211.

- Dashwood Manor Care Home (RG23 7GF) — ≈ 3.49 km. Coords 51.232313, -1.144191.

- Basingstoke & North Hampshire Hospital (RG24 9NA) — ≈ 3.29 km. Approx coords 51.28056, -1.11000.

- Beggarwood Surgery (RG22 4AQ) — ≈ 3.56 km. Coords 51.229553, -1.141779.

Open Space / Water (for odour/noise/air & hydrology relevance)

- Down Grange Meadow (Local Nature Reserve) — ≈ 2.07 km WSW. Coords 51.24367, -1.13135.

- River Loddon at Eastrop Park — ≈ 2.58 km NE (nearest main watercourse reach). Coords ≈51.26639, -1.07611.

Please see appendix A for a map of sensitive receptors in relation to the site.

# 4. Risk Assessment Methodology

This ERA follows Environment Agency guidance for environmental permits. Each risk is evaluated using a source–pathway–receptor model and rated for Likelihood and Consequence to determine overall Risk. Mitigation measures reduce risks to ALARP (As Low As Reasonably Practicable).

Likelihood (L): Extremely Unlikely, Unlikely, Possible, Likely, Very Likely.

Consequence (C): Negligible, Minor, Moderate, Major, Severe.

Risk Rating (qualitative): Low (L), Medium (M), High (H).

# 5. Environmental Risk Assessment

| Hazard / Source | Receptor(s) | Pathway | Consequence | Likelihood | Risk | Mitigation / Controls | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Release of process water (spill/leak) | Soil; groundwater | Loss of containment; floor defects | Minor–Moderate | Unlikely | Low | Impermeable floor; closed-loop system; bunded storage; routine inspections; spill kits; staff training | Low |
| Uncontrolled discharge to foul sewer | Foul sewer; WWTP | Misconnection/overflow | Minor–Moderate | Extremely Unlikely | Low | Physical separation from foul sewer; no process drains; lockable isolation; drainage map and permits-to-work | Low |
| Aerosol/mist from UHP jetting | On-site staff; nearby receptors | Airborne dispersion | Negligible–Minor | Unlikely | Low | Fully enclosed jetting cabinet/enclosure; local extraction/ventilation; PPE and RPE as required | Low |
| Noise from UHP equipment and handling | Nearest residential; schools | Airborne transmission / building leakage | Minor nuisance | Unlikely | Low | Indoor operations; acoustic shielding; daytime operations; maintenance of equipment; periodic noise checks | Low |
| Odour from waste tyres/water system | Residential; schools; open space users | Airborne dispersion | Minor nuisance | Extremely Unlikely | Low | Wet process; clean storage; regular housekeeping; prompt removal; water treatment/bleed-off if required | Low |
| Dust/debris from handling or cutting | Air quality; local roads | Airborne dispersion; track-out | Minor nuisance | Unlikely | Low | Wet cutting; housekeeping; sweeping; wheel checks on egress; covered skips/containers | Low |
| Litter from vehicles (ingress/egress) | Local roads; public realm | Escape from vehicles | Minor nuisance | Possible | Low | Load containment (nets/covers); driver checks; signed gate policy; periodic road sweeping; contractor briefings | Low |
| Mud/track-out to highway | Highway users; drainage | Vehicle movements | Minor safety/amenity | Unlikely | Low | Hardstanding; good housekeeping; wheel checks; spill kits; contractor requirements | Low |
| Storage of waste tyres – accidental fire | Air quality; health; property | Ignition → smoke/runoff | Major–Severe | Unlikely | Medium | Fire Prevention Plan (FPP); stockpile limits; separation distances; ignition control; hot-work permits; routine patrols | Low |
| Deliberate ignition (arson/vandalism) | Air quality; health; property | Unauthorised access | Major–Severe | Unlikely | Medium | Perimeter fencing; controlled access; CCTV; security lighting; alarm response; secure doors | Low |
| Firewater runoff during fire | Soil; groundwater; surface water; sewer | Overland flow; drainage | Major | Extremely Unlikely | Low | Firewater containment plan; penstock/valve closure; sacrificial containment; spill booms; liaison with FRS | Low |
| Smoke plume from fire | Residential; schools; sensitive receptors | Airborne dispersion | Major (acute) | Extremely Unlikely | Low | FPP measures preventing fire; emergency response plan; notify authorities; shelter advice via FRS | Low |
| Storage/use of oils, fuels, hydraulic fluids | Soil; groundwater | Spill/leak | Moderate | Unlikely | Low | Bunded storage to >110%; drip trays; ADR-compliant containers; maintenance of hoses; spill response | Low |
| Vehicle collision with tanks/plant | Containment; staff | Impact leading to breach | Moderate | Extremely Unlikely | Low | Traffic management plan; barriers; speed limits; trained drivers; segregation of pedestrians/plant | Low |
| Extreme weather (storm, wind, heat) | Site infrastructure; neighbours | Damage/overturning; odour risk | Minor–Moderate | Unlikely | Low | Preventive maintenance; secure storage; weather warnings monitoring; emergency checks post-event | Low |
| Power failure leading to loss of control | Process stability; containment | Pump stoppage/overflow | Minor | Extremely Unlikely | Low | Fail-safe controls; level alarms; isolation valves; manual shut-offs; UPS for critical alarms if applicable | Low |
| Pests/vermin (standing water/tyres) | Public health; amenity | Access to habitat | Minor | Extremely Unlikely | Low | Keep tyres dry; routine inspections; prompt removal; pest control contract if required | Low |
| Visual impact | Local community | Line of sight | Minor nuisance | Unlikely | Low | Industrial context; operations indoors; tidy yard; screening where practicable | Low |
| Light spill (if operating in darkness) | Residential; ecology | Artificial light at night | Minor | Extremely Unlikely | Low | Directional lighting; timers; maintain luminaires; limit night operations | Low |
| Drain misconnection / wrong valve position | Sewer; controlled waters | Human error | Moderate | Extremely Unlikely | Low | Drainage plan; valve position checks; lock-off; permits-to-work; supervision and shift handover | Low |
| Housekeeping failure / waste accumulation | Amenity; pests; fire load | Operational lapse | Minor–Moderate | Unlikely | Low | Daily inspections; KPI for housekeeping; supervision; corrective actions tracking | Low |

# 6. Monitoring, Inspection & Maintenance

The following routine checks ensure controls remain effective:
• Daily: Housekeeping; spill kit stocked; valve positions; enclosure doors closed; litter check at gate; wheel/track-out check.
• Weekly: Bund integrity visual check; container condition; tyre stockpile separation distances; security checks (CCTV, fencing, locks).
• Monthly: Plant and equipment maintenance; ventilation performance check; drainage plan review and signage.
• Quarterly: Training refreshers/toolbox talks; emergency drill (spill/fire); review of complaints register and corrective actions.
• Annually: Management review of ERA and EMS; tank integrity check (if applicable).

# 7. Incident Response & Reporting

Incidents and near-misses are managed through the site Environmental Management System (EMS). Spill Response: stop the source, contain at source using spill kits and booms, protect drains, recover waste to suitable containers, and dispose via authorised contractor. Fire Response: follow the Fire Prevention Plan (FPP), raise alarm, contact emergency services, and implement firewater containment by closing valves/penstocks as trained. Significant incidents will be reported to the relevant authorities in accordance with permit conditions and EMS procedures.

# 8. Residual Risk Summary

With the stated controls in place, all risks to identified receptors are reduced to Low or Negligible. The closed-loop process, impermeable surfacing, containment controls, Fire Prevention Plan measures, and management systems ensure that impacts are prevented or minimised to a level acceptable to the Environment Agency.

Appendix A - Sensitive Receptors Map