CBE Plus – Environmental Permit Application

Appendix 10 – Emission Points

| Emission Point Reference | Source | Document Reference | Parameter | Quantity | Unit |
| --- | --- | --- | --- | --- | --- |
| A2 | LEV for Plating Line | Monitoring Proposal CES Environmental Instruments (Appendix 23 Stack Emissions 2025) | Ammonia (NH3) | 0.5 | mg/m³ |
|  |  |  | Hydrogen Chloride (HCl) | 0.0 | mg/m³ |
|  |  |  | Nickel (Ni) | 0.0022 | mg/m³ |
| A4 | LEV for Plating Line | Monitoring Proposal CES Environmental Instruments (Appendix 23 Stack Emissions 2025) | Ammonia (NH3) | 0.5 | mg/m³ |
|  |  |  | Hydrogen Chloride (HCl) | 0.0 | mg/m³ |
|  |  |  | Nickel (Ni) | 0.0001 | mg/m³ |
| A5 | LEV for Stripping Line | Monitoring Proposal CES Environmental Instruments (Appendix 23 Stack Emissions 2025) | Oxides of Nitrogen (NO and NO2 expressed as NO2) | 1.66 | mg/m³ |
|  |  |  | Nickel (Ni) | 0.0002 | mg/m³ |
| S1 | LEV for Spray booth | Monitoring Proposal CES Environmental Instruments (Appendix 23 Stack Emissions 2025) | Particulate Matter (PM) | 0.235 | mg/m³ |
|  |  |  | Velocity / Volume Flow | 9251 | m³/hr |
|  |  |  | Total Organic Compounds (TOCs) | 89.51 | mg/m³ |

Table 2: Point source emissions to water/foul drain

| B1 | Effluent Treatment Plant | CBEPB208 - CBE Plus Effluent Discharge Consent Limits. 2025 Discharge information in Appendix 17 ETP Discharge Log | Temperature | 43.3 | °C |
| --- | --- | --- | --- | --- | --- |
|  |  |  | pH | 6 – 10.5 | pH |
|  |  |  | Settled Chemical Oxygen Demand | 3,000 | mg/l |
|  |  |  | Settleable Solids | 250 | mg/l |
|  |  |  | Total Copper | 1 | mg/l |
|  |  |  | Total load of Copper discharged in 24 hours | 0.015 | kg |
|  |  |  | Total Cyanide | 2 | mg/l |
|  |  |  | Total Nickel | 4 | mg/l |
|  |  |  | Total load of Copper discharged in 24 hours | 0.06 | kg |
|  |  |  | Total Zinc | 1 | mg/l |