Chemical Container Management Inspection Checklist

Chemical Containers

|  | - All containers are kept sealed and closed with adequate headspace. |
| --- | --- |
|  | - All containers are labeled to identify contents and hazards, or labeled as empty (except new containers) and labels are legible. |
|  | - All containers are in good condition, structurally sound (not bulging or leaking), and clean and free of accumulated material and residue. |
|  | - All containers are compatible with the material being stored. |
| COMMENTS: |  |

Chemical Container Storage Areas

|  | - Chemical storage area and secondary containment are dry, clean and free from accumulated liquid and debris, vegetation, trash, leaves, dirt, etc. |
| --- | --- |
|  | - Sufficient aisle space is being provided to allow for physical inspection of containers. |
|  | - Spill response material is compatible with all materials stored in chemical container storage area and stored in adequate amounts to contain potential spills. |
|  | - Chemical Container Storage Areas are impervious to leaks and secondary containment capacity is either (whichever is greater) 10% of aggregate stored volume or 110% of largest container. |
|  | - Incompatible chemicals are kept separate. (See MSDS sheets for guidance) |
|  | - Flammable and combustible liquids stored inside a building are located within an approved storage cabinet or inside a storage room that complies with government regulations |
|  | - All Chemical Container Storage Areas have adequate secondary containment (as required by Spill Prevention and Control). |
|  | - Containers are stacked at a height consistent with load bearing capability. |
|  | - Appropriate additional measures are taken to protect storm drains / site drains. |
| COMMENTS: |  |

Empty Chemical Containers

|  |  | - Containers are recycled or disposed of in accordance with HSEMS 407. |
| --- | --- | --- |
|  |  | - Containers are not made available to employees for personal use. |
|  |  | - Contract arrangements exist if feasible. |
|  |  | - Containers are emptied to the maximum extent practicable before recycling. |
|  |  | - Containers to be re-used on site comply with the following: |
|  |  | - Thoroughly empty and re-labeled |
|  |  | - Compatible with new contents |
| COMMENTS: |  |  |

Corrective Actions from this Inspection

| Issue | Corrective Action | Responsible Person | Date due |
| --- | --- | --- | --- |

| Date | Signed as checked | Management Sign Off |
| --- | --- | --- |