Alderley Park Block 21 Waste Area

Permit Owner & Operator – Bruntwood SciTech-Alderley Park Ltd

WASTE MANAGEMENT SYSTEM

Document History

| Version | Date | Page Number(s) | Author/ Owner | Approved | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | 08/02/2017 | All | Mike Broadfoot | Mike Broadfoot | 1st Version |
| 2 | 12/06/2017 | All | Mike Broadfoot | Mike Broadfoot | 2nd Version |
| 3 | 30/01/2018 | All | Mike Broadfoot | Mike Broadfoot | 3rd Version |
| 4 | 20/11/2018 | All | Mike Broadfoot | Mike Broadfoot | 4th Version |
|  | 27/08/2019 | 6,7,10 | Mike Broadfoot | Mike Broadfoot | Amended |
|  | 15/02/2021 | 5 | Mike Broadfoot | Mike Broadfoot | Updated |
|  | 10/08/2021 | 5, 6, 7, 9 ,10, 14 and 17 | Mike Broadfoot | Mike Broadfoot | Updated |
|  | 01/07/2024 | 5, 6. 7, 10 - 18 | Mike Broadfoot | Mike Broadfoot | Updated |
| 5 | 11/08/2025 | All | Mike Broadfoot | Mike Broadfoot | 5th Version |

CONTENTS:

- GENERAL CONSIDERATIONS

Site operator/permit holder

Site history and planning status

Permit Area

Hours of Operation

Waste types and quantities

Management structure & responsibilities

Technical Competence

Staff training and competence

Health, Safety & Environmental Policy

Business Continuity

2.0	SITE ENGINEERING AND INFRASTRUCTURE

Access and parking

Site Security

Site office

Waste Transfer Compound

Storage of materials on site

Treatment – Autoclave

Drainage Infrastructure

Rejected Wastes

Vehicles, Plant & Equipment

Site Identification Board

Washing and disinfecting containers

3.0	SITE OPERATIONS

Clinical Waste/Infectious Waste

Biological-waste

Treatment Process

Duty of Care

Permit to Surrender

Display of Environmental Permit

Managing Documents and Records

Permitted Activities

Permitted Categories and Types of Waste

4.0	ENVIRONMENTAL CONTROL, MONITORING AND REPORTING

Accident and Incident Reporting

Vehicle & Plant Breakdowns

Site Inspections

Spillages

Surface and Ground Water

Containment Bunding

Odour

Dust

Noise and Vibration

Pests

Litter

5.0	APPENDICIES

Appendix A 	- Permit Transfer GB3603XP

Appendix A1   - Environmental Permit (EPR/AB3709FN)

Appendix B 	- Standard Rules (SR2008No25)

Appendix C 	- Generic Risk Assessment

Appendix D 	- Planning Permission Notice

Appendix E 	- sub-basement room of 21D67

Appendix F	- Waste site map

Appendix G	- TCM Competence (Certificates) original

Appendix H	- TCM Competence (Certificates) Validation

Appendix I	- Site Rules

Appendix J 	- APL Health, Safety and Environmental Policy

Appendix K 	- Site Drainage Plan

Appendix L	- Autoclave Daily Inspection Sheet

Appendix M 	- Spillage Clearance Procedure

Appendix M 	- AP EPR permit procedures

6.0	REFERENCES

Environment Agency guidance:

Environemtal Permitting Regulations 2010 (as amended)

(https://www.gov.uk/topic/environmental-management/environmental-permits)

Controlling and monitoring you emissions for an Environmental Permit (https://www.gov.uk/guidance/control-and-monitor-emissions-for-your-environmental-permit)

Health and Safety Executive legislation:

- Health & Safety at Work etc Act 1974

  - The Management of Health and Safety at Work Regulations 1999

  - The Control of Substances Hazardous to Health Regulations 2002 (as amended)

  - Health and Safety (First Aid) Regulations 1981

Site Emergency Procedures:

- Alderley Park, Health, Safety and Environment, Emergency Procedure

- Alderley Park, Health, Safety and Environment, Evacuation Procedure

- APL Business Continuity Plan for W&LF

APL Risk Assessments (RA) and Method Statements (MS):

- Waste section lockdown process flow Bruntwood waste 005

- Bio-hazardous waste process flow Bruntwood waste 008

- Hazardous Waste Process flow, Bruntwood waste 009 (to be rewritten for new permit)

- Non-hazardous and inert waste Process flow, Bruntwood waste 011

- Clinical Waste Process flow, Bruntwood waste 011

- Recycling Product Process flow Process flow, Bruntwood waste 014

- Battery disposal Process flow, Bruntwood waste 015 (to be updated)

- WEEE and Hazardous WEEE waste Process flow, Bruntwood waste 017 (to be updated)

- Collection of Infectious/Biological waste APL/Waste/RA-002 Bruntwood/Waste MS-002

- Disinfecting yellow Sulo bins Bruntwood waste/RA-013, Bruntwood/waste/MS-013

- Use of WLF Autoclave APL/waste/RA-001, Bruntwood/waste/MS-001

- Disposal of Clinical bins Bruntwood/waste/MS-004

- Cleaning internal floor of transfer station Bruntwood/waste/MS-012

- Fork lift truck duties APL/waste/RA 011, Bruntwood/waste/MS 011

- Spillage control – Bruntwood/waste/RA-014, Bruntwood/waste/MS-014

- Refuelling vehicles Bruntwood/waste/RA-016

- Noise from operation activities Bruntwood/waste/RA-024

1.0	GENERAL CONSIDERATIONS

Site operator/permit holder

The site, which is the subject of this site management system, is managed and operated by Bruntwood Management Services Limited (BMSL) as a service to its customers on site.

The Permit number is (GB3603XP).

A copy of the permit (Appendix A and A1) the site is operated in accordance with the waste operations described in standard rules SR2008 No25 (Appendix B) Bespoke permit applied for and associated Generic Risk Assessment Bespoke risk assessment completed (Appendix C).

The site has been registered with the Environment Agency as required under the Hazardous Waste (England and Wales) Regulations 2005. The Hazardous Waste Producer code is ALDERL (SK10 4TG) in line with the bespoke permit

The site will receive waste from site customers.  The contact names for the Waste facility are:

Andrew Butterworth (Chief Commercial Officer, Permit holder)	07808 714510

Mike Broadfoot TechMCIWM (TCM)				           07387 417925

Mike Broadfoot (Waste, Logistics Manager)			           07387 417925

Dawn White (Senior operations Manager, Bruntwood SciTech)	07801 622733

Louise Carney (Senior Health, Safety & Risk Manager)		07553 756449

John Topping (Risk Manager)			      		           07387 419882

Site history and planning status

The Alderley Park Waste Facility is situated in Block 21 within the Waste Logistics Facility.  The site address is Alderley Park Waste Logistics Facility, Block 21, Alderley Park, Macclesfield, and Cheshire, SK10 4TG. There are no residential buildings located within 400 meters of the site.

The site was granted planning permission by the Borough of Macclesfield to operate a clinical waste & healthcare waste treatment and transfer station on 30th May 2007.

A copy of the planning decision notice is attached to this document in Appendix D.

Permit Area

The boundary of the Environmental Permit is shown in the permit.  It includes the indoor waste area on the Basement floor of Block 21. All references to ‘the site’ in this working plan shall mean this area and the associated infrastructure, plant and equipment.

A bespoke permit is to replace the current permit, as requested by EA, permit no. 25 is to be terminated.

The bespoke permits boundary will be to the perimeter that encompasses the waste and logistics yard and two rooms inside, these are 21B85 and 21D67. This boundary must be reviewed.

Statement that follows is in preparation of the Permit. (Application with environmental agency)

The scope of the bespoke permit is to encompass the sub-basement room of 21D67 for the sorting of clinical/healthcare waste, plan of which is attached in Appendix E. In addition, an allotted area of the waste/logistics yard will be secured for the storage of clinical waste before collection as seen in Appendix F. This will also take in the current waste exemption T4 and S2

Hours of Operation

The site will be operated during the following hours for the receipt of waste and all other operations:

Monday to Friday 0600 – 15:00

Waste types and quantities

Permitted wastes are limited to non-hazardous and hazardous clinical and healthcare waste. The type of wastes accepted at the site is detailed in tables 2.3a, 2.3b, and 2.3c of the Standard Rules SR2008No25_75kte.

The waste is collected from various locations across Alderley Park.  The maximum quantity of hazardous waste to be received, stored, treated or repackaged for disposal shall not exceed, individually or aggregated 10 tonnes per day. The treatment of non-hazardous waste for disposal shall not exceed 50 tonnes per day. The total quantity of waste that can be accepted at the site must be less than 75,000 tonnes a year.

The application for the bespoke permit is intended to cover the above and also include the storage of hazardous waste under the Hazardous waste regulations 2005. This process will involve collection and storage. Transport, packing and disposal is by an approved competent supplier. This permit will also encompass WEEE waste (Shipping container to be provided for storage), Non-hazardous and recycling.

The only form of compaction on site is via a polystyrene briquette machine that operates on a corkscrew mechanism and friction compacts the product.

Waste inputs records are collated on a daily basis and inputted into an Excel spreadsheet. This real-time data is monitored to ensure that the permit conditions are not breached. Treatment of infectious clinical waste is by Autoclave in order to render the clinical waste safe, this is located in 21B78/21B85 on the basement floor of the same facility.

Management structure & responsibilities

The Environmental Permit (GB3603XP) is registered to Bruntwood Management Services Ltd “the operator” to operate waste operations described in standard rules SR2008 No25. The reprehensive for the permit owner is the Alderley Park Site Head of operations (the permit owner is Andrew Butterworth Chief Commercial Officer). The TCM and the Site Risk Manager are responsible for the management system and together with property managers ensures that the facility is maintained in compliance with the Permit.

The Manager (TCM Holder) is responsible for operations and compliance with the Environmental Permit.  Some brief descriptions of the responsibilities of those staff that are involved in operating the Alderley Park Waste Facility are outlined in Table 1 below.

Table 1: Responsibility of Role Function

| Position | Company | Responsibility/General management (Ref: EPR Compliance Assessment Report section C) |
| --- | --- | --- |
| Permit Owner | Bruntwood | To ensure that the management and operation of activities undertaken at the Alderley Park Waste Facility are carried out in accordance with the agreed procedures and with the Environmental Permit. |
| Waste Manager TCM holder | Bruntwood SciTec | The Waste Manager will have overall responsibility for onsite management and operations including being responsible for health and safety. The waste manager will ensure that: • The site is available to receive waste; • The arrangements for the removal of wastes from site is acceptable; and • The site is operating within the parameters of the Environmental Permit. |
| Risk Management (HS&E) | Bruntwood SciTec | Internal auditing of the waste facility to ensure compliance with Environmental Permit conditions. |
| The TCM holder | Bruntwood SciTec | The TCM holder will manage the operations onsite. ensure that: - All clinical waste and healthcare waste collection, treatment and transfer station operations are managed in compliance with the permit. - In the role of TCM, audit compliance of the permitted site. - The autoclave is operated, maintained and serviced in accordance with manufacturer’s instructions. - Ensure that legislations and regulations are followed in accordance with the scheme. - Ensure documentation is within compliance standards - Report to the permit holder any major issues for the running of the site. |
| Waste supervisor | Bruntwood SciTec | Is responsible and monitors for the day-to-day operations on site. |
| Facility Team Members (Waste Operators) | Bruntwood SciTec | Day to day operations (collections) of hazardous waste, Clinical, Radioactive, WEEE and Dry Waste and recyclables |

Whenever the site is open to receive or dispatch wastes, or is carrying out any of the waste management operations, it is supervised by at least one member of staff who is suitably trained and fully conversant with the requirements of the permit regarding:

- waste acceptance and control procedures;

- operational controls;

- maintenance checklist;

- record-keeping;

- emergency action plans;

- accident management plans; and

- Notifications to the Environment Agency.

Technical Competence

Operations at the site would be under the overall control of a technically competent person who holds the relevant Technical Competence Manager (TCM) under the Waste Management Industry Training and Advisory Board (WAMITAB) CIWM scheme. The technically competent person must demonstrate continuing competence by passing a periodic assessment which tests the candidate’s understanding of recent developments within the waste industry. A copy of the technically competent person’s TCM Certificate is attached at Appendix G and H.

The current Technical Competence Manager (TCM) clarified to:

1.	Level 4 Diploma in Waste Management Operations: Managing Physical & Chemical Treatment - Hazardous Waste - 4MPTH

2.	Level 4 Diploma in Waste Management Operations: Managing Thermal Treatment – Hazardous Clinical Waste – 4MTTHC

3.	Level 4 Diploma in Waste Management Operations: Managing Transfer – Hazardous Clinical Waste – 4MTSHC

For the Alderley Park Waste Facility permit (GB3603XP) a minimum attendance of 20% (i.e. 9 hours) is required per week.  Any changes in the technically competent management of the site including the name of the incoming person, together with evidence that such person has the required technical competence, will be submitted to the Environment Agency in writing within 5 days of the change in management.

Training and competence

All new employees are provided with Permit Regulation training, Environmental Legislation training and HS&E induction training. In addition, they are provided with site specific training as detailed in the training skills matrix dependent upon which area they are working. This ensures that they are trained and competent to carry out their duties.  All staff is familiar with the site emergency procedures.

Health & Safety

All operations on the Alderley Park site and permitted area will be carried out in accordance with the relevant requirements of the Health & Safety at Work etc Act 1974 and all other applicable legislation.   Site rules for employees, visitors and contractors will be shown to all site users and must be signed prior to using the site. Anyone not complying with the conditions of use will be asked to leave the site. A copy of the Site Rules is attached in Appendix I.

Fire extinguishers will be kept on site to deal with fires as stated in the Emergency Plan.  A first aid kit is kept on site and will be maintained to the required standard, i.e. as stated in the Health and Safety (First Aid) Regulations 1981.

APL HS&E Policy is attached in Appendix J

Alderley Park – Health, Safety and Environment, Emergency Procedures

The Management of Health and Safety at Work Regulations 1999 set out the need for all employers to have procedures for emergency situations and to ensure that all site personnel are aware of them. In order to meet the legal and company requirements, systems must be in place to identify the potential for, and responses to, emergency situations, and for preventing and mitigating the likely illness, injury or damage that may be associated with them.

This document outlines the arrangements for dealing with emergencies, involving when necessary external emergency services, on Alderley Park and includes escalation to crisis management processes.

Alderley Park – Health, Safety and Environment, Evacuation Procedures

The purpose of this document is to define the method of raising the alarm, and evacuation of both able bodied and disabled/temporary disabled people from buildings, in the event of an emergency situation at Alderley Park.

The document is applicable across all functions and staff, including visitors and contractors working on the Alderley Park site.

2.0	SITE ENGINEERING AND INFRASTRUCTURE

Access and parking

Access to the Alderley Park site is gained from Congleton Road (A34). The location of the site is shown on the site plan. There is sufficient parking space for all visitors available at Alderley Park in designated car parks.

Site Security

APL has engaged an approved supplier to carry out a background checking service for all new employees that provides proof that employees have been screened to BS7858:2012 – the recognised national standard.

In order to prevent unauthorised access to the Alderley Park Waste Facility, site security measures have been put in place. The permitted area is situated within the building and is only accessed by security cleared personnel via security fob system. The site is protected by 3m high palisade fencing around the perimeter.  There are signs warning people not to enter the site. Gates are lock when waste logistic facility is closed.

The access gates/roller shutter doors are closed at all times when the site is not operational (outside of working hours) and security lighting is used as appropriate. Gates and fencing are inspected at the commencement of each working day. Any defects or damage which compromises the integrity of the enclosure would be made secure by temporary repair by the end of the working day. Permanent repairs would be affected as soon as practicable. All inspections, defects, damage and repairs would be recorded in the site diary.

In the event of the fire alarm being activated area will be made safe and evacuated.  Additional security measures are provided on site through the use of CCTV, the areas covered by the CCTV system are 21B85/21B78, 21D67 and all external areas.

With regards to the bespoke permit. During operating hours the area is staffed and CCTV monitored this includes all areas of the waste logistics yard. Roller shutter doors are closed after hours and the waste section only opens to accept waste or remove. Additional CCTV is now installed, this watches the sub-basement waste area doorway. The site is protected by 3m high palisade fencing around the perimeter. This fenced area is locked out of hours and the onsite security takes over the monitoring of the waste facility.

Site office

The site office is used by operatives and the storage of documents is in the purpose built archive room within the site. A copy of the Environmental Permit and Management System are held in the site office.

Surface Water Drainage

Water collected from roadways and roofs will be directed via a series of gullies and gutters into the surface water drainage system as shown on the site drainage plan. A copy of the site drainage plan is attached in Appendix K.

Foul Water Drainage (combined drain for foul and trade effluent)

Trade Effluent is discharged into the public sewer under Consent to Discharge issued by United Utilities Water Plc. This is a legal document and contains the conditions relating to the effluent to be discharged. All companies that discharge effluent to drain are required to sign up to the site agreement with BMSL.

Waste transfer compound

The waste transfer compound comprises of a concrete impermeable floor surface laid to suitable falls to aid drainage. The surface is of sufficient strength for tipping of loads, storage of wastes and operating loading plant.

Storage of materials on site

Clinical waste - all clinical waste containers are stored in a purpose built area within the sub-basement. A maximum of 42 x 770 litre wheeled containers are stored in the area at any one time the remainder are stored in the fenced and gated area of the waste yard. Skips can be located in the waste yard for the storage and disposal of recyclables; scrap metal, wood, cardboard bales WEEE etc.

| 08 | WASTES FROM THE MANUFACTURE, FORMULATION, SUPPLY AND USE (MFSU) OF COATINGS (PAINTS, VARNISHES AND VITREOUS ENAMELS), ADHESIVES, SEALANTS AND PRINTING INKS |  |
| --- | --- | --- |
| 08 01 | wastes from MFSU and removal of paint and varnish |  |
| 08 01 11* | waste paint and varnish containing organic solvents or other hazardous substances | MH |
| 14 | WASTE ORGANIC SOLVENTS, REFRIGERANTS AND PROPELLANTS (except 07 and 08) |  |
| 14 06 | waste organic solvents, refrigerants and foam/aerosol propellants |  |
| 14 06 01* | chlorofluorocarbons, HCFC, HFC | AH |
| 14 06 02* | other halogenated solvents and solvent mixtures | AH |
| 14 06 03* | other solvents and solvent mixtures | AH |
| 15 | WASTE PACKAGING, ABSORBENTS, WIPING CLOTHS, FILTER MATERIALS AND PROTECTIVE CLOTHING NOT OTHERWISE SPECIFIED |  |
| 15 01 | packaging (including separately collected municipal packaging waste) |  |
| 15 01 01 | paper and cardboard packaging | AN |
| 15 01 02 | plastic packaging | AN |
| 15 01 03 | wooden packaging | AN |
| 15 01 04 | metallic packaging | AN |
| 15 01 07 | glass packaging | AN |
| 15 02 | absorbents, filter materials, wiping cloths and protective clothing |  |
| 18 | WASTES FROM HUMAN OR ANIMAL HEALTH CARE AND/OR RELATED RESEARCH (except kitchen and restaurant wastes not arising from immediate health care) |  |
| 18 01 | wastes from natal care, diagnosis, treatment or prevention of disease in humans |  |
| 18 01 01 | sharps (except 18 01 03) | AN |
| 18 01 02 | 2 Body parts and organs including blood bags and blood preserves (except 18 01 03) | AN |
| 18 01 03* | wastes whose collection and disposal is subject to special requirements in order to prevent infection | AH |
| 18 01 04 | wastes whose collection and disposal is not subject to special requirements in order to prevent infection(for example dressings, plaster casts, linen, disposable clothing, diapers) | AN |
| 18 01 06* | chemicals consisting of or containing hazardous substances | MH |
| 18 01 07 | chemicals other than those mentioned in 18 01 06 | MN |
| 18 01 08* | cytotoxic and cytostatic medicines | AH |
| 18 01 09 | medicines other than those mentioned in 18 01 08 | AN |
| 18 02 | wastes from research, diagnosis, treatment or prevention of disease involving animals |  |
| 18 02 01 | sharps (except 18 02 02) | AN |
| 18 02 02* | wastes whose collection and disposal is subject to special requirements in order to prevent infection | AH |
| 20 | MUNICIPAL WASTES (HOUSEHOLD WASTE AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |  |
| 20 01 | separately collected fractions (except 15 01) |  |
| 20 01 01 | paper and cardboard | AN |
| 20 01 02 | glass | AN |
| 20 01 11 | textiles | AN |
| 20 01 21* | fluorescent tubes and other mercury-containing waste | AH |
| 20 01 25 | edible oil and fat | AN |
| 20 01 27* | paint, inks, adhesives and resins containing hazardous substances | MH |
| 20 01 33* | batteries and accumulators included in 16 06 01, 16 06 02 or 16 06 03 and unsorted batteries and accumulators containing these batteries | AH |
| 20 01 36 | discarded electrical and electronic equipment other than those mentioned in 20 01 21, 20 01 23 and 20 01 35 | AN |
| 20 01 39 | plastics | AN |
| 20 01 40 | metals | AN |
| 20 03 | other municipal wastes |  |
| 20 03 07 | bulky waste | AN |

Radioactive waste

Radioactive waste is transferred into this facility.  N.B. The Radioactive Permits at Alderley Park for open and sealed sources are also in the process of being transferred to APL.

Treatment – Autoclave (Getinge Autoclave)

The autoclave is a fully automated/computerised process. Use is covered by method statement and risk assessment APL/waste/MS-001, RA-001. The autoclave takes approximately 1.5-2 hours per cycle.

Tests are carried out as it is being brought back to use on a Monday morning and results are recorded. During the autoclave cycle, temperature and pressure are monitored and printouts produced and kept. Chemical indicator strips are included in each batch for monitoring and again are recorded and kept with the records for that batch. Steam is provided for the process from the boilers and if the temperature drops below those required, the autoclave stops and won’t open – the cycle is restarted when the temperature returns to that required. A daily inspection sheet is attached to the autoclave. A copy is attached at Appendix L.  A daily record of the Autoclave performance is kept. This includes breakdowns and failures that have occurred which are also recorded on the autoclave percentage of failure spreadsheet.

Biological Indicator Testing of the Autoclave at block 21 is carried out by a competent service provider (Evotec UK ltd) every month. Following the testing Evotec provide a Biological Challenge Test Report.  Furthermore, Autoclave instrumentation is tested and calibrated by Getinge Ltd. Following testing Getinge Ltd provide a validation report.

Rejected Wastes

If it is suspected that waste which does not conform to that permitted under the Environmental Permit has been deposited at the site, it would be placed in a designated quarantine area. This waste would be removed as soon as possible to a suitably permitted facility. A record would be kept in the site diary of all rejected wastes. The waste producer and the Environment Agency would be notified of the non-conformance. All permit regulations will be followed.

Vehicles, plant and equipment

The vehicles, plant and equipment used at the site are detailed in Table 2:

Table 2:

| Vehicles & Plant | Activity |
| --- | --- |
| 3 ton luton van | Collection cardboard waste and plastic waste |
| Fork-lift Truck Diesel | Loading and unloading |
| Fork-lift Truck Electric | Loading and unloading |
| Cardboard baling machine | Operation, Loading and unloading |
| Double baling machine for drinks cans and plastic milk bottles | Operation, Loading and unloading |
| Polystyrene briquette Machine | Operation, Loading and unloading |

Vehicle drivers and FLT operators are required to complete pre-start check sheets before they start their shifts. The completed checklists are handed into the waste supervisor. The waste supervisor will arrange for any defects to be rectified as a matter of urgency.   All vehicles are provided by Bruntwood Management Services and maintained in accordance with manufacturer’s recommendations. The FLT is maintained every 12 months in accordance with the Lifting Operations and Lifting Equipment Regulations (LOLER). FLT Operators are trained and their licence is refreshed every 3 years.  All items of plant are on a maintenance schedule. The cardboard baler is provided by Bagnal and Morris and is maintained to the manufactures specification by Bagnal and Morris. All operator are trained by Bagnal and Morris to operate the machine and Risk assessments and Method statements are available.

Site Identification Board

A site identification board which is easily readable from outside the Waste Facility entrance during hours of daylight is provided at the main site entrance in accordance with the Permit requirement. The identification board is regularly inspected and in the event of damage or defect that significantly affects the legibility of the board it would be repaired or replaced within a timescale agreed with the Environment Agency.

The board displays the following information:

- site name and address;

- permit holder;

- permit number;

- environment Agency national telephone numbers; and

- Days and hours site is open to receive waste.

3.	SITE OPERATIONS

Assessment of Best Available Techniques.  BAT requirements in relation to operations and maintenance are defined in the Sector Guidance Note SGN S5.06 Treatment and Recovery of Hazardous Waste and the EA “Getting the Basics Right” document. BAT for operations and maintenance are outlined in the table 3 below.

| Table 3: BAT Assessment – Operations and Maintenance |  |  |
| --- | --- | --- |
| BAT Requirement | Site Justification |  |
| - Waste Acceptance | - Waste materials will only be accepted in accordance with the site permit. - The waste facility will operate in defined areas for material off loading and storage. Waste will only be accepted if there is sufficient storage capacity available. - The operator has contingency measures in place for alternative waste reception facilities in the event that the site cannot accept waste. - All loads will be visually inspected by site personnel during off-loading. |
| - Waste Storage | - Waste storage areas will be located away from watercourses. - All healthcare and hazardous Waste materials will be stored in leak proof containers with lids in designated bunded area. - WEEE will be store in containers on an impermeable surface. - All non-hazardous wastes are stored in an area where a 3m high palisade fencing surrounds the area and is locked out of hours. |
| - Operational Control | - The operator will have defined procedures for the control of operations, fugitive emissions monitoring and plant maintenance to mitigate/control any potential adverse impact on the environment. As seen the waste flows, standard operating procedures (SOP) - Audits of controls will be undertaken through the management system to ensure on-going effectiveness against defined performance measures. |
| - Maintenance | - The operator has a defined procedure for identifying, reviewing and prioritising plant and equipment preventative maintenance regimes. - The preventative maintenance system will cover all plant and equipment whose failure could lead to impact on the environment. - The maintenance system will include a performance monitoring facility and overall maintenance will be audited as part of the management system. |

Preliminary procedures

Guidance is given by the site management to all employees, sub-contractors, other waste carriers and customers regarding the waste types which are acceptable at the waste transfer site. The site is used primarily for the receipt of waste from Alderley Park Limited clients.

Waste collection procedures

Guidance is provided to all persons on site regarding the waste types which are acceptable at the waste transfer site (the site runs a 100% pre-acceptance policy). The site is used primarily for the receipt of waste from tenant companies based on site.

Bruntwood management services operate Three waste collection vehicles; Vehicle with a tail-lift and two FLT’s onsite. Drivers have received training in all aspects of safe collection of waste. All collected waste is either held in designated collection points or taken to the waste transfer facility for sorting, treatment, storage and disposal. A record is kept of all waste types collected.

Sorting and Storage

Clinical waste materials will be stored in leak proof containers with lids in designated area. Manually sorted in the permitted area, and repackaging will take place.

Treatment

All treatment activities take place within a building with impermeable surface and sealed drainage system. The permit does not permit the mixing of hazardous waste. The permit also does not permit the burning of wastes, either in the open, inside buildings or in any form of incinerator.

Duty of care

When transferring non-hazardous waste, a waste transfer note is completed. This document details of the waste type, quantity, method of carriage (i.e. skip, vehicle), date/time of the movement, operators detail as well as the carrier’s details. It describes the waste and the appropriate code from the European Waste Catalogue (EWC). The competent staff member and the waste carrier both sign the transfer note to show that the transfer has taken place. Records must be kept for 2 years.

Hazardous Waste

When transferring hazardous waste, a hazardous waste consignment note must be completed. The same information as above must be included. The list of wastes can be found at: www.environment-agency.gov.uk/ewc.  The five parts of the Consignment Note must be filled out in the correct order, by the appropriate person.

Before the waste is removed, the:

- Producer (or holder) of the waste must complete Parts A and B on each copy of the note, and gives all copies to the carrier.

- Carrier must check that journey origin and destination given in Part A, and the waste details given in Part B are correct. The carrier then enters their details in, and signs, Part C. All copies are handed back to the consignor.

- Consignor (i.e. producer or holder) must then confirm that the information supplied to the carrier in Parts A and B is correct, and sign Part D. The consignor keeps a copy and hands two back to the carrier.

After the carrier has removed the waste, the:

- Carrier delivers the waste to the consignee with two copies of the Consignment Note. The consignee checks the waste and the documentation, and either accepts or rejects the waste. The consignee completes Part E to show their decision; keeping one copy and handling one back to the carrier.

- Consignee must, once a quarter, send consignee returns confirming receipt of the waste to the producer/holder identified in Part A2, and the Environment Agency.

Waste Exemptions

The following Waste Exemptions will be utilised and applied for where applicable:

Exemptions T4 (Preparatory treatments (baling, sorting, shredding etc) (non-agricultural waste only)) Exemption reference: WEX383930

Exemptions S1 (Storage of waste in secure container (non-agricultural waste only)) and S2 (Storage of waste in a secure place (non-agricultural waste only))

Rejected Waste

If the waste has been incorrectly described on the Waste Consignment Note, the consignment must be rejected. However, if your permit or exemption allows you to receive the waste, according to the correct description, you can still accept it onto site, but you must do the following:

- Contact the producer/holder and confirm the new waste description and that you can receive the waste;

- Make a copy of the original Consignment Note;

- Complete the original copy in the manner described above;

- Add ‘R’ onto the Consignment Note number of the new note;

- Enter the correct details for the waste description;

- State that the waste has now been accepted;

- Give one copy of the new note to the carrier;

- Send a copy of the new note to the consignor and/or producer.

The details of both consignments must be entered onto the consignee return.

Washing and disinfecting containers

Clinical waste containers taken to the Bolton Hospital Incinerator at Bolton are emptied and then put onto a bin washing machine that rotates, cleans and disinfects the containers ready to be returned. The driver only collects on a ‘one for one’ return basis, i.e. deliver 10 containers, and return with 10 containers.

The internal waste storage area floor, hand rails and fitments are cleaned and disinfected on a rota system every 2 weeks.

Reusable containers at Alderley Park are inspected and any debris or residue is removed from containers Method statement (Bruntwood/waste/MS-013) and Risk Assessment (Bruntwood/waste/RA-013). The containers are then sprayed with disinfectant to neutralise the growth of disease-carrying microorganisms ready for re-use.  Any defects to containers are reported to the site waste manager and recorded. The site Waste manager arranges for the defective containers to either be repaired or scrapped in accordance with Method statement (Bruntwood/waste/MS-004).

Permit Surrender

To assist in permit surrender, records would be maintained to demonstrate how the land beneath the site has been protected at all times between the date of permit issue and the end of permit operations. Records to be maintained would include:

- maintenance of impermeable surfacing;

- maintenance of drains; and

- actions taken to clean up incidents and spillages.

Display of Environmental Permit

A copy of the Environmental Permit and Standard Rules SR2008No25_75kte is kept available for reference by all staff and contractors whose work may have an impact on the environment.

Managing Documentation and Records

Records pertaining to the site operation will be maintained in either electronic or hard copy form as follows:

a) electronic records will be saved to the internal shared network drive; and

b) paper records will be retained in designated filing system in the site office.

Records will be available on request. Operational records will be retained for at least 3 years in line with the current Environment Agency guidelines.  Controls are in place to ensure that all documents are issued, revised and maintained in accordance with the conditions of the permit. The documents that are included within the scope of the controls are as follows:

- policies, procedures and responsibilities;

- targets;

- maintenance records;

- monitoring records;

- results of audits;

- results of reviews;

- complaints, accidents and incidents records and reports; and

- Training records.

A record of the types and quantities of wastes accepted and dispatched from site will be maintained. A summary of the types and quantities of wastes deposited at the site will be provided to the Environment Agency at the agreed frequency and format.

4.0	ENVIRONMENTAL CONTROL, MONITORING AND REPORTING

All Environmental Accidents and Incidents are reported and investigated to establish learning.

Vehicle & Plant Breakdowns

In the event of breakdown of vehicles or fork lift trucks repairs will be actioned as a matter of urgency. If repairs cannot be resolved quickly an alternative vehicle or fork-lift truck will be sourced until the defect can be repaired.

Site inspections

The inspection form will be completed by a person who is familiar with the requirements of the permit for the site. All details of defects, problems and repairs will be recorded on the form on the day that each event occurs. Detailed comments will be recorded in the site diary.

Any major defects found during the daily site inspection which are likely to lead to a breach of permit conditions will be repaired by the end of the working day in which they are found where possible. If a repair is not possible by the end of the working day, the Environment Agency will be contacted by Bruntwood management services to agree a suitable timescale for repair.  All repairs to site security fencing will be reported on the discovery of the damage and the site will be made secure until the repair has been affected.

Spillages

Spillages are dealt with locally by operators using the spill kit.  Additional support is available from the Site Emergency Response Team by calling 01625238666

All site surfaces will be inspected daily when the site is in operation. Debris will be swept as required and placed in a skip.  Any spillages of fuel/oil will be cleared immediately by depositing sand or absorbents on the affected area. The sand or absorbents will be placed in a container to be taken to a suitably licensed site for disposal. All spillages of waste and windblown litter will be cleared by the end of the working day in which they occur. Spillage clearance procedures are detailed in Appendix M.

Drainage management.

Bruntwood management services are committed to the management and maintenance of drainage systems, to prevent contamination of the ground and business interruption through flooding or loss of service.

All areas of the site used for waste handling operations have impermeable concrete surfacing and sealed drainage. The site is operated to prevent fugitive emissions to surface water and groundwater. Liquids can only be discharged into a sewer subject to a consent issued by the local water company. Liquids may be taken off-site in a tanker for disposal or recovery.

Odour

There would be no point source emissions to air from waste management operations at the facility. Odour has been assessed as a fugitive emission. Incoming wastes would be subject to inspection by site personnel who would be aware of the permitted wastes that can be accepted at the facility and those which could be potentially malodorous and which could require special attention.

Dust

Due to the nature of the wastes accepted at the Waste Facility it is not anticipated that excessively dusty waste would arrive at the facility. Visual inspections would be carried out by the site operatives during the working day, especially during dry and windy conditions. The Site Manager would assess the need for remedial action and implement such action where necessary.

Noise & Vibration

It is not anticipated that site operations will cause a noise nuisance or vibration problem because the scale, location and vehicle/plant type used on site of the operation.

Pests

The facility would be inspected by both site management and operatives for infestations of pests, vermin and insects on a routine basis. If specific waste is found to be responsible for attracting scavengers, pests or infestation, this waste would be removed from the proposed waste facility as soon as practicable.  A specialist pest control contractor would be engaged if required.

Litter

All waste handling and storage would take place within the confines of the Waste Facility building. The boundary of the site and its environs would be regularly checked and any windblown litter collected and disposed of appropriately. It would be the responsibility of the site staff to constantly monitor the site for any signs of escaping materials either from within the storage bays themselves or from vehicles delivering or removing materials to and from the site. Inspections would be carried out on a daily basis and a record maintained within the site diary.

Appendices

| Appendix A | Appendix A1 | Appendix B | Appendix C | Appendix D |
| --- | --- | --- | --- | --- |
| Appendix E | Appendix F | Appendix G | Appendix H | Appendix I |
|  |  | TCM originals | TCM validation |  |
| Appendix J | Appendix K | Appendix L | Appendix M | Appendix N |