Packaging For Recycling/Reuse

EWCs Accepted – 15-01-10*, 150102, 150104

Container Sizes – IBCs, 25L, 60L, 120L, 205L, <25L, <1L only with prior consent and additional info supplier

Conditions - <1% contamination by volume, contaminants must be readily removable in aqueous conditions. No POPs, No Water/Air Reactive, Non-Highly Toxic, Non-malodorous, no residues of prescribed substances, No-pressurised containers, no metallic or fine organic powders that may present an explosion / reactivity hazard. Materials may be assessed upon receipt to confirm suitability by waste acceptance testing through onsite laboratory or information on chemical composition

Materials – Plastic, Steel, Glass

HP Codes Accepted – HP2 (with pre-treatment), HP3 (with pre-treatment) HP4, HP5, HP6 (cat 3,4 only), HP7, HP8, HP10, HP11, HP12, HP13, HP14, HP15

Hazard Statements Unsuitable for Treatment – Not accepted for Treatment

HP1 (or reactivity) – H200, H201, H202, H203, H204, H205

HP3 – H220, H221, H222, H223, H230, H231, H232, H240, H241, H242, H250, H251, H260, H261

HP6 – H300, H310, H330

Gases – H229, H280

Supplemental Hazards – EUH001, EUH006, EUH014, EUH018, EUH029

Suitable for Pre-treatment (see pre-treatment process)

HP2 – H270, H271, H272

HP3 - H225, H226, H228, H242

Pre-acceptance checks

Material must be clearly labelled with nature of previous contents and full list supplied for technical assessment. Containers must be nominally empty. Material must be presented palletised or in suitable containers to enable offloading a forklift truck

Containers must have lids and be sealed if they contain any liquid residues

Containers must be transported in accordance with ADR as appropriate

Acceptance Checks

Materials received are checked for compliance against pre-acceptance information supplied by customer against pre-acceptance information for suitability for washing process by site chemists. Any containers containing excess residues (>1%) and quarantined and customer is informed. Any containers received falling outside of scope of pre-acceptance information are quarantined pending approval of suitability to process or routing to third party disposal site (such as Highly Toxic materials or pressurised containers). Materials with excess residues will be assessed in-line with site waste acceptance procedures and residues sampled to enable compliant disposal. Where containers are received without previous contents being identified these are quarantined pending supplementary information supplied by the customer.