010 Environmental Risk Assessment

| Farm Name: Wheaton Ashton Farm | Applicant: Belmont Farms Ltd |
| --- | --- |

Date:  	3rd May 2024

Prepared by: 	The Farm Consultancy Group, using information provided by Mr Michael Rolston

| Source of emission | Emission e.g. odour, noise, dust, ammonia, run-off, spillage | Receptor e.g. air, water, land, humans, plants | Description of impact and duration of impact i.e. short term (ST), medium (MT) or long term (LT) | Significance of negative impacts: Major +++ Moderate ++ Minor + Nil o | Mitigation/management measures e.g. installation planning, technical measures |
| --- | --- | --- | --- | --- | --- |
| Pig production inc. cleaning out, feed storage, use of machinery | Ammonia | Air Land Plants | Direct toxic effect on trees (ST) Increased acidification of soil close to housing (LT) Changes to sensitive ecosystems (LT) | ++ + + + | Feed targeted to maximise FCR and minimise amount of ammonia produced. Reduced crude protein diet to match pig and sow requirement. Slatted floor system with frequent slurry removal. Roof high velocity ventilation. Pre application request completed by EA- detailed ammonia modelling prepared and screens out |
|  | Odour | Humans | Nuisance (ST) | + | See Odour Management Plan No odour complaints received. |
|  | Dust | Humans Plants Land Water Air | Nuisance (ST) Contributes to odours (ST) Health issues – inhalation (LT) Covers leaves reducing photosynthesis (ST) Nutrient enrichment of soils (LT) Nutrient enrichment (MT) Impacts on air quality (ST) | + o + o o o + | Regular cleaning of dust to prevent build- up within buildings, on roofs and around vents as part of disease control strategy. Production of dust minimised through no bedding required with this system. Dry feed system used predominantly automated feeding system with closed pipework prevents dust. |
|  | Noise | Humans | Nuisance (ST) | + | Reference Noise Management Plan No Odour Complaints Received |
|  | Zoonoses, notifiable diseases. | Humans Livestock | Bio security risks (ST) | ++ | Secure site with visitor policy (visitor book in place). Livestock monitored for signs of disease, and incidents reported quickly. Defra bio security guidance followed by all staff, contractors, and visitors. |
|  | Spillage of feed, litter, slurry, manure. | Land Water | Nitrogen and Phosphorus levels in excess of crop requirements in soil (ST) – (all slurry and manure currently exported so actual risk minimal) Potential for increased mineral or metal content of soils (LT) Possible toxic effects on wildlife (ST) Increased biochemical oxygen demand (BOD) of watercourses (ST) | ++ ++ +++ +++ | Slurry storage facilities in place, with all liquid contained. Any spills cleaned up and disposed of safely. As quick as practically possible. NVZ – regulations followed; all slurry exported. Watercourses nearby so risk of contamination increased. |
|  | Used disinfectants. | Water | Possible toxic effects on wildlife (ST) Increased biochemical oxygen demand (BOD) of watercourses (ST) | ++ ++ | Disposal into slurry lagoon. Use of Defra/NOAH approved disinfectants. Watercourses nearby so risk of contamination increased. |
| Use of vehicles | Soil | Land | Soil compaction (ST) Transfer of soil across and off the installation (ST) | o + | The majority of traffic moves on purpose built concrete or hardstanding tracks. Trafficking avoided when soil conditions are not favourable. Careful selection of machinery and tyres. Tyres operated at correct pressures. |
|  | Spillage of materials in transit. | Water | Increased biochemical oxygen demand (BOD) of watercourses (ST) Nutrient leaching from soil to surface waters and groundwater (LT) Nutrient enrichment (eutrophication) of watercourses and groundwater (LT) | o to ++ o to ++ o to ++ | Dusty materials moved within sealed containers or covered vehicles. Integrity of vehicles checked for leakages. Watercourses nearby so risk of contamination increased. |
|  | Odour, Noise | Humans | Nuisance (ST) | + | Reference Odour Management Plan and Noise Management Plan. |
| Disposal of carcasses inc. Fallen Stock Scheme. | Odour | Humans | Nuisance (ST) | + | Use of covered/sealed containers. Carcasses disposed of as soon as practically possible by collection. At least once a week. |
|  | Emissions of acid gasses | Plants Land Air Humans | Direct toxic effect on trees (LT) Nutrient enrichment of soils (LT) Increased acidification of soil close to housing (LT) Changes to sensitive ecosystems (LT) Production of greenhouse gasses (LT) Irritation of eyes, nose and throat (ST) | o o o o + + | Equipment maintained to optimum standards as per SVS approval. Machinery and equipment sited as far as possible from neighbours. Compliant with legislation Animal By-products Regulations. |
|  | Pests | Humans Livestock | Nuisance caused by vermin or flies (ST) Biosecurity risks (ST) Biosecurity risks (ST) | + ++ ++ | Use of covered containers. All equipment disinfected after use. Carcasses disposed of as soon as practically possible. (Weekly Collections) |
| Waste storage e.g. dirty water tanks, manure and slurry Stores | Odour | Humans | Nuisance (MT) | ++ | Slurry storage compliant with NVZ regulations, Frequent slurry removal reduces odour build up in the pig buildings. Slurry Store Slurry lagoons. Slurry tanks being installed. |
|  | Ammonia and acid | Air Land Plants | Direct toxic effect on trees (ST) Nutrient enrichment of soils (LT) Increased acidification of soil close to housing (LT) Changes to sensitive ecosystems (LT) | + + o + | Slurry storage compliant with NVZ regulations, the slurry lagoons do not require stirring due to the slurry separator system. Slurry tanks to be installed. Frequent slurry removal reduces odour build up in the pig buildings. Slurry reception pit covered with pipework below slurry level. Slurry lagoons. Slurry tanks being installed. |
|  | Dirty water | Water | Increased biochemical oxygen demand (BOD) of watercourses (ST) Nutrient leaching from soil to surface waters and groundwater (LT) Nutrient enrichment (eutrophication) of watercourses and groundwater (LT) | ++ ++ ++ | Tanks regularly checked for integrity. Drainage in place so all dirty water pumped slurry lagoon. Minimal dirty water produced. Site produces clean water, or any dirty water is part of the slurry system. |
|  | Odour | Humans | Nuisance (ST) | ++ | All slurry exported off unit. Pipework constructed as per SSAFO regulations and weekly maintenance check carried out with any repair work completed immediately. |
|  | Noise | Humans | Nuisance (ST) | + | All slurry exported off unit. Pipework constructed as per SSAFO regulations and weekly maintenance check carried out with any repair work completed immediately. |
|  | Dust | Humans | Nuisance (ST) Health issues – inhalation (LT) | o o | All slurry exported off unit. Pipework constructed as per SSAFO regulations and weekly maintenance check carried out with any repair work completed immediately. |
| AD Biogas plant | Odour Ammonia Dust Spillages | Air Land Water | Direct toxic effect on trees (ST) Nutrient enrichment of soils (LT) Increased acidification of soil close to housing (LT) Changes to sensitive ecosystems (LT) | ++ | Serviced as required per manufacturers guidance Daily recording of performance parameters All pipework bolted down and protected from accidental damage. |
| Waste storage e.g. dirty water tanks, manure and slurry lagoons and input tanks | Odour | Humans | Nuisance (MT) | ++ | Slurry storage compliant with NVZ regulations, emptied out to reduce odour. Ridged cover to be installed on slurry storage tanks. Frequent slurry removal reduces odour build up in the pig buildings. Reference Odour Management Plan. |
|  | Ammonia | Air Land Plants | Direct toxic effect on trees (ST) Nutrient enrichment of soils (LT) Increased acidification of soil close to housing (LT) Changes to sensitive ecosystems (LT) | + + o + | Slurry storage compliant with NVZ regulations, emptied out to reduce odour, only stirred when emptying to allow crust to develop. Reference Odour Management Plan. |
|  | Dirty water | Water | Increased biochemical oxygen demand (BOD) of watercourses (ST) Nutrient leaching from soil to surface waters and groundwater (LT) Nutrient enrichment (eutrophication) of watercourses and groundwater (LT) | ++ ++ ++ | Tanks regularly checked for integrity. Minimal dirty water produced. Soya tank bunded. |
| Non-organic waste storage and disposal | Waste materials, packaging etc | Water Land Air | Residues and products of degradation | + | Policy to avoid where possible. Dedicated storage areas and facilities. Collection by licensed contractors for recycling or disposal. |
| Fire | Odour | Air Humans | Nuisance (ST) Direct Toxic effect (MT) | + | No unauthorised storage of chemicals and up to date list of inventories made available for the fire services. Call fire services as soon as safely possible and do not tackle fires alone. |
|  | Ammonia | Air Humans Plants | Direct toxic effect on trees (ST) Nutrient enrichment of soils (ST) Increased acidification of soil close to housing (ST) | + | No unauthorised storage of chemicals and up to date list of inventories made available for the fire services. Call fire services as soon as safely possible and do not tackle fires alone. |
|  | Dirty water | Water | Increased biochemical oxygen demand (BOD) of watercourses (ST) Nutrient leaching from soil to surface waters and groundwater (LT) Nutrient enrichment (eutrophication) of watercourses and groundwater (LT) | + | No unauthorised storage of chemicals and up to date list of inventories made available for the fire services. Call fire services as soon as safely possible and do not tackle fires alone. All water used will be directed to the slurry tanks for removal. |
| Power Failure | Odour | Air Humans | Nuisance (ST) Odour from inefficient biomass boiler and unventilated poultry houses. | ++ | Plan to get power back on as soon as possible. Use Accident Management Plan and use back-up generator as soon as possible. |
|  | Ammonia | Air Humans Plants | Direct toxic effect on trees (ST) Nutrient enrichment of soils (ST) Increased acidification of soil close to housing (ST) | + | Plan to get power back on as soon as possible. Use Accident Management Plan and use back-up generator as soon as possible. |
|  | Dirty water | Water | Increased biochemical oxygen demand (BOD) of watercourses (ST) Nutrient leaching from soil to surface waters and groundwater (LT) Nutrient enrichment (eutrophication) of watercourses and groundwater (LT) | o | Plan to get power back on as soon as possible. Use Accident Management Plan and use back-up generator as soon as possible. - all yard water drains into the dirty water catchment tanks. |
|  | Noise | Air Humans | Nuisance (ST) | + | Plan to get power back on as soon as possible. Use Accident Management Plan and use back-up generator as soon as possible. Stop using generator as soon as mains power returns |