Site Condition Report

| 1.0 SITE DETAILS |  |
| --- | --- |
| Applicants Name | Hazledene Ltd |
| Activity Address | Butlers Bank Farm Shawbury Shrewsbury Shropshire SY4 4HG |
| National Grid Reference | 415852,544121 |
| Document Reference for Site Condition Report at permit application and surrender | 12th March 2026 |
|  | Location Plan |
|  | Layout/Drainage Plan |

| 2.0 Condition of the Land at Permit issue |  |
| --- | --- |
| Environmental setting including: Geology Hydrogeology Surface Waters | The pullet rearing unit lies on level ground to the northeast of Edgebolton, with the surrounding area being flat. Predominant land use is arable and grazing. Field pattern is semi large scale with a medium pattern of field boundaries. There are nearby residences within 100 metres from the poultry house, the nearest being 46m to the west of the installation boundary. Topography and Drainage Butlers Bank Farm poultry operations lie on a level area to the northeast of Edgebolton, draining to French drains, leading to two soakaways. The poultry houses are between 70 and 75 metres above sea level. Careful choice of approved building material colours help to minimise the visual intrusion normally associated with poultry units. Geology and Hydrogeology The British Geological Survey shows the solid geology beneath the site to be of the Permian period, comprising Bridgenorth Sandstone Formation - Sandstone Searches indicate the site is not located in a Ground Water Protection Zone. Site is located over a Bedrock Aquifer – Principal. Site is located over a Surface Aquifer – Secondary A. Ground water vulnerability – Medium/High. The site is situated in a Surface NVZ area. Site is located in a drinking water surface safeguard zone The site, and clean water drainage is directed to French drains and two soakaways. Average rainfall for this area is 1150mm. Hydrology General drainage flow is in a southerly direction. River Roden is situated 960m to the west of the installation boundary. |
| Pollution history including: Pollution incidents that may have affected land Historical land uses and associated contaminants Any visual/olfactory evidence of existing contamination Evidence of damage to pollution prevention measures | None noted Polluting substances – None noted Previous use – Arable farming. None noted None noted |
| Evidence of historical contamination | N/A |
| 3.0 Permitted Activities | Double decker house, for pullet rearing. This will give a total of 64,000 places. Heating will be provided by direct fired LPG heaters. The working area where vehicles operate is laid to concrete and hard standing. Feed is delivered in covered lorries and stored on site in vermin proof steel galvanised bins. Manure is removed from house by a belt system twice weekly. Immediately following depopulation, litter is removed off site, and used on operator controlled land under a manure management plan adhering to the DEFRA Codes of Good Agricultural Practices and sufficient available spreading acreage. The houses are then washed and disinfected prior to the cycle beginning again. Underground storage tanks will have been installed to catch all wash waters. Dead birds are removed from the houses and stored in freezers awaiting collection by a licensed agent. Diesel fuel storage is in a bunded tank. |
| Non permitted activities undertaken | N/A |
| Document References | Location Plan Layout/Drainage Plan H1 Assessment |
| 4.0 Changes to Activity | N/A |
| 5.0 Measures taken to protect Land | Site will be operated in compliance with “how to comply” routine maintenance schedules are followed and recorded and with any abnormal operations recorded. |
| 6.0 Pollution Incidents | N/A |
| 7.0 Soil, gas and water quality monitoring | No monitoring will be undertaken at the installation. |