Site Condition Report

| 1.0 SITE DETAILS |  |
| --- | --- |
| Applicants Name | Rodenhurst Hall Ltd |
| Activity Address | Rodenhurst Hall Farm, Roddington, Telford, Shropshire SY4 4QU |
| National Grid Reference | 358816,315867 |
| Document Reference for Site Condition Report at permit application and surrender | 8th July 2020 |
|  | Location Plan |
|  | Layout/Drainage Plan |

| 2.0 Condition of the Land at Permit issue |  |
| --- | --- |
| Environmental setting including: Geology Hydrogeology Surface Waters | The poultry unit lies on level ground to the north of Roddington. Predominant land use is arable farming. Field pattern is semi large scale with a strong pattern of field boundaries. There are sensitive receptors within 400m of site. Topography and Drainage The poultry houses are between 50 and 60 metres above sea level. Soil bunding and tree planting will help to minimise the visual intrusion normally associated with poultry units. Site clean water drainage all goes to soakaway trenches. Geology and Hydrogeology According to the BGS Extract Map shows the site geology to be of Permian period of Rock Type – Bridgnorth Sandstone Formation. Searches indicate the site is underlain with a Bedrock Aquifer – Principal. The Site is situated in a Groundwater Vulnerability Zone, classified Med/Med High. The site is situated within a Surface Nitrate Vulnerable Zone. The site is not situated in, or within 250m of its boundary, to a Groundwater/Source protection zone. Hydrology General drainage is in an easterly direction. Average rainfall for this area is 1100 mm. The River Roden lies some 240m to the north east. |
| Pollution history including: Pollution incidents that may have affected land Historical land uses and associated contaminants Any visual/olfactory evidence of existing contamination Evidence of damage to pollution prevention measures | None noted Polluting substances – None noted Previous use prior to 2020 – Arable Farming/Grazing. None noted None noted |
| Evidence of historical contamination | N/A |
| 3.0 Permitted Activities | Planning permission is being sought for 4 poultry houses for Broiler chicken. This will give a total of 200,000 places. Ground source heat pump heating will provide hot water for heating the poultry houses. The working area where vehicles operate is laid to concrete. Dust deposited on grassed area by end gable fans will prevent run off to water course. Feed is delivered in covered lorries and stored on site in vermin proof steel galvanised bins. Immediately following depopulation, litter is removed off site and used on operator controlled land. The houses are then washed and disinfected prior to the cycle beginning again. Underground storage tanks will have been installed to catch all wash waters. Dead birds are removed from the houses and stored in a temperature controlled store awaiting incineration in a licensed incinerator Diesel fuel storage is in a bunded tank/storage area. |
| Non permitted activities undertaken | N/A |
| Document References | Location Plan Layout/Drainage Plan H1 Assessment |
| 4.0 Changes to Activity | N/A |
| 5.0 Measures taken to protect Land | Site will be operated in compliance with “how to comply” routine maintenance schedules are followed and recorded and with any abnormal operations recorded. |
| 6.0 Pollution Incidents | N/A |
| 7.0 Soil, gas and water quality monitoring | No monitoring will be undertaken at the installation. |