Document Reference: Environmental Risk Assessment Towens Westonzoyland

ERA 06102025

Date: 06 October 2025

Prepared for: Towens of Weston Ltd

Prepared by: Towens of Weston Ltd & MTS Environmental Ltd

Quality Control

| Revision No. | Date Revised | Amendments | Authored By | Sign Off | Approved By | Sign Off |
| --- | --- | --- | --- | --- | --- | --- |
| 1.0 | 09/11/22 | Original Draft for permit variation | Kasia Haywood |  | Luke Bridges |  |
| 2.0 | 16/10/24 | Amendments due to changes | Hattie Miller |  | Luke Bridges |  |
| 3.0 | 06/10/2025 | Amendments due to changes | Hattie Miller |  | John Telling |  |

DISCLAIMER: This document was prepared by MTS Environmental Ltd solely on behalf of the Client (Towens of Weston Ltd). MTS accepts no responsibility or liability for any use that is made of this document other than the purpose for which it was originally commissioned and prepared.

This document may contain information of a specialised and/or highly technical nature and the Client is advised to seek clarification on any elements which may be unclear to it. Information, advice, recommendations and opinions in this document should only be relied upon in the context of the whole document and any documents referenced explicitly herein and should then only be used within the context of the appointment.

Information reported herein may be based on the interpretation of public domain data collected by MTS and/or information provided by the Client. These data have been accepted in good faith as being accurate and valid

Contents

1. Introduction	3

2. Environmental Risk Assessment	3

2.1. Methodology	3

2.2. Potential Hazards	3

2.3. Pathways	4

2.4. Receptors	4

2.4. Risk Assessment	5

3. Summary	5

## 1. Introduction

Towens of Weston Ltd is applying for a variation to their existing environmental permit (permit number: EPR/HB3004KS) for its Westonzoyland site at Former Airfield, Land off A372, Westonzoyland, TA7 0JS. The permit is being changed to amend the activities on site to include washing. The main activities on site will still consist of soils and aggregate recovery and recycling to produce products to be re-used in construction, highways and utilities.

This Environmental Risk Assessment (ERA) is limited to a qualitative assessment of the potential risks to the environment and human health specifically related to the activities, including the proposed activities, undertaken at the Towens of Weston Ltd Westonzoyland site. This report will identify any significant risks and detail the measures that Towens of Weston Ltd will adopt to appropriately manage any risk of pollution.

## 2. Environmental Risk Assessment

### 2.1. Methodology

This report has been prepared following the Environment Agency’s Risk Assessment guidance. It specifically relates to the potential risks associated with odour; noise and vibration; fugitive emissions and accidents and incidents.

This risk assessment addresses the above risks and is based on the following methodology:

      - Identification of potential risks

      - Identification of all potential receptors to these risks

      - An assessment of each risk type.

The environmental risk assessment (Appendix A) assesses the risks to the environment and human health from activities carried out at the Towens of Weston Ltd Westonzoyland depot and identifies the pollutant linkage i.e. source – pathway – receptor for each risk type.

### 2.2. Potential Hazards

The potential hazards resulting from the activities carried out at the Towens of Weston Ltd Westonzoyland depot have been considered, as provided in Appendix A, and are summarised below:

- Odour:

- Waste materials

- Noise and vibration:

- Engine noise from vehicles

- Use of reverse vehicle warnings

- Use of plant and machinery

- Fugitive emissions:

- Particulate matter i.e. dust

- Scavenging birds, pests, and vermin

- Mud and litter

- Accidents:

- Fire

- Leaks and spillages

- Flooding

- Unauthorised access

### 2.3. Pathways

The pathways identified for each risk type are shown in Table 1:

Table 1: Potential Pathways

| Risk Type | Pathway |
| --- | --- |
| Odour | Air |
| Noise and vibration | Air |
| Fugitive emissions | Air |
| Accidents | Air |
|  | Surface water run-off |
|  | Infiltration |
|  | Percolation |

### 2.4. Receptors

Receptors within 1000m of the application site have been identified and are shown in Table 2 below and in the Sensitive Receptor Plan (Appendix B). The main pathway for the identified sources is the air and as such, atmospheric conditions can affect dispersion rates and the potential risk. Therefore, the location of each receptor in relation to the site may influence the potential impact of the risk, as summarised in Table 2.

Table 2: Location of potential receptors in relation to waste operations

| Receptor | Distance from site (m) | Direction |
| --- | --- | --- |
| Residential |  |  |
| Caravan Park | 155m | South East |
| Properties off A372 | 860m | North West |
| Manor Farm | 900m | North |
| Springway Farm | 260m | North |
| Property on Place Drove | 670m | South West |
| Designated Land and Waterways |  |  |
| Site of Special Scientific Interest (SSSI) – Langmead and Weston Level | 90m | South |
| SSSI – Langmead and Weston Level | 700m | South West |
| Priority Habitat Inventory (PHI) – Lowland Meadows | 185m and further | South |
| PHI – Lowland Dry Acid Grassland | 955m | South East |
| PHI – Coastal and Floodplain Grazing Marsh | 40m | South |
| PHI – Deciduous Woodland | 870m | East |
| Important Plant Areas Plantlife (GB) | 90m | South |
| Higher Level Stewardship Target Area – Somerset Levels and Moors | 0m | All directions |
| Rhynes | 145m | South and West |
| Sensitive Land Uses |  |  |
| Middlezoy Aerodrome | 400m | East |
| Farm | 115m | North West |
| Westonzoyland Allotments | 680m | North West |
| Playing Field | 700m | North West |
| Westonzoyland Airfield | 450m | North |
| Control Tower | 765m | North |
| Industrial/Commercial |  |  |
| Chard H J & Sons | 100m | East |
| Kleen Kutt | 264m | East |
| Bridgwater Motorcycle Training | 590m | North West |
| Grandfields Westonzoyland Motor Track and Learner Driver Area | 900m | East |
| T&K Motorcycle Training | 910m | East |
| Hall G – Vehicle Repair | 950m | North |
| Regency | 170m | North East |
| Slabs R Us | 210m | North |
| Public Rights of Way |  |  |
| Restricted Byway | 90m | South West |
| Public Footpath | 620m | South East |
| Infrastructure/utilities |  |  |
| A372 | 390m | North |
| Protected species |  |  |
| Priority Species - Curlew | 0m | North and West |
| Priority Species - Lapwing | 0m | All directions |
| Priority Species - Redshank | 0m | North and West |
| Priority Species - Snipe | 920m | West |
| Groundwater |  |  |
| The site is not within a source protection zone or drinking water safeguard zone |  |  |

### 2.4. Risk Assessment

The Environmental Risk Assessment (Appendix A) looks at each specific hazard identified and assesses the likelihood of those hazards impacting on nearby receptors. This is achieved by fulfilling the following objectives:

      - Identify the location and nature of each hazard

      - Identify the specific receptors potentially at risk and assess the sensitivity of each receptor

      - Provide an assessment of the risk posed to each sensitive receptor

      - Identify management and monitoring techniques to remove or mitigate the risk

      - Provide recommendations for more detailed assessments where necessary.

## 3. Summary

The Environmental Risk Assessment indicates that if the appropriate outlined management techniques are implemented at the site to protect nearby sensitive receptors, the proposed activities as part of the permit variation will have no significant impacts in terms of odour, noise and fugitive emissions, and the likelihood of accidents is minimal.

Appendix A – Environmental Risk Assessment

Table A1: Odour Risk Assessment and Management Plan

| What is the risk? |  |  | Managing the risk | Assessing the risk |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hazard | Receptor | Pathway | Risk Management | Probability of Exposure | Consequence | What is the overall risk? |
| Odorous Waste Types | Local population in residential dwellings and businesses listed in Table 2 SSSI Site Staff | Air transport then inhalation | Permitted waste types stored onsite are not putrescible and so have a low odour potential. All non-conforming or odorous wastes are stored in a weatherproof, covered building in the quarantine area. There will be strict waste acceptance procedures in place to minimise the risk of non-compliant wastes being accepted. Details of the waste acceptance procedures are provided in the Environmental Management System (EMS). All site operatives will be vigilant regarding identifying non- compliant wastes and any non-conformances or odour issues will be reported to the Site Manager. | Very unlikely as the waste types accepted on site do not give off odour unless heated and the material will be stored at ambient temperature | Odour annoyance and complaints | Low |

Table A2: Noise and Vibration Risk Assessment and Management Plan

| What is the risk? |  |  | Managing the risk | Assessing the risk |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hazard | Receptor | Pathway | Risk Management | Probability of Exposure | Consequence | What is the overall risk? |
| Noise and vibrations from loading and unloading of waste | Local population in residential dwellings, sensitive land uses, and businesses listed in Table 2 SSSI | Air and vibration | All noise generating activities will be undertaken between the hours of 07:00 to 17:00 Monday to Friday and 07:00 to 13:00 on Saturday, except for emergency repairs. All plant and machinery will have effective silencers where practicable and be maintained in accordance with the manufacturer’s requirements to minimise the risk of mechanical failure which could result in increased noise emissions. The loading/unloading of wastes will be undertaken in a controlled manner to keep noise/vibration to a minimum. Vehicles will be directed by site operatives to minimise the drop height when depositing loads at the site. All noise and vibration generating activity will be monitored closely and site operatives will be vigilant and report any excessive noise or vibration issues to the Site Manager. The site boundary has a 2.5m high concrete block wall to reduce potential noise levels and contain noise within the site boundary. Noise to be managed in accordance with site-specific Noise Management Plan. | Intermittent noise disturbance | Noise annoyance and complaints | Low |
| Vehicle movements on site | Local population in residential dwellings, sensitive land uses, and businesses listed in Table 2 SSSI | Air | Loads will only be delivered to the site during working hours (07:00 to 17:00 Monday to Friday and 07:00 to 13:00 on Saturday). The delivery of waste will take place in a controlled manner by limiting drop heights to keep noise to a minimum. Designated one-way haul road for vehicles on site to avoid contact with materials. Speed limit of 10mph on site. The site boundary has a 2.5m high concrete block wall to reduce potential noise levels and contain noise within the site boundary. All plant and machinery will have effective silencers where practicable and be maintained in accordance with the manufacturer’s requirements to minimise the risk of mechanical failure which could result in increased noise emissions. An anti-idling policy ensures that all equipment and vehicles when not in regular use shall be switched off. The Site Manager will be responsible for ensuring the above measures are implemented. Anti-beeping policy in place on site to prevent unnecessary vehicle noise. All noise generated by vehicle movements will be monitored closely and site operatives will be vigilant and report any excessive noise or vibration issues to the Site Manager. Noise to be managed in accordance with site-specific Noise Management Plan. | Intermittent during operating hours | Intermittent noise and vibration disturbance | Low |
| Use of plant and machinery. | Local population in residential dwellings, sensitive land uses, and businesses listed in Table 2 SSSI | Air | All noise generating activities will take place during working hours (07:00 to 17:00 Monday to Friday and 07:00 to 13:00 on Saturday), except for emergency repairs. All plant and machinery will have effective silencers where practicable and be maintained in accordance with the manufacturer’s requirements. All equipment and vehicles, when not in regular use, shall be switched off. The Site Manager will be responsible for ensuring the above measures are implemented. Processing activities will be done on a campaign basis to avoid continuous on/off use and noise nuisance. Minimal plant and machinery is on site, consisting only of a Matec Aggretec 150 plant, shovel, excavator, crusher and mobile water bowser. Locations have been considered to minimise noise production: Matec Aggretec 150 and crusher are located in the north western side away from sensitive receptors. The site boundary has a 2.5m high concrete block wall to reduce potential noise levels and contain noise within the site boundary. | Intermittent during operating hours. | Intermittent noise and vibration disturbance. | Low |
|  |  |  | All noise generating activity will be monitored closely and site operatives will be vigilant and report any excessive noise or vibration issues to the Site Manager. Noise to be managed in accordance with site-specific Noise Management Plan. |  |  |  |
| Noise from reversing vehicle warnings. | Local population in residential dwellings, sensitive land uses, and businesses listed in Table 2 SSSI | Air | All noise generating activities will take place during working hours (07:00 to 17:00 Monday to Friday and 07:00 to 13:00 on Saturday) except for emergency repairs. All noise and vibration generating activity will be monitored closely and site operatives will be vigilant and report any excessive noise or vibration issues to the Site Manager. Designated one-way haul road for vehicles on site to minimise the need for reversing except when tipping into the tipping area or collecting materials from bays. The site boundary has a 2.5m high concrete block wall to reduce potential noise levels and contain noise within the site boundary. Noise to be managed in accordance with site-specific Noise Management Plan. | Intermittent during operating hours. | Intermittent noise disturbance. | Low |
| Noise from processing of waste materials (crushing, screening) | Local population in residential dwellings, sensitive land uses, and businesses listed in Table 2 SSSI | Air | All noise generating activities will take place during working hours (07:00 to 17:00 Monday to Friday and 07:00 to 13:00 on Saturday) except for emergency repairs. Crushing activities will not generate levels of noise above that originating from the surrounding roads and industrial area. All plant and machinery will have effective silencers where practicable and be maintained in accordance with the manufacturer’s requirements to minimise the generation of noise. All plant and equipment will be switched off when not in regular use. Processing activities will be done on a campaign basis to avoid continuous on/off use and noise nuisance. Minimal plant and machinery is on site, consisting only of a Matec Agrretec 150 plantshovel, excavator, crusher, and mobile water bowser. Locations have been considered to minimise noise production: Matec Aggretec 150 and crusher are located in the north western side away from sensitive receptors. The site boundary has a 2.5m high concrete block wall to reduce potential noise levels and contain noise within the site boundary. All noise and vibration generating activity will be monitored closely and site operatives will be vigilant and report any excessive noise or vibration issues to the Site Manager. Noise to be managed in accordance with site-specific Noise Management Plan. | Intermittent during operating hours | Intermittent noise disturbance | Low |

Table A3: Fugitive emissions risk assessment and management plan

| What is the risk? |  |  |  | Managing the risk | Assessing the risk |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hazard | Receptor |  | Pathway | Risk Management | Probability of Exposure | Consequence | What is the overall risk? |
| To Air |  |  |  |  |  |  |  |
| Dust emissions from vehicle movements | Local population in residential dwellings, sensitive land uses, and businesses listed in Table 2 Site Staff Users of roads listed in Table 2 SSSI |  | Air transport then deposition | Wastes being delivered to the site will be covered or sheeted to prevent the generation of dust while the waste is in transit. Vehicle speeds will be limited onsite and the access road to 10mph to prevent re-suspension and movement of dust. All equipment and vehicles when not in regular use shall be switched off to minimise the risk of dust emissions that may arise from idling. Designated one-way haul road for vehicles on site to avoid contact with materials. The implementation of dust suppression systems including the use of spray on crushing plant, mobile water bowser with spray hose and regular maintenance of haul roads and site surface with a water bowser and road sweeper. The site perimeter is surrounded by a 2.5m high concrete lego block wall which will act as a screen for dust so no dust escapes from the site boundary. Dust will be managed in accordance with the Dust Management Plan prepared for the site. The Site Manager undertakes a daily visual assessment of dust levels and all site operatives will be vigilant and report any problems to the Site Manager. | Unlikely due to measures in place | Local nuisance i.e. dust on cars, clothing, and vegetation. Nutrient enrichment. | Low |
| Dust emissions generated during unloading of waste from HGVs. |  | Local population in residential dwellings, sensitive land uses, and businesses listed in Table 2 Site staff Users of roads listed in Table 2 SSSI | Air transport then deposition | A water bowser will be used to dampen site haul roads, site surface, storage bays and stockpiles if necessary. The loading/unloading of wastes will be undertaken in a controlled manner by limiting drop heights to keep dust emissions to a minimum. Wastes will be stored in bays to prevent wind whipping. Designated one-way haul roads and paths for HGVs to avoid contact with waste. Drop heights will be minimised to reduce the generation of dust whilst the waste is being handled. The site is enclosed within a 2.5m high concrete wall surrounding the site perimeter will act as a screen for dust so no dust escapes from the site boundary. Dust will be managed in accordance with the Dust Management Plan prepared for the site. | Dust could potentially reach nearby properties when a strong wind blows in their direction. Management actions should prevent this happening | Local nuisance i.e. dust on cars, clothing, and vegetation. Nutrient enrichment. | Low |
|  |  |  |  | The Site Manager will undertake a daily visual assessment of dust levels and all site operatives will be vigilant and report any problems to the Site Manager. Operations will temporarily cease when winds are likely to generate dust emissions from wastes and materials. |  |  |  |
| Dust from haul road |  | Local population in residential dwellings, sensitive land uses, and businesses listed in Table 2 Users of roads listed in Table 2 SSSI | Air transport then deposition | The use of modern plant and regular maintenance shall be practiced to reduce emissions. The implementation of dust suppression systems including the use of spray on crushing plant, mobile water bowser with spray hose and regular maintenance of haul roads and site surface with a water bowser and road sweeper. The haul road on the site will be wetted down daily. Designated one-way haul roads and paths for HGVs to avoid contact with waste. Dust will be managed in accordance with the Dust Management Plan prepared for the site. The Site Manager undertakes a daily visual assessment of dust levels and all site operatives will be vigilant and report any problems to the Site Manager. | Unlikely due to measures in place | Local nuisance i.e. dust on cars, clothing, and vegetation. | Low |
| Dust emissions from the processing of waste materials (crushing and screening) |  | Local population in residential dwellings, sensitive land uses, and businesses listed in Table 2 Users of roads listed in Table 2 SSSI | Air transport than deposition | All plant is regularly maintained to reduce emissions. The implementation of dust suppression systems including the use of spray on crushing plant, mobile water bowser with spray hose and regular maintenance of haul roads and site surface with a water bowser and road sweeper. The site benefits from its own plant so operations can be done on smaller volumes more frequently to ensure that stockpiles are kept to a smaller size. This reduces dust/litter from stockpiles and protects the surrounding receptors. No crushing takes place in very dry or windy conditions. All plant and equipment will be switched off when not in regular use. All processing activities conducted within the 2.5m high boundary concrete lego block wall to prevent dust escaping and increase the number of dust barriers between receptors. Processing activities will be done on a campaign basis to avoid continuous on/off use and dust nuisance. Minimal plant and machinery is on site, consisting only of a Matec Agrretec 150 plant, shovel, excavator, crusher, and mobile water bowser. Locations have been considered to minimise noise production: Matec Aggretec 150 and crusher are located in the northwestern side away from sensitive receptors. The site is enclosed within a 2.5m high concrete wall to prevent dust escaping from the site. Dust will be managed in accordance with the Dust Management Plan prepared for the site. The Site Manager undertakes a daily visual assessment of dust levels and all site operatives will be vigilant and report any problems to the Site Manager | Unlikely due to measures in place | Local nuisance i.e. dust on cars, clothing, and vegetation. Nutrient enrichment. | Low |
| Release of particulate matter (dusts), vapours and polluting gases |  | Local population in residential dwellings, sensitive land uses, and businesses listed in Table 2 Site staff Users of roads listed in Table 2 SSSI | Air transport then inhalation | Permitted waste types do not include dusts, powders or loose fibres and waste is not typically dusty unless it is stored during prolonged dry periods when damping down is carried out where required. Asbestos containing waste is not accepted on site, so the release of asbestos fibres is extremely low. All non-conforming waste is stored in a quarantine area to prevent cross contamination of waste and is removed from site within 14 days. Operatives manually handling waste will wear suitable personal protective equipment (PPE) where required to mitigate against exposure to substances. The potential sources of fugitive emissions to air have been identified and a Dust Management Plan has been prepared to prevent any potential dust emissions from reaching any nearby receptors. The Site Manager undertakes a daily visual assessment of dust levels and all site operatives will be vigilant and report any problems to the Site Manager | Unlikely due to measures in place | Respiratory illness including. lung cancer and mesothelioma. | Low |

| To Water |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Contaminated rainwater run-off. | Surface water and groundwater SSSI | Water | Hazardous wastes are not accepted on site so the risk of contaminated run off is low. Any non-conforming wastes will be stored in a separate bay on an impermeable surface in an area of sealed drainage. The site surface is impermeable concrete which prevents the leaching of contaminants into groundwater. In the event of a spill, emergency procedures as outlined in the EMS will be followed. The onsite sustainable drainage system collects all surface water run off from site via gullies, settlement tanks and holding tanks. The run-off water collected is re-used on site. Fuel and oil will be stored on site in a fully bunded bowser within the secure fuel store container. The site is secure through 2.5m high concrete lego block walls, secure lockable gates, intruder alarms and CCTV so theft or damage which may cause spills is minimal. A comprehensive flood risk assessment and flood containment plan has been conducted which the site will operate in accordance with. | Very unlikely | Contamination of groundwater surface water bodies | Low |
|  |  |  | There are strict waste acceptance procedures in place at the site to prevent the acceptance of non-conforming waste types. Details of the waste acceptance procedures are provided in the EMS. |  |  |  |
| Pest/Scavenging Birds |  |  |  |  |  |  |
| Birds and pests | Local population in residential dwellings, sensitive land uses, and businesses listed in Table 2 SSSI | Air transport and over ground | Permitted wastes stored onsite are not putrescible and will therefore not be attractive to pests or scavenging birds. Green waste is not accepted on site so there will be no useable nesting sites available. Hazardous wastes are not accepted on site and any non-conforming wastes are stored within a weatherproof, covered building, on an impermeable surface, which is secured outside of operating hours so will not be accessible to birds or pests. There are strict waste acceptance procedures in place at the site to prevent the acceptance of non-conforming waste types. Details of the waste acceptance procedures are provided in the EMS. The site is enclosed within 2.5m high secure fencing and perimeter walls to deny the access of pests. Limited open stockpiles to attract birds and pests. Materials are stored in segregated bays to prevent cross-contamination and encouraging odour that may attract birds and pests. | Very unlikely due to the nature of the waste material | Nuisance to local receptors within 1km of the environmental permit boundary. | Low |
|  |  |  | The Site Manager will undertake regular reviews of pests and scavenging birds at the site. All site operatives will be vigilant and report any problems to the Site Manager. |  |  |  |

| Mud |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Mud from vehicle movements | Users of local highways listed in Table 2 | Tracked on vehicle wheels. | The implementation of dust suppression systems including the use of spray on crushing plant, dust sprinklers and regular maintenance of haul roads and site surface with a water bowser and road sweeper. If mud is deposited on the access road and/or highway then a road sweeper will be employed if necessary. All vehicles exiting the site would be checked for exterior mud or debris before leaving the site. The site benefits from a wheel wash station consisting of a jet wash hose and brush. Designated one-way haul road for HGVs on site to minimise contact with materials and tracking of mud. The amount of mud on local roads will be monitored daily by site operatives. | Unlikely due to measures in place. | Local nuisance. Mud on roads is unsightly and can increase the likelihood of road traffic accidents. | Low |
| Litter |  |  |  |  |  |  |
| Litter | All receptors listed in Table 2. | Air transport then deposition | Waste types received by the site generally do not contain litter that could become windblown. Any waste types that may generate litter will be stored in dedicated bays. Operatives will be vigilant, and any litter reported will be removed immediately. All incoming loads will be sheeted and remain sheeted until they are ready to be tipped. The site perimeter is enclosed by 2.5m perimeter concrete walls to prevent any litter being blown from the site. Designated one-way haul road for HGVs to avoid contact with waste. There are strict waste acceptance procedures in place to prevent the acceptance of non-conforming waste types. Details of the waste acceptance procedures are provided in the EMS. Working areas will be regularly cleared and inspected to minimise litter. Housekeeping measures are in place during operating hours. | Unlikely due to measures in place. | Local nuisance | Low |

Table A4: Accident and Incident Risk Assessment and Management Plan

| What is the risk? |  |  | Managing the risk | Assessing the risk |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hazard | Receptor | Pathway | Risk Management | Probability of Exposure | Consequence | What is the overall risk? |
| Fire or failure to contain firewater | Air transport then inhalation or deposition Groundwater and surface water. Local residents listed in Table 2 SSSI | Infiltration and contamination of surface water | No combustible waste is accepted on site. If any incidental combustible waste is brought to site, then it will be transferred and stored in the quarantine area in containers made of fire-resistant materials to prevent fires spreading. Hazardous and combustible wastes are not accepted on site. No waste shall be burnt on site and the use of welding/cutting tools (tools with a naked flame) are sanctioned first by the site manager/competent person. All site operatives are required to recognise signs of smouldering waste at the point of reception. Such wastes shall remain in the container and removed to the quarantine area. The site manager shall be informed. There will be strict waste acceptance procedures in place at the site to prevent the acceptance of non-conforming waste types. Details of the waste acceptance procedures are provided in the EMS. The operator will undertake routine maintenance of equipment in accordance with manufacturer’s guidance. This will minimise the risk of mechanical failure which may result in an increased risk of combustion. | Unlikely | Contamination of local groundwater and/or surface water. | Low |
|  |  |  | Site notices and training will be undertaken regarding fire hazards. |  |  |  |
|  |  |  | Site Manager will be responsible for actions in the event of a fire. Fire booms, spill kits, fire blankets and extinguishers, sand and buckets available on site. The site benefits from a sealed drainage system with settlement tanks and holding tanks which is closed loop and prevent fire water from entering the local watercourse. The drainage system and use of fire booms provides a sufficient capacity volume for containing firewater in a worst-case scenario. |  |  |  |
| Leaks and spillages of oil or fuel. | Groundwater and surface water. SSSI | Infiltration | The operator will undertake regular maintenance of plant equipment in accordance with manufacturer’s guidance. This will minimise the risk of mechanical failure which may result in leaks. All fuel, oil and lubricants stored on site will be double-bunded and stored in a secure container. The storage will be maintained and inspected in accordance with the manufacturer’s recommendations. Daily vehicle / plant checks to ensure any fuel/oil leaks etc. are repaired as soon as possible. Hazardous and combustible wastes are not accepted on site. The site benefits from a sealed drainage system with settlement tanks and holding tanks which is closed loop and prevent fire water from entering the local watercourse. Spill kits are readily available on site in case of a spill, these use absorbent mats which soak up any contaminating hydrocarbons. The emergency response outlined in the EMS will be followed. The Site Manager will be responsible for ensuring effective remediation and documenting any incident. | Unlikely due to measures in place. | Contamination of land and watercourses. | Low |
| Flooding | Groundwater | Infiltration and Percolation | The site is in an area at risk of flooding from rivers or surface waters. A Flood Risk Assessment and flood containment plan has been developed to manage this risk. The whole site is surfaced in impermeable concrete. Hazardous wastes are not accepted on site and any non-conforming wastes will be stored in a separate bay on an impermeable surface with a positive sealed drainage system. Waste types stored in open bays are inert or non-hazardous so in the event that surface water comes into contact with these wastes, significant pollution or contamination of groundwater or surface water is considered unlikely. The site benefits from a sealed drainage system with settlement tanks and holding tanks which is closed loop and prevent fire water from entering the local watercourse. | Unlikely due to measures in place in the nature of the proposed development. | Disruption to works operations Contamination of local groundwater and/or surface water | Low |
| Vandalism | Local population in residential dwellings, sensitive land uses, and businesses listed in Table 2 | Unauthorised entry to the site | The site is gated with CCTV and intruder alarms, is surrounded by 2.5m high security walls, fencing and vegetation. Access to the waste area will be restricted to trained depot staff. Hazardous wastes are not accepted on site. All fuel, oil and lubricants will be double-bunded and stored within a secure container. The storage will be maintained and inspected in accordance with the manufacturer’s recommendations. Any identified damage to the site security will be recorded and temporarily repaired as necessary before the end of the working day. Permanent repair or replacement will be undertaken as soon as practicable. Procedures are in place which require all visitors to the site to sign in on arrival and sign out on departure. | Unlikely due to measures in place. | Release of polluting materials to air, water or land. | Low |
| All on-site hazards from wastes; machinery and vehicles | Local human population gaining unauthorised entry to the site, site staff and contractors | Direct physical contact | Activities will be managed and operated in accordance with an EMS which will include measures to prevent unauthorised access. Hazardous waste are not accepted on site and any non-conforming wastes will be stored in a weatherproof, covered building which is locked outside of operating hours. | There is always a risk of accidents, but measures have been put in place to reduce the risk associated with site activities. | Injury or health effects | Low |

Appendix B – Sensitive Receptor Plan