Dust Management Plan

Contents

Introduction	3

Our Values	4

Obligations	4

Brief History	4

Main Regulations Addressed	5

Site Setting	5

Site Design and Receptor List	6

Wind Rose and Source of Weather Data	8

Dust-Emitting Materials Stored Onsite	9

Waste Storage	10

Potential Sources of Dust and Emission Points	10

Control Measures	10

Sensitive Receptors	13

Training	14

Complaints	14

Appendix A – Site Plan Layout	15

Appendix B – Site Map	16

Appendix C – Site Plan Footprint	17

# Introduction

Roles and Responsibilities

L&C Skip Hire (the operator) has the overall responsibility for ensuring this plan is adhered to, with specific responsibilities for:

- Ensuring that staff and contractors working onsite receive adequate information, instruction and training regarding the content of this plan.

- Ensuring the adequate provision of resources, e.g., personal protective equipment (PPE), dust suppression media.

- Ensuring that provision and maintenance of the above is adequate.

Overall responsibility for dust prevention and management lies with the senior management of L&C Skip Hire.

Purpose

The primary purpose of this Dust Management Plan (DMP) is to identify potential sources of dust and the associated controls required to prevent or minimise the risk of dust emissions arising as a result of site activities.

Scope

This Dust Management Plan is a standalone document that also forms part of the EMS with the individual waste acceptance and handling procedures for the site forming part of the DMP. This DMP has been prepared in accordance with the latest Environment Agency guidance. It relates to the storage of potentially malodorous waste, both pre-and post-treatment, at the site. All waste materials stored at the site have been considered in this plan.

The measures prescribed in the EA approved DMP will be regularly reviewed to ensure that operations on site remain compliant with the DMP at all times. The ongoing review will also ensure that the approved document remains in conformance with the latest version of the guidance. This DMP will also meet the insurance requirements for the business.

Any changes to operations that will impact upon the DMP will be submitted to the Environment Agency prior to implementation.

Following this DMP will ensure that:

- Potentially malodorous sources of waste are identified.

- Control measures for the control of malodorous sources are implemented.

- The impact of the effects of dust on people, the environment and other businesses is eliminated or minimised.

# Our Values

We recognise the potential impact of our business on the society and environment in which we operate. We aim to ensure that our values are widely communicated throughout each business activity and that they provide the foundations on which we will continue to develop and implement a robust corporate responsibility at all levels.

As the company grows, it understandably faces more complex and challenging situations. What we do as a business must go hand in hand with how we behave in the face of health, safety and environmental challenges, and addressing them effectively will help us get the business right.

# Obligations

Our obligations are:

- To ensure that current procedures remain effective and relevant to a changing business.

- To ensure that information relevant to health and safety is provided to our employees.

- The provision of employee training relevant to the role undertaken.

- The design and maintenance of safe facilities and processes.

- To ensure that our operations are safe for employees, contractors, members of the public and other visitors to our facility.

# Brief History

The business started operations in June 2010 and has quickly become a reliable and efficient waste management company providing a skip hire service in Teesside and the surrounding areas for both commercial and residential.

The facility is a waste transfer station currently permitted to accept no more than 75te per year of household, commercial and industrial waste, with asbestos storage. This permit application is for a variation to allow operations to be expanded to accept waste electrical and electronic equipment (WEEE).

The company has previously been audited against ISO standards 14001 and 9001, and OHSAS 18001, having achieved certification in April 2017.

This Dust Management Plan has been generated in support of a new environmental permit variation to cover additional EWC codes, particularly those pertaining to WEEE. L&C Skip Hire intends to expand its operations and construct a new shed for the sorting of WEEE.

# Main Regulations Addressed

The following guidance, legislation and regulations apply to this Dust Management Plan:

- Environment Agency core guidance on dust management and the controlling and monitoring of emissions for environmental permits.

- Environmental Protection Act 1990 (Part III) on statutory nuisance.

- Health and Safety at Work etc. Act 1974.

- The Provision and Use of Work Equipment Regulations 1998.

- The Management of Health and Safety at Work Regulations 1999.

# Site Setting

The site is situated on Smiths Dock Road, South Bank, TS6 6UJ.

The site comprises approximately 0.6Ha and is situated approximately 520 metres north of the nearest residential receptors. Adjacent businesses include scrap handling and scaffolding companies, as well as the Teesworks industrial estate and freeport. The Teesmouth and Cleveland Coast SPA and SSSI are situated approximately 650 metres to the north of the site boundary.

The site predominantly consists of an open yard, with a covered bay for the receipt and processing of received waste. This waste is then separated by a conveyor stream that runs along the western boundary, first of all separating out fine material before the larger fractions are separated by site personnel in a closed building. A quarantine zone for asbestos wastes is also located on this part of the site. Facilities for the handling of WEEE are to be constructed along the south-western boundary of the site. A small workshop for the servicing of site equipment is also in this area. Storage facilities, the site weighbridge, welfare facilities and offices are located on the eastern side of the site.

The site is fully enclosed with CCTV.

The site consists of hardstanding, concrete structures where all activities are carried out. There are no instances of unmade ground within the site boundary. Drainage consists of sealed interceptors, as well as connection to the foul network maintained by Northumbrian Water. There are no direct discharges to the River Tees.

L&C Skip Hire operate under an existing environmental management system (EMS), which identifies the potential sources of pollution from site activities, and implements the required controls to eliminate or minimise the effects of these activities on human health and the environment.

The DMP forms part of the EMS.

# Site Design and Receptor List

This section identifies potential sensitive receptors located within one kilometre of the site. Sensitivity may be assessed using the below criteria as set out by the Institute of Air Quality Management’s guidance for the assessment of dust soiling:

High – users can reasonably expect enjoyment of a high level of amenity; or the appearance, aesthetics or value of their property would be diminished by soiling; and the people or property would reasonably be expected to be present continuously, or at least regularly for extended periods, as part of the normal pattern of use of the land. Indicative examples include dwellings, museums and other culturally important collections, medium and long term car parks and car showrooms.

Medium – users would expect to enjoy a reasonable level of amenity, but would not reasonably expect to enjoy the same level of amenity as in their home; or the appearance, aesthetics or value of their property could be diminished by soiling; or the people or property wouldn’t reasonably be expected to be present here continuously or regularly for extended periods as part of the normal pattern of use of the land. Indicative examples include parks and places of work.

Low – the enjoyment of amenity would not reasonably be expected; or property would not reasonably be expected to be diminished in appearance, aesthetics or value by soiling; or there is transient exposure, where the people or property would reasonably be expected to be present only for limited periods of time as part of the normal pattern of use of the land. Indicative examples include playing fields, farmland (unless commercially-sensitive horticultural), footpaths, short term car parks and roads.

| No. | Receptor | Receptor Type | Direction from Site | Approx. distance from the site boundary to the receptor boundary (m) | Sensitivity to Dust / Particulates |
| --- | --- | --- | --- | --- | --- |
| 1 | Smiths Dock Road | Infrastructure | East | 23 | Low |
| 2 | J Gunn Scaffolding | Commercial | South | 47 | Medium |
| 3 | Teesworks | Industrial/Commercial | East | 85 | Medium |
| 4 | Saltburn-Darlington Railway Line | Infrastructure | South | 141 | Low |
| 5 | German Auto Breakers | Commercial | South | 208 | Medium |
| 6 | Asda South Bank | Commercial/Retail | South | 404 | Medium |
| 7 | SK Chilled Foods | Commercial | South West | 423 | Medium |
| 8 | AVG Biogas Plant | Industrial | South West | 442 | Low |
| 9 | PD Ports | Industrial/Commercial | North West | 486 | Medium |
| 10 | Teesport Commerce Park | Industrial/Commercial | North West | 503 | Medium |
| 11 | A66 | Infrastructure | South | 507 | Low |
| 12 | Residential Dwelling | Residential Dwelling | South | 561 | High |
| 13 | St Peter’s Catholic Church | Place of Worship | South West | 590 | High |
| 14 | David Fox Transport | Haulage | East South East | 697 | Medium |
| 15 | River Tees | SPA, SSSI, Ramsar | North | 736 | High |
| 16 | Golden Boy Green Community Centre | Community Centre | South South East | 830 | High |

# Wind Rose and Source of Weather Data

There is no onsite weather station. The wind rose presented below shows the prevailing wind direction for Teesside, taken from Iowa State University Iowa Environmental Mesonet (IEM). This is generated from observations between 01/01/1973 to 03/01/2026.

The wind rose shows the prevailing wind direction to be from the south or south-west. This corresponds with observations from the site and surrounding areas. The infrastructure of the site is such that the covered bay for the initial receipt and processing of wastes acts as a barrier for any particulate matter that may be affected by the prevailing wind direction. Furthermore, the site is enclosed by palisade fencing.

# Dust-Emitting Materials Stored Onsite

| Dusty and Potentially Dusty Material | Dust Potential (High, Medium, Low Risk) | Maximum Quantity Onsite (m3) | Maximum Time Held Onsite | Location of Dusty Materials |
| --- | --- | --- | --- | --- |
| Municipal Waste | High | 28.2 | 1 week | Shed |
| Plastics | High (light fractions) | 28.2 | 1 week | Skip, western boundary |
| Green Wastes, including soils | High | 28.2 | 1 week | Skip, western boundary |
| Cardboard/Paper | Medium | 28.2 | 1 week | Skip, western boundary |

The most and worst-case scenario of total waste being stored onsite at any one time is 130 tonnes.

# Waste Storage

Unprocessed waste arrives at site and is unloaded. The waste is pushed into the shed where it is then sorted by means of recycling into the following recyclable materials:

- Recycled plastics.

- Cardboard.

- Wood

- Other recyclable materials.

Once this practice has been carried out stockpiles are created. Waste is turned periodically by mechanical means to prevent the possibility of self-combustion. This is recorded in the site diary.  Waste that needs to be stored elsewhere is stored in containers outside of the building.

Taking into account the addition of new waste streams as part of this permit application, the maximum allowable waste accepted per annum will be 100,000 tonnes. This averages out at approximately 8,300 tonnes of waste arriving per month. Given the fact the waste is being transported out when manual recycling has been carried out, it is estimated that no more than 130 tonnes of waste will be on site at any one time.

# Potential Sources of Dust and Emission Points

| Location/Emission Points | Activity | Waste Type | Risk |
| --- | --- | --- | --- |
| Site Entrance | Arrival/departure of vehicles | All | Medium |
| Treatment Shed | Unloading/sorting | Municipal | High |
| Segregation Area | Sorting/storage | Municipal | High |
| Plastic Skips | Storage/unloading/loading | Plastics | Low |
| Cardboard Skips | Storage/unloading/loading | Cardboard | Medium |
| Green Waste Skips | Storage/unloading/loading | Green waste | High |

# Control Measures

| Dusty/Potentially Dusty Material | Control Measures | Monitoring Frequency and Parameters | Trigger Level | Action to be Taken if Outside Normal Parameters |
| --- | --- | --- | --- | --- |
| Incoming/outgoing movements | Deliveries to be scheduled to minimise queuing | Scheduling and monitoring | Vehicles queuing at site entrance; particulate emissions | Rescheduling. Engines to be switched off when not in use |
|  | Cleaning of vehicles | Constant / visual inspection / maintenance | Residue build-up | Vehicles to be cleaned |
|  | Prevention of build-up of material on site roads | Constant / housekeeping practices | Build-up of dusty material on road surfaces; particulates liberated by vehicle movements | Roads to be cleaned; site briefings and Toolbox Talks. Source of materials to be confirmed in case material is being introduced from offsite due to large-scale remediation work taking place on neighbouring site; liaison with Redcar & Cleveland Council as required |
| Loading/unloading of dusty material | Lighter fractions to be bagged at collection point. Tipping activities to be undertaken in enclosed area with minimum drop heights | Constant / acceptance procedures | Lighter fractions leaving unloading area; significant volume of smaller fractions (e.g., individual plastic bottles, household waste) arriving onsite in unbagged state | Load to be rejected or, if accepted, quarantined / isolated / prioritised for treatment. Damping down as required |
|  | Material to be stored onsite for minimum period of time to prevent build-up of waste within unloading area, material to be processed upon arrival | Constant / routine, regular collections | Build-up of material resulting in poor housekeeping with potential increase in dust potential | Scheduled collections to be undertaken at increased frequency / discussions with waste producer |
| Processing of waste (picking area) | Enclosed within sealed building / first in first out for processed wastes | Constant / ensure waste is processed within dedicated area | Waste not processed, allowed to build up | Site briefings, Toolbox Talks / prioritise processing / suspend collections if necessary to allow backlog to be cleared |
| Storage/handling of light fractions, including municipal and green wastes | First in, first out, limited storage time | Constant / site inspections / oldest waste removed first | Non-compliance identified by site management | Site briefings, Toolbox Talks, prioritise processing, temporarily suspend collections, water suppression to dampen affected area |
|  | Storage in enclosed shed (canopy building) and skips | Constant / activities undertaken in designated, enclosed area / daily inspections | Non-compliance with procedure / damage to building resulting in gaps in structure | Site briefings, Toolbox Talks, repairs as soon as practicable |
| Storage of DMR | First in, first out, limited storage time, covered storage | Constant / site inspections / oldest waste removed first | Non-compliance identified by site management | Site briefings, Toolbox Talks, prioritise processing, temporarily suspend collections |
|  | Storage in enclosed shed (canopy building) / storage in skips | Constant / activities undertaken in designated, enclosed area / daily inspections | Non-compliance with procedure / damage to building resulting in gaps in structure | Site briefings, Toolbox Talks, repairs as soon as practicable |
| General housekeeping | Bays and containers regularly emptied and cleaned. Site roads kept clean | Constant / site inspections, including site perimeter / housekeeping; information logged in site diary | Build-up of waste | Site briefings, Toolbox Talks, prioritise cleaning |

# Sensitive Receptors

There are a number of receptors within one kilometre of the site, including commercial, industrial and residential receptors, as well as the Teesmouth and Cleveland Coast SPA, SSSI and Ramsar site. Any businesses and residents in close proximity to the site and in the direct path of the prevailing wind will be notified in the event of a significant incident. Where deemed necessary, the local media will be contacted to inform local residents of the incident and to avoid the area.

The prevailing wind direction in the area is predominantly south-westerly, meaning that the receptors most likely to be affected by a significant incident include the Teesworks freeport and industrial estate, and the River Tees itself, which comprises the aforementioned protected areas.

# Training

Briefing on the contents of this DMP is undertaken for all new personnel with refreshers as required, alongside familiarisation with the site EMS.

# Complaints

All complaints will be recorded as part of the site environmental management system. The Site Manager is responsible for ensuring that these are fully investigated, signed and dated with remedial actions identified. The following information will be recorded:

- Contact details where available (name, address, telephone number, email)

- Date and time of complaint

- Date and time of event (if known)

- Description of dust/windblown material

- Duration (if known)

- Frequency

- Weather conditions (general, wind speed, direction)

- Activities being undertaken (if known, any unusual activities either on or offsite)

- Remedial actions identified

Where required, the Environment Agency will be notified of any event that contravenes the environmental permit within 24 hours of the breach being identified. Activities identified as giving rise to dust will be temporarily suspended until emissions are brought back under control.

This DMP and its associated controls will be reviewed and updated accordingly following any incident investigation that validates a dust complaint.

If notified by the Environment Agency that site activities are giving rise to dust, the DMP will again be reviewed and updated.

Where significant dust issues are identified, immediate neighbours (within 200 metres of the site) will be contacted to inform them of the situation and the action being taken. The Environment Agency will also be notified.

Complaints from neighbouring properties will be logged and investigated and feedback provided.

# Appendix A – Site Plan Layout

# Appendix B – Site Map

# Appendix C – Site Plan Footprint