Dust and Emissions Management Plan

High View Farm Waste Transfer Station Environmental Permit Application

West London Composting Limited

Prepared by:

SLR Consulting Limited

Treenwood House, Rowden Lane, Bradford on Avon, BA15 2AU

SLR Project No.: 402.065523.00001

10 October 2025

Revision: V2

Basis of Report

This document has been prepared by SLR Consulting Limited (SLR) with reasonable skill, care and diligence, and taking account of the timescales and resources devoted to it by agreement with West London Composting Limited (the Client) as part or all of the services it has been appointed by the Client to carry out. It is subject to the terms and conditions of that appointment.

SLR shall not be liable for the use of or reliance on any information, advice, recommendations and opinions in this document for any purpose by any person other than the Client. Reliance may be granted to a third party only in the event that SLR and the third party have executed a reliance agreement or collateral warranty.

Information reported herein may be based on the interpretation of public domain data collected by SLR, and/or information supplied by the Client and/or its other advisors and associates. These data have been accepted in good faith as being accurate and valid.

The copyright and intellectual property in all drawings, reports, specifications, bills of quantities, calculations and other information set out in this report remain vested in SLR unless the terms of appointment state otherwise.

This document may contain information of a specialised and/or highly technical nature and the Client is advised to seek clarification on any elements which may be unclear to it.

Information, advice, recommendations and opinions in this document should only be relied upon in the context of the whole document and any documents referenced explicitly herein and should then only be used within the context of the appointment.

Table of Contents

Basis of Report	i

1.0	INTRODUCTION	1

1.1	Scope of DMP	1

1.2	Key Guidance	2

2.0	BASELINE ENVIRONMENT	3

2.1	Site Location	3

2.2	Other Sources of Dust	3

2.3	Air Quality	3

2.4	Sensitive Receptors	4

2.4.1	Sensitive Receptors	4

2.5	Meteorological Conditions	7

3.0	OPERATIONS AT THE SITE	9

3.1	Site Operations	9

3.2	Waste Acceptance	9

3.3	Storage	10

3.4	On-Site Transportation	11

3.5	Off-Site Transportation / Import and Export Operations	11

3.6	Mobile Plant and Equipment	12

3.7	Potential Dust Sources and Magnitude	12

4.0	DUST AND PM10 MANAGEMENT	15

4.1	Source-Pathway-Receptor Routes	15

4.2	Control of Fugitive Dust/Particulate Emissions	16

4.3	Other Considerations	20

4.4	Responsibility for Implementation	20

4.5	Visual Dust Monitoring	21

5.0	DUST COMPLAINTS PROCEDURE	23

5.1	Engagement with the Community	23

5.2	Management Responsibilities	24

5.3	DMP Update and Review	24

List of Tables

Table 1 Other Potential Sources of Dust Emissions	3

Table 2 Defra Mapped Background Particulate Matter Concentrations	4

Table 3 Sensitive Human Receptors	4

Table 4 Dust Release Inventory	12

Table 5 Source-Pathway-Receptor Routes	15

Table 6 Control Measures for Dust/PM10 and Other Emissions	17

Table 7 Dust Management Responsibilities	20

Table 8	Proposed Non-Hazardous Waste Types to be Accepted at the Site	26

Table 9  Proposed Clinical Waste Types to be Accepted at the Site	35

Table 10  Proposed Asbestos Waste to be Accepted at the Site	36

Table 11 Dust Complaint Form	38

Table 12 Dust Event Form	41

List of Figures

Figure 1 Site Setting in Relation to Sensitive Human Receptors and Ecological Designation	6

Figure 2 Wind Rose for Heathrow Meteorological Station (5-year average)	7

Drawings

Drawing 001 	Site Location Plan

Drawing 002 	Environmental Permit Boundary and Site Layout

Drawing 003	Site Setting Plan

Appendices

Appendix A European Waste Catalogue (EWC) Codes

Appendix B Dust Complaint Form

Appendix C Dust Event Log

# INTRODUCTION

West London Composting Limited (WLC) has retained SLR Consulting Limited (SLR) to prepare a Dust and Emission Management Plan (DMP) for a bespoke Environmental Permit (EP) variation application for the proposed Waste Transfer Station (WTS), located at High View Farm, New Years Green Lane, Harefield, Middlesex, UB9 6LX, hereafter referred to as the ‘Site’. The facility already operates under an existing Waste Management Licence (WML) (No. GTL GRU017) and is operated by WLC.

The Facility will undertake waste bulking and transfer operations only, managing up to 75,000 tonnes per annum (tpa) of waste including the following waste types:

Sweepings;

Clinical / offensive waste;

Bulky waste;

Dry mixed recyclables (DMR);

Plasterboard or wood;

Residual

Category 3 Co-mingled (Mixed food and green waste)

Food waste;

Asbestos waste;

End-of-life tyres; and

Metal waste.

The majority of waste will be stored in designated concrete bays or containers within a fully enclosed WTS building which will benefit from impermeable surfacing. Wastes stored outside of the WTS building will comprise asbestos, end-of-life tyres and metal waste only.  Wastes outside of the WTS building will be stored in enclosed skips.

## Scope of DMP

It is recognised that activities at the Site may result in the release of fugitive dust emissions, which have the potential to diminish amenity in the local area through deposition (dust soiling) and visible dust clouds. Smaller dust particles have the potential to increase local ambient concentrations of suspended particulate matter (PM10 and PM2.5).

Therefore, it is a requirement to control activities at the Site in order to prevent or mitigate potential releases of dust. The DMP provides a proactive approach to the effective management of dust during the Facility works and operation.

This DMP sets out the potential sources of dust at the Site, the measures in place to control dust generation and monitor releases, and the management and monitoring actions that will be taken in response to a dust event. The determination of receptor sensitivity and dust emission magnitude has been determined with reference to the Institute of Air Quality Management (IAQM) Guidance on the Assessment of Mineral Dust Impacts for Planning, herein referred to as the IAQM Mineral Dust Guidance.

The DMP is a ‘live document’, in this respect the dust control measures, and management procedures contained within it will be updated on a periodic basis. This DMP will be kept in the Site office and be available to all employees. The DMP will be implemented throughout the duration of the facilities’ operation.

## Key Guidance

In developing the DMP, key guidance documents that have been consulted include:

The Mineral Industry Research Organisation (MIRO), Good practice guide: control and measurement of nuisance dust and PM10 from the extractive industries (2011); and

IAQM, Guidance on the Assessment of Mineral Dust Impacts for Planning (2016).

# BASELINE ENVIRONMENT

## Site Location

The Site is located on High View Farm, Middlesex, UB9 6LX centred on National Grid Reference (NGR) TQ 07093 88015. The town of Ruislip is located approximately 2.5km south-east of the site.

The area surrounding the Site comprises predominantly agricultural / open land. Grand Union Canal is located approximately 2km west at its closest point. There are several ancient woodlands within 2km of the Site, including an SSSI.

The Site will be accessed via a track leading to Newyears Green Lane which is located north of the EP boundary.

The Site’s location is illustrated on Drawing 001, and the EP Boundary and Site Layout are illustrated in Drawing 002. Local receptors within a 500m radius of the Site are shown on Drawing 003.

## Other Sources of Dust

A review of other potential sources of dust in the Site locale has been undertaken through use of aerial imagery. Other potential sources of dust emissions within the local area are as follows:

Table 1 Other Potential Sources of Dust Emissions

| Site Name | Distance from Site (m) | Direction from Site | Grid Reference |  |
| --- | --- | --- | --- | --- |
|  |  |  | x | y |
| Superior Stone | Adjacent | West | 507064 | 188002 |
| Envar / West London Composting | Adjacent | East | 507130 | 187978 |
| Pylon Farm | 140 | North | 507034 | 188223 |
| Logs U Like | 155 | North | 507244 | 188142 |
| Crow’s Nest Farm / Country Compost | 400 | East | 507495 | 187873 |

## Air Quality

The Site is located within the administrative area of London Borough of Hillingdon and does not lie within an Air Quality Management Areas (AQMAs). The closest AQMA to the Site is approximately 800m south of the facility and is declared for Nitrogen dioxide. AQMAs are areas of sensitivity, declared by local authorities where an Air Quality Assessment Level (AQAL) (including for PM10) is not likely to be achieved. As such, the Site setting can be considered to be not sensitive to air quality.

The Department for Environment, Food and Rural Affairs (Defra) maintains a nationwide model of existing and future background air quality concentrations at a 1km grid square resolution which is routinely used to support LAQM requirements and air quality assessments. The data sets include annual average concentration estimates for PM10 and PM2.5 using a base year of 2018 (the year in which comparisons between modelled and monitoring are made).

The predicted Defra mapped background concentrations for 2024 for the London Borough of Hillingdon are presented in Table 2.

Table 2 Defra Mapped Background Particulate Matter Concentrations

| Grid Square (X,Y) | Year | Annual Mean Concentration (µg/m3) |  |
| --- | --- | --- | --- |
|  |  | PM10 | PM2.5 |
| 507500, 188500 | 2024 | 12.3 | 7.5 |
| AQAL |  | 40 | 20 |

Mapped background concentrations for the 1km grid square containing the Site are ‘well below’ the respective air AQALs.

## Sensitive Receptors

The IAQM Mineral Dust Guidance states two key screening distance for dust when determining human and ecological receptors:

For soft rock quarries (i.e., Sands and Gravel), adverse dust impacts are uncommon beyond 250m; and

For hard rock quarries (i.e., Granite), adverse dust impacts are uncommon beyond 400m.

These screening distances are typically measured to nearest dust generating source or activity. For the purpose of this DMP, the precautionary 400m screening distance has been applied. This represents a highly conservative approach given that the dust emission potential associated with WTS activities is typically much lower than the quarrying industry.

In addition, as presented in Table 2, it has been established that local PM10 concentrations are ‘well below’ the AQALs. As such, consideration of both human and ecological receptors within 400m of the Site boundary is considered sufficient.

### Sensitive Receptors

There are a number of sensitive human and ecological receptors within 400m of the Site as presented in Table 3 and illustrated in Figure 1.

The sensitivity of receptors to dust has been determined with reference to the IAQM Mineral Dust Guidance.

Table 3 Sensitive Human Receptors

| Receptor | Receptor Name | Receptor Type | Receptor Dust Sensitivity | UK NGR |  | Distance from Permit Boundary (m) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | X | Y |  |
| R1 | Superior Stone Limited | Commercial | Medium | 507066 | 188002 | Adjacent |
| R2 | MS Joinery London | Commercial | Medium | 507059 | 188013 | 20 |
| R3 | Aesir Automotive Ltd BMW specialist | Commercial | Medium | 507067 | 188047 | 25 |
| R4 | Westglaze Limited | Commercial | Medium | 507081 | 188082 | 20 |
| R5 | Residential properties | Residential | High | 507107 | 188109 | 30 |
| R6 | Lami Auto | Commercial | Medium | 507005 | 188064 | 80 |
| R7 | Mackenzies Removals and Storage | Commercial | Medium | 507208 | 188054 | 80 |
| R8 | Smart Glazing & Home Improvements | Commercial | Medium | 507213 | 188082 | 100 |
| R9 | Grade II Listed Buildings | Listed Buildings | Medium | 507117 | 188189 | 140 |
| R10 | Residential properties | Residential | High | 507117 | 188189 | 150 |
| R11 | Logs U Like | Commercial | Medium | 506949 | 188092 | 150 |
| R12 | Residential properties | Residential | High | 507005 | 188165 | 160 |
| R13 | Ace Grab Hire and Haulage | Commercial | Medium | 507160 | 188198 | 145 |
| R14 | JM Motors | Commercial | Medium | 507298 | 188095 | 185 |
| R15 | Residential properties | Residential | High | 507326 | 188090 | 210 |
| R16 | Boward Tree Surgery | Commercial | Medium | 506836 | 188041 | 210 |
| R17 | Residential properties | Residential | High | 506890 | 187804 | 280 |
| R18 | Crow’s Nest Farm | Residential | High | 507372 | 187874 | 330 |
| R19 | Newyears Green Covert | Woodland | Medium | 506795 | 188451 | 400 |
| R20 | Country Compost | Commercial | Medium | 507497 | 187870 | 430 |

Figure 1 Site Setting in Relation to Sensitive Human Receptors and Ecological Designation

## Meteorological Conditions

The most important climatic parameters governing the release and dispersal of fugitive emissions from the facility are wind speed, wind direction and rainfall:

Wind direction determines the broad direction of dispersal;

Wind speed affects ground level concentrations by increasing the initial dilution of pollutants in the emission. It will also affect the potential for dust entrainment; and

Rainfall naturally supresses dust release; >0.2mm of rainfall a day considered sufficient to suppress dust.

A wind rose for Heathrow meteorological station (5-year average 2018, 2019, 2020, 2022 & 2023), located approximately 11km south of the Site is presented in Figure 2. The wind rose shows winds from the southwest are most frequent. Therefore, locations to the northeast of the facility are most likely to be impacted by potential dust emissions.

Figure 2 Wind Rose for Heathrow Meteorological Station (5-year average)

Relevant rainfall data applicable to the Site has been obtained from the Meteorological Office website of UK mapped climate averages for 1991-2020. The nearest representative site with rainfall data is Northwood (Hertfordshire). The annual average days of rain ≥0.1mm for the area of the Site is 118.08 days per year, comprising approximately 32% of the year.

# OPERATIONS AT THE SITE

This section identifies the activities, potential dust source and PM emissions at the facility. The operational layout is illustrated in Drawing 002.

## Site Operations

The facility is designed to receive a variety of waste types, with limited manual sorting undertaken at the Site prior to bulk removal. Waste will be delivered to the facility in secure, steel sided lorries, skips and other vehicles. The facility will accept approximately 75,000 tpa of waste, comprising of:

Sweepings;

Clinical / offensive waste;

Bulky waste;

Dry mixed recyclables (DMR);

Plasterboard or wood;

Residual;

Category 3 Co-mingled (Mixed food and green waste);

Food waste;

Asbestos waste;

End-of-life tyres; and

Metal waste.

The Site will purely be a storage and transfer site used to bulk up waste prior to transfer to a suitably permitted alternative facility for further recovery or disposal.

The WTS building will benefit from a fast action roller shutter door that will be closed during waste tipping and handling. No treatment of waste will occur, and the waste will be stored for a minimal length of time 5 days. During storage, minimal handling and compaction of the waste will occur.

## Waste Acceptance

The facility will accept predominantly non-hazardous mixed waste including co-mingled recyclable materials, bulky waste, paper and cardboard, residual waste, street sweepings, garden waste, clinical waste, and food waste. The majority of the waste is considered to have a low dust potential. Details of proposed waste for acceptance and associated European Waste Catalogue (EWC) Codes are presented in full in Appendix A European Waste Catalogue (EWC) Codes. It is acknowledged that some of the accepted waste types have the potential to be dustier than others.

All waste received at the Site will be monitored at entry (report to a weighbridge/site office) and visually inspected to ensure compliance with the permitted waste types for the facility and to identify any particularly dusty loads. All waste will undergo visual inspection during deposition within the WTS building to confirm its description and composition against the relevant documentation. Any abnormal loads will be rejected or placed into the quarantine area and containerised if appropriate prior to removal from site. Rejected wastes will be transported off site within 36 hours, and potentially odorous loads will be removed from the quarantine area within 24 hours.

## Storage

A maximum of 1,500 tonnes will be stored on Site at any one time.  A maximum of 50 tonnes of hazardous waste will be stored at any one time.

- All waste inside the WTS building will be stored within designated concrete bays, or containers. The building will benefit from impermeable surfacing, and sealed drainage.

- All waste outside the WTS building will be stored in enclosed skips and comprise of asbestos, tyres and metal waste only. Surfacing outside the building comprises of hardstanding.  Enclosed skips will prevent the ingress of rainwater to waste.

The following summarises the key waste storage measures to be adopted on Site:

Waste will be stored in locations that minimise the unnecessary handling of waste (i.e. within close proximity of the treatment plant input and output area);

Waste handling will be carried out by competent staff using appropriate equipment;

The majority of waste will be stored within the building and is therefore away from any watercourses and sensitive receptors.  Wastes stored outside the building will be contained within enclosed skips to prevent the escape of waste and ingress of rainwater. This also ensures that all waste will be securely stored preventing unauthorised access and vandalism;

The quantity of stored waste will be monitored against the allowed maximum capacities;

Waste will be processed as soon as possible and stored on Site for a maximum of 5 days. Treatment will be prioritised for treatment of removal off-Site based on the following:

Its type;

Its age on arrival;

The date of arrival; and

The duration of storage on Site.

Clinical waste will be stored for a maximum of 5 days (typically removed every 2-3 days) prior to transfer off Site to a suitably permitted alternative facility for further recovery or disposal.

Green waste will be stored for a maximum of 4 days prior to removal.

Food waste will be stored for a maximum of 2 days prior to removal.

Food, co-mingled food / green waste, and green waste will be stored in a dedicated bunker inside the transfer building before being transferred off-site to a suitably licensed facility. There will be no treatment of this waste on site.

Green waste incoming loads will be weighed-in on the weighbridge and directed to the dedicated green waste bunker, the loads will be inspected to ensure they meet the correct green waste specifications. Mobile plant pushes the tipped waste up to ensure all material is inside the bunker. The green waste storage area will benefit from impermeable surfacing and a sealed drainage system. The onsite sweeper is used to clear any debris from the loading and clear the potential limited volumes of leachate from the bunker, which will then be disposed of at permitted disposal sites.

Bulky waste with POPs will be stored in one bay inside the WTS building.. Waste containing POPs shall be identified and segregated from other wastes and stored on an impermeable surface with sealed drainage system, to prevent cross-contamination. All wastes containing POPs will be included in the waste transfer note with the correct waste code. This waste will then be sent to a suitably authorised disposal or recovery site to either completely destroy the POPs or irreversibly transform the POPs.

Waste stored outside of the WTS building will comprise of asbestos, metal and tyres only. Asbestos, tyres and metals will be stored in skips outside, which will use covers to prevent rainwater ingress which can lead to contaminated runoff and also reduces the release of asbestos fibres. Asbestos waste shall be double bagged, or where necessary, securely wrapped. It will be kept within clearly identified, segregated, sealed, secure, lockable bulk containers (i.e. skips). It will not be stored loose or in bays.

All wastes will be subject to inspections and checking against the declaration on the waste transfer note.

Waste will be treated on a first-in-first-out basis unless more recently received wastes are prioritised because they pose a higher risk of pollution;

Good housekeeping measures would ensure that stockpiles are suitably managed to stay within the designated bays. Housekeeping equipment is stored within the Housekeeping Store located in the north of the Site as shown on Drawing 002

Storage areas will benefit from daily cleaning using brooms and weekly washdowns using hoses;

All waste storage containers and bays will be clearly labelled to ensure the segregation of waste.  ;

Storage areas will be inspected weekly to ensure there is no loss of containment; and

Any spillages will be cleared and logged in the site diary immediately.

Due to the type of inputs, it is expected the bulk of wastes will be evenly spread throughout the year apart from green waste which will be seasonal. All waste will be stored for a maximum of 5 days (typically removed every 2-3 days) prior to transfer off-Site to a suitably permitted alternative facility for further recovery or disposal.

The proposed waste lists for the Site, including waste types and EWC Codes are listed in Appendix A.

## On-Site Transportation

There will be periodic vehicle movements during operational hours. Vehicle movements will arise from waste import, handling, stockpiling and export operations. The vehicle movements at the Site will be primarily as a result of refuse collection vehicles (RCVs) and tippers importing or exporting waste, as well as mobile plant in operation for handling and stockpiling operations. Waste vehicles will be restricted and not permitted to drive over areas of unmade ground. This will be ensured by clear signposting of vehicle routes through the Site.

## Off-Site Transportation / Import and Export Operations

There will be periodic vehicle movements to / from the facility on to the local road network during operational hours. Vehicles entering / leaving the facility on to / from the local road network would arise as a result of waste import / export operations. The vehicles leaving the Site will typically comprise RCVs and tippers.

The areas of the facility which will be accessed by the RCVs and tippers accessing the facility will be hard paved. Waste vehicles will be restricted and not permitted to drive over areas of unmade ground. This will be ensured by clear signposting of vehicle routes through the Site.

## Mobile Plant and Equipment

Particulate matter can be a by-product of internal combustion, and the facility is expected to utilise items of plant with internal combustion engines such as mobile tele-handlers.

The exact type, model and emission rating of mobile plant and equipment operating at the facility is not known at this stage. It is anticipated that tele-handlers will be Euro VI compliant. It is also assumed the internal plant and machinery will meet the most recent Non-Road Mobile Machinery (NRMM) emission standards. Due to the size and nature of the facility, only a small number of operational plant are anticipated to be in use at any one time.

## Potential Dust Sources and Magnitude

In review of the Site operations outlined above, potential sources of dust at the facility and their potential magnitude of dust emissions have been identified and are presented in Table 4. It is understood the facility is unlikely to have waste types that would be especially dusty. The potential magnitude of dust emissions for each source has been determined in reference to the IAQM Mineral Dust Guidance, as well as consideration of the control measures expected to be in place. It is anticipated that the majority of the material to be received at the facility will have a small potential for dust emissions.

Table 4 Dust Release Inventory

| Dust Source | Potential Magnitude of Dust Emissions | Reasons / Controls |
| --- | --- | --- |
| Road vehicles entering and leaving the facility, tracking material out onto the public highway. | Small | The onsite roads including access will be paved; Waste vehicles will travel to / from the Site on Newyears Green Lane and the A40 – a major paved road; Municipal waste and recyclables generally have a low dust potential, reducing the significance of dust re-suspension by vehicles; and Daily visual dust monitoring on the local road network will be carried out by all members of staff throughout their shift with any potential emissions of dust reported to the Facility Manager |
| Internal vehicle / plant movements within the Site on the impermeable surface | Small | On-Site vehicle movements will be on paved roads; Municipal waste and recyclables generally have a low dust potential, reducing the significance of dust re-suspension by vehicles; and Daily visual monitoring of internal paved roads will be carried out by all members of staff throughout their shift with any potential emissions of dust reported to the Facility Manager. |
| Exhaust emissions from on-Site vehicles / plant and from off-Site heavy goods vehicles (HGVs) | Small | It is assumed the internal plant and machinery will meet the most recent NRMM emission standards. The RCS fleet accessing the Site is expected to be Euro VI compliant. |
| Dust from waste acceptance | Small | The facility will accept, predominantly non-hazardous mixed waste including co-mingled recyclable materials, bulky waste, paper and cardboard, residual waste, street sweepings, garden waste, clinical waste, and food waste. Due to the nature of these wastes, the potential risk of dust emissions from acceptance handling and storage of the wastes are low. Municipal waste and recyclables generally have a low dust potential. Fast action roller shutter doors will be closed during tipping and waste handling help maintain containment during vehicle ingress / egress. |
| Dust from handling of waste | Small | Tipping heights are minimised where possible. Fugitive dust releases are contained within the building, preventing wind whipping (through application of a physical barrier around the stockpile). |
| Litter from waste storage and debris falling from loaded (covered) vehicles. | Small | Vehicles are covered when entering or exiting the facility (sheeting or enclosed vehicles). Any excessive litter material at the facility or on the highways will be cleared using a mechanical sweeper and/or litter picker if required. |
| Green waste materials. | Medium | Green waste can have a high dust potential if stored for extended periods (i.e. if allowed to dry out and decompose). Green waste is removed from the Facility within five days. It is understood no green waste shredding will be undertaken at the facility. |
| Storage of mixed municipal waste | Medium | Mixed municipal waste generally has low dust potential. Fugitive releases during periods of dry / windy weather. All waste types are stored indoors or within impermeable concrete areas. Water suppression will be used to dampen the stockpile where required to mitigate fugitive releases (i.e. during periods of dry / windy weather). |
| Exhaust emissions from on-Site vehicles / plant and from off-Site HGVs | Small | The RCV fleet accessing the Site is expected to be Euro VI compliant. It is assumed the internal plant and machinery will meet the most recent NRMM emission standards. |

The Site Manager will be responsible for implementing appropriate risk management measures in accordance with the operational techniques (OT) document (402.065523.00001_OT and WAP) and this DMP.

# DUST AND PM10 MANAGEMENT

Potential emissions of dust and PM10 from the Site operations are mitigated through adoption of a range of operational and designed-in dust control measures. These measures have been determined in consideration of the Source-Pathway-Receptor (SPR) routes, as outlined in the section below.

## Source-Pathway-Receptor Routes

The pathway for the majority of the releases is atmospheric dispersion; primarily from the dust / particulate source (e.g. wind whipping and handling operations). The SPR routes are detailed in Table 5

.

Table 5 Source-Pathway-Receptor Routes

| Source | Pathway | Receptor | Type of Impact | Where Relationship Can be Interrupted |
| --- | --- | --- | --- | --- |
| Road vehicles entering and leaving the facility, tracking material out onto the public highway. | Material falling from lorries. Track out of material from the facility onto the public road network by HGVs. | There are medium and high sensitivity receptors within 10m of Newyears Green Lane The nearest medium sensitivity receptor (R1) located adjacent to the Site boundary (i.e. close distance). The nearest high sensitivity receptor (R2) located within 65m of the Site boundary (i.e. intermediate distance). | Visual soiling, also consequent resuspension as airborne particulates | Internal haul routes are paved / tarmacked; therefore the accumulation of debris on vehicles whilst on Site is anticipated to be minimal. All HGVs transferring material to / from the facility shall be covered (contained vehicles or sheeted). Roads will be swept and cleaned whenever necessary In the event that mud, debris or waste arising from the facility is deposited outside the Site, the affected area will be cleaned, and traffic will be isolated from sources of mud and debris within the Site. Daily visual inspection of the facility by Site management will identify any problem with mud which will be cleaned up as soon as possible. Where necessary road cleaning equipment will be deployed. The facility will benefit from good housekeeping and all areas of the facility will be cleaned on a daily basis. |
| Internal vehicle / plant movements within the Site on the impermeable surface | Atmospheric dispersion. |  | Airborne particulates | Operational plant are anticipated to be Euro VI compliant / meet with the most recent NRMM emission standards. Only a small number of operational plant will be in use at any given time. |
| Dust from acceptance of waste. |  |  | Visual soiling, also consequent resuspension as airborne particulates | The majority of waste will be accepted to and stored within a fully enclosed building (fast action roller shutter doors will be in operation). Waste stored outside the building is stored in enclosed skips. Water suppression will be used to dampen stockpiles where required. |
| Dust from handling of waste |  |  |  | Drop heights will be minimised to prevent dust emissions. |
| Litter from waste storage and debris falling from loaded (covered) vehicles. |  |  | Visual soiling, also consequent resuspension as airborne particulates | Strict waste acceptance will ensure that only authorised waste accepted. The facility will benefit from good housekeeping and all areas of the facility will be cleaned on a daily basis. All vehicles leaving the facility will be inspected to ensure that they are clear of loose waste. Any excessive litter material at the facility or on the highways will be cleared using a mechanical sweeper and/or litter picker if required. |

## Control of Fugitive Dust/Particulate Emissions

Dust control measures have been determined in consideration of the source-pathway-receptor routes outlined in Section 4.1 above. The key designed-in and operational dust control measures in place at the facility are summarised below:

The WTS building is set back from nearby sensitive receptors (i.e. R1) Waste acceptance procedures are in place to avoid receipt of unsuitable (i.e. excessively dusty) waste types;

Wastes stored outside of the WTS building are stored in enclosed skips;

Particularly dusty loads will be classified as non-conforming waste and the appropriate procedure will be followed;

Internal haul routes are maintained, therein minimising accumulation of dust/dirt on vehicles visiting the facility and reducing the potential risk of trackout of dust and dirt from the Site onto the public road network;

Tipping, stockpiling and bulk removal of waste will be undertaken within the WTS building which is fully enclosed accessed via roller shutter doors;

Drop heights for the tipping or handling of waste types will be minimised where possible to reduce resuspension of dust;

Material storage areas will comprise an impermeable surface, thus minimising suspension of dust from stockpiling and handling operations;

Daily walkovers will be undertaken by the Site supervisor who will respond immediately in the event of any significant dust problem; and

Any complaints from neighbours or other persons will be investigated and dealt with as necessary (road sweeping, water suppression and/or varied operational practice).

Further details on all dust control measures implemented are outlined in Table 6.

Table 6 Control Measures for Dust/PM10 and Other Emissions

| Abatement Measure | Description / Effect | Overall Consideration and Implementation | Trigger for Implementation |
| --- | --- | --- | --- |
| Facility Processes | Majority of operations are taking place in an enclosed building eliminating pathways to sensitive receptors. Wastes outside of WTS building will be stored in enclosed skips. | In combination with other measures to reduce dust and particulate generation this assists to reduce the effectiveness of the pathway between the source and the receptor. | Implemented at all times that the facility is operational. |
| Speed limit and minimisation of vehicle movements | Reducing vehicle movements reduces emissions from vehicles. Speed limits will be implemented for vehicles using the Site (i.e. 5mph) along with traffic calming measures. | Implement as part of good practice and incorporated into training / induction process. Clearly presented around the facility. | Used at all times that the facility is operational. |
| Minimising drop heights for material | Minimisation of the height at which materials are handled reduces the distance over which debris, dust and particulates could be blown and dispersed by winds. | Implement as part of good practice and incorporated into the training process. | Implemented at all times that the facility is operational. During periods of prolonged dry and windy weather conditions, consideration given to visual assessment of dust plumes being generated from existing drop heights and reduced / ceased as required. |
| Good housekeeping | A consistent, regular housekeeping regime will be in place to ensure the Site is regularly checked and issues remedied to prevent and remove dust and particulate build up: The WTS building floor will be swept daily after loading operations have ceased. All bays and bay walls will be cleaned/swept out (as appropriate) three times per week (as when bays are completely empty). All roads and operational areas will be checked on a daily basis and swept daily, as necessary, in line with daily inspections to reduce dust emissions. | Easy to implement and requires minimal equipment. Encourages a sense of pride and satisfaction amongst the staff which promotes vigilance and a positive culture. | Implemented at all times that the facility is operational. |
| Sheeting of loaded vehicles (unless enclosed) | Prevents the escape of debris, dust and particulates from vehicles as they travel. | Vehicles would be checked upon entering and prior to leaving the facility. | Implemented at all times that the facility is operational. |
| Surfacing of vehicle routes | Haul roads and access roads are maintained. Operational areas inside the building have an impermeable surface. Hardstanding outside of the WTS building will be maintained. | Impermeable surfaces reflect industry best practice. | Surfaces are periodically inspected for signs of wear or damage. Remedial works will be commissioned as required. |
| Special measures for materials with a high dust potential | It is anticipated that the majority of the material to be received at the facility will have a small-to-medium potential for the generation of dust emissions. Where it is identified that materials with a high dust potential have been received (i.e. very dry green material), a number of special measures will be put in place to reduce the handling and retention time of that material. | Where materials are identified to have a high dust potential are received/stored at the facility (such as very dry green waste), dust suppression (hose pipe from adjacent jet wash) will be utilised, to keep the material damp, thus reducing the dust potential. The facility operations are managed such that materials identified to have a high dust potential are removed from the facility within 24-hours. | Implemented where materials are identified to have a high dust potential. |
| Marking of stockpile base | Clear delineation of stockpile areas minimises the risk of vehicles traversing across loose particulates on the ground and causing re-suspension or re-distribution across the facility. | Easy method to implement, with clear line marking provided on the impermeable concrete at the storage areas. | Implemented at all times when the facility is operational. |
| Restriction of vehicles on unmade ground | Vehicles will not be able to traverse unmade areas of ground at the Site. This significantly reduces the potential for material to be tracked across the Site and resuspended. | HGV access is limited to the haul routes and is clearly signposted. | Implemented at all times when the Site is operational. |
| Dust Suppression | Dust suppression will be achieved through implementation of water-based suppression systems. Water suppression can be a highly effective way of reducing the dust potential at-source, eliminating the pathway to the receptors. | Water suppression will be available for all waste storage areas. Water suppression will comprise of a Screenpod S65 Dust Cannon which can be moved around the site as required. The suppression will have a fixed water supply from the mains. There will be water tanks on Site to provide the required water for suppression in case of mains failure | Implemented as required, to be determined by the Waste & Recycling Supervisors by monitoring of meteorological conditions (i.e. blowing towards receptors, lower wind speed and dry) and identification of material received with a high dust potential. In the event that the water suppression is not operational for a short period of time (i.e. malfunction or maintenance) where it is identified to be required, handling and processing operations would be temporarily suspended. |
| Visual Dust Monitoring | Visual dust monitoring provides a cost-effective method of monitoring that allows for pro-active, immediate response to dust generating events. | Daily visual assessment is undertaken by operatives for airborne or deposited dust. Daily assessments include the following areas: Perimeter walk around for visible dust plumes travelling off-Site; If required, off-Site walkover surveys; Storage areas; and Facility haul roads, access road and public highway near Site exit. Facility operatives who undertake visual assessments have appropriate training. Details recorded would include (as a minimum): Weather conditions (qualitative wind speed, direction, rainfall); Current operations (location of activities); Identification of any significant dust or dispersion beyond the site boundary; and Additional mitigation measures put in place, if required. | In the event that visual dust monitoring identifies dust being transported beyond the facility boundary and mitigation measures fail to resolve the issue, all dust generating activities will cease until the source of the dust has been identified and steps taken to prevent the off-Site emissions. Additional visual monitoring will be undertaken where: Particularly dusty conditions are detected by operational staff; Dust emissions are evident near the facility boundary during any activity; and In response to complaints being received – in this situation off-Site monitoring must also be carried out at appropriate locations. |

## Other Considerations/Contingency

##### Water Usage/Availability

Usage of water for dust suppression is sporadic and short-term as its requirement is dependent on the dust potential of received material. Water for dust suppression is available for the mains water supply. In the event of a failure in the mains water supply, the facility will have provision of  water tanks which may be used for dust suppression where required.

##### Site Closure

In the event that dust control measures on the Site are not sufficient to prevent the emissions of dust from the Site boundary, the Site Manager can make the decision to close the Site and stop operations if required. Potential deliveries of waste will be informed on the Site closure and alternative arrangements made.

## Responsibility for Implementation

A suitably trained Waste & Recycling Supervisor will be present at the facility when operational, who is responsible for the implementation of dust management measures where required. Responsibilities are allocated to specific personnel to ensure dust generation is effectively controlled as outlined in Table 7.

Table 7 Dust Management Responsibilities

| Actions | Responsibility |
| --- | --- |
| Monitoring meteorological forecast | Waste & Recycling Supervisor |
| Routine daily visual dust monitoring | Waste & Recycling Supervisor |
| Routine monthly visual dust monitoring | Waste & Recycling Supervisor |
| Coordinating plant area cleaning | Waste & Recycling Supervisor |
| Application of plant dust suppression | Waste & Recycling Supervisor |
| Completing dust event forms | Waste & Recycling Supervisor |
| Liaison with public and regulator | Waste & Recycling Supervisor, Waste & Recycling Area Manager |
| Coordinating DMP updates | Waste & Recycling Supervisor, Waste & Recycling Area Manager |
| *The procedure for the Waste & Recycling Supervisor to review feedback from visual monitoring will be to review the visual monitoring record in the Site Logbook. |  |

All personnel at the facility understand their responsibility to ensure the generation of dust is minimised. Each employee is made aware of the importance of dust control and the most effective measures available to minimise such emissions either as part of the induction process, or as a specific training exercise. Training incorporates the following aspects:

Waste types that can be accepted at the facility, as outlined within the associated permit;

Identification of high dust potential waste;

Key activities with the highest potential for dust generation;

Methodology of visual dust assessments;

Importance of unofficial visual dust assessments during everyday work and how to report visible dust emissions;

How to respond to a complaint from a member of the public;

The complaints protocol and escalation method;

What to do in the event of a dust emission incident, and who to inform;

The importance of the DMP, its ‘active’ format and its location;

Any dust monitoring methods incorporated on Site at the time;

Overview of the prevailing winds and how this affects daily operations;

Key aspects to look out for during routine operations with regard to dust generating activities;

Cleaning regime on Site (routine and intermittent);

Regime of maintenance of onsite plant;

Routine measures that can be incorporated into daily work schedules to minimise dust and emissions (i.e. no idling, minimise drop heights, traversing across base of stockpiles, covering of loads); and

Additional measures that can be undertaken to minimise dust and emissions (i.e. notification of relevant person visual dust plumes are identified, remedial actions).

Refresher training is expected to be provided every two years.

## Visual Dust Monitoring

Visual dust monitoring provides a cost-effective method of monitoring that allows for pro-active, immediate response to dust generating events.

Visual assessment is undertaken on a daily basis by the facilities operatives for airborne or deposited dust. Facility operatives who undertake the visual dust assessments will receive appropriate training. Daily assessments include, as a minimum, a visual assessment of the following areas (identified as areas / activities with the highest potential for dust generation):

Perimeter walk around;

If required, offsite walkover surveys;

Material storage areas;

Internal haul routes; and

Access road and public highway near Site exit.

Based upon the size of the facility, it is considered viable for daily monitoring to include a walkover of the entire perimeter (permit boundary) as the routine. Fixed dust monitoring locations are shown on Drawing 002.

All visual monitoring is recorded in the daily logbook and made available to the EA as required. Details recorded include (as a minimum):

Weather conditions (qualitative wind speed, direction, rainfall);

Current site operations (location of activities);

Identification of any significant dust within the Facility or dispersion beyond the Facility boundary; and

Additional mitigation measures put in place, if required.

An increase in the frequency and scale of visual monitoring will be undertaken where:

Particularly dusty conditions are detected by operational staff;

Dust emissions are evident near the boundary during any activity; and / or

In response to complaints being received – in this situation off Site monitoring will also be carried out at appropriate locations.

The results of the visual dust monitoring will be monitored by the facility management. Where it is identified that significant dust levels are present on-site, or dust is visible beyond the facility boundary, Site management will ensure that the appropriate mitigation measures are adopted in response. In the event that visual dust monitoring identifies dust being transported beyond the facility boundary and mitigation measures fail to resolve the issue, all dust generating activities will cease until the source of the dust has been identified and steps taken to prevent the off-Site emissions.

In the event that continuous off-Site dust emissions are detected (i.e. more than 2 days in a row) alongside complaints being received by members of the public, correspondence with the EA will be undertaken to discuss subsequent steps.

It is not proposed to undertake any visual dust monitoring outside of the operational hours of the facility. However, if monitoring was specifically required outside of the operational hours, a third-party monitoring company could be commissioned to undertake monitoring.

# DUST COMPLAINTS PROCEDURE

Complaints may be notified to the facility either during or after an event, directly by the complainant or indirectly through a regulator (such as the Local Planning Authority or Environmental Health Department) who was notified.

Complaint records will include the following (recorded in the Site logbook):

Date, time, and name of complainant (if given);

Nature of complaint;

Locality of complaint; and

A summary of the investigation and actions taken and the outcome.

Complaint response will have the objective of investigating the incident and preventing any continuing issue, for example by putting in place additional control measures to prevent re-occurrence of the incident and updating the DMP. Complainants will be informed of the findings of investigations and the actions taken (if contact details are provided at the time the complaint is made).

Investigations will include but not be limited to:

Visit by facility personnel to location of complainant to verify issue (if the complaint is made ‘after’ rather than ‘during’ a dust event this may not be possible);

Review of activities at time of incident to investigate potential sources;

If dust event is occurring or a recurring event undertake more frequent targeted on-site and off-site visual monitoring and record findings;

Review of control measures and management actions at the time of the incident;

Review of meteorological conditions at the time of the incident; and

Reporting of findings (using Appendix B Example Dust Event Form or Site logbook).

All complaints will be acknowledged within 2 working days and a response provided in line with the Councils Complaints Procedure. An example Dust Event Form is included in Appendix B. Where a number of complaints are received (or recurring complaints are received), the complaints investigation would be escalated to the Waste & Recycling Area Manager, who will lead an investigation seeking to rectify the issue at the earliest opportunity. The Waste & Recycling Area Manager may engage the services of a specialist contractor to investigate where appropriate.

## Engagement with the Community

The Waste & Recycling Area Manager (or nominated representative) will act as liaison with the regulator and local community for issues relating to dust nuisance.

The nominated representative will respond promptly to all complaints by undertaking an investigation into the dust event, including weather conditions, operations on Site and mitigation measures in place at the time of the complaint.

Complainants will be informed of the investigation.

Following the receipt of a complaint, the details of the complaint will be recorded (an example of a compliant record form is presented in Appendix B), a Dust Event Form will be completed, and the results of the subsequent investigation kept in the Facilities Logbook.

Liaison with local residents and business can be undertaken through posts on the local council / community council’s websites as appropriate.

## Management Responsibilities

There will be a trained Waste & Recycling Supervisor present at the facility during operational hours, responsible for dealing with complaints (i.e. receipt, recording and investigation).

Contact details will be available at all times at the facility entrance, with details (including a phone number / email address) provided for both operational hours and out-of-hour periods.

## DMP Update and Review

This DMP is a controlled document, and forms part of the Environmental Management System (EMS). The DMP will be reviewed on an annual basis. However, the DMP is intended to be a ‘live’ document which serves as a reference during daily operations, and as such will be updated on a more frequent basis should the following occur:

Significant changes are made to the plant or operational practices;

The regulator requests that the DMP is updated; or

Complaints are received, which on subsequent investigation result in the identification of further control measures or remedial action, in addition to those set out within this DMP

European Waste Catalogue (EWC) Codes

Table 8	Proposed Non-Hazardous Waste Types to be Accepted at the Site

| EWC Code | Description |
| --- | --- |
| 01 | WASTES RESULTING FROM EXPLORATION, MINING, QUARRYING, AND PHYSICAL AND CHEMICAL TREATMENT OF MINERALS |
| 01 01 | Wastes from mineral excavation |
| 01 01 01 | Wastes from mineral metalliferous excavation |
| 01 01 02 | Wastes from mineral non-metalliferous excavation |
| 01 03 | Wastes from physical and chemical processing of metalliferous minerals |
| 01 03 06 | Tailings other than those mentioned in 01 03 04 and 01 03 05 |
| 01 03 09 | Red mud from alumina production other than the wastes mentioned in 01 03 07 |
| 01 04 | Wastes from physical and chemical processing of non-metalliferous minerals |
| 01 04 08 | Waste gravel and crushed rocks other than those mentioned in 01 04 07 |
| 01 04 09 | Waste sand and clays |
| 01 04 11 | Wastes from potash and rock salt processing other than those mentioned in 01 04 07 |
| 01 04 12 | Tailings and other wastes from washing and cleaning of minerals other than those mentioned in 01 04 07 and 01 04 11 |
| 01 04 13 | Wastes from stone cutting and sawing other than those mentioned in 01 04 07 |
| 02 | WASTES FROM AGRICULTURE HORTICULTURE AQUACULTURE FORESTRY HUNTING AND FISHING FOOD PREPARATION AND PROCESSING |
| 02 01 | Wastes from agriculture, horticulture, forestry, hunting and fishing |
| 02 01 03 | Plant-tissue waste |
| 02 01 04 | Waste plastics (except packaging) |
| 02 01 07 | Wastes from forestry |
| 02 01 10 | Waste metal |
| 02 02 | Wastes from the preparation and processing of meat, fish and other foods of animal origin |
| 02 02 03 | Materials unsuitable for consumption or processing |
| 02 03 | Wastes from fruit, vegetables, cereals, edible oils, cocoa, coffee, tea and tobacco preparation and processing; conserve production; yeast and yeast extract production, molasses preparation and fermentation |
| 02 03 04 | Materials unsuitable for consumption or processing |
| 02 04 | Wastes from sugar processing |
| 02 04 01 | Soil from cleaning and washing beet |
| 02 04 02 | Off-specification calcium carbonate |
| 02 05 | Wastes from the dairy products industry |
| 02 05 01 | Materials unsuitable for consumption or processing |
| 02 06 | Wastes from the baking and confectionery industry |
| 02 06 01 | Materials unsuitable for consumption or processing |
| 02 06 02 | Wastes from preserving agents |
| 02 07 | Wastes from the production of alcoholic and non-alcoholic beverages (except coffee, tea, and cocoa) |
| 02 07 01 | Wastes from washing, cleaning and mechanical reduction of raw materials |
| 02 07 02 | Wastes from spirits distillation |
| 02 07 04 | Materials unsuitable for consumption or processing |
| 03 | WASTES FROM WOOD PROCESSING, AND THE PRODUCTION OF PANELS AND FURNITURE, PULP, PAPER AND CARDBOARD |
| 03 01 | Wastes from wood processing and the production of panels and furniture |
| 03 01 01 | Waste bark and cork |
| 03 01 05 | Sawdust, shavings, cuttings, wood, particle board and veneer other than those mentioned in 03 01 04 |
| 03 03 | Wastes from pulp, paper and cardboard production and processing |
| 03 03 01 | Waste bark and wood |
| 03 03 07 | Mechanically separated rejects from pulping of waste paper and cardboard |
| 03 03 08 | Wastes from sorting of paper and cardboard destined for recycling |
| 03 03 10 | Fibre rejects, fibre-, filler- and coating-sludges from mechanical separation |
| 04 | WASTES FROM THE LEATHER, FUR AND TEXTILE INDUSTRIES |
| 04 01 | Wastes from the leather and fur industry |
| 04 01 08 | Waste tanned leather (blue sheetings, shavings, cuttings, buffing dust) containing chromium |
| 04 01 09 | Wastes from dressing and finishing |
| 04 02 | Wastes from the textile industry |
| 04 02 21 | Wastes from unprocessed textile fibres |
| 04 02 22 | Wastes from processed textile fibres |
| 06 | WASTES FROM INORGANIC CHEMICAL PROCESSES |
| 06 09 | Wastes from the MSFU of phosphorous chemicals and phosphorous chemical processes |
| 06 09 02 | Phosphorous slag |
| 06 09 04 | Calcium-based reaction wastes other than those mentioned in 06 09 03 |
| 06 11 | Wastes from the manufacture of inorganic pigments and opacifiers |
| 06 11 01 | Calcium-based reaction wastes from titanium dioxide production |
| 07 | WASTES FROM ORGANIC CHEMICAL PROCESSES |
| 07 02 | Wastes from the MFSU of plastics, synthetic rubber and man-made fibres |
| 07 02 13 | Waste plastic |
| 09 | WASTES FROM THE PHOTOGRAPHIC INDUSTRY |
| 09 01 | Wastes from the photographic industry |
| 09 01 07 | Photographic film and paper containing silver or silver compounds |
| 09 01 08 | Photographic film and paper free of silver of silver compounds |
| 09 01 10 | Single-use cameras without batteries |
| 09 01 12 | Single-use cameras containing batteries other than those mentioned in 09 01 11 |
| 10 | WASTES FROM THERMAL PROCESSES |
| 10 01 | Wastes from power stations and other combustion plants (except 19) |
| 10 01 01 | Bottom ash, slag and boiler dust (excluding boiler dust mentioned in 10 01 04) |
| 10 01 05 | Calcium-based reaction wastes from flue-gas desulpherisation in solid form |
| 10 01 07 | Calcium-based reaction wastes from flue-gas desulpherisation in sludge form |
| 10 01 15 | Bottom ash, slag and boiler dust from co-incineration other than those mentioned in 10 01 14 |
| 10 01 19 | Wastes from gas cleaning other than those mentioned in 10 01 05, 10 01 07 and 10 01 18 |
| 10 01 24 | Sands from fluidised beds |
| 10 02 | Wastes from the iron and steel industry |
| 10 02 01 | Wastes from the processing of slag |
| 10 02 02 | Unprocessed slag |
| 10 02 08 | Solid wastes from gas treatment other than those mentioned in 10 02 07 |
| 10 02 10 | Mill scales |
| 10 02 14 | Filter cakes from gas treatment other than those mentioned in 10 02 13 |
| 10 02 15 | Other filter cakes |
| 10 03 | Wastes from aluminium thermal metallurgy |
| 10 03 02 | Anode scraps |
| 10 03 05 | Waste alumina |
| 10 03 16 | Skimmings other than those mentioned in 10 03 15 |
| 10 03 18 | Carbon-containing wastes from anode manufacture other than those mentioned in 10 03 17 |
| 10 03 24 | Solid wastes from gas treatment other than those mentioned in 10 03 23 |
| 10 03 26 | Filter cakes from gas treatment other than those mentioned in 10 03 25 |
| 10 03 28 | Wastes from cooling-water treatment other than those mentioned in 10 03 27 |
| 10 03 30 | Wastes from treatment of salt slags and black drosses other than those mentioned in 10 03 29 |
| 10 04 | Wastes from lead thermal metallurgy |
| 10 04 10 | Wastes from cooling-water treatment other than those mentioned in 10 04 09 |
| 10 05 | Wastes from zinc thermal metallurgy |
| 10 05 01 | Slags from primary and secondary production |
| 10 05 09 | Wastes from cooling-water treatment other than those mentioned in 10 05 08 |
| 10 05 11 | Dross and skimmings other than those mentioned in 10 05 10 |
| 10 06 | Wastes from copper thermal metallurgy |
| 10 06 01 | Slags from primary and secondary production |
| 10 06 02 | Dross and skimmings from primary and secondary production |
| 10 06 10 | Wastes from cooling-water treatment other than those mentioned in 10 05 10 |
| 10 07 | Wastes from silver, gold and platinum thermal metallurgy |
| 10 07 01 | Slags from primary and secondary production |
| 10 07 02 | Dross and skimmings from primary and secondary production |
| 10 07 03 | Solid wastes from gas treatment |
| 10 07 05 | Filter cakes from gas treatment |
| 10 07 08 | Wastes from cooling-water treatment other than those mentioned in 10 07 07 |
| 10 08 | Wastes from other non-ferrous thermal metallurgy |
| 10 08 09 | Other slags |
| 10 08 11 | Dross and skimmings other than those mentioned in 10 08 10 |
| 10 08 13 | Carbon-containing wastes from anode manufacture other than those mentioned in 10 08 12 |
| 10 08 14 | Anode scrap |
| 10 08 18 | Filter cakes from flue-gas treatment other than those mentioned in 10 08 17 |
| 10 08 20 | Wastes from cooling-water treatment other than those mentioned in 10 08 19 |
| 10 09 | Wastes from casting of ferrous pieces |
| 10 09 03 | Furnace slag |
| 10 09 06 | Casting cores and moulds which have not undergone pouring other than those mentioned in 10 09 05 |
| 10 09 08 | Casting cores and moulds which have undergone pouring other than those mentioned in 10 09 07 |
| 10 09 14 | Waste binders other than those mentioned in 10 09 13 |
| 10 09 16 | Waste crack-indicating agent other than those mentioned in 10 09 15 |
| 10 10 | Wastes from casting of non-ferrous pieces |
| 10 10 03 | Furnace slag |
| 10 10 06 | Casting cores and moulds which have not undergone pouring, other than those mentioned in 10 10 05 |
| 10 10 08 | Casting cores and moulds which have undergone pouring, other than those mentioned in 10 10 07 |
| 10 10 14 | Waste binders other than those mentioned in 10 10 13 |
| 10 10 16 | Waste crack-indicating agent other than those mentioned in 10 10 15 |
| 10 11 | Wastes from manufacture of glass and glass products |
| 10 11 03 | Waste glass-based fibrous materials |
| 10 11 10 | Waste preparation mixture before thermal processing, other than those mentioned in 10 11 09 |
| 10 11 12 | Waste glass other than those mentioned in 10 11 11 |
| 10 11 16 | Solid wastes from flue-gas treatment other than those mentioned in 10 11 15 |
| 10 11 18 | Filter cakes from flue-gas treatment other than those mentioned in 10 11 17 |
| 10 12 | Wastes from manufacture of ceramic goods, bricks, tiles and construction products |
| 10 12 01 | Waste preparation mixture before thermal processing |
| 10 12 05 | Filter cakes from gas treatment |
| 10 12 06 | Discarded moulds |
| 10 12 08 | Waste ceramics, bricks, tiles and construction products (after thermal processing) |
| 10 12 10 | Solid wastes from gas treatment other than those mentioned in 10 12 09 |
| 10 12 12 | Wastes from glazing other than those mentioned in 10 12 11 |
| 10 13 | Wastes from the manufacture of cement, lime, and plaster and articles and products made from them |
| 10 13 01 | Waste preparation mixture before thermal processing |
| 10 13 04 | Wastes from calcination and hydration of lime |
| 10 13 07 | Filter cakes from gas treatment |
| 10 13 10 | Wastes from asbestos-cement manufacture other than those mentioned in 10 13 09 |
| 10 13 11 | Wastes from cement-based composite materials other than those mentioned in 10 13 09 |
| 10 13 13 | Solid wastes from gas treatment other than those mentioned in 10 13 12 |
| 10 13 14 | Waste concrete |
| 11 | WASTES FROM CHEMICAL SURFACE TREATMENT AND COATING OF METALS AND OTHER MATERIALS; NON-FERROUS HYDRO METALLURGY |
| 11 01 | Wastes from chemical surface treatment and coating of metals and other materials (for example galvanic processes, zinc coating processes, pickling processes, etching, phosphating, alkaline degreasing, anodising) |
| 11 01 10 | Filter cakes other than those mentioned in 11 01 09 |
| 11 01 14 | Degreasing wastes other than those mentioned in 11 01 13 |
| 11 02 | Wastes from non-ferrous hydrometallurgical processes |
| 11 02 03 | Wastes from the production of anodes for aqueous electrolytical processes |
| 11 02 06 | Wastes from copper hydrometallurgical processes other than those mentioned in 11 02 05 |
| 11 05 | Wastes from hot galvanising processes |
| 11 05 01 | Hard zinc |
| 11 05 02 | Zinc ash |
| 12 | WASTES FROM SHAPING AND PHYSICAL AND MECHANICAL SURFACE TREATMENT OF METALS AND PLASTICS |
| 12 01 | Wastes from shaping and physical and mechanical surface treatment of metals and plastics |
| 12 01 01 | Ferrous metal filings and turnings |
| 12 01 03 | Non-ferrous metal filings and turnings |
| 12 01 05 | Plastic shavings and turnings |
| 12 01 13 | Welding wastes |
| 12 01 17 | Waste blasting material other than those mentioned in 12 01 16 |
| 12 01 21 | Spent grinding bodies and grinding materials other than those mentioned in 12 01 20 |
| 15 | WASTE PACKAGING: ABSORBENTS, WIPING CLOTHS, FILTER MATERIALS AND PROTECTIVE CLOTHING NOT OTHERWISE SPECIFIED |
| 15 01 | Packaging (including separately collected municipal packaging waste) |
| 15 01 01 | Paper and cardboard packaging |
| 15 01 02 | Plastic packaging |
| 15 01 03 | Wooden packaging |
| 15 01 04 | Metallic packaging |
| 15 01 05 | Composite packaging |
| 15 01 06 | Mixed packaging |
| 15 01 07 | Glass packaging |
| 15 01 09 | Textile packaging |
| 15 02 | Absorbents, filter materials, wiping cloths and protective clothing |
| 15 02 03 | Absorbents, filter materials, wiping cloths, and protective clothing other than those mentioned in 15 02 02 |
| 16 | WASTES NOT OTHERWISE SPECIFIED IN THE LIST |
| 16 01 | End-of-life vehicles from different means of transport (including off-road machinery) and wastes from dismantling of end-of-life vehicles and vehicle maintenance (except 13, 14, 16 06 and 16 08) |
| 16 01 03 | End-of-life tyres |
| 16 02 | Wastes from electrical and electronic equipment |
| 16 02 14 | Discarded equipment other than those mentioned in 16 02 09 to 16 02 13 |
| 16 02 16 | Components removed from discarded equipment other than those mentioned in 16 02 15 |
| 16 03 | Off-specification batches an unused products |
| 16 03 04 | Inorganic wastes other than those mentioned in 16 03 03 |
| 16 03 06 | Organic wastes other than those mentioned in 16 03 05 |
| 16 06 | Batteries and accumulators |
| 16 06 04 | Alkaline batteries (except 16 06 03) |
| 16 06 05 | Other batteries and accumulators |
| 16 11 | Waste linings and refractories |
| 16 11 02 | Carbon-based linings and refractories from metallurgical processes other than those mentioned in 16 011 01 |
| 16 11 04 | Other linings and refractories from metallurgical processes other than those mentioned in 16 11 03 |
| 16 11 06 | Linings and refractories from non-metallurgical processes other than those mentioned in 16 11 05 |
| 17 | CONSTRUCTION AND DEMOLITION WASTES (INCLUDING EXCAVATED SOIL FROM CONTAMINATED SITES) |
| 17 01 | Concrete, bricks, tiles and ceramics |
| 17 01 01 | Concrete |
| 17 01 02 | Bricks |
| 17 01 03 | Tiles and ceramics |
| 17 01 07 | Mixtures of concrete, bricks, tiles and ceramics other than those mentioned in 17 01 06 |
| 17 02 | Wood, glass and plastic |
| 17 02 01 | Wood |
| 17 02 02 | Glass |
| 17 02 03 | Plastic |
| 17 03 | Bituminous mixtures, coal tar and tarred products |
| 17 03 02 | Bituminous mixtures other than those mentioned in 17 03 01 |
| 17 04 | Metals (including their alloys) |
| 17 04 01 | Copper, bronze, brass |
| 17 04 02 | Aluminium |
| 17 04 03 | Lead |
| 17 04 04 | Zinc |
| 17 04 05 | Iron and steel |
| 17 04 06 | Tin |
| 17 04 07 | Mixed metals |
| 17 04 11 | Cables other than those mentioned in 17 04 10 |
| 17 05 | Soil (including excavated soil from contaminated sites) stones and dredging spoil |
| 17 05 04 | Soil and stones other than those mentioned in 17 05 03 |
| 17 05 08 | Track ballast other than those mentioned in 17 05 07 |
| 17 06 | Insulation materials and asbestos-containing construction materials |
| 17 06 04 | Insulation materials other than those mentioned in 17 06 01 and 17 06 03 |
| 17 08 | Gypsum-based construction material |
| 17 08 02 | Gypsum-based construction materials other than those mentioned in 17 08 01 |
| 17 09 | Other construction and demolition wastes |
| 17 09 04 | Mixed construction and demolition wastes other than those mentioned in 17 09 01, 17 09 02, and 17 09 03 |
| 19 | WASTES FROM WASTE MANAGEMENT FACILITIES, OFF-SITE WASTE WATER TREATMENT PLANTS AND PREPARATION OF WATER INTENDED FOR HUMAN CONSUMPTION/INDUSTRIAL USE |
| 19 01 | Wastes from incineration or pyrolysis of waste |
| 19 01 02 | Ferrous materials removed from bottom ash |
| 19 01 12 | Bottom ash and slag other than those mentioned in 19 01 11 |
| 19 01 18 | Pyrolysis wastes other than those mentioned in 19 01 17 |
| 19 01 19 | Sands from fluidised beds |
| 19 02 | Wastes from physico/chemical treatments of waste (including dechromatation, decyanidation, neutralisation) |
| 19 02 03 | Premixed wastes composed only of non-hazardous waste |
| 19 02 10 | Combustible wastes other than those mentioned in 19 02 08 and 19 02 09 |
| 19 04 | Vitrified waste and wastes from vitrification |
| 19 04 01 | Vitrified waste |
| 19 05 | Wastes from aerobic treatment of solid wastes |
| 19 05 01 | Non-composted fraction of municipal and similar wastes |
| 19 05 02 | Non-composted fraction of animal and vegetable waste |
| 19 05 03 | Off-specification compost |
| 19 12 | Wastes from the mechanical treatment of waste (for example sorting, crushing, compacting, pelletising) not otherwise specified |
| 19 12 01 | Paper and cardboard |
| 19 12 02 | Ferrous metal |
| 19 12 03 | Non-ferrous metal |
| 19 12 04 | Plastic and rubber |
| 19 12 05 | Glass |
| 19 12 07 | Wood other than that mentioned in 19 12 06 |
| 19 12 08 | Textiles |
| 19 12 09 | Minerals (for example sand, stones) |
| 19 12 10 | Combustible waste (refuse derived fuel) |
| 19 13 | Wastes from soil and groundwater remediation |
| 19 13 02 | Solid wastes from soil remediation other than those mentioned in 19 13 01 |
| 20 | MUNICIPAL WASTES (HOUSEHOLD WASTE AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |
| 20 01 | Separately collected fractions (except 15 01) |
| 20 01 01 | Paper and carboard |
| 20 01 02 | Glass |
| 20 01 08 | Biodegradable kitchen and canteen waste |
| 20 01 10 | Clothes |
| 20 01 11 | Textiles |
| 20 01 34 | Batteries and accumulators other than those mentioned in 20 01 33 |
| 20 01 36 | Discarded electrical and electronic equipment other than those mentioned in 20 01 21, 20 10 23, and 20 01 35 |
| 20 01 38 | Wood other than that mentioned in 20 01 37 |
| 20 01 39 | Plastics |
| 20 01 40 | Metals |
| 20 01 41 | Wastes from chimney sweeping |
| 20 02 | Garden and park wastes (including cemetery waste) |
| 20 02 01 | Biodegradable waste |
| 20 02 02 | Soil and stones |
| 20 03 | Other municipal wastes |
| 20 03 01 | Mixed municipal waste |
| 20 03 02 | Waste from markets |
| 20 03 03 | Street-cleaning residues |
| 20 03 07 | Bulky waste |

Table 9  Proposed Clinical Waste Types to be Accepted at the Site

| EWC Code | Description |
| --- | --- |
| 09 | WASTES FROM THE PHOTOGRAPHIC INDUSTRY |
| 09 01 | Wastes from the photographic industry |
| 09 01 01* | Water-based developer and activator solutions |
| 09 01 02* | Water-based offset plate developer solutions3 |
| 09 01 03* | Solvent based developer solutions3 |
| 09 01 04* | Fixer solutions3 |
| 09 01 05* | Bleach and bleach fixer solutions3 |
| 09 01 07 | Photographic film and paper containing silver or silver compounds3 |
| 09 01 08 | Photographic film and paper free of silver or silver compounds3 |
| 18 | WASTES FROM HUMAN OR ANIMAL HEALTHCARE AND/OR RELATED RESEARCH (EXCEPT KITCHEN AND RESTAURANT WASTES NOT ARISING FROM IMMEDIATE HEALTH CARE) |
| 18 01 | Wastes from natal care, diagnosis, treatment or prevention of disease in humans |
| 18 01 01 | Sharps (except 18 01 03) |
| 18 01 02 | Body parts and organs including blood bags and blood preserves (except 18 01 03) |
| 18 01 03* | Wastes whose collection and disposal is subject to special requirements in order to prevent infection (e.g. dressings, plaster casts, linen, disposable clothing, nappies) |
| 18 01 04 | Wastes whose collection and disposal is not subject to special requirements in order to prevent infection |
| 18 01 06* | Chemicals consisting of or containing hazardous substances |
| 18 01 07 | Chemicals other than those mentioned in 18 01 06 |
| 18 01 08* | Cytotoxic and cytostatic medicines |
| 18 01 09 | Medicines other than those mentioned in 18 01 08 |
| 18 01 10* | Amalgam waste from dental care |
| 18 02 | Wastes from research, diagnosis, treatment or prevention of disease involving animals |
| 18 02 01 | Sharps (except 18 02 02) |
| 18 02 02* | Wastes whose collection and disposal is subject to special requirements in order to prevent infection |
| 18 02 03 | Wastes whose collection and disposal is not subject to special requirements in order to prevent infection |
| 18 02 05* | Chemicals consisting of or containing hazardous substances |
| 18 02 06 | Chemicals other than those mentioned in 18 02 05 |
| 18 02 07* | Cytotoxic and cytostatic medicines |
| 18 02 08 | Medicines other than those mentioned in 18 02 07 |
| 20 | MUNICIPAL WASTES (HOUSEHOLD AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |
| 20 01 | Separately collected fractions (except 15 01) |
| 20 01 31* | Cytotoxic and cytostatic medicines |
| 20 01 32 | Medicines other than those mentioned in 20 01 31 |
| 20 01 99 | Other fractions not otherwise specified (consisting of nappies and absorbent hygiene products (AHPs) only) |

Table 10  Proposed Asbestos Waste to be Accepted at the Site

| EWC Code | Description |
| --- | --- |
| 17 | CONSTRUCTION AND DEMOLITION WASTES (INCLUDING EXCAVATED SOIL FROM CONTAMINATED SITES) |
| 17 06 | insulation materials and asbestos-containing construction materials |
| 17 06 01* | Insulation materials containing asbestos |
| 17 06 03* | Construction materials containing asbestos |

Dust Complaint Form

Table 11 Dust Complaint Form

| Complainant Details |  |
| --- | --- |
| Complainant Name: |  |
| Address and postcode: |  |
| Complainant contact details (telephone/ email): |  |
| Date & time of complaint: |  |
| Complaint reference number: |  |
| Complaint details: |  |
| Investigation Details |  |
| Investigation carried out by: |  |
| Investigator position/role: |  |
| Date & time of investigation: |  |
| Weather conditions at time of complaint and investigation: |  |
| Wind speed and direction at time of complaint and investigation: |  |
| Investigation findings: |  |
| Feedback given to NRW and/or local authority? |  |
| Date feedback given: |  |
| Feedback given to complainant and/or public? |  |
| Date feedback given: |  |
| Review and improve |  |
| Improvements needed to prevent a reoccurrence: |  |
| Proposed date for completion of required improvements: |  |
| Actual date of completion (to be filled in once completed): |  |
| If proposed date for completion of improvements was missed, state why: |  |
| Does the DMP need updating? |  |
| Date that the DMP was updated (if applicable): |  |
| Closure |  |
| Site supervisor review date: |  |
| Site supervisor signature (to confirm no further action required): |  |

Dust Event Log

Table 12 Dust Event Form

| Dust Event Form |  |
| --- | --- |
| Name of Author |  |
| Description of Event e.g. Complaint registered (name and address), or location of the visible dust crossing the site boundary |  |
| Date / Time |  |
| Activities taking place during time of event |  |
| Dust mitigation techniques employed at time of event |  |
| Summary of weather conditions leading up to and during the event |  |
| Details of corrective actions |  |
| Notes |  |

IAQM, Guidance on the Assessment of Mineral Dust Impacts for Planning, v1.1, 2016.

MIRO, Good practice guide: control and measurement of nuisance dust and PM10 from the extractive industries, February 2011.

The Meteorological Office, https://www.metoffice.gov.uk/research/climate/maps-and-data/uk-climate-averages [accessed December 2024].

This is limited to wastes of this type arising from medical practices or associated research activities.