Environmental Risk Assessment

High View Farm Waste Transfer Station Environmental Permit Application

West London Composting Limited

Prepared by:

SLR Consulting Limited

Treenwood House, Rowden Lane, Bradford on Avon, BA15 2AU

SLR Project No.: 402.065523.00001

14 October 2025

Revision: V2

Basis of Report

This document has been prepared by SLR Consulting Limited (SLR) with reasonable skill, care and diligence, and taking account of the timescales and resources devoted to it by agreement with West London Composting Limited (the Client) as part or all of the services it has been appointed by the Client to carry out. It is subject to the terms and conditions of that appointment.

SLR shall not be liable for the use of or reliance on any information, advice, recommendations and opinions in this document for any purpose by any person other than the Client. Reliance may be granted to a third party only in the event that SLR and the third party have executed a reliance agreement or collateral warranty.

Information reported herein may be based on the interpretation of public domain data collected by SLR, and/or information supplied by the Client and/or its other advisors and associates. These data have been accepted in good faith as being accurate and valid.

The copyright and intellectual property in all drawings, reports, specifications, bills of quantities, calculations and other information set out in this report remain vested in SLR unless the terms of appointment state otherwise.

This document may contain information of a specialised and/or highly technical nature and the Client is advised to seek clarification on any elements which may be unclear to it.

Information, advice, recommendations and opinions in this document should only be relied upon in the context of the whole document and any documents referenced explicitly herein and should then only be used within the context of the appointment.

Table of Contents

Basis of Report	i

1.0	INTRODUCTION	1

1.1	Methodology	1

2.0	IDENTIFYING THE RISKS	3

3.0	SITE SETTING AND RECEPTORS	4

3.1	Site Setting	4

3.1.1	Agricultural / Open Land	4

3.1.2	Commercial and Industrial	4

3.1.3	Residential	5

3.1.4	Local Transport Network	5

3.1.5	Surface Water Features	5

3.2	Geology, Hydrogeology and Hydrology	5

3.2.1	Geology	5

3.2.2	Hydrogeology	5

3.2.3	Hydrology	5

3.3	Ecology	6

3.3.1	European / Internationally Designated Sites	6

3.3.2	Nationally / Locally Designated Sites	6

3.4	Cultural Heritage	7

3.5	Receptors	7

3.6	Windrose	9

4.0	ENVIRONMENTAL RISK ASSESSMENT	11

5.0	CONCLUSION	22

Tables

Table 3-1 Surrounding Land Use	4

Table 3-2 Receptors	7

Table 4-1 Odour Risk Assessment and Management Plan	12

Table 4-2 Noise Risk Assessment and Management Plan	14

Table 4-3 Fugitive Emissions Risk Assessment and Management Plan	15

Table 4-4 Accidents Risk Assessment and Management Plan	19

-

-

-

-

-

-

Figures

Figure A: Windrose for Heathrow Meteorological Station (5-year average)	10

Drawings

Drawing 001 	Site Location Plan

Drawing 002 	Environmental Permit Boundary and Site Layout

Drawing 003	Site Setting Plan

Appendices

Appendix A	Bayhurst Wood/Ruislip Woods SSSI – Screening Assessment

# INTRODUCTION

West London Composting Limited (WLC) has retained SLR Consulting Limited (SLR) to prepare a bespoke Environmental Permit (EP) variation application for the proposed Waste Transfer Station (WTS), located in High View Farm, New Years Green Lane, Harefield, Middlesex, UB9 6LX, hereafter referred to as the ‘Site’. The facility already operates under an existing Waste Management Licence (WML) (No. GTL GRU017) and is operated by WLC,

This Environmental Risk Assessment (ERA) is a simple assessment of the risks to the environment and human health from accidents, odour, noise and fugitive emissions that may be associated with the proposed activities at the WTS.

## Methodology

The assessment has been completed in accordance with the Environment Agency (EA) Technical Guidance ‘Risk Assessments for your Environmental Permit’, last updated August 2022. The aim of the assessment is to identify any significant risks and to demonstrate that the risk of pollution or harm will be acceptable by taking the appropriate measures to manage these risks. The EA Guidance requires all receptors that are near the Site and could reasonably be affected by the activities to be identified and considered as part of the assessment.

This ERA uses the following approach for identifying and assessing the risks from the proposed High View Farm WTS:

Step 1	Identify and consider risks for your Site and the sources of the risks.

Step 2	Identify the receptors at risk from your Site.

Step 3	Identify the possible pathways from the sources of the risks to the receptors.

Step 4	Assess risks relevant to your specific activity and check they are acceptable and can be screened out.

Step 5  State what you will do to control the risks if they are too high.

Step 6 Submit your risk assessment as part of your EP application.

Section 2.0 of this document is a screening step to identify the receptors at risk as part of this assessment.

Section 3.0 identifies people or parts of the environment that could be harmed (at potentially significant risk) by the activity. The ERA for a bespoke EP application requires all receptors that are near the Site and could reasonably be affected by the activities to be identified and considered as part of the assessment.

Therefore, for the purpose of this report:

A 2km radius from the Site’s EP boundary has been adopted in reviewing RAMSAR, SAC, SPA and SSSIs and sensitive receptors of ecological importance along with features such as Sites of cultural and natural heritage; and

A radius of 500m from the Site’s EP boundary has been adopted for all other potentially sensitive local receptors (for example, residential, commercial, industrial, agricultural and surface water receptors)

The potentially sensitive receptors are illustrated on Drawing 003 and described in Table 3-2 below.

Section 4.0 of this document presents the assessment and demonstrates that any risks of pollution or harm will be mitigated to manage the risk.

This ERA should be read in conjunction with the following documents submitted with this EP application:

Application forms:

Parts A, C2, C4, and F1;

Drawings:

Drawing 001 Site Location Plan;

Drawing 002 Environmental Permit Boundary and Site Layout; and

Drawing 003 Site Setting Plan.

Non-Technical Summary (NTS);

Operating Techniques (OT) and Waste Acceptance Procedures (WAP);

Fire Prevention Plan (FPP);

Dust Management Plan (DMP);

Noise Management Plan (NMP);

Odour Management Plan (OMP);

Environmental Management System (EMS) Summary; and

Site Condition Report (SCR).

# IDENTIFYING THE RISKS

Step 2 is a general screening step to identify the potential risks to the environment from the development. The following is generally considered to require assessment for bespoke operations:

Amenity and Accidents;

Site Waste (installations only);

Global Warming Potential;

Odour;

Climate Change;

Noise; and

Point source emissions to air, water and land.

- There will be no point source emissions to groundwater, surface water, air or land resulting from the proposed High View Farm WTS, and neither will there be any Site waste arising or global warming potential.

- Therefore, only ‘Amenity and Accidents’ remains applicable for assessment in this instance, and includes the consideration of odour, noise and vibration, fugitive emissions (including dust, mud, litter and pests) and accidents.

# SITE SETTING AND RECEPTORS

This section identifies the Site setting and potentially sensitive receptors in the vicinity of the Site.

## Site Setting

The Site is located on High View Farm, Middlesex, UB9 6LX centred on National Grid Reference (NGR) TQ 07093 88015. The town of Ruislip is located approximately 2.5km south-east of the Site.

The area surrounding the Site comprises predominantly agricultural / open land and commercial / industrial premises. Grand Union Canal is located approximately 2km west of the Site at its closest point. There are several designated ecological habitats within 2km of the Site, including several ancient woodlands and a SSSI.

The Site will be accessed via a track leading to Newyears Green Lane which is located north of the EP boundary.

The Site’s location is illustrated on Drawing 001, and the EP Boundary and Site Layout are illustrated in Drawing 002. Local receptors within a 500m radius of the Site are shown on Drawing 003.

A summary of the immediate surrounding land use is provided in Table 3-1.

Table 3-1 Surrounding Land Use

| Boundary | Description |
| --- | --- |
| North | Adjacent to the north are industrial premises within the same industrial complex as the Site. |
| East | Immediately to the east lies industrial premises. The land beyond this predominantly comprises open / agricultural land. |
| South | Open / agricultural land lies immediately south of the Site. |
| West | Immediately to the west lies industrial premises within the same industrial complex as the Site. |

The immediate surrounding land uses are described in further detail below.

### Agricultural / Open Land

The area surrounding the Site comprises predominantly agricultural / open land. The Site is bounded on the southern EP boundary by agricultural / open land.

### Commercial and Industrial

Within 500m of the Site, there are thirteen commercial / industrial premises, the closest are as follows:

Adjacent to the Site’s western boundary lies a commercial premises belonging to Superior Stone Limited; and

Adjacent on the Site’s eastern boundary is a waste facility operated by Envar. This recycling centre will be cleared and operations ceased prior to operations at the Site beginning.

A HS2 construction site lies approximately 220m north-west of the Site.

### Residential

There are a limited number of residential properties within 500m of the proposed Site. The closest residential receptors are individual properties situated approximately 30m north, 120m north-west, 150m north-east, and 280m south-west of the Site.

### Local Transport Network

Newyears Green Lane is located approximately 80m from Site’s northern EP boundary. In addition to this, there are two unnamed roads located 250m south-west and 400m north-west of the Site. There are also two unnamed tracks, approximately 20m south and 260m south-west of the Site.

### Surface Water Features

There are a number of surface water features within 500m of the Site, including a lagoon, a stream and an unnamed small lake. The closest of these is a small lake approximately 100m west of the Site.

## Geology, Hydrogeology and Hydrology

### Geology

A review of the British Geological Survey (BGS) map, reveals that the Site is underlain by a bedrock of London Clay Formation (clay, silt and sand). This is a sedimentary bedrock formed between 56 and 47.8 million years ago during the Palaeogene period.

There are no superficial geological deposits underlying the Site.

### Hydrogeology

#### Aquifer Designations

There are no bedrock deposits underlying the Site according to the Multi-Agency Geographical Information for the Countryside (MAGIC) map.

There are no superficial aquifer classifications underlying the Site.

#### Source Protection Zones

The Site is located within a zone I and zone II Source Protection Zone.

### Hydrology

#### Groundwater Vulnerability

The Groundwater Vulnerability layer on MAGIC map reveals that the Site lies within an area classified as ‘unproductive’.

#### Flood Zone

The Flood Map for Planning confirms that the Site lies within Flood Zone 1, which is defined as “land having a less than 1 in 1,000 annual probability of river or sea flooding”.

## Ecology

### European / Internationally Designated Sites

#### Sites of Special Scientific Interest (SSSI)

There are two SSSIs which lie within a 2km radius of the Site. The SSSI’s are described as follows;

Bayhurst Wood/Ruislip Woods SSSI lies 460m to the north-east of the Site and is designated for areas of semi-natural ancient woodland and other semi-natural habitats found there; and

Fray’s Farm Meadows SSSI is located approximately 1.87km south-west of the Site and is designated for the unimproved wet alluvial grassland found there.

WLC has prepared a Screening Assessment specifically in regard to the risk of operations affecting the Bayhurst Wood/Ruislip Woods SSSI. The Screening Assessment is provided as Appendix A of this ERA.  Given the distance of the SSSI from the operations and the controls to be put in place, there is considered to be no likely significant effect on the SSSI.

#### Other Receptors

A review of MAGIC map identifies that none of the following receptors are located within 2km of the Site:

Ramsar;

Special Areas of Conservation;

Special Protection Area; and

Marine Conservation Zone.

### Nationally / Locally Designated Sites

The following national designated Sites were identified within 2km of the Site;

#### Ancient Woodland;

There are five areas of Ancient & Semi-Natural Woodland within 2km of the Site. The closest of these is Newyears Green Covert which lies 400m south-west of the Site.

#### Local Nature Reserves

There are two Local Nature Reserves within 2km of the Site: Frays Valley, located 1490m south-west of the Site, and Denham Quarry Park, located 1.6km south-west of the Site.

#### National Nature Reserve

There is one National Nature Reserve located within 2km of the Site: Ruislip Woods, located 480m north-east of the Site.

#### Other Receptors

A review of MAGIC map identifies that none of the following receptors are located within 2km of the Site:

Areas of Outstanding Natural Beauty

## Cultural Heritage

A review of MAGIC confirmed that the following receptors were present within a 2km radius of the Site:

#### Listed Buildings

There are a number of listed buildings within 2km of the proposed Site. The closest of each listed building grade are as follows:

Grade II listed St Leonard’s Farmhouse, is located 140m to the north;

Grade I listed Breakspear House, is located 1.9km to the north-west; and

Grade II* listed Great Barn to west of Manor Farm Yard, located 1.87km east.

#### Scheduled Monuments

There are three scheduled monuments located within 2km of the proposed Site boundary. The closet of these is Brakenbury Farm, located approximately 760m to the south.

#### Other Receptors

A review of MAGIC map identifies that none of the following receptors are located within 2km of the Site:

World Heritage Sites;

Registered Battlefields; and

Registered Parks and Gardens

## Receptors

Local receptors within 500m of the Site are recorded in Table 3-2 below, along with natural and cultural receptors within 2km.

Table 3-2 Receptors

| Receptor Name | Receptor Type | Direction | Approximate Distance from Permit Boundary (m) |
| --- | --- | --- | --- |
| Local Receptors within 500m of the proposed EP boundary, as shown on Drawing 003 |  |  |  |
| Agricultural / Open Land | Agricultural receptors | South | Adjacent |
| Superior Stone Limited | Commercial premises | West | Adjacent |
| Agricultural / Open Land | Agricultural receptors | South-west | Adjacent |
| Unnamed track | Local transport network | South | Adjacent |
| Westglaze Limited | Commercial premises | North-west | 20 |
| Aesir Automotive Ltd BMW specialist | Commercial premises | West | 25 |
| Residential properties | Residential properties | North | 30 |
| Lami Auto | Commercial premises | West | 80 |
| Mackenzies Removals and Storage | Commercial premises | North-east | 80 |
| Newyears Green Lane | Local transport network | North | 80 |
| Agricultural / Open Land | Agricultural receptors | East | 95 |
| Small Lake | Surface water receptor | West | 100 |
| Smart Glazing & Home Improvements | Commercial premises | North-east | 100 |
| Residential properties | Residential properties | North | 110 |
| Ace Grab Hire and Haulage | Commercial premises | North | 145 |
| Logs U Like | Commercial premises | West | 150 |
| Residential properties | Residential properties | North-east | 150 |
| Industrial Site | Industrial premises | North-east | 165 |
| JM Motors | Commercial premises | North-east | 185 |
| Unnamed track | Local transport network | South-west | 200 |
| Boward Tree Surgery | Commercial premises | West | 210 |
| Residential properties | Residential properties | North-east | 210 |
| HS2 construction site | Industrial premises | North-west | 220 |
| Unnamed road | Local transport network | South-west | 230 |
| Residential properties | Residential properties | South-west | 280 |
| Stream | Surface water receptor | North-west | 310 |
| Crows Nest Farm | Residential properties | East | 330 |
| Lagoon | Surface water receptor | North-west | 380 |
| Unnamed road | Local transport network | North-west | 400 |
| Country Compost | Commercial premises | East | 430 |
| Unnamed lake | Surface water receptor | South-east | 480 |
| Ecological and Cultural Heritage Receptors within 2km of the proposed EP boundary |  |  |  |
| St Leonard's Farmhouse | Listed Buildings | North | 140 |
| Newyears Green Covert | Ancient Woodland | South-west | 400 |
| Bayhurst Wood Country Park | Ancient Woodland | North-west | 450 |
| Ruislip Woods | SSSI’s (Sites of Special Scientific Interest) (England) National Nature Reserve | North-east | 460 |
| Crow's Nest Farmhouse | Listed Buildings | East | 520 |
| Brakenbury Farm | Scheduled Monuments | South | 760 |
| Mad Bess Wood | Ancient Woodland | North | 980 |
| Medieval Moated Site | Scheduled Monuments | South | 1150 |
| Frays Valley | Local Nature Reserves (England) | South-west | 1490 |
| Park Wood | Ancient Woodland | North-east | 1550 |
| Ruislip Motte and Bailey | Scheduled Monuments | East | 1730 |
| The Pinnocks Wood | Ancient Woodland | South-west | 1740 |
| Great Barn to west of Manor Farm Yard* | Listed Buildings | East | 1860 |
| Breakspear House | Listed Buildings | North-west | 1870 |
| Denham Quarry Park | Local Nature Reserves (England) | South-west | 1600 |
| Fray's Farm Meadows | SSSI’s (Sites of Special Scientific Interest) (England) | South-west | 1870 |
| Dovecote to northwest of Breakspear House* | Listed Buildings | North-west | 1930 |

## Windrose

A wind rose for Heathrow meteorological station (5-year average 2018, 2019, 2020, 2022 & 2023), located approximately 11km south of the Site is presented in Figure A. The wind rose shows winds from the southwest are most frequent. Therefore, locations to the northeast of the Site are most likely to be impacted by potential odour emissions.

Figure A: Windrose for Heathrow Meteorological Station (5-year average)

# ENVIRONMENTAL RISK ASSESSMENT

The following tables assess the Site in terms of potential hazards posed, receptors and pathways along with management and assessment of the identified risks.

The probability of exposure is the likelihood of the receptors being exposed to the hazard and is defined as low, medium, or high. These terms are qualified as follows:

Low: exposure is unlikely, barriers in place to mitigate against exposure.

Medium: exposure is fairly probable, barriers to exposure less controllable.

High: exposure is probably, direct exposure likely with few barriers.

The methodology outlined in Section 1.1 of this report is the basis on which it is determined whether the proposed High View Farm WTS will lead to significant impact on the surrounding environment. Where a conclusion of ‘not significant’ has been reached, it is proposed that the mitigation and management measures that will be in place at the Site will be sufficient to ensure that there will be no impact at the surrounding environment.

Table 4-1 Odour Risk Assessment and Management Plan

| What Do You Do That Can Harm and What Could Be Harmed |  |  | Managing The Risk | Assessing The Risk |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hazard | Receptor | Pathway | Risk Management | Probability of Exposure | Consequences | What is the overall risk |
| What has the potential to cause harm? | What is at risk/What do I wish to protect? | How can the hazard get to the receptor? | What measures will you take to reduce the risk? Who is responsible for what? | How likely is the contact? | What is the harm that can be caused? | What is the risk that still remains? The balance of probability and consequence |
| Odour from receipt and handling of wastes including clinical waste and food waste. Odour from the storage of wastes including clinical waste, food waste and green waste. | Sensitive receptors listed in Table 3-2 including residential, commercial, agricultural recreational and ecological receptors. | Air | The proposed Site will accept, predominantly non-hazardous mixed waste including co-mingled recyclable materials, bulky waste, paper and cardboard, residual waste, street sweepings, garden waste, clinical waste, and food waste, all of which will be handled and stored within the new WTS building. Wastes stored outside in enclosed skips will comprise asbestos, end-of-life tyres and metal wastes only. Whilst most of the waste has a low odour potential, it is recognised that the clinical waste, food waste and green waste have a higher odour potential. Higher odour potential wastes are handled and stored inside the WTS building. Outside waste storage will not comprise wastes with high odour potential. The Site will be operated in accordance with the with the OMP included as part of the EP variation application. Odour mitigation and management measures as detailed in the OMP are summarised below: Clinical waste types will be kept segregated from the other wastes streams at all times; The majority of wastes will be accepted and stored within a dedicated WTS building. The WTS building will be fully enclosed, to ensure ingress of odour from the building is minimised; Wastes stored outside of the WTS building will comprise of wastes with low odour potential only; Fast-acting roller shutter doors will be installed on the new WTS building, and kept closed during tipping and unloading of wastes; Strict waste acceptance procedures will be adhered to, to ensure only permitted wastes are accepted on Site; All waste storage containers and bays within the new WTS building will be clearly labelled to ensure the segregation of waste; Odour masking sprays will be fitted within the new WTS building; Potentially odorous wastes will be a stored for minimal periods of time, with clinical waste stored for a maximum of 5 days. The Site will be monitored for odours by Site personnel throughout the working week. In the event that odours are detected, investigations will be undertaken to determine the cause and appropriate remedial action taken. The Site Manager will be responsible for implementing risk management measures in accordance with the Operating Techniques (OT) document (402.065523.00001_OTWAP), and OMP (402.065523.00001_OMP). | Medium | Odour Nuisance and loss of amenity. | Low |

Table 4-2 Noise Risk Assessment and Management Plan

| What Do You Do That Can Harm and What Could Be Harmed |  |  | Managing The Risk | Assessing The Risk |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hazard | Receptor | Pathway | Risk Management | Probability of Exposure | Consequences | What is the overall risk |
| What has the potential to cause harm? | What is at risk/What do I wish to protect? | How can the hazard get to the receptor? | What measures will you take to reduce the risk? Who is responsible for what? | How likely is the contact? | What is the harm that can be caused? | What is the risk that still remains? The balance of probability and consequence |
| Noise from the delivery and handling of waste at the Site. | Sensitive receptors listed in Table 3-2 including residential, commercial, agricultural recreational and ecological receptors. | Air | The Site is located within an area largely surrounded by agricultural / open land and other industrial / commercial premises. The closest sensitive receptor to noise is ‘Superior Stone Limited’ located adjacent on the western boundary, and the closest residential receptor is along Newyears Green Lane. The following procedures will be in place at the Site, as detailed in the NMP to ensure that noise from the acceptance and handling of waste on Site is minimised; The majority of waste will be accepted and stored within an enclosed WTS building; Site operations will be restricted to hours specified in the planning consent; All plant will be switched off when not in use; Plant will be selected & operated to minimise noise. All Site plant and machinery will be operated and maintained in accordance with manufacturer’s specifications; If horns or alarms on Site plant or infrastructure, or delivery vehicles are deemed to cause unacceptably high levels of noise, alternative technologies will be explored and implemented; Speed limits will be implemented for vehicles using the Site; Traffic calming measures will be implemented to enforce speed limits; and Site access roads and operational areas will be maintained and repaired to minimise emissions of noise due to uneven and poor surfacing. Auditory inspections will be carried out daily & in response to complaints. A record of the inspection findings and any complaints will be made in the Site diary. The Site Manager will be responsible for implementing risk management measures in accordance with the OT document (402.065523.00001_OTWAP), and NMP (402.065523.00001_NMP). | Low | Noise disturbance and loss of amenity. | Low |

Table 4-3 Fugitive Emissions Risk Assessment and Management Plan

| What Do You Do That Can Harm and What Could Be Harmed |  |  | Managing The Risk | Assessing The Risk |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hazard | Receptor | Pathway | Risk Management | Probability of Exposure | Consequences | What is the overall risk |
| What has the potential to cause harm? | What is at risk/What do I wish to protect? | How can the hazard get to the receptor? | What measures will you take to reduce the risk? Who is responsible for what? | How likely is the contact? | What is the harm that can be caused? | What is the risk that still remains? The balance of probability and consequence |
| To Air: |  |  |  |  |  |  |
| Dust from vehicle movements and operation of plant machinery. Dust from acceptance, handling, and storage of waste. | Sensitive receptors listed in Table 3-2 including residential, commercial, agricultural recreational and ecological receptors. | Air | The proposed Site will accept, predominantly non-hazardous mixed waste including co-mingled recyclable materials, bulky waste, paper and cardboard, residual waste, street sweepings, garden waste, clinical waste, and food waste. Due to the nature of these wastes, the potential risk of dust emissions from acceptance handling and storage of the wastes are low. Wastes stored outside of the building will comprise asbestos, end-of-life tyres and metal waste only. These wastes will be stored in enclosed skips. The Site will be operated in accordance with the DMP included as part of the EP application. Dust mitigation and management measures as detailed in the DMP are summarised below: The majority of waste will be accepted to and stored within a new fully enclosed WTS building; The new WTS building will have fast action roller shutter doors that will be closed during tipping and waste handling; Wastes stored outside of the building will comprise asbestos, end-of-life tyres and metal waste only. These wastes will be stored in enclosed skips to prevent the emission of dust; Speed limits will be implemented for vehicles using the Site; Traffic calming measures will be implemented to enforce speed limits & reduce emissions of dust; Site access roads and operational areas will be maintained and repaired to minimise emissions of dust due to uneven and poor surfacing; All roads and operational areas will be swept where necessary to reduce dust emissions; All vehicles delivering waste to the Site shall be sheeted to minimise emissions of dust; and Drop heights will be minimised to prevent emissions of dust. Daily visual monitoring will be carried out by all members of staff throughout their shift with any potential emissions of dust reported to the Site Manager. The Site Manager will be responsible for implementing risk management measures in accordance with the OT document (402.065523.00001_OTWAP), and DMP (402.065523.00001_DMP). | Low | Nuisance and harm to human health | Negligible |
| Bioaerosols from the storage of wastes | Sensitive receptors listed in Table 3-2 including residential, commercial, agricultural recreational and ecological receptors. | Air | It is recognised that the acceptance and storage of green waste, and food waste could lead to the release of bioaerosols. To prevent the release of bioaerosols, all green and food waste will be stored internally within the fully enclosed WTS building. The WTS building will benefit from roller shutter doors that will be closed during waste tipping, and handling. No treatment of waste will occur, and the waste will be stored for a minimal length of time (max 5 days). During storage, minimal handling and compaction of the waste will occur. Strict waste acceptance procedures will be implemented to ensure that only permitted wastes are accepted on Site. The Site Manager will be responsible for implementing risk management measures in accordance with the OT document (402.065523.00001_OT). | Negligible. | Nuisance and harm to human health | Negligible |
| To Water: |  |  |  |  |  |  |
| Contaminated run off. | Sensitive receptors listed in Table 3-2 including residential, commercial, agricultural recreational and ecological receptors. Groundwater. | Land | All waste inside the building will be stored on impermeable surfacing, which will minimise the exposure of wastes to the elements and reduce the risk of contaminated run off and generation of dirty process water. Dirty process water will be contained within the sealed drainage system, preventing uncontrolled risk of contaminated run off from the Site. Any contaminated run off within the building would be contained in the building and would be regularly collected by tanker vehicles for removal from Site and off-Site treatment. Waste stored outside the building will comprise of asbestos, metal and tyres. These wastes will be stored in enclosed skips to prevent the ingress of water. Surfacing of the Site outside of the WTS building comprises hardstanding. The Site Manager will be responsible for implementing risk management measures in accordance with the OT document (SLR Ref: 402.065523.00001_OT). | Low | Contamination | Very Low |
| Pests |  |  |  |  |  |  |
| Birds, pests and insects. | Sensitive receptors listed in Table 3-2 including residential, commercial, agricultural recreational and ecological receptors. | Land, Water and Air | Strict waste acceptance procedures will be implemented to ensure that only authorised wastes are accepted. In the event that non-conforming wastes are delivered to Site, they will be isolated and removed from Site at the earliest opportunity. All waste with the potential to attract pests will be stored within the fully enclosed WTS building, minimising the risk of pests. Wastes to be stored outside of the WTS building are not considered to be Timeframes for storage of wastes will be kept as low as practically possible, and all waste is stored for a maximum of 5 days. In the event that birds, vermin or insects are identified at the Site, a specialist pest control contractor will be employed to undertake remedial measures. The Site Manager will be responsible for implementing risk management measures in accordance with the OT document (SLR Ref: 402.065523.00001_OT). | Medium | Nuisance, potential risk to health | Low |
| Mud/Litter |  |  |  |  |  |  |
| Litter from waste | Sensitive receptors listed in Table 3-2 including residential, commercial, agricultural recreational and ecological receptors. | Air | The following management techniques will be employed at the Site, to ensure that the risk of generation of litter from wastes is minimised: Strict waste acceptance procedures will ensure that only authorised wastes are accepted; The Site will benefit from good housekeeping and all areas of the Site will be cleaned on a daily basis; All Site vehicles leaving operational areas will be inspected to ensure that they are clear of loose waste; The majority of wastes will be accepted to and stored within an enclosed WTS building. Wastes stored outside of the building will be contained within enclosed skips only. The Site and its immediate surrounding will be inspected on a daily basis and action will be taken to maintain the area free of significant accumulations of litter and debris. Any excessive litter material at the facility or on the highways will be cleared using a mechanical sweeper and / or litter picker if required. The Site Manager will be responsible for implementing risk management measures in accordance with the OT document (SLR Ref: 402.065523.00001_OT). | Low | Nuisance from litter. Dangerous conditions on roads. | Negligible |
| Mud from vehicle movements | Local Road Network | Transferral of mud on vehicles wheels | All access roads will be tarmacked, and as such the risk of mud track out from traffic and plant machinery movements will be low. Despite this, the following management techniques will be employed at the Site, to ensure that the risk of mud track out is minimised: Areas of hardstanding and impermeable surfacing will be maintained to be free of significant quantities of mud and debris; All vehicles will be covered when loads are entering and exiting the facility; Roads will be swept and cleaned whenever necessary; and In the event that mud, debris or waste arising from the Site is deposited outside the Site boundary, the affected area will be cleaned, and traffic will be isolated from sources of mud and debris within the Site. Daily visual inspection of the Site by Site management will identify any problem with mud which will be cleaned up as soon as possible. Where necessary road cleaning equipment will be deployed. The Site Manager will be responsible for implementing risk management measures in accordance with the OT document (SLR Ref: 402.065523.00001/OT). | Low | Nuisance from mud. Dangerous conditions on roads. | Negligible |

Table 4-4 Accidents Risk Assessment and Management Plan

| What Do You Do That Can Harm and What Could Be Harmed |  |  | Managing The Risk | Assessing The Risk |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hazard | Receptor | Pathway | Risk Management | Probability of Exposure | Consequences | What is the overall risk |
| What has the potential to cause harm? | What is at risk/What do I wish to protect? | How can the hazard get to the receptor? | What measures will you take to reduce the risk? Who is responsible for what? | How likely is the contact? | What is the harm that can be caused? | What is the risk that still remains? The balance of probability and consequence |
| Leakage of fuel and oils | Local surface water features including rivers, streams and drains. Groundwater. | Runoff and percolation through ground | The majority of waste will be stored within the enclosed WTS building, which will benefit from impermeable surfacing and sealed drainage. All waste outside of the WTS building will comprise of asbestos, end-of-life tyres and metal waste only. Waste outside of the WTS building will be stored in enclosed skips. Tanks used for the storage of fuel and maintenance oil, will be constructed so that any leaks/spillages will be contained. Tanks will be surrounded by a leakage containment bund capable of containing at least 110% of the volume of the largest tank within the bund or 25% of the total tank volume within the bund, whichever is the greater. Storage tanks will be constructed to the appropriate British Standard. Tanks will be inspected visually on a daily basis by the Site staff to ensure the continued integrity of the tanks and identify the requirement for any remedial action. Minor spillages will be cleaned up immediately, using sand or proprietary absorbent to clean up liquids and placed in alternative containers. Materials suitable for absorbing and containing minor spillages will be maintained on Site. The Site staff will undertake daily monitoring for evidence of spillage and leakage. Alongside regular visual inspections, the tanks will be fitted with level indicators to prevent overfilling. In the event of a major spillage immediate action will be taken to contain the spillage and prevent liquid from entering surface water drains and the unsurfaced ground. The spillage will be cleared immediately and placed in containers for off-Site disposal and the EA will be notified. The Site Manager will be responsible for implementing risk management measures in accordance with OT document (SLR Ref: 402.065523.00001_OT). | Low | Contamination of surroundings | Low |
| Unauthorised waste | Sensitive receptors listed in Table 3-2 including residential, commercial, recreational, ecological and agricultural receptors. | Via air (odours) Overland (to sewers, surface and groundwater) | Upon delivery waste will be subject to strict waste acceptance procedures to identify, reject and/or segregate potentially non-conforming waste. Only waste authorised by the EP will be accepted at the Site. All wastes will be subject to inspection and checking against the declaration on the waste transfer note. In the event that unauthorised waste is delivered to the Site, the waste will be segregated and stored in the designated quarantine area within the WTS building prior to export from Site to an alternative suitably permitted facility. The Site Manager will be responsible for implementing risk management measures in accordance with the OT document (402.065523.00001_OT). | Low | Odour nuisance Water contamination | Low |
| Fire | Sensitive receptors listed in Table 3-2 including residential, commercial, recreational, ecological and agricultural receptors. Site personnel. | Air and Land | The Site will be operated in accordance with the FPP, included within Section 5 of this EP application. | Low | Harm to human health, harm to operations, pollution of surroundings. | Low |
| Flooding | Sensitive receptors listed in Table 3-2 including residential, commercial, agricultural recreational and ecological receptors. Site personnel. | Land | The Site is located within a Flood Zone 1, which means it has a low probability of flooding. Evacuation procedures will be implemented in the event of flooding. The Site Manager will be responsible for implementing risk management measures in accordance with the OT document (SLR Ref: 402.065523.00001_OT). | Low | Harm to human health, contamination of groundwater and surface water. | Very low |
| Security and Vandalism | Sensitive receptors listed in Table 3-2 including residential, commercial, agricultural recreational and ecological receptors. | Air, Land and Water | The Site will benefit from the following security measures: The Site will be manned during operational hours by Site staff who will undertake inspections; An internal and external CCTV monitoring system which can be monitored on Site or remotely; A 2.4m high steel palisade security fence, surrounding the EP boundary; and Gated entrances to prevent the access of unauthorised persons. The Site will be inspected daily by the operations staff to identify deterioration and damage and the need for any repairs. The Site will be maintained and repaired to ensure its continued integrity. In the event that damage is sustained repairs will be made by the end of the working day. If this is not possible, suitable measures will be taken to prevent any unauthorised access to the Site and permanent repairs will be affected as soon as practicable. All visitors will be required to register in the visitor’s book and sign out again on exit. This minimises the risk of unauthorised visitors being present at the Site. Operational procedures, including regular inspections, ensure continual monitoring of security provision at the Site. The Site Manager will be responsible for implementing risk management measures in accordance with the OT document (SLR Ref: 402.065523.00001_OT). | Low | Nuisance, Contamination and harm to human health. | Low |

# CONCLUSION

This ERA has been undertaken as described by the EA regulatory guidance. The assessment is provided as part of the variation application for a bespoke EP for the High View Farm WTS.

This qualitative risk assessment, in addition to the specific impact assessments and management plans, has considered odour, noise, fugitive emissions, dust, releases to water, litter, and potential for accidents and incidents. The assessment concludes that with the implementation of the risk management measures described above, and in the separate management plans potential hazards from the proposed development are not likely to be significant and no further assessment is required.

Bayhurst Wood/Ruislip Woods -Screening Assessment

Risk assessments for your environmental permit - GOV.UK (www.gov.uk)

British Geological Society, geology viewer map https://geologyviewer.bgs.ac.uk/ accessed in November 2024

Multi-Agency Geographical Information for the Countryside Map, available at www.magic.gov.uk, accessed in November 2024

Gov.uk, Flood Map for Planning, available at https://flood-map-for-planning.service.gov.uk/, accessed in November 2024