Odour Management Plan

High View Farm Waste Transfer Station Environmental Permit Application

West London Composting Limited

Prepared by:

SLR Consulting Limited

Treenwood House, Rowden Lane, Bradford on Avon, BA15 2AU

SLR Project No.: 402.065523.00001

Client Reference No: UK.10483

14 October 2025

Revision: 2

Revision Record

| Revision | Date | Prepared By | Checked By | Authorised By |
| --- | --- | --- | --- | --- |
| 1.0 | 30 January 2025 | RM | RJ | RJ |
| 1.1 | 14 February 2025 | GW | GS | GS |
| 2 | 14 October 2025 | GW | GS | GS |

Basis of Report

This document has been prepared by SLR Consulting Limited (SLR) with reasonable skill, care and diligence, and taking account of the timescales and resources devoted to it by agreement with West London Composting Limited (the Client) as part or all of the services it has been appointed by the Client to carry out. It is subject to the terms and conditions of that appointment.

SLR shall not be liable for the use of or reliance on any information, advice, recommendations and opinions in this document for any purpose by any person other than the Client. Reliance may be granted to a third party only in the event that SLR and the third party have executed a reliance agreement or collateral warranty.

Information reported herein may be based on the interpretation of public domain data collected by SLR, and/or information supplied by the Client and/or its other advisors and associates. These data have been accepted in good faith as being accurate and valid.

The copyright and intellectual property in all drawings, reports, specifications, bills of quantities, calculations and other information set out in this report remain vested in SLR unless the terms of appointment state otherwise.

This document may contain information of a specialised and/or highly technical nature and the Client is advised to seek clarification on any elements which may be unclear to it.

Information, advice, recommendations and opinions in this document should only be relied upon in the context of the whole document and any documents referenced explicitly herein and should then only be used within the context of the appointment.

Table of Contents

1.0	Introduction	1

1.1	Scope of Odour Management Plan	1

1.2	Key Guidance	2

2.0	Baseline Environment	2

2.1	Location	2

2.2	Other Potential Sources of Odour	2

2.3	Sensitive Receptors	3

2.4	Meteorological Conditions	4

3.0	Operations at High View WTS	6

3.1	Site Operations	6

3.2	Hours of Operation	6

3.3	Waste Acceptance	6

3.4	Storage	6

3.5	On-Site Transportation / Material Handling	7

3.6	Off-Site Transportation / Import and Export Operations	7

4.0	Potential odour Sources and Magnitude	8

4.1	Source	8

4.2	Pathway	10

4.3	Receptors	10

4.4	Significance of Effects	11

5.0	Control Measures and Process Monitoring	12

6.0	Monitoring and Maintenance	16

6.1	Monitoring of Ambient Odours	16

6.2	Control Measures during Routine Maintenance	16

6.3	Monitoring Meteorological Conditions	16

6.4	Recording of Results and Reporting	17

6.5	Notifying the EA	17

7.0	Contingencies	18

7.1	Foreseeable Events	18

7.2	Detection of Odour in Response to Complaints	19

7.3	Out of Hours Contact Details	19

7.4	Receipt of an Odour Complaint	19

8.0	Document Updates and Reviews	21

8.1	Responsible Staff	21

8.2	General Procedures for Training and Competency of Staff	21

8.3	Odour Management Plan Review	21

Tables in Text

Table A:	Sensitive Human Receptors	3

Table B:	Pathway Effectiveness Summary	10

Table C:	SPR Summary	11

Table D:	Appropriate Techniques / BAT	12

Table E:	Foreseeable Events	18

Table A-1 Proposed Non-Hazardous Waste Types to be Accepted at the Site	1

Table A-2 Proposed Clinical Waste Types to be Accepted at the Site	10

Table A-3 Proposed Asbestos Waste to be Accepted at the Site	11

-

-

-

Figures in Text

Figure A: Site Setting	3

Figure B: Sensitive Receptors	4

Figure C: Windrose for Heathrow Meteorological Station (5-year average)	5

Appendices

Appendix A	European Waste Catalogue (EWC) Codes

Appendix B	Odour Assessment Form

Appendix C	Odour Complaints Reporting Form

Appendix D	Odour Survey Methodology

Appendix E	Odour Risk of each EWC Code

Appendix F	Off-site Odour Monitoring

Drawings

Drawing 001 	Site Location Plan

Drawing 002 	Environmental Permit Boundary and Site Layout

Drawing 003	Site Setting Plan

# Introduction

West London Composting Limited (WLC) has retained SLR Consulting Limited (SLR) to prepare an Odour Management Plan (OMP) for the proposed Waste Transfer Station (WTS) (the ‘Facility’), located in High View Farm, New Years Green Lane, Harefield, Middlesex, UB9 6LX (the ‘Site’).

The Facility will undertake waste bulking and transfer operations, managing up to 75,000 tonnes per annum (tpa) of mixed waste, a small proportion of which will consist of clinical waste (approximately 10,000 tpa) including nappies and sharps.

The activities that will be carried out at the site as defined under Annex II of the Waste Framework Directive can be summarised as follows:

R3: Recycling or reclamation of organic substances which are not used as solvents;

R4: Recycling or reclamation of metals and metal compounds;

R5: Recycling or reclamation of other inorganic materials;

R13: Storage pending recovery or disposal.

D9: Physico-chemical treatment not specified elsewhere which results in final compounds or mixtures which are discarded by means of any of the operations numbered D1 to D8 and D10 to D12.

D14: Repackaging prior to submission to any of the operations numbered D1 to 13.

D15: Storage pending any of the operations numbered D1 to D14 (excluding temporary storage, pending collection, on the site where it is produced).

All waste will be stored in designated concrete bays or containers within the enclosed WTS building which will benefit from impermeable surfacing and a sealed drainage system throughout. All activities would take place within the enclosed building providing a level of containment to potential odours.

## Scope of Odour Management Plan

It is recognised that activities at the Facility may result in the release of fugitive odour emissions, which have the potential to diminish amenity in the local area.

Therefore, it is a requirement to control activities at the WTS in order to prevent or mitigate potential odours. The OMP provides a proactive approach to the effective management of odour during the WTS operations.

This OMP sets out the potential sources of odour at the WTS, the measures in place to control odour generation and monitor releases, and the management and monitoring actions that will be undertaken. The determination of receptor sensitivity and odour emission magnitude has been determined with reference to the Institute of Air Quality Management (IAQM) Guidance on the Assessment of Odour for Planning.

The OMP is a ‘live document’, in this respect the odour control measures, and management procedures contained within it will be updated on a periodic basis. This OMP will be kept in the Facility office and be available to all employees. The OMP will be implemented throughout the duration of the Facilities’ operation.

## Key Guidance

In developing the OMP, key guidance documents that have been consulted include:

Environment Agency (EA), H4 Odour Management Guidance; and

IAQM, Guidance on the Assessment of Odour Impacts for Planning.

# Baseline Environment

## Location

The Site is located off New Years Green Lane, UB9 6LX centred on National Grid Reference (NGR) TQ 07093 88015. The Site is located approximately 24.5km north-west of London, and 2km north of Ickenham which is part of the London Borough of Hillingdon.

The Site is bounded by industrial premises to the north, east and west of the site, with predominantly open agricultural land beyond. To the south of the site lies open / agricultural land, as illustrated in Figure A.

The Site is accessed via the New Years Green Lane which runs north of the Site. The closest residential receptor is north of New Years Green Lane situated approximately 30m north.

The Site’s location is illustrated on Drawing 001, and the EP Boundary and Site Layout are illustrated in Drawing 002. Local receptors within a 500m radius of the Site are shown on Drawing 003.

## Other Potential Sources of Odour

A review of other potential sources of odour in the Site locale has been undertaken through use of aerial imagery.

There is potential for agricultural odours to arise from farming activities on the surrounding fields. Local receptors would be familiar with associated odours and be desensitised to them. The odour character would be markedly different to that from the Facility (i.e. waste odours).

On this basis odour emissions from other premises have not been considered within this OMP.

Figure A: Site Setting

## Sensitive Receptors

Receptors in proximity to the Site with a sensitivity to odours have been identified and presented Table A. The sensitivity of receptors to odours has been determined with reference to the IAQM odour Guidance. The closest human receptors considered to be sensitive to odour are presented and illustrated in Figure B.

Table A:	Sensitive Human Receptors

| Receptor | Receptor Type | Odour Sensitivity | UK NGR (m) |  | Distance from Boundary |
| --- | --- | --- | --- | --- | --- |
|  |  |  | X | Y |  |
| R1 | Industrial | Low | 507065 | 188032 | <50m |
| R2 | Residential | High | 507107 | 188112 | 35m |
| R3 | Residential | High | 507119 | 188183 | 110m |
| R4 | Residential | High | 507215 | 188116 | 100m |
| R5 | Residential | High | 506973 | 188153 | 150m |
| R6 | Industrial | Low | 507301 | 188061 | 170m |
| R7 | Industrial | Low | 507420 | 187878 | 340m |
| R8 | Residential | High | 506794 | 187951 | 250m |
| R9 | Industrial | Low | 506904 | 187830 | 190m |

Figure B: Sensitive Receptors

## Meteorological Conditions

The most important climatic parameters governing the release and dispersal of fugitive odour emissions from the Facility are wind speed and direction:

Wind direction determines the broad direction of dispersal; and

Wind speed affects ground level concentrations by increasing the initial dilution of pollutants in the emission.

A wind rose for Heathrow meteorological station (5-year average 2018, 2019, 2020, 2022 & 2023), located approximately 11km south of the Site is presented in Figure C. The wind rose shows winds from the southwest are most frequent. Therefore, locations to the northeast of the Site are most likely to be impacted by potential odour emissions.

Figure C: Windrose for Heathrow Meteorological Station (5-year average)

# Operations at High View WTS

This section identifies the activities and potential odour sources at the Facility. The operational layout of the Facility is shown in Drawing 002.

## Site Operations

The Site operations would be limited to the receipt, storage and transfer of these waste types prior to bulk export to suitably permitted facilities for further recovery or disposal. Wastes are typically delivered to the Site in secure, steel sided lorries, as well as a range of other smaller vehicles.

The Facility will accept approximately 75,000 tpa of waste, comprising:

Dry mixed recyclables;

Clinical waste;

Food waste;

Green waste;

Metal waste;

End-of-life tyres;

Asbestos waste; and

General waste.

## Hours of Operation

The Facility will be operational between 06:00 to 18:00 Monday to Sunday (excluding bank holidays).

## Waste Acceptance

Further details of accepted waste and associated European Waste Catalogue (EWC) Codes are presented in full in Appendix A.

## Storage

A maximum of 1,500 tonnes of waste will be stored at the Site at any one time. All waste will within the fully enclosed WTS building be stored in designated concrete bays or containers which will benefit from impermeable surfacing and a sealed drainage system. Clinical waste types will be kept segregated from the other wastes streams at all times and stored within enclosed containers. A maximum of 50 tonnes of clinical waste will be stored on site at any one time.  Wastes stored outside of the WTS building will comprise of tyres, asbestos and metal waste only. All waste outside the building will be stored in enclosed skips to prevent the ingress of rainwater.

In consideration of the nature of the waste types to be received, it is expected that the volumes of most waste types received will be consistent throughout the year. Green waste is an exception, for which the volumes received would vary seasonally.

Waste received at the WTS would be stored for a maximum of 5 days  prior to export. Food waste will be stored for a maximum of 2 days. Green waste for a maximum of 4 days. Clinical waste for a maximum of 5 days.

The quantity of stored waste will be monitored against the allowed maximum capacities. This will be calculated by recording the volume of waste entering the site and the application of standard EA conversion factors as appropriate or via a weighbridge.

Good housekeeping measures (as outlined in Table C) would ensure that stockpiles are suitably managed to stay within the designated bays.

All waste storage containers and bays will be clearly labelled to ensure the segregation of waste.

## On-Site Transportation / Material Handling

There would be periodic vehicle movements along the hard-standing areas of the Site during operational hours. Vehicle movements would arise from waste import, handling, stockpiling and export operations. The vehicle movements at the Facility would be primarily as a result of Refuse Collection Vehicles (RCVs) and street cleaning caged vans / tippers importing or exporting waste, as well as mobile plant in operation for handling and stockpiling operations.

## Off-Site Transportation / Import and Export Operations

There would be periodic vehicle movements to/from the Facility on to the local road network during operational hours. Vehicles entering/leaving the Facility to/from the local road network would arise as a result of waste import/export operations. The vehicle entering/leaving the Site would typically comprise RCVs and street cleaning caged vans / tippers.

The areas of the Facility which would be accessed by the RCVs and street cleaning caged vans / tippers accessing the Site would be hard paved. RCVs and tippers transferring material to / from the Facility shall be covered (contained or sheeted) where possible.

# Potential odour Sources and Magnitude

In order to consider the likely significance of effects of potential odours arising from the Facility operations upon nearby sensitive receptors it important to consider the following:

Source: i.e. magnitude of release;

Pathway: i.e. the prevailing meteorological conditions and distance / direction of receptors in relation to the facility; and

Receptors: i.e. the sensitivity of receptors to potential odours.

Further details on the source, pathway and receptors (SPR) are outlined within the sections below.

## Source

The application of good working practices and process control is of fundamental importance in eliminating and minimising the quantities of odours formed at the Facility and their subsequent release to atmosphere. This section provides an inventory of all potential odour sources under the full range of normal operating conditions.

The overall aim in the operation of the Facility is to apply Best Available Techniques (BAT) at all stages of operations. For this reason, the Facility is operated and managed in accordance with the accepted hierarchy of preferred controls, that is:

- Prevent the formation or emission of odorous compounds in the first place;

- Where this is not practicable, minimise the release of odour;

- Abate excessive emissions; then

- Dilute any residual odour by effective dispersion in the atmosphere.

Potential odour sources associated with the operation of the Facility have been identified in consideration of the Facility operations. The key potential odour sources identified at the Facility are as follows:

General waste;

Clinical /offensive waste;

Food waste; and

Green waste.

The wastes with potential for odour will only be stored with the proposed building on the Site. Dry mixed recyclables are generally associated with a negligible odour potential, and as such have not been considered further within the OMP. Waste stored outside of the WTS building, which includes asbestos, tyres and metal only is considered to have a negligible odour potential and thus is not considered further as part of this OMP.

Further details on the specific odour sources identified are outlined in the sections below. An assessment of the odour risk from each waste type to be accepted at the Site is provided in Appendix E.

### Mixed Municipal Waste

Mixed municipal waste is typically associated with moderately or highly offensive odours. The odour potential is dependent on the quantity in storage, the state of material decomposition upon arrival at Site and the time stored prior to off-site export. Given that food waste is collected separately, the proportion of food waste co-mingled within the mixed municipal waste is likely to be low.

Mixed municipal waste would be deposited, stored and loaded for export within an enclosed building, thus providing a level of containment to odours. Further, as detailed in Section 5.0, the waste will be retained for a limited time and the WTS building shall utilise odour masking sprays.

As such, the associated odour potential of mixed municipal waste at the Facility is considered ‘small’.

### Clinical Waste

Clinical waste is typically associated with moderately or highly offensive odours. The odour potential and character is dependent on the type of waste present and the state of material decomposition upon arrival.

Clinical waste will be kept segregated from other waste streams at all times and stored within an enclosed containers within the enclosed WTS building, thus providing an additional level of containment to odours (further to that of the WTS building). Clinical waste will be stored for a maximum of five days before being moved on.

As such, the odour potential of clinical waste at the Facility is considered ‘small’.

### Food Waste

Food waste, which has the potential to be moderately or highly offensive with a likely odour character of a rotting/putrid nature (i.e. rotten eggs, dairy (rancid), dead animal (rotten)) and animal origin based (i.e. fishy, poultry). The odour potential is dependent on the state of material decomposition upon arrival as well as the quantity in storage and the time stored prior to off-site export.

Food waste would be deposited and stored within the enclosed WTS building, thus providing a level of containment to odours. The food waste would then be moved off-site normally within 48 hours to minimise odour. Given the above, the associated odour potential of food waste at the Facility is considered ‘small’.

### Green Waste

Green waste is typically associated with slightly offensive odours with a typical ‘grassy’ or ‘musty’ character. The odour potential is dependent on the quantity in storage, the state of material decomposition upon arrival at Site and the time stored prior to off-site export.

Green waste would be deposited and stored within a dedicated bay and would typically be removed from Site within 4 days. The bay will be fully cleared before refilling. Furthermore, in consideration of the similarity of green waste odours to those currently present within the site setting (agricultural), the sensitivity of nearby residential receptors to green-waste type odours is likely to be low.

As such, the associated odour potential of green waste at the Facility is considered ‘small’.

### Overall Source Odour Potential

In consideration of the key odour sources identified, the anticipated waste throughputs, retention times and the level of containment provided, the source odour potential of the operations is considered ‘small’.

## Pathway

The pathway by which odours may impact upon receptor locations is a result of atmospheric dispersion. In general, high wind speeds lead to emitted odour being rapidly dispersed and diluted due to turbulence, and conversely low wind speeds inhibit the dilution of odours.

Prevailing wind directions are considered in assessing the likelihood and management of emission risks. In consideration of the local meteorological conditions, prevailing wind in the Site locale are anticipated to be from the southwest. Consequently, the potential impact of emissions is likely to be greater to the northeast of the Site.

Wind speed analysis for Heathrow meteorological station show that low wind speeds (i.e. <5m/s) have been observed for approximately 67% of the hours between 2018 and 2023 (excluding 2021).

The effectiveness of dispersion/dilution of odours from the sources identified at the Site is considered to be low, in line with open processes with low-level releases.

There is one residential receptor within a distance of 100m of the Site boundary, the majority of high sensitivity receptors (see Table A) are considered remote to the potential odour sources identified. A summary of the pathway effectiveness for each receptor is presented in Table B.

Table B:	Pathway Effectiveness Summary

| Receptor ID | Receptor Type | Distance from Site (approx. m) | Wind Direction linking Source to Receptora | Frequency of Winds from Source to Receptorb |  | Pathway Effectiveness |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Low Windsc | Notes |  |
| R1 | Industrial | 15m | E – NE | 2.7% | Infrequent | Mod. Effective |
| R2 | Residential | 35m | S-SSE | 8.8% | Mod. Frequent | Highly Effective |
| R3 | Residential | 110m | S-SSE | 6.9% | Mod. Frequent | Mod. Effective |
| R4 | Residential | 100m | NE | 3.8% | Infrequent | Mod. Effective |
| R5 | Residential | 150m | SE | 2.6% | Infrequent | Ineffective |
| R6 | Industrial | 170m | WSW | 5.4% | Mod. Frequent | Mod. Effective |
| R7 | Industrial | 340m | NW | 3.4% | Infrequent | Ineffective |
| R8 | Residential | 250m | ENE | 2.7% | Infrequent | Ineffective |
| R9 | Industrial | 190m | NE | 3.8% | Infrequent | Ineffective |
| Table Notes - Wind direction blowing from (i.e. the inverse of the direction from the WTS). - Determined in reference to meteorological data from the Heathrow Airport recording station, as presented in Section 2.4). - Defined as wind speeds below 5m/s. |  |  |  |  |  |  |

## Receptors

In reference to the IAQM Odour Guidance, residential properties are of a high sensitivity, commercial properties are of medium sensitivity and industrial uses of a low sensitivity to odours. Sensitive receptors considered are presented in Table A, whilst their locations are illustrated in Figure B.

## Significance of Effects

The Source-Pathway-Receptor (S-P-R) conceptual model outlined within the IAQM Odour Guidance has been used to assess the likely odour effect as a result of potential odours arising from operations from the Facility at the nearest sensitive receptors.

A summary of the SPR and associated significance of effects on the identified sensitive receptors is presented in Table C.

Table C:	SPR Summary

| Receptor ID | Receptor Type | Odour Sensitivity | Source Odour Potential | Pathway Effectiveness | Odour Exposure | Likely Odour Effect |
| --- | --- | --- | --- | --- | --- | --- |
| R1 | Industrial | Low | Small | Mod. Effective | Negligible | Negligible |
| R2 | Residential | High | Small | Highly Effective | Low Risk | Slight Adverse |
| R3 | Residential | High | Small | Mod. Effective | Negligible | Negligible |
| R4 | Residential | High | Small | Mod. Effective | Negligible | Negligible |
| R5 | Residential | High | Small | Ineffective | Negligible | Negligible |
| R6 | Industrial | Low | Small | Mod. Effective | Negligible | Negligible |
| R7 | Industrial | Low | Small | Ineffective | Negligible | Negligible |
| R8 | Residential | High | Small | Ineffective | Negligible | Negligible |
| R9 | Industrial | Low | Small | Ineffective | Negligible | Negligible |

As shown in Table C, the likely odour effect at receptor R2 is ‘slight adverse’, and a ’negligible’ odour effect is predicted at all other sensitive receptors. In reference to the IAQM Odour Guidance, a ‘slight’ or ‘negligible’ odour effect corresponds to a ‘not significant’ effect.

Notwithstanding this, a range of odour control measures are proposed to minimise potential odours from the WTS operations, as presented in Section 5.0 below.

# Control Measures and Process Monitoring

The odour control measures employed at the Facility are detailed in Table D.

Table D:	Appropriate Techniques / BAT

| Potentially Odorous Process / Material | Control measures (Appropriate Measure / BAT) | Monitoring frequency | Monitoring procedure and optimum process parameters | Trigger level |
| --- | --- | --- | --- | --- |
| Waste Reception | Waste rejection procedure | Constant, ongoing throughout shift. | All waste received at the Site is monitored at entry (report to a weighbridge) to ensure compliance with the permitted waste types for the facility. Any abnormal loads are rejected. Highly odorous loads identified are directed to the front of the queue for tipping (to reduce residence time outside of the main building). | Abnormal or highly odorous loads identified. |
|  | Containment of food waste |  | All vehicles delivering or collecting food waste shall be sheeted or have sealable containers, which will not be opened until inside. | Visual Inspection |
|  | Use of the vehicular access doors |  | Fast-acting roller shutter doors will be installed on the WTS building and kept closed when not in use. | Vehicular access door open, but not in use. |
| Storage of Waste | Containment of odour sources - general | Constant, ongoing throughout shift. A daily record of tonnage in and out of the entire site is maintained. Inspected weekly to ensure there is no loss of containment. | Waste storage and handling operations are undertaken within the main WTS building. This building is enclosed, thereby reducing fugitive odour emissions. All waste storage containers and bays within the WTS building will be clearly labelled to ensure the segregation of waste. A maximum of 1,500 tonnes of waste will be stored on site at any one time | N/A |
|  |  |  |  | Volume of waste stored constantly monitored |
|  | Containment of odour sources – food waste |  | Food waste will be stored in enclosed containers within dedicated bays. | N/A |
|  | Containment of odour sources –clinical waste |  | Clinical waste will be stored in enclosed containers within dedicated bays. No more than 50 tonnes of clinical waste (primarily EWC 18 01 04) will be stored on site before being removed for recovery. Sharps (primarily EWC 18 01 01) will be stored within specified sharps containers. All other wastes will be bulked up and moved offsite on a regular basis, following the First In, First Out (FIFO) principles. | Non-compliance |
|  | Containment of odour sources – green waste |  | Up to 100 tonnes of green waste will be stored within the building at any one time & will be managed as per FIFO principles. | N/A |
|  | Use of the vehicular access doors |  | Fast-acting roller shutter doors will be installed on the WTS building and kept closed when not in use, where possible. | Vehicular access door open, but not in use. |
|  | Minimising of retention time |  | The length of time waste is stored at the site is minimised – waste will be on site for a maximum 5 days. Food waste will typically be removed within 2 days Green waste will typically be removed within 4 days | N/A |
|  | Good housekeeping | Daily Weekly | Storage areas will benefit from daily cleaning using brooms. Storage areas will benefit from weekly cleaning using hoses. | N/A |
|  | Masking Odours | N/A | Odour masking sprays will be fitted within the WTS building. | N/A |
|  | Backup Machines | Daily | In case of machine breakdown alternative machines can quickly be supplied as WLC own a large fleet. Therefore, no onsite delays to waste handling are anticipated. | Machine breakdown |
| Spillage | Good housekeeping | Constant, ongoing throughout shift. | Any spillages will be cleared and logged in the site diary immediately. | Spillage |
| Litter across Site | Litter picking / good housekeeping. | Constant, ongoing throughout shift. | Periodic litter picking is undertaken around the Site as required to keep litter levels low. Good housekeeping practises are encouraged. | Excessive levels of litter around the Site. |

# Monitoring and Maintenance

Monitoring of process controls, odour containment, odorous releases, and dispersion pathways are as described in the sections below.

## Monitoring of Ambient Odours

Monitoring ambient odour provides a broad indication of the effectiveness of the odour management as a whole, i.e. odour minimisation, containment, treatment and dispersion. This is a reactive process and should be considered as a final indicator of odour control effectiveness.

The assessment is “sensory” in that the human noise is used as the detector – a sound approach considering that no analytical instrument can give a unified measure of a complex mixture of compounds in the same way that a human experiences odour.

Sniff-testing is employed for the following reasons:

as part of a weekly survey at the Site boundary during normal operations, to confirm the effective performance of odour control measures in place;

at the Site boundary during periods of adverse meteorological conditions, breakdowns or during other abnormal events to evaluate the effectiveness of the control measures in place and the likelihood that odour complaints will be received; and

in the event that complaints are received, at the locations of sensitive receptors as part of the complaint investigation procedure.

‘Sniff-tests’ will follow the procedure detailed within Appendix D as set out within the H4 Odour Guidance. The locations of odour monitoring points outside of the Site boundary are shown on the plan as presented in Appendix F.  Monitoring points within the Site boundary are shown on Drawing 002. It is noted that the odour monitoring locations are the same as those proposed for visual dust monitoring on the Site.

## Control Measures during Routine Maintenance

The facility will have no need for planned maintenance shutdown; typically, individual pieces of equipment will be able to be isolated from the process to allow for service / maintenance.

During necessary maintenance works (e.g. replacing or repairing a roller door), there is the potential for reduced containment of odours.

## Monitoring Meteorological Conditions

If odour complaints are received the Site Manager or other designated responsible person will record daily weather conditions in the Site Diary from online data sources (i.e. local forecast), including wind direction, wind speeds and ambient temperatures.

The recording of meteorological data can also be an effective management tool when used for the following:

during routine operations, to plan where boundary monitoring should be focussed to assess odour impacts;

at the time of abnormal events (i.e. breakdown) to predict where odour impacts could potentially occur; and

in the investigation of odour complaints or to verify community observations.

## Recording of Results and Reporting

All waste received at the Site is monitored at entry (report to a weighbridge) to ensure compliance with the permitted waste types for the facility and to identify any particularly odorous loads.

Daily records are maintained and include the following details (where applicable):

results of inspections and any olfactory monitoring carried out by site personnel;

operational problems including date, time, duration and cause of problem;

complaints received including address (if available); and

details of corrective actions taken and any subsequent changes to operational procedures.

The weekly sniff-tests undertaken will be made on the Odour Assessment Form presented in Appendix B which will be filed and kept on site for inspection by the EA as and when required.

In the event that odour is detected at the Site boundary, this will be noted in the Site diary and the Site Manager (or any appointed representative) will be informed to allow for appropriate steps to be taken to mitigate the odour.

## Notifying the EA

In the event that an accident or incident occurs, the Operator will notify the EA as soon as practicably possible using the emergency 24hr phone line (0800 807060). The Site Manager for the facility will also notify the Regulatory Officer should any complaints be received directly to the Site and advise what remedial measures have been undertaken. Copies of any complaints will be made available for EA to review

# Contingencies

In accordance with EA’s Guidance on OMPs, contingency plans have been defined to react to situations where monitoring indicates that a potential odour source is not completely under control, or that adverse impact has occurred.

This includes accidents (or incidents) which would result in the loss of control of odorous substances and have the potential to cause an unacceptable short-term impact on the local community but are not considered an emergency situation.

## Foreseeable Events

Table E below outlines some of the foreseeable ‘abnormal events’ which might occur at the Site and the associated contingencies and recovery steps to address these events.

Table E:	Foreseeable Events

| Abnormal event | Recovery steps |
| --- | --- |
| Power Failure | In the event of power failure, operations would be suspended, and all external doors would be closed manually. |
|  | During a power failure, further waste deliveries would not be received at the Site. Where possible waste deliveries would be diverted to other sites. |
| Compromised odour containment | If doors are stuck open or building fabric is damaged, then the following contingency measures will be implemented: Arrangements made to temporarily re-establish containment (i.e. patching), prior to arrangement of long-term repairs; and Minimise the presence of odorous materials e.g. transferring existing material off-site as soon as practicable. Odour surveys would be undertaken more regularly until an effective fix is implemented |
| Damage to Vehicular access door | Engage with contractors to undertake remedial actions as soon as possible. |
| Over capacity | Each day a review will be carried out of the stock in comparison to expected incoming material and material removal. In the event that the material storage areas are not considered to have sufficient capacity, the Site Manager will consider the option for diverting incoming material to other waste management facilities to prevent build-up of material beyond capacity. |
| Flood | In the event of flooding municipal and food waste will be rapidly removed from the Facility, where possible. Widespread flooding of the Facility may also prevent the operation of key electrical equipment and vehicular access. Under such extreme conditions no further operations would be undertaken (i.e. opening of doors) and no further waste would be received. Widespread flooding may prevent access to Facility. In such a situation could not be received or exported from the Facility. |
| Staff Shortage | If long-term staff shortage (or a prolonged and widespread period of staff absence) occurs the operator would cease receiving deliveries of waste and suspend Facility operations. |
| Abnormal Meteorological Conditions | Extreme cold / snowfall: Employ snow clearing equipment. Divert incoming waste streams where feasible High temperatures and stable conditions (promoting generation of odour) Review of requirements for activities that involve opening of the waste reception building doors and reduce frequency and duration of door opening if practicable. |

In the event of Site emergency, an assessment will be made to determine whether the incident has the potential for off-Site environmental impacts and the Operations Director or Operations Manager will be notified in line with the categorisation criteria without delay and the Site’s Incident Response Plan will be followed.

## Detection of Odour in Response to Complaints

The olfactory survey methodology, as detailed in Appendix D, will be followed and the likely source(s) of the detected odour identified by determining the sources of greatest odour intensity, contingency actions will be implemented as identified above.

The first assessment of an odour at the Site boundary will be whether the odour has or is likely to leave Site, if it has not and is not likely to leave Site the problem that caused the odour shall be remedied to prevent continuation of odour. All information regarding action taken will be recorded on the external Odour Assessment Form (Appendix B).

If an odour at a level which is likely to cause pollution (i.e. high intensity and/or offensiveness) is likely to leave the Site boundary or has already left the Site boundary, the Site Manager (or representative) will be notified immediately.

The olfactory survey will be repeated on consecutive days after initiation of corrective actions, until odour has reduced to an acceptable level.

## Out of Hours Contact Details

An Emergency Duty Standby Number will be made available which will always be answered in the event of an emergency.

## Receipt of an Odour Complaint

### Complaint Logging

A phone number for members of the public to contact WLC with any complaints will be visible on the Site board at the entrance. Following the receipt of a complaint WLC will endeavour to contact the complainant to provide feedback on actions taken to both assess the event and convey any remedial actions.

All complaints will be recorded on an Odour Complaint Form such as that presented in Appendix C. Information that will be recorded will include the following:

Date and time at which the odour complaint was received and detected;

Location / address of complainant (where provided); and

A description of the odour observed by the complainant (where provided).

Following an odour complaint, a trained member of staff will undertake a sniff test, recording the results on an Odour Monitoring Form such as that presented in Appendix B. Where possible the sniff test will be undertaken by a member of staff that does not routinely work within the WTS building (and will not therefore be accustomed to the characteristic malodours that might arise from the Site). If an odour which can be attributed to the Site operations is encountered during the sniff test, the source of the detected odours will be investigated by the Site Management Team and the outcome recorded.

Investigations will include the likely source and cause of the odour and a review of the meteorological data. Suitable remedial action will be investigated, where required. The complainant will be informed of any action taken and all actions will be recorded.

Should no odours which can be attributed to the Site operations be observed:

A record of the sniff test will be made;

The meteorological conditions during the test will be checked; and

Suitable feedback would be provided to the complainant.

### Complaint Investigation

The following actions will be taken upon receipt of an odour complaint:

- The Site Manager will be informed of the odour complaint as soon as possible, including the location, time and date (if reported) of the complaint being lodged; and

The Site Manager (or an appointed representative) will undertake the following assessment process:

Review of the operations and control systems at the Site prior to and at the time of the complaint to:

Determine if material was being received at the Site at the time of the complaint;

Determine if highly odorous material was being received, stored or removed at the time of the complaint;

Determine if any abnormal operating conditions were occurring;

Determine if any accidents or incidents requiring contingency actions were being undertaken; and

Determine if any emergency situations existed at that time.

Review of the meteorological conditions (wind speed and direction) prior to and at the time of the complaint – to establish whether a pathway can be established between the Site and the complainant; and / or

Review the previous history of complaints at the location identified.

The Site Manager (or appointed representative) will visit the complaint location as soon as practicable in order to subjectively determine odour presence / absence and. If presence of an odour which could be attributed to the Site operations is detectable, the odour characteristics and intensity would be determined in accordance with the procedure detailed in Appendix D, and a complaint form (such as the one presented in Appendix C) would be filled-out.

The EA will be informed in line with Permit requirements.

# Document Updates and Reviews

## Responsible Staff

The Site has a formally documented management structure. It is the responsibility of every manager/supervisor, with the support of the environmental professionals, to identify environmental risks that are relevant to the Site and determine if a particular activity or service is environmentally significant.

Once identified, it is the responsibility of the Site Manager to highlight the significant aspects to all relevant employees and contractors. The Site Manager is also responsible for monitoring and managing all activities under their control to improve environmental performance.

Work instructions, job descriptions and procedures exist for critical areas of the Site activities have been issued to or made available to personnel responsible for undertaking these tasks.

## General Procedures for Training and Competency of Staff

Staff are familiarised with the content of the OMP as part of the induction process, or as a specific training exercise. Refresher training is provided as required, whenever the OMP is updated, or following an incident (whichever is sooner). The OMP is made available to all site staff as a reference document, either electronically or via a hard copy retained within the site office.

## Odour Management Plan Review

This OMP is a controlled document, and forms part of the Sit’s Environmental Management System.

In line with the recommendations of the EA’s H4 Odour Management guidance, the OMP will be reviewed on an annual basis, as a minimum.

However, the OMP is intended to be a live document which serves as a reference during daily operations, and as such would be updated on a more frequent basis should the following occur:

Significant changes are made to the plant or operational practices;

There is a change to the management structure, designation of responsibility or training provision;

the EA requests that the OMP is updated in their role as regulator; or

Complaints are received, which on subsequent investigation result in the identification of further control measures or remedial action, in addition to those set out within this OMP.

- European Waste Catalogue (EWC) Codes

Table A-1 Proposed Non-Hazardous Waste Types to be Accepted at the Site

| EWC Code | Description |
| --- | --- |
| 01 | WASTES RESULTING FROM EXPLORATION, MINING, QUARRYING, AND PHYSICAL AND CHEMICAL TREATMENT OF MINERALS |
| 01 01 | Wastes from mineral excavation |
| 01 01 01 | Wastes from mineral metalliferous excavation |
| 01 01 02 | Wastes from mineral non-metalliferous excavation |
| 01 03 | Wastes from physical and chemical processing of metalliferous minerals |
| 01 03 06 | Tailings other than those mentioned in 01 03 04 and 01 03 05 |
| 01 03 09 | Red mud from alumina production other than the wastes mentioned in 01 03 07 |
| 01 04 | Wastes from physical and chemical processing of non-metalliferous minerals |
| 01 04 08 | Waste gravel and crushed rocks other than those mentioned in 01 04 07 |
| 01 04 09 | Waste sand and clays |
| 01 04 11 | Wastes from potash and rock salt processing other than those mentioned in 01 04 07 |
| 01 04 12 | Tailings and other wastes from washing and cleaning of minerals other than those mentioned in 01 04 07 and 01 04 11 |
| 01 04 13 | Wastes from stone cutting and sawing other than those mentioned in 01 04 07 |
| 02 | WASTES FROM AGRICULTURE HORTICULTURE AQUACULTURE FORESTRY HUNTING AND FISHING FOOD PREPARATION AND PROCESSING |
| 02 01 | Wastes from agriculture, horticulture, forestry, hunting and fishing |
| 02 01 03 | Plant-tissue waste |
| 02 01 04 | Waste plastics (except packaging) |
| 02 01 07 | Wastes from forestry |
| 02 01 10 | Waste metal |
| 02 02 | Wastes from the preparation and processing of meat, fish and other foods of animal origin |
| 02 02 03 | Materials unsuitable for consumption or processing |
| 02 03 | Wastes from fruit, vegetables, cereals, edible oils, cocoa, coffee, tea and tobacco preparation and processing; conserve production; yeast and yeast extract production, molasses preparation and fermentation |
| 02 03 04 | Materials unsuitable for consumption or processing |
| 02 04 | Wastes from sugar processing |
| 02 04 01 | Soil from cleaning and washing beet |
| 02 04 02 | Off-specification calcium carbonate |
| 02 05 | Wastes from the dairy products industry |
| 02 05 01 | Materials unsuitable for consumption or processing |
| 02 06 | Wastes from the baking and confectionery industry |
| 02 06 01 | Materials unsuitable for consumption or processing |
| 02 06 02 | Wastes from preserving agents |
| 02 07 | Wastes from the production of alcoholic and non-alcoholic beverages (except coffee, tea, and cocoa) |
| 02 07 01 | Wastes from washing, cleaning and mechanical reduction of raw materials |
| 02 07 02 | Wastes from spirits distillation |
| 02 07 04 | Materials unsuitable for consumption or processing |
| 03 | WASTES FROM WOOD PROCESSING, AND THE PRODUCTION OF PANELS AND FURNITURE, PULP, PAPER AND CARDBOARD |
| 03 01 | Wastes from wood processing and the production of panels and furniture |
| 03 01 01 | Waste bark and cork |
| 03 01 05 | Sawdust, shavings, cuttings, wood, particle board and veneer other than those mentioned in 03 01 04 |
| 03 03 | Wastes from pulp, paper and cardboard production and processing |
| 03 03 01 | Waste bark and wood |
| 03 03 07 | Mechanically separated rejects from pulping of waste paper and cardboard |
| 03 03 08 | Wastes from sorting of paper and cardboard destined for recycling |
| 03 03 10 | Fibre rejects, fibre-, filler- and coating-sludges from mechanical separation |
| 04 | WASTES FROM THE LEATHER, FUR AND TEXTILE INDUSTRIES |
| 04 01 | Wastes from the leather and fur industry |
| 04 01 08 | Waste tanned leather (blue sheetings, shavings, cuttings, buffing dust) containing chromium |
| 04 01 09 | Wastes from dressing and finishing |
| 04 02 | Wastes from the textile industry |
| 04 02 21 | Wastes from unprocessed textile fibres |
| 04 02 22 | Wastes from processed textile fibres |
| 06 | WASTES FROM INORGANIC CHEMICAL PROCESSES |
| 06 09 | Wastes from the MSFU of phosphorous chemicals and phosphorous chemical processes |
| 06 09 02 | Phosphorous slag |
| 06 09 04 | Calcium-based reaction wastes other than those mentioned in 06 09 03 |
| 06 11 | Wastes from the manufacture of inorganic pigments and opacifiers |
| 06 11 01 | Calcium-based reaction wastes from titanium dioxide production |
| 07 | WASTES FROM ORGANIC CHEMICAL PROCESSES |
| 07 02 | Wastes from the MFSU of plastics, synthetic rubber and man-made fibres |
| 07 02 13 | Waste plastic |
| 09 | WASTES FROM THE PHOTOGRAPHIC INDUSTRY |
| 09 01 | Wastes from the photographic industry |
| 09 01 07 | Photographic film and paper containing silver or silver compounds |
| 09 01 08 | Photographic film and paper free of silver of silver compounds |
| 09 01 10 | Single-use cameras without batteries |
| 09 01 12 | Single-use cameras containing batteries other than those mentioned in 09 01 11 |
| 10 | WASTES FROM THERMAL PROCESSES |
| 10 01 | Wastes from power stations and other combustion plants (except 19) |
| 10 01 01 | Bottom ash, slag and boiler dust (excluding boiler dust mentioned in 10 01 04) |
| 10 01 05 | Calcium-based reaction wastes from flue-gas desulpherisation in solid form |
| 10 01 07 | Calcium-based reaction wastes from flue-gas desulpherisation in sludge form |
| 10 01 15 | Bottom ash, slag and boiler dust from co-incineration other than those mentioned in 10 01 14 |
| 10 01 19 | Wastes from gas cleaning other than those mentioned in 10 01 05, 10 01 07 and 10 01 18 |
| 10 01 24 | Sands from fluidised beds |
| 10 02 | Wastes from the iron and steel industry |
| 10 02 01 | Wastes from the processing of slag |
| 10 02 02 | Unprocessed slag |
| 10 02 08 | Solid wastes from gas treatment other than those mentioned in 10 02 07 |
| 10 02 10 | Mill scales |
| 10 02 14 | Filter cakes from gas treatment other than those mentioned in 10 02 13 |
| 10 02 15 | Other filter cakes |
| 10 03 | Wastes from aluminium thermal metallurgy |
| 10 03 02 | Anode scraps |
| 10 03 05 | Waste alumina |
| 10 03 16 | Skimmings other than those mentioned in 10 03 15 |
| 10 03 18 | Carbon-containing wastes from anode manufacture other than those mentioned in 10 03 17 |
| 10 03 24 | Solid wastes from gas treatment other than those mentioned in 10 03 23 |
| 10 03 26 | Filter cakes from gas treatment other than those mentioned in 10 03 25 |
| 10 03 28 | Wastes from cooling-water treatment other than those mentioned in 10 03 27 |
| 10 03 30 | Wastes from treatment of salt slags and black drosses other than those mentioned in 10 03 29 |
| 10 04 | Wastes from lead thermal metallurgy |
| 10 04 10 | Wastes from cooling-water treatment other than those mentioned in 10 04 09 |
| 10 05 | Wastes from zinc thermal metallurgy |
| 10 05 01 | Slags from primary and secondary production |
| 10 05 09 | Wastes from cooling-water treatment other than those mentioned in 10 05 08 |
| 10 05 11 | Dross and skimmings other than those mentioned in 10 05 10 |
| 10 06 | Wastes from copper thermal metallurgy |
| 10 06 01 | Slags from primary and secondary production |
| 10 06 02 | Dross and skimmings from primary and secondary production |
| 10 06 10 | Wastes from cooling-water treatment other than those mentioned in 10 05 10 |
| 10 07 | Wastes from silver, gold and platinum thermal metallurgy |
| 10 07 01 | Slags from primary and secondary production |
| 10 07 02 | Dross and skimmings from primary and secondary production |
| 10 07 03 | Solid wastes from gas treatment |
| 10 07 05 | Filter cakes from gas treatment |
| 10 07 08 | Wastes from cooling-water treatment other than those mentioned in 10 07 07 |
| 10 08 | Wastes from other non-ferrous thermal metallurgy |
| 10 08 09 | Other slags |
| 10 08 11 | Dross and skimmings other than those mentioned in 10 08 10 |
| 10 08 13 | Carbon-containing wastes from anode manufacture other than those mentioned in 10 08 12 |
| 10 08 14 | Anode scrap |
| 10 08 18 | Filter cakes from flue-gas treatment other than those mentioned in 10 08 17 |
| 10 08 20 | Wastes from cooling-water treatment other than those mentioned in 10 08 19 |
| 10 09 | Wastes from casting of ferrous pieces |
| 10 09 03 | Furnace slag |
| 10 09 06 | Casting cores and moulds which have not undergone pouring other than those mentioned in 10 09 05 |
| 10 09 08 | Casting cores and moulds which have undergone pouring other than those mentioned in 10 09 07 |
| 10 09 14 | Waste binders other than those mentioned in 10 09 13 |
| 10 09 16 | Waste crack-indicating agent other than those mentioned in 10 09 15 |
| 10 10 | Wastes from casting of non-ferrous pieces |
| 10 10 03 | Furnace slag |
| 10 10 06 | Casting cores and moulds which have not undergone pouring, other than those mentioned in 10 10 05 |
| 10 10 08 | Casting cores and moulds which have undergone pouring, other than those mentioned in 10 10 07 |
| 10 10 14 | Waste binders other than those mentioned in 10 10 13 |
| 10 10 16 | Waste crack-indicating agent other than those mentioned in 10 10 15 |
| 10 11 | Wastes from manufacture of glass and glass products |
| 10 11 03 | Waste glass-based fibrous materials |
| 10 11 10 | Waste preparation mixture before thermal processing, other than those mentioned in 10 11 09 |
| 10 11 12 | Waste glass other than those mentioned in 10 11 11 |
| 10 11 16 | Solid wastes from flue-gas treatment other than those mentioned in 10 11 15 |
| 10 11 18 | Filter cakes from flue-gas treatment other than those mentioned in 10 11 17 |
| 10 12 | Wastes from manufacture of ceramic goods, bricks, tiles and construction products |
| 10 12 01 | Waste preparation mixture before thermal processing |
| 10 12 05 | Filter cakes from gas treatment |
| 10 12 06 | Discarded moulds |
| 10 12 08 | Waste ceramics, bricks, tiles and construction products (after thermal processing) |
| 10 12 10 | Solid wastes from gas treatment other than those mentioned in 10 12 09 |
| 10 12 12 | Wastes from glazing other than those mentioned in 10 12 11 |
| 10 13 | Wastes from the manufacture of cement, lime, and plaster and articles and products made from them |
| 10 13 01 | Waste preparation mixture before thermal processing |
| 10 13 04 | Wastes from calcination and hydration of lime |
| 10 13 07 | Filter cakes from gas treatment |
| 10 13 10 | Wastes from asbestos-cement manufacture other than those mentioned in 10 13 09 |
| 10 13 11 | Wastes from cement-based composite materials other than those mentioned in 10 13 09 |
| 10 13 13 | Solid wastes from gas treatment other than those mentioned in 10 13 12 |
| 10 13 14 | Waste concrete |
| 11 | WASTES FROM CHEMICAL SURFACE TREATMENT AND COATING OF METALS AND OTHER MATERIALS; NON-FERROUS HYDRO METALLURGY |
| 11 01 | Wastes from chemical surface treatment and coating of metals and other materials (for example galvanic processes, zinc coating processes, pickling processes, etching, phosphating, alkaline degreasing, anodising) |
| 11 01 10 | Filter cakes other than those mentioned in 11 01 09 |
| 11 01 14 | Degreasing wastes other than those mentioned in 11 01 13 |
| 11 02 | Wastes from non-ferrous hydrometallurgical processes |
| 11 02 03 | Wastes from the production of anodes for aqueous electrolytical processes |
| 11 02 06 | Wastes from copper hydrometallurgical processes other than those mentioned in 11 02 05 |
| 11 05 | Wastes from hot galvanising processes |
| 11 05 01 | Hard zinc |
| 11 05 02 | Zinc ash |
| 12 | WASTES FROM SHAPING AND PHYSICAL AND MECHANICAL SURFACE TREATMENT OF METALS AND PLASTICS |
| 12 01 | Wastes from shaping and physical and mechanical surface treatment of metals and plastics |
| 12 01 01 | Ferrous metal filings and turnings |
| 12 01 03 | Non-ferrous metal filings and turnings |
| 12 01 05 | Plastic shavings and turnings |
| 12 01 13 | Welding wastes |
| 12 01 17 | Waste blasting material other than those mentioned in 12 01 16 |
| 12 01 21 | Spent grinding bodies and grinding materials other than those mentioned in 12 01 20 |
| 15 | WASTE PACKAGING: ABSORBENTS, WIPING CLOTHS, FILTER MATERIALS AND PROTECTIVE CLOTHING NOT OTHERWISE SPECIFIED |
| 15 01 | Packaging (including separately collected municipal packaging waste) |
| 15 01 01 | Paper and cardboard packaging |
| 15 01 02 | Plastic packaging |
| 15 01 03 | Wooden packaging |
| 15 01 04 | Metallic packaging |
| 15 01 05 | Composite packaging |
| 15 01 06 | Mixed packaging |
| 15 01 07 | Glass packaging |
| 15 01 09 | Textile packaging |
| 15 02 | Absorbents, filter materials, wiping cloths and protective clothing |
| 15 02 03 | Absorbents, filter materials, wiping cloths, and protective clothing other than those mentioned in 15 02 02 |
| 16 | WASTES NOT OTHERWISE SPECIFIED IN THE LIST |
| 16 01 | End-of-life vehicles from different means of transport (including off-road machinery) and wastes from dismantling of end-of-life vehicles and vehicle maintenance (except 13, 14, 16 06 and 16 08) |
| 16 01 03 | End-of-life tyres |
| 16 02 | Wastes from electrical and electronic equipment |
| 16 02 14 | Discarded equipment other than those mentioned in 16 02 09 to 16 02 13 |
| 16 02 16 | Components removed from discarded equipment other than those mentioned in 16 02 15 |
| 16 03 | Off-specification batches an unused products |
| 16 03 04 | Inorganic wastes other than those mentioned in 16 03 03 |
| 16 03 06 | Organic wastes other than those mentioned in 16 03 05 |
| 16 06 | Batteries and accumulators |
| 16 06 04 | Alkaline batteries (except 16 06 03) |
| 16 06 05 | Other batteries and accumulators |
| 16 11 | Waste linings and refractories |
| 16 11 02 | Carbon-based linings and refractories from metallurgical processes other than those mentioned in 16 011 01 |
| 16 11 04 | Other linings and refractories from metallurgical processes other than those mentioned in 16 11 03 |
| 16 11 06 | Linings and refractories from non-metallurgical processes other than those mentioned in 16 11 05 |
| 17 | CONSTRUCTION AND DEMOLITION WASTES (INCLUDING EXCAVATED SOIL FROM CONTAMINATED SITES) |
| 17 01 | Concrete, bricks, tiles and ceramics |
| 17 01 01 | Concrete |
| 17 01 02 | Bricks |
| 17 01 03 | Tiles and ceramics |
| 17 01 07 | Mixtures of concrete, bricks, tiles and ceramics other than those mentioned in 17 01 06 |
| 17 02 | Wood, glass and plastic |
| 17 02 01 | Wood |
| 17 02 02 | Glass |
| 17 02 03 | Plastic |
| 17 03 | Bituminous mixtures, coal tar and tarred products |
| 17 03 02 | Bituminous mixtures other than those mentioned in 17 03 01 |
| 17 04 | Metals (including their alloys) |
| 17 04 01 | Copper, bronze, brass |
| 17 04 02 | Aluminium |
| 17 04 03 | Lead |
| 17 04 04 | Zinc |
| 17 04 05 | Iron and steel |
| 17 04 06 | Tin |
| 17 04 07 | Mixed metals |
| 17 04 11 | Cables other than those mentioned in 17 04 10 |
| 17 05 | Soil (including excavated soil from contaminated sites) stones and dredging spoil |
| 17 05 04 | Soil and stones other than those mentioned in 17 05 03 |
| 17 05 08 | Track ballast other than those mentioned in 17 05 07 |
| 17 06 | Insulation materials and asbestos-containing construction materials |
| 17 06 04 | Insulation materials other than those mentioned in 17 06 01 and 17 06 03 |
| 17 08 | Gypsum-based construction material |
| 17 08 02 | Gypsum-based construction materials other than those mentioned in 17 08 01 |
| 17 09 | Other construction and demolition wastes |
| 17 09 04 | Mixed construction and demolition wastes other than those mentioned in 17 09 01, 17 09 02, and 17 09 03 |
| 19 | WASTES FROM WASTE MANAGEMENT FACILITIES, OFF-SITE WASTE WATER TREATMENT PLANTS AND PREPARATION OF WATER INTENDED FOR HUMAN CONSUMPTION/INDUSTRIAL USE |
| 19 01 | Wastes from incineration or pyrolysis of waste |
| 19 01 02 | Ferrous materials removed from bottom ash |
| 19 01 12 | Bottom ash and slag other than those mentioned in 19 01 11 |
| 19 01 18 | Pyrolysis wastes other than those mentioned in 19 01 17 |
| 19 01 19 | Sands from fluidised beds |
| 19 02 | Wastes from physico/chemical treatments of waste (including dechromatation, decyanidation, neutralisation) |
| 19 02 03 | Premixed wastes composed only of non-hazardous waste |
| 19 02 10 | Combustible wastes other than those mentioned in 19 02 08 and 19 02 09 |
| 19 04 | Vitrified waste and wastes from vitrification |
| 19 04 01 | Vitrified waste |
| 19 05 | Wastes from aerobic treatment of solid wastes |
| 19 05 01 | Non-composted fraction of municipal and similar wastes |
| 19 05 02 | Non-composted fraction of animal and vegetable waste |
| 19 05 03 | Off-specification compost |
| 19 12 | Wastes from the mechanical treatment of waste (for example sorting, crushing, compacting, pelletising) not otherwise specified |
| 19 12 01 | Paper and cardboard |
| 19 12 02 | Ferrous metal |
| 19 12 03 | Non-ferrous metal |
| 19 12 04 | Plastic and rubber |
| 19 12 05 | Glass |
| 19 12 07 | Wood other than that mentioned in 19 12 06 |
| 19 12 08 | Textiles |
| 19 12 09 | Minerals (for example sand, stones) |
| 19 12 10 | Combustible waste (refuse derived fuel) |
| 19 13 | Wastes from soil and groundwater remediation |
| 19 13 02 | Solid wastes from soil remediation other than those mentioned in 19 13 01 |
| 20 | MUNICIPAL WASTES (HOUSEHOLD WASTE AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |
| 20 01 | Separately collected fractions (except 15 01) |
| 20 01 01 | Paper and carboard |
| 20 01 02 | Glass |
| 20 01 08 | Biodegradable kitchen and canteen waste |
| 20 01 10 | Clothes |
| 20 01 11 | Textiles |
| 20 01 34 | Batteries and accumulators other than those mentioned in 20 01 33 |
| 20 01 36 | Discarded electrical and electronic equipment other than those mentioned in 20 01 21, 20 10 23, and 20 01 35 |
| 20 01 38 | Wood other than that mentioned in 20 01 37 |
| 20 01 39 | Plastics |
| 20 01 40 | Metals |
| 20 01 41 | Wastes from chimney sweeping |
| 20 02 | Garden and park wastes (including cemetery waste) |
| 20 02 01 | Biodegradable waste |
| 20 02 02 | Soil and stones |
| 20 03 | Other municipal wastes |
| 20 03 01 | Mixed municipal waste |
| 20 03 02 | Waste from markets |
| 20 03 03 | Street-cleaning residues |
| 20 03 07 | Bulky waste |

Table A-2 Proposed Clinical Waste Types to be Accepted at the Site

| EWC Code |  | Description |  |
| --- | --- | --- | --- |
| 09 |  | WASTES FROM THE PHOTOGRAPHIC INDUSTRY |  |
| 09 01 |  | Wastes from the photographic industry |  |
| 09 01 01* |  | Water-based developer and activator solutions |  |
| 09 01 02* |  | Water-based offset plate developer solutions3 |  |
| 09 01 03* |  | Solvent based developer solutions3 |  |
| 09 01 04* |  | Fixer solutions3 |  |
| 09 01 05* |  | Bleach and bleach fixer solutions3 |  |
| 09 01 07 |  | Photographic film and paper containing silver or silver compounds3 |  |
| 09 01 08 |  | Photographic film and paper free of silver or silver compounds3 |  |
| 18 |  | WASTES FROM HUMAN OR ANIMAL HEALTHCARE AND/OR RELATED RESEARCH (EXCEPT KITCHEN AND RESTAURANT WASTES NOT ARISING FROM IMMEDIATE HEALTH CARE) |  |
| 18 01 |  | Wastes from natal care, diagnosis, treatment or prevention of disease in humans |  |
| 18 01 01 |  | Sharps (except 18 01 03) |  |
| 18 01 02 |  | Body parts and organs including blood bags and blood preserves (except 18 01 03) |  |
| 18 01 03* |  | Wastes whose collection and disposal is subject to special requirements in order to prevent infection (e.g. dressings, plaster casts, linen, disposable clothing, nappies) |  |
| 18 01 04 |  | Wastes whose collection and disposal is not subject to special requirements in order to prevent infection |  |
| 18 01 06* |  | Chemicals consisting of or containing hazardous substances |  |
| 18 01 07 |  | Chemicals other than those mentioned in 18 01 06 |  |
| 18 01 08* |  | Cytotoxic and cytostatic medicines |  |
| 18 01 09 |  | Medicines other than those mentioned in 18 01 08 |  |
| 18 01 10* |  | Amalgam waste from dental care |  |
| 18 02 |  | Wastes from research, diagnosis, treatment or prevention of disease involving animals |  |
| 18 02 01 |  | Sharps (except 18 02 02) |  |
| 18 02 02* |  | Wastes whose collection and disposal is subject to special requirements in order to prevent infection |  |
| 18 02 03 |  | Wastes whose collection and disposal is not subject to special requirements in order to prevent infection |  |
| 18 02 05* |  | Chemicals consisting of or containing hazardous substances |  |
| 18 02 06 |  | Chemicals other than those mentioned in 18 02 05 |  |
| 18 02 07* |  | Cytotoxic and cytostatic medicines |  |
| 18 02 08 |  | Medicines other than those mentioned in 18 02 07 |  |
| 20 |  | MUNICIPAL WASTES (HOUSEHOLD AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |  |
| 20 01 |  | Separately collected fractions (except 15 01) |  |
| 20 01 31* |  | Cytotoxic and cytostatic medicines |  |
| 20 01 32 |  | Medicines other than those mentioned in 20 01 31 |  |
| 20 01 99 |  | Other fractions not otherwise specified (consisting of nappies and absorbent hygiene products (AHPs) only) |  |

Table A-3 Proposed Asbestos Waste to be Accepted at the Site

| EWC Code |  | Description |  |
| --- | --- | --- | --- |
| 17 |  | CONSTRUCTION AND DEMOLITION WASTES (INCLUDING EXCAVATED SOIL FROM CONTAMINATED SITES) |  |
| 17 06 |  | insulation materials and asbestos-containing construction materials |  |
| 17 06 01* |  | Insulation materials containing asbestos |  |
| 17 06 03* |  | Construction materials containing asbestos |  |

Odour Assessment Form

| Background Information |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Person Undertaking Survey (& Position) |  |  |  |  |  |  |  |  |
| Date |  |  |  | Time |  |  |  |  |
| Survey Results |  |  |  |  |  |  |  |  |
| Location |  |  | Intensity (1-6) (see below) | Persistence (A-E) (see below) |  |  |  | Odour Characteristic (e.g. waste, farm, fuel etc) |
| If odour is strong / persistent additional information to be detailed below: |  |  |  |  |  |  |  |  |
| Intensity |  |  |  |  |  |  |  |  |
| 0 | No detectable odour |  |  |  |  |  |  |  |
| 01 | Very faint odour (some doubt as to whether the odour is actually present) |  |  |  |  |  |  |  |
| 2 | Faint odour (barely noticeable) |  |  |  |  |  |  |  |
| 3 | Distinct odour (odour easily detected) |  |  |  |  |  |  |  |
| 4 | Strong odour (bearable but offensive) |  |  |  |  |  |  |  |
| 5 | Very strong odour (instinct to walk away) |  |  |  |  |  |  |  |
| 6 | Extremely strong odour highly likely to cause annoyance (May induce nausea) |  |  |  |  |  |  |  |
| Persistence |  |  |  |  |  |  |  |  |
| A |  |  | Occasional |  |  |  | Less than 10% of the time |  |
| B |  |  | Intermittent |  |  |  | 10-30% of the time |  |
| C |  |  | Frequent |  |  |  | 30-50% of the time |  |
| D |  |  | Persistent |  |  |  | 50-75% of the time |  |
| E |  |  | Constant |  |  |  | >75% of the time |  |
| If during the survey the odour is strong or persistent at any location on the Site boundary, the following information requires completion regarding plant operation. |  |  |  |  |  |  |  |  |
| Waste Delivery |  | Was waste received at the Site during the period the complaint was received? |  |  |  |  |  |  |
|  |  | If yes, were the correct waste acceptance procedures followed? |  |  |  |  |  |  |
| Waste Processing |  | Were waste processing operations undertaken during the period the complaint was received? |  |  |  |  |  |  |
| Waste Storage |  | Any unusual waste storage operations (i.e. increased volumes, alternative storage location, etc)? |  |  |  |  |  |  |
|  |  | If yes, provide details |  |  |  |  |  |  |

Odour Complaints Reporting Form

| Odour Complaints Recording Form |  |  |  |
| --- | --- | --- | --- |
| Reference Number: |  |  |  |
| Date: |  |  |  |
| Name and address of caller: |  |  |  |
| Tel No. of caller: |  |  |  |
| Location of caller in relation to installation: |  |  |  |
| Time and date of complaint: |  |  |  |
| Date, time and duration of offending odour: |  |  |  |
| Caller’s description of odour, e.g. comparison with other odours, strong/weak, continuous, fluctuating: |  |  |  |
| Has the caller any other comments about the offending odour? |  |  |  |
| Site Research into Complaint |  |  |  |
| Weather conditions (e.g. dry, rain fog, snow): |  |  |  |
| Wind strength and direction (e.g. light, steady, strong, gusting): |  |  |  |
| Any previous complaints relating to this odour? |  |  |  |
| Any other relevant information: |  |  |  |
| Potential odour sources that could give rise to the complaint |  |  |  |
| Operating conditions at the time offending odour occurred (i.e. any unusual operations?): |  |  |  |
| Follow Up |  |  |  |
| Date and time caller contacted: |  |  |  |
| Action taken: |  |  |  |
| Amendment required to Odour Management Plan? Y/N - if Y provide details |  |  |  |
| Form completed by: |  | Signed: |  |

Odour Survey Methodology

The exact locations for offsite monitoring are selected based on the prevailing wind direction and proximity to receptors.

The monitoring will be extended to the surrounding locality if odour likely to cause annoyance is detected at the Site boundary.

At each location observations shall be made concerning odour intensity, persistence and character, time, date, weather conditions and any ‘abnormal’ Site operating conditions at the time of the survey. Surveys shall be carried out in accordance with the monitoring protocol contained within EA’s H4 Odour Guidance.

The odour assessor should not be subject to significant Site odours in the 30-minutes prior to the assessment, or food, drink or cigarettes within the last hour. This is to ensure that monitors are not suffering from odour fatigue and will be sensitive to Site odours. Furthermore, the following exclusions shall apply:

Staff members that are regularly exposed to Site odours for longer than 30 minutes; and

Any staff members known or suspected of having a very poor sense of smell should not be used for odour monitoring.

The inspections shall be undertaken as follows:

- The person should walk slowly and breathe normally and begin their assessment at areas of expected low odour concentration, i.e. upwind of the Site, and should move to areas of high odour concentration. If odour is detected while walking, the intensity should be recorded as at least 3 (distinct), or higher.

If an odour cannot be detected whilst walking, the person should periodically stand still and inhale deeply facing upwind. If odour is then detected, but can only be detected in this manner, the odour ‘intensity’ should be recorded as 2 (faint).

Following detection of any odour of intensity 3 or above at the Site boundary during an odour inspection, the following measures will be taken:

The olfactory survey will deviate to determine the extent of plume downwind (at or above an intensity level 3) and at potential receptors affected; and

An on-site inspection shall be carried out seeking to trace any observed odour back to source so that the appropriate corrective and/or preventative action can be taken (with regard to Contingency Measures detailed in Section 4.0).

On-site inspections would be undertaken by continuing the olfactory survey methodology onto the Site to inspect all potential odour sources.

The Site Manager shall be notified immediately of any detected odours that are considered to have the potential to give rise to significant off-site odour impact (intensity 3 at a receptor location). The contingency measures detailed within Section 4.0 will be followed.

Odour Risk of each EWC Code

The EWC codes to be accepted at the Site have different levels of potential to be odorous. West London Composting Ltd has considered the odour risk from each waste type in “normal” and “abnormal” circumstances, whereby:

- a “normal” circumstance can be considered the most common way one may experience these wastes; and

- an “abnormal” circumstance may be in case where waste which has been left a long time in a transfer station or trailer.

The EWC codes to be accepted and their potential to be odorous in normal and abnormal circumstances are detailed in the following table.

| LoW Code | Description | Waste Stream | Odour drivers | Normal (managed) risk |  | Abnormal (e.g., damaged containers/prolonged dwell/heat) risk |  | Notes/assumptions |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17 06 01* | Insulation materials containing asbestos | Asbestos | Mineral fibres/inert matrix | Negligible |  | Negligible |  | Odour not expected; haz controls apply (separate assessment). |  |
| 17 06 03* | Construction materials containing asbestos | Asbestos | Mineral fibres/inert matrix | Negligible |  | Negligible |  | Odour not expected; haz controls apply (separate assessment). |  |
| 09 01 01* | Water-based developer and activator solutions | Clinical | Chemical odours (developer/fixer) | Low |  | Medium |  | Keep sealed; |  |
| 09 01 02* | Water-based offset plate developer solutions3 | Clinical | Chemical odours (developer/fixer) | Low |  | Medium |  | Keep sealed; |  |
| 09 01 03* | Solvent based developer solutions3 | Clinical | Chemical odours (developer/fixer) | Low |  | Medium |  | Keep sealed; |  |
| 09 01 04* | Fixer solutions3 | Clinical | Chemical odours (developer/fixer) | Low |  | Medium |  | Keep sealed; |  |
| 09 01 05* | Bleach and bleach fixer solutions3 | Clinical | Chemical odours (developer/fixer) | Low |  | Medium |  | Keep sealed; |  |
| 09 01 07 | Photographic film and paper containing silver or silver compounds | Clinical | Inert film/paper; minimal odour | Negligible |  | Low |  |  |  |
| 09 01 08 | Photographic film and paper free of silver or silver compounds3 | Clinical | Inert film/paper; minimal odour | Negligible |  | Low |  |  |  |
| 18 01 01 | Sharps (except 18 01 03) | Clinical | Sealed sharps, no putrescibles | Negligible |  | Low |  |  |  |
| 18 01 02 | Body parts and organs including blood bags and blood preserves (except 18 01 03) | Clinical | Putrescible biological tissue | Medium |  | High |  | Must remain sealed and expedited offsite. Containerised |  |
| 18 01 03* | Wastes whose collection and disposal is subject to special requirements in order to prevent infection (e.g. dressings, plaster casts, linen, disposable clothing, nappies) | Clinical | Soiled dressings/clinical textiles; residual fluids | Medium |  | High |  | Containerised; segregated; rapid turnaround. |  |
| 18 01 04 | Wastes whose collection and disposal is not subject to special requirements in order to prevent infection | Clinical | Soiled non-infectious clinical items | Low |  | Medium |  | Containerised; sealed; housekeeping critical. |  |
| 18 01 06* | Chemicals consisting of or containing hazardous substances | Clinical | Chemical/medicine odours possible | Low |  | Low |  | Sealed containers; |  |
| 18 01 07 | Chemicals other than those mentioned in 18 01 06 | Clinical | Chemical/medicine odours possible | Low |  | Low |  | Sealed containers; |  |
| 18 01 08* | Cytotoxic and cytostatic medicines | Clinical | Chemical/medicine odours possible | Low |  | Low |  | Sealed containers; |  |
| 18 01 09 | Medicines other than those mentioned in 18 01 08 | Clinical | Chemical/medicine odours possible | Low |  | Low |  | Sealed containers; |  |
| 18 01 10* | Amalgam waste from dental care | Clinical | inert | Negligible |  | Negligible |  | Sealed containers; |  |
| 18 02 01 | Sharps (except 18 02 02) | Clinical | Sealed sharps, no putrescibles | Negligible |  | Low |  | Containerised; odour negligible unless spillage. |  |
| 18 02 02* | Wastes whose collection and disposal is subject to special requirements in order to prevent infection | Clinical | Soiled dressings/clinical textiles; residual fluids | Medium |  | High |  | Containerised; segregated; rapid turnaround FIFO |  |
| 18 02 03 | Wastes whose collection and disposal is not subject to special requirements in order to prevent infection | Clinical | Soiled non-infectious clinical items | Low |  | Medium |  | Containerised; sealed; housekeeping critical. |  |
| 18 02 05* | Chemicals consisting of or containing hazardous substances | Clinical | Chemical/medicine odours possible | Low |  | Medium |  | Sealed containers; |  |
| 18 02 06 | Chemicals other than those mentioned in 18 02 05 | Clinical | Chemical/medicine odours possible | Low |  | Medium |  | Sealed containers; |  |
| 18 02 07* | Cytotoxic and cytostatic medicines | Clinical | No significant putrescibles identified | Low |  | Low |  |  |  |
| 18 02 08 | Medicines other than those mentioned in 18 02 07 | Clinical | No significant putrescibles identified | Low |  | Low |  |  |  |
| 20 01 31* | Cytotoxic and cytostatic medicines | Clinical | No significant putrescibles identified | Low |  | Low |  |  |  |
| 20 01 32 | Medicines other than those mentioned in 20 01 31 | Clinical | No significant putrescibles identified | Low |  | Low |  |  |  |
| 20 01 99 | Other fractions not otherwise specified (consisting of nappies and absorbent hygiene products (AHPs) only) | Clinical | Absorbent hygiene products (nappies); residual organics | Medium |  | High |  | Keep sealed/containerised in bags; expedite turnaround FIFO. |  |
| 01 01 01 | Wastes from mineral metalliferous excavation | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 01 01 02 | Wastes from mineral non-metalliferous excavation | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 01 03 06 | Tailings other than those mentioned in 01 03 04 and 01 03 05 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 01 03 09 | Red mud from alumina production other than the wastes mentioned in 01 03 07 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 01 04 08 | Waste gravel and crushed rocks other than those mentioned in 01 04 07 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 01 04 09 | Waste sand and clays | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 01 04 11 | Wastes from potash and rock salt processing other than those mentioned in 01 04 07 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 01 04 12 | Tailings and other wastes from washing and cleaning of minerals other than those mentioned in 01 04 07 and 01 04 11 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 01 04 13 | Wastes from stone cutting and sawing other than those mentioned in 01 04 07 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 02 01 03 | Plant-tissue waste | Non-hazardous | Green/plant waste; can heat/ferment if held | low |  | Medium |  | FIFO, storage limits, 14 day turnaround |  |
| 02 01 04 | Waste plastics (except packaging) | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 02 01 07 | Wastes from forestry | Non-hazardous | Green brash/wood; can heat/ferment if held | Low |  | Low |  |  |  |
| 02 01 10 | Waste metal | Non-hazardous | Inert metals | Negligible |  | Negligible |  |  |  |
| 02 02 03 | Materials unsuitable for consumption or processing | Non-hazardous | Putrescible organics (food/dairy/animal) | Medium |  | High |  | FIFO; 48 h storage; acceptance criteria |  |
| 02 03 04 | Materials unsuitable for consumption or processing | Non-hazardous | Putrescible organics (food/dairy/animal) | Medium |  | High |  | FIFO; 48 h storage; acceptance criteria |  |
| 02 04 01 | Soil from cleaning and washing beet | Non-hazardous | Watery residues; some organics/alcohols | Negligible |  | low |  |  |  |
| 02 04 02 | Off-specification calcium carbonate | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 02 05 01 | Materials unsuitable for consumption or processing | Non-hazardous | Putrescible organics (food/dairy/animal) | Medium |  | High |  | FIFO; 48 h storage; acceptance criteria |  |
| 02 06 01 | Materials unsuitable for consumption or processing | Non-hazardous | Putrescible organics (food/dairy/animal) | Medium |  | High |  | FIFO; 48 h storage; acceptance criteria |  |
| 02 06 02 | Wastes from preserving agents | Non-hazardous | Watery residues; some organics/alcohols | Low |  | Medium |  | FIFO; 48 h storage; acceptance criteria |  |
| 02 07 01 | Wastes from washing, cleaning and mechanical reduction of raw materials | Non-hazardous | Watery residues; some organics/alcohols | Low |  | Medium |  | FIFO; 48 h storage; acceptance criteria |  |
| 02 07 02 | Wastes from spirits distillation | Non-hazardous | Watery residues; some organics/alcohols | Low |  | Medium |  | FIFO; 48 h storage; acceptance criteria |  |
| 02 07 04 | Materials unsuitable for consumption or processing | Non-hazardous | Putrescible organics (food/dairy/animal) | Medium |  | High |  | FIFO; 48 h storage; acceptance criteria |  |
| 03 01 01 | Waste bark and cork | Non-hazardous | Wood/biomass low odour; may sour if wet | Negligible |  | Low |  | FIFO; 48 h storage; acceptance criteria |  |
| 03 01 05 | Sawdust, shavings, cuttings, wood, particle board and veneer other than those mentioned in 03 01 04 | Non-hazardous | Wood/biomass low odour; may sour if wet | Negligible |  | Low |  |  |  |
| 03 03 01 | Waste bark and wood | Non-hazardous | Wood/biomass low odour; may sour if wet | Negligible |  | Low |  |  |  |
| 03 03 07 | Mechanically separated rejects from pulping of waste paper and cardboard | Non-hazardous | Wet paper rejects; starch/ink residues | Negligible |  | Low |  |  |  |
| 03 03 08 | Wastes from sorting of paper and cardboard destined for recycling | Non-hazardous | Paper/textiles can sour if wet; may be soiled | Negligible |  | Low |  |  |  |
| 03 03 10 | Fibre rejects, fibre-, filler- and coating-sludges from mechanical separation | Non-hazardous | Wet fibre/filler sludges; mild organics | Negligible |  | Low |  |  |  |
| 04 01 08 | Waste tanned leather (blue sheetings, shavings, cuttings, buffing dust) containing chromium | Non-hazardous | Soiled/wet fibres/leather can sour but unlikelly to be in signifiant amounts | Negligible |  | Low |  |  |  |
| 04 01 09 | Wastes from dressing and finishing | Non-hazardous | Soiled/wet fibres/leather can sour but unlikelly to be in signifiant amounts | Negligible |  | Low |  |  |  |
| 04 02 21 | Wastes from unprocessed textile fibres | Non-hazardous | Soiled/wet fibres/leather can sour but unlikelly to be in signifiant amounts | Negligible |  | Low |  |  |  |
| 04 02 22 | Wastes from processed textile fibres | Non-hazardous | Soiled/wet fibres/leather can sour but unlikelly to be in signifiant amounts | Negligible |  | Low |  |  |  |
| 06 09 02 | Phosphorous slag | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 06 09 04 | Calcium-based reaction wastes other than those mentioned in 06 09 03 | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 06 11 01 | Calcium-based reaction wastes from titanium dioxide production | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 07 02 13 | Waste plastic | Non-hazardous | Non-putrescible polymers | Low |  | Low |  |  |  |
| 09 01 07 | Photographic film and paper containing silver or silver compounds | Non-hazardous | Inert film/paper; minimal odour | Negligible |  | Low |  |  |  |
| 09 01 08 | Photographic film and paper free of silver of silver compounds | Non-hazardous | Inert film/paper; minimal odour | Negligible |  | Low |  |  |  |
| 09 01 10 | Single-use cameras without batteries | Non-hazardous | Inert film/paper; minimal odour | Negligible |  | Low |  |  |  |
| 09 01 12 | Single-use cameras containing batteries other than those mentioned in 09 01 11 | Non-hazardous | Inert film/paper; minimal odour | Negligible |  | Low |  |  |  |
| 10 01 01 | Bottom ash, slag and boiler dust (excluding boiler dust mentioned in 10 01 04) | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 01 05 | Calcium-based reaction wastes from flue-gas desulpherisation in solid form | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 01 07 | Calcium-based reaction wastes from flue-gas desulpherisation in sludge form | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 01 15 | Bottom ash, slag and boiler dust from co-incineration other than those mentioned in 10 01 14 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 01 19 | Wastes from gas cleaning other than those mentioned in 10 01 05, 10 01 07 and 10 01 18 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 01 24 | Sands from fluidised beds | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 02 01 | Wastes from the processing of slag | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 02 02 | Unprocessed slag | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 02 08 | Solid wastes from gas treatment other than those mentioned in 10 02 07 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 02 10 | Mill scales | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 02 14 | Filter cakes from gas treatment other than those mentioned in 10 02 13 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 02 15 | Other filter cakes | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 03 02 | Anode scraps | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 03 05 | Waste alumina | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 03 16 | Skimmings other than those mentioned in 10 03 15 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 03 18 | Carbon-containing wastes from anode manufacture other than those mentioned in 10 03 17 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 03 24 | Solid wastes from gas treatment other than those mentioned in 10 03 23 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 03 26 | Filter cakes from gas treatment other than those mentioned in 10 03 25 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 03 28 | Wastes from cooling-water treatment other than those mentioned in 10 03 27 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 03 30 | Wastes from treatment of salt slags and black drosses other than those mentioned in 10 03 29 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 04 10 | Wastes from cooling-water treatment other than those mentioned in 10 04 09 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 05 01 | Slags from primary and secondary production | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 05 09 | Wastes from cooling-water treatment other than those mentioned in 10 05 08 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 05 11 | Dross and skimmings other than those mentioned in 10 05 10 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 06 01 | Slags from primary and secondary production | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 06 02 | Dross and skimmings from primary and secondary production | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 06 10 | Wastes from cooling-water treatment other than those mentioned in 10 05 10 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 07 01 | Slags from primary and secondary production | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 07 02 | Dross and skimmings from primary and secondary production | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 07 03 | Solid wastes from gas treatment | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 07 05 | Filter cakes from gas treatment | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 07 08 | Wastes from cooling-water treatment other than those mentioned in 10 07 07 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 08 09 | Other slags | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 08 11 | Dross and skimmings other than those mentioned in 10 08 10 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 08 13 | Carbon-containing wastes from anode manufacture other than those mentioned in 10 08 12 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 08 14 | Anode scrap | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 08 18 | Filter cakes from flue-gas treatment other than those mentioned in 10 08 17 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 08 20 | Wastes from cooling-water treatment other than those mentioned in 10 08 19 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 09 03 | Furnace slag | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 09 06 | Casting cores and moulds which have not undergone pouring other than those mentioned in 10 09 05 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 09 08 | Casting cores and moulds which have undergone pouring other than those mentioned in 10 09 07 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 09 14 | Waste binders other than those mentioned in 10 09 13 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 09 16 | Waste crack-indicating agent other than those mentioned in 10 09 15 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 10 03 | Furnace slag | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 10 06 | Casting cores and moulds which have not undergone pouring, other than those mentioned in 10 10 05 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 10 08 | Casting cores and moulds which have undergone pouring, other than those mentioned in 10 10 07 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 10 14 | Waste binders other than those mentioned in 10 10 13 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 10 16 | Waste crack-indicating agent other than those mentioned in 10 10 15 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 11 03 | Waste glass-based fibrous materials | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 11 10 | Waste preparation mixture before thermal processing, other than those mentioned in 10 11 09 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 11 12 | Waste glass other than those mentioned in 10 11 11 | Non-hazardous | Inert glass | Negligible |  | Negligible |  |  |  |
| 10 11 16 | Solid wastes from flue-gas treatment other than those mentioned in 10 11 15 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 11 18 | Filter cakes from flue-gas treatment other than those mentioned in 10 11 17 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 12 01 | Waste preparation mixture before thermal processing | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 12 05 | Filter cakes from gas treatment | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 12 06 | Discarded moulds | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 12 08 | Waste ceramics, bricks, tiles and construction products (after thermal processing) | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 12 10 | Solid wastes from gas treatment other than those mentioned in 10 12 09 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 12 12 | Wastes from glazing other than those mentioned in 10 12 11 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 13 01 | Waste preparation mixture before thermal processing | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 13 04 | Wastes from calcination and hydration of lime | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 13 07 | Filter cakes from gas treatment | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 13 10 | Wastes from asbestos-cement manufacture other than those mentioned in 10 13 09 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 13 11 | Wastes from cement-based composite materials other than those mentioned in 10 13 09 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 13 13 | Solid wastes from gas treatment other than those mentioned in 10 13 12 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 10 13 14 | Waste concrete | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 11 01 10 | Filter cakes other than those mentioned in 11 01 09 | Non-hazardous | Light hydrocarbon/chemical odours possible | Low |  | Medium |  | uncommon waste, FIFO, acceptance criteria |  |
| 11 01 14 | Degreasing wastes other than those mentioned in 11 01 13 | Non-hazardous | Light hydrocarbon/chemical odours possible | Low |  | Medium |  | uncommon waste, FIFO, acceptance criteria |  |
| 11 02 03 | Wastes from the production of anodes for aqueous electrolytical processes | Non-hazardous | Light hydrocarbon/chemical odours possible | Low |  | Medium |  | uncommon waste, FIFO, acceptance criteria |  |
| 11 02 06 | Wastes from copper hydrometallurgical processes other than those mentioned in 11 02 05 | Non-hazardous | Light hydrocarbon/chemical odours possible | Low |  | Medium |  | uncommon waste, FIFO, acceptance criteria |  |
| 11 05 01 | Hard zinc | Non-hazardous | Light hydrocarbon/chemical odours possible | Low |  | Medium |  | uncommon waste, FIFO, acceptance criteria |  |
| 11 05 02 | Zinc ash | Non-hazardous | Light hydrocarbon/chemical odours possible | Low |  | Medium |  | uncommon waste, FIFO, acceptance criteria |  |
| 12 01 01 | Ferrous metal filings and turnings | Non-hazardous | Inert metals | Negligible |  | Negligible |  |  |  |
| 12 01 03 | Non-ferrous metal filings and turnings | Non-hazardous | Inert metals | Negligible |  | Negligible |  |  |  |
| 12 01 05 | Plastic shavings and turnings | Non-hazardous | Non-putrescible polymers | Low |  | Low |  |  |  |
| 12 01 13 | Welding wastes | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 12 01 17 | Waste blasting material other than those mentioned in 12 01 16 | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 12 01 21 | Spent grinding bodies and grinding materials other than those mentioned in 12 01 20 | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 15 01 01 | Paper and cardboard packaging | Non-hazardous | Mostly clean empties | Low |  | Low |  |  |  |
| 15 01 02 | Plastic packaging | Non-hazardous | Non-putrescible polymers | Low |  | Low |  |  |  |
| 15 01 03 | Wooden packaging | Non-hazardous | Mostly clean empties | Low |  | Low |  |  |  |
| 15 01 04 | Metallic packaging | Non-hazardous | Inert metals | Negligible |  | Negligible |  |  |  |
| 15 01 05 | Composite packaging | Non-hazardous | Mostly clean empties | Low |  | Low |  |  |  |
| 15 01 06 | Mixed packaging | Non-hazardous | Residual food/contaminants in mix | Negligible |  | low |  |  |  |
| 15 01 07 | Glass packaging | Non-hazardous | Inert glass | Negligible |  | Negligible |  |  |  |
| 15 01 09 | Textile packaging | Non-hazardous | Paper/textiles can sour if wet; may be soiled | Negligible |  | low |  |  |  |
| 15 02 03 | Absorbents, filter materials, wiping cloths, and protective clothing other than those mentioned in 15 02 02 | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 16 01 03 | End-of-life tyres | Non-hazardous | Rubber odour only (not putrescible) | Low |  | Low |  |  |  |
| 16 02 14 | Discarded equipment other than those mentioned in 16 02 09 to 16 02 13 | Non-hazardous | Non-putrescible components | Low |  | Low |  |  |  |
| 16 02 16 | Components removed from discarded equipment other than those mentioned in 16 02 15 | Non-hazardous | Non-putrescible components | Low |  | Low |  |  |  |
| 16 03 04 | Inorganic wastes other than those mentioned in 16 03 03 | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 16 03 06 | Organic wastes other than those mentioned in 16 03 05 | Non-hazardous | Organic off-spec wastes | Medium |  | High |  | Would be required to go through pre acceptance and be pre-assessed by TCM |  |
| 16 06 04 | Alkaline batteries (except 16 06 03) | Non-hazardous | Inert casings; negligible odour | Negligible |  | Low |  |  |  |
| 16 06 05 | Other batteries and accumulators | Non-hazardous | Inert casings; negligible odour | Negligible |  | Low |  |  |  |
| 16 11 02 | Carbon-based linings and refractories from metallurgical processes other than those mentioned in 16 011 01 | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 16 11 04 | Other linings and refractories from metallurgical processes other than those mentioned in 16 11 03 | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 16 11 06 | Linings and refractories from non-metallurgical processes other than those mentioned in 16 11 05 | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 17 01 01 | Concrete | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 17 01 02 | Bricks | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 17 01 03 | Tiles and ceramics | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 17 01 07 | Mixtures of concrete, bricks, tiles and ceramics other than those mentioned in 17 01 06 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 17 02 01 | Wood | Non-hazardous | Wood/biomass low odour; may sour if wet but generally in massive form unlikley to have odour | Low |  | Medium |  |  |  |
| 17 02 02 | Glass | Non-hazardous | Inert glass | Negligible |  | Negligible |  |  |  |
| 17 02 03 | Plastic | Non-hazardous | Non-putrescible polymers | Low |  | Low |  |  |  |
| 17 03 02 | Bituminous mixtures other than those mentioned in 17 03 01 | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 17 04 01 | Copper, bronze, brass | Non-hazardous | Inert metals | Negligible |  | Negligible |  |  |  |
| 17 04 02 | Aluminium | Non-hazardous | Inert metals | Negligible |  | Negligible |  |  |  |
| 17 04 03 | Lead | Non-hazardous | Inert metals | Negligible |  | Negligible |  |  |  |
| 17 04 04 | Zinc | Non-hazardous | Inert metals | Negligible |  | Negligible |  |  |  |
| 17 04 05 | Iron and steel | Non-hazardous | Inert metals | Negligible |  | Negligible |  |  |  |
| 17 04 06 | Tin | Non-hazardous | Inert metals | Negligible |  | Negligible |  |  |  |
| 17 04 07 | Mixed metals | Non-hazardous | Inert metals | Negligible |  | Negligible |  |  |  |
| 17 04 11 | Cables other than those mentioned in 17 04 10 | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 17 05 04 | Soil and stones other than those mentioned in 17 05 03 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 17 05 08 | Track ballast other than those mentioned in 17 05 07 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 17 06 04 | Insulation materials other than those mentioned in 17 06 01 and 17 06 03 | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 17 08 02 | Gypsum-based construction materials other than those mentioned in 17 08 01 | Non-hazardous | Gypsum generating H2S if wet & mixed with organics | Low |  | High |  | Segregate from organics; keep dry. Acceptance criteria |  |
| 17 09 04 | Mixed construction and demolition wastes other than those mentioned in 17 09 01, 17 09 02, and 17 09 03 | Non-hazardous | Minor residues; | Negligible |  | Negligible |  |  |  |
| 19 01 02 | Ferrous materials removed from bottom ash | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 19 01 12 | Bottom ash and slag other than those mentioned in 19 01 11 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 19 01 18 | Pyrolysis wastes other than those mentioned in 19 01 17 | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 19 01 19 | Sands from fluidised beds | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 19 02 03 | Premixed wastes composed only of non-hazardous waste | Non-hazardous | Mixed/combustible outputs with residual organics | Medium |  | High |  | due to variability would always require pre-assessment |  |
| 19 02 10 | Combustible wastes other than those mentioned in 19 02 08 and 19 02 09 | Non-hazardous | Mixed/combustible outputs with residual organics | Medium |  | High |  | due to variability would always require pre-assessment |  |
| 19 04 01 | Vitrified waste | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 19 05 01 | Non-composted fraction of municipal and similar wastes | Non-hazardous | MBT fine fractions; residual putrescibles | Medium |  | High |  | Dense material which would be turned around within max 7 days FIFO |  |
| 19 05 02 | Non-composted fraction of animal and vegetable waste | Non-hazardous | Putrescible organics (food/dairy/animal) | High |  | High |  | Containerised or palletised. Reloaded out within 3 days. Pre-assessment due to variability |  |
| 19 05 03 | Off-specification compost | Non-hazardous | Off-spec compost; anaerobic potential | Low |  | Low |  |  |  |
| 19 12 01 | Paper and cardboard | Non-hazardous | Paper/textiles can sour if wet; may be soiled | Low |  | Low |  |  |  |
| 19 12 02 | Ferrous metal | Non-hazardous | Inert metals | Negligible |  | Negligible |  |  |  |
| 19 12 03 | Non-ferrous metal | Non-hazardous | Inert metals | Negligible |  | Negligible |  |  |  |
| 19 12 04 | Plastic and rubber | Non-hazardous | Non-putrescible polymers | Low |  | Low |  |  |  |
| 19 12 05 | Glass | Non-hazardous | Inert glass | Negligible |  | Negligible |  |  |  |
| 19 12 07 | Wood other than that mentioned in 19 12 06 | Non-hazardous | Wood/biomass low odour; may sour if wet | Low |  | Low |  |  |  |
| 19 12 08 | Textiles | Non-hazardous | Paper/textiles can sour if wet; may be soiled | Low |  | Low |  |  |  |
| 19 12 09 | Minerals (for example sand, stones) | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 19 12 10 | Combustible waste (refuse derived fuel) | Non-hazardous | RDF unlikley to have residual fines or organics | Low |  | Low |  |  |  |
| 19 13 02 | Solid wastes from soil remediation other than those mentioned in 19 13 01 | Non-hazardous | Mineral/inert; no organics | Negligible |  | Negligible |  |  |  |
| 20 01 01 | Paper and carboard | Non-hazardous | Paper/textiles can sour if wet; may be soiled | Low |  | Low |  |  |  |
| 20 01 02 | Glass | Non-hazardous | Inert glass | Negligible |  | Negligible |  |  |  |
| 20 01 08 | Biodegradable kitchen and canteen waste | Non-hazardous | Putrescible organics (food/dairy/animal) | High |  | High |  | Dense material which would be turned around within max 48 working hours FIFO |  |
| 20 01 10 | Clothes | Non-hazardous | Paper/textiles can sour if wet; may be soiled | Low |  | Low |  |  |  |
| 20 01 11 | Textiles | Non-hazardous | Paper/textiles can sour if wet; may be soiled | Low |  | Low |  |  |  |
| 20 01 34 | Batteries and accumulators other than those mentioned in 20 01 33 | Non-hazardous | Inert casings; negligible odour | Negligible |  | Low |  |  |  |
| 20 01 36 | Discarded electrical and electronic equipment other than those mentioned in 20 01 21, 20 10 23, and 20 01 35 | Non-hazardous | Non-putrescible components | Low |  | Low |  |  |  |
| 20 01 38 | Wood other than that mentioned in 20 01 37 | Non-hazardous | Wood/biomass low odour; may sour if wet | Low |  | Low |  |  |  |
| 20 01 39 | Plastics | Non-hazardous | Non-putrescible polymers | Low |  | Low |  |  |  |
| 20 01 40 | Metals | Non-hazardous | Inert metals | Negligible |  | Negligible |  |  |  |
| 20 01 41 | Wastes from chimney sweeping | Non-hazardous | Soot/tar smell possible; not putrescible | Low |  | Low |  |  |  |
| 20 02 01 | Biodegradable waste | Non-hazardous | Green/plant waste; can heat/ferment if held | Medium |  | High |  | FIFO - turnaround time max storage 14 days |  |
| 20 02 02 | Soil and stones | Non-hazardous | Mostly mineral; possible organic taint | Low |  | Low |  |  |  |
| 20 03 01 | Mixed municipal waste | Non-hazardous | Variable black-bag content; residual organics | Medium |  | High |  | Assumes local collections with food segregated. If food present, treat as MED/HIGH. FIFO in place. Max storage amounts. Timed storage max 7 days |  |
| 20 03 02 | Waste from markets | Non-hazardous | Putrescible organics (food/dairy/animal) | Medium |  | High |  | Dense material which would be turned around within max 48 working hours FIFO |  |
| 20 03 03 | Street-cleaning residues | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |
| 20 03 07 | Bulky waste | Non-hazardous | No significant putrescibles identified | Low |  | Low |  |  |  |

.

Off-site Odour Monitoring

Guidance on the Assessment of Odour for Planning, IAQM, July 2018.

H4 Odour Management, How to comply with your environmental permit, Environment Agency, March 2011

This is limited to wastes of this type arising from medical practices or associated research activities.