WRM

wrm-ltd.co.uk

01943 468138

Site Condition Report

v1.0

Environmental and sustainability solutions provided to

GB Liming Ltd

WRM-LTD.CO.UK

This report was prepared by Walker Resource Management Ltd (WRM’) within the terms of its engagement and in direct response to a scope of services. This report is strictly limited to the purpose and the facts and matters stated in it and does not apply directly or indirectly and must not be used for any other application, purpose, use or matter. In preparing the report, WRM may have relied upon information provided to it at the time by other parties. WRM accepts no responsibility as to the accuracy or completeness of information provided by those parties at the time of preparing the report. The report does not take into account any changes in information that may have occurred since the publication of the report. If the information relied upon is subsequently determined to be false, inaccurate, or incomplete then it is possible that the observations and conclusions expressed in the report may have changed. WRM does not warrant the contents of this report and shall not assume any responsibility or liability for loss whatsoever to any third party caused by, related to, or arising out of any use or reliance on the report howsoever. No part of this report, its attachments or appendices may be reproduced by any process without the written consent of WRM. All enquiries should be directed to WRM.

| Document Title | Site Condition Report |  |
| --- | --- | --- |
| Client | GB Liming Ltd |  |
| Revision | v1.0 |  |
| Date | 07/04/2026 |  |
| Document Reference | Site Condition Report |  |
| Project Reference | 1578/F05 |  |
| Author: Bryanna Heaton |  | Reviewer: Graeme Kennett |

Copyright ©

All material on these pages, including without limitation text, logos, icons and photographs, is copyright material of Walker Resource Management Ltd (WRM). Use of this material may only be made with the express, prior, written permission of WRM. This document was produced solely for use by the named client to whom the document refers. The methodology (if any) contained in this report is provided to you in confidence and must not be disclosed or copied to third parties without the prior written agreement of WRM. Disclosure of that information may constitute an actionable breach of confidence or may otherwise prejudice our commercial interests.

| Revision | Details | Date |
| --- | --- | --- |
| 0.1 | Initial Draft | 13/01/2026 |
| 0.2 | Update Following Information Return | 19/02/2026 |
| 1.0 | Final version for submission | 07/04/2026 |

Revision Log

CONTENTS

Contents

1.0	Introduction	1

2.0	condition of land at permit issue	2

3.0	permitted Activities	9

4.0	changes to the activity	11

5.0	Measures Taken to Protect Land	11

6.0	Pollution Incidents that may have had an impact on land, and their remediation	12

7.0	Soil Gas and Water Quality Monitoring (where undertaken)	12

8.0	decommissioning and removal of pollution risk	12

9.0	Reference Data and Remediation (where relevant)	13

10.0	Statement of Site Condition	13

# Introduction

| Name of the applicant | GB Liming Ltd |
| --- | --- |
| Activity address | GB Liming Ltd Weston Road Honeybourne Evesham WR11 7QF |
| National grid reference | SP 11152 42124 |
| Document reference and dates for Site Condition Report at permit application and surrender | Date at application: Site Condition Report – 19/02/2026 |
| Document references for site plans (including location and boundaries) | The following figures and drawings are included in the accompanying documents: - EPR-001_Site_Plan - Site Layout Drafts - Honeybourne (8.12.25) |

# condition of land at permit issue

| Environmental setting | GB Liming Ltd (hereon ‘GBL’) currently holds no Environmental Permit for the operation of the site. The site is seeking a Bespoke Environmental Permit for the operation of a biosolids liming facility. The site is situated on an unnamed track off Weston Road (B4035) within the predominantly rural area of Honeybourne, approximately 7km east of Evesham Town Centre. It is surrounded by rural land use with the nearest residential receptor located approximately 500m southwest of the site. Weston Industrial Estate is situated approximately 1km southeast of the site. The site is not located within; - 500 metres of a European site, Ramsar, SSSI or Marine Conservation Zone (MCZ); - 50 metres of a National Nature Reserve, Local Nature Reserve, Local Wildlife Site, Ancient woodland or Scheduled Ancient Monument; - 50 metres of a site that has species or habitats protected under the Biodiversity Action Plan that the Environment Agency considers at risk to this activity; - 250 metres within the presence of Great Crested Newts where it is linked to the breeding ponds of the newts by good habitat; - 10 metres of a watercourse. The northern portion of the site is predominantly comprised of the site weighbridge and the weighbridge office / store which is underlain by concrete hardstanding. 4 biosolids received bays are situated in the northeastern portion of the site on the impermeable concrete hardstanding. The lime rig and biosolids reception area is situated in the northwestern portion of the site on impermeable concrete. The northern portion of the site is considered the ‘Dirty area’. The central portion of the site comprises 8 limed biosolids storage bays inclusive of mellowing bays situated on impermeable concrete hardstanding. The southern portion of the site comprises the 50m3 underground run off tank and the 4” dock for waste water extraction via road tanker. Geology The site is indicated by the British Geological Survey (BGS) online viewer (GeoIndex) and sheet mapping (Sheet No. 200 Stratford upon Avon 1974 1:50,000) to be underlain by the following sequence: - Superficial deposits of Head-Clay, silt, sand and gravel across the entirety of the site comprising Clay, Silt, Sand and Gravel considered likely to be underlain by; - Solid geology of the Blue Lias Formation And Charmouth Mudstone Formation (Undifferentiated) across the entirety of the site comprising mustone. There are no publicly available BGS recorded boreholes located on-site. Hydrology The closest surface water feature to the site is located 743m to the south of the site. In addition, the following data is presented within the Envirocheck Report: - There are twelve discharge consents to freshwater/river within 1000m of the site. The closest listing relates to the release of Sewage Discharges – Final/Treated Effluent to the Ditch Trib Of Bretforton Brook located approximately 717m to the west. The status of this listing is recorded as New Consent (Water Resources Act 1991, Section 88 & Schedule 10 as amended by Environment Act 1995). - The remaining listings are located in excess of 720m from the site and relate to various discharges, including Sewage Discharges and Crude Effluent. Hydrogeology The superficial deposits of Head-Clay, silt, sand and gravel deposits are classified by the EA as a Secondary Undifferentiated Aquifer. The solid geology of the Blue Lias Formation And Charmouth Mudstone Formation is classified by the EA as a Secondary Undifferentiated Aquifer. In addition, the EA provides the following classification and data: - There are no site sensitivity land uses or source protection zones. Flooding The site is indicated by the Environment Agency to be: - At a very low risk of flooding from rivers and the sea, defined as a 0.1% chance of a flood each year. - The site is at a very low risk of surface water flooding defined as a 0.1% chance of a flood each year. - The site is in an area where there is a potential for groundwater flooding to occur at surface. - The site is in an area where flooding from reservoirs is unlikely. |
| --- | --- |

| Pollution history including: pollution incidents that may have affected land historical land-uses and associated contaminants any visual or olfactory evidence of existing contamination evidence of damage to pollution prevention measures | Historical Land Use 1885 - The site is characterised of woodland. Honeybourne Grounds is situated approximately 280m northeast of the site boundary including associated residential dwellings. - There are two public footpaths running through the site from the southwest to the northeast. - There is a lodge situated approximately 150m south of the site and another situated approximately 150m east of the site. - The surrounding area of the site is characterised predominantly of agricultural land. - Buckle Street is situated approximately 800m east of the site. 1903 – 1905 - The site is no longer characterised of woodland and is now characterised of agricultural land. - There is no notable change to the surrounding area of the site. 1924 - No notable change to the site. - The Great Western Railway was developed and is situated approximately 900m east of the site. - No other notable changes to the surrounding area of the site. - Western Subedge Station is situated approximately 500m southeast of the site. 1938 - 1948 - No notable change to the site. - No other notable changes to the surrounding area of the site. 1955 – 1968 - The site is now part of a wilder Airfield site. - No other changes surrounding the site. 1971 – 1974 - An unnamed track has now been developed to the immediate to the north and south of the site. - Two ancillary structures characterised by industrial units approximately 100m north of the site. - A transport depot, tower farm and associated tower farmhouse are situated approximately 300 – 800m east of the site. - Residential and industrial developments approximately 800m to the east of the site including sewage works, factories and packing depots. - Increased residential developments approximately 1.5km northwest of the site. 1993 – 2000 - Another ancillary structure and tanks have been developed approximately 100m north of the site. - Airfield is disused. - Further development of Honeybourne Industrial Estate situated approximately 300m east of the site. - Continued minor industrial development to the east of the site with Weston Industrial Estate now being named. - No further development to the area surrounding the site. 2006 – 2025 - Another ancillary structure has been developed approximately 100m north of the site. - Willersy solar farm is situated approximately 1.2km southwest of the site. - No more notable developments to the area surrounding the site. On-Site Summary From at least 1885 until approximately 1955 the site is indicated to have been occupied by fields. From at least 1955 until approximately 1968 the site was characterised by a now disused airfield. From 1968 to present the site has been characterised of an underdeveloped industrial site with several ancillary structures developed to the north of the site. Surrounding Area Summary Since at least 1885 until 1971 the surrounding area has generally comprised agricultural fields with the most notable development being the Great Western Railway line. From 1971 to present, there has been gradual industrial developments to the east of the site which are now established industrial areas. Pollution Incidents - There is one Pollution Incident to Controlled Waters listing located within 1000m of the site. The listing is located approximately 208m east of the site and relates to the release of oils - diesel in 1998. This listing is recorded as category 3 – minor incident. - There are no Substantiated Pollution Incident Register listings located within 1000m of the site. Waste Register - There are four Licenced Waste Management Facilities (Locations) records within 500m of the site. The closest listing relates to a Metals Recycling Site (Mixed) (License: 48168), recorded as being issued in 1992 and the status is recorded as expired. All other remaining listings are categorised as metals recycling sites, waste transfer facilities, HCI Waste TS, treatment and asbestos, vehicle depollution, and ELVs recycling sites with licenses that have been expired, issued, superseded or modified. - There is one Registered Waste Transfer Site listing within 1000m of the site. The listing is located approximately 682m to the east and relates to a waste transfer site that accepts Household/Commercial/Industrial Waste. The licence for this facility is recorded as Operational as far as is known. Industrial Activity - There are 15 Contemporary Trade Directory Entries records within 500m of the site. The closest entry is located approximately 257m south of the site and relates to Concrete Manufacturers & Distributors. The status of this record is listed as active. Further from site there are 14 listings within 500m of the site that relate to various active and inactive commercial and industrial activities, including but not limited to Garage Services, Engineers, and Distribution Services. - There are four Manufacturing and Production (Points of Interest) records located within 500m of the site. The closest listing relates to Industrial Features (Tanks) and is located approximately 425m east of the site. All other listings relate to industrial features either tank(s), business parks and industrial estates or unspecified works or factories. - There is an airport and landing strip (Public Infrastructure) located approximately 451m east of the site known as Honeybourne Airfield. |
| --- | --- |

`

| Evidence of historic contamination, for example, historical site investigation, assessment, remediation and verification reports (where available) | There are no previous third-party Site investigation, Assessment, Remediation or Verification reports for the Site relevant to historical contamination. |
| --- | --- |
| Baseline Soil and Groundwater Reference Data | There have been no ground investigations carried out on site As such, no baseline levels of contamination have been established. However, rural soil chemistry levels have been estimated for the site as follows: Arsenic – 15 - 25mg/kg Cadmium – <1.8mg/kg Chromium – 60 – 90mg/kg Lead – <100mg/kg Nickel – 30 – 45mg/kg |
| Supporting Information | - Appendix A: Historical Site Maps - Appendix B: Environmental Setting Maps - Appendix C: Datasheet |

# permitted Activities

| Permitted Activities | GB Liming Ltd (hereon ‘GBL’) currently currently holds no Environmental Permit for the operation of the site. The site is seeking a Bespoke Environmental Permit for the operation of a biosolids liming facility. All incoming vehicles delivering feedstock to the facility will enter the site via the entrance. Documentation will be checked by the inputs operative to ensure that the material being delivered is of the correct type. The relevant documentation will be signed by the operative to confirm the acceptance/receipt of the material prior to the driver being allowed to proceed to the dedicated unloading area and will be sorted into dedicated storage areas. Biosolids will then be treated using alkaline materials (typically quicklime or hydrated lime) which raises the pH and temperature of the material for a defined period (raise pH to above 12 for at least 2 hours), which: - Reduces pathogens to safe levels - Stabilises the material and slows biological breakdown - Significantly reduces odour potential - Improves handling characteristics The treated material is commonly referred to as lime-stabilised biosolids. When held at such elevated pH for more than two hours, this technique can deliver Enhanced Treated biosolids. The process parameters (lime dosage, mixing effectiveness, retention time) are controlled through documented procedures and operator competence, ensuring consistent outcomes. Compliant biosolids will then be loaded and removed for application to land under The Sludge (Use in Agriculture) Regulations 1989. |
| --- | --- |
| Non-Permitted Activities Undertaken | There will be no non-permitted activities undertaken on-site. |
| Document References for: - Plan showing activity layout - Historical land use maps - Site sensitivity maps | The following figures and drawings are included in the accompanying documents: - EPR-001_Site_Plan - Site Layout Drafts - Honeybourne (8.12.25) - Appendix A: Historical Site Maps - Appendix B: Environmental Setting Maps - Appendix C: Datasheet |

# changes to the activity

| Have there been any changes to the activity boundary? |  | If yes, provide a plan showing the changes to the activity boundary |
| --- | --- | --- |
| Have there been any changes to the permitted activities? |  | N/A |
| Have any ‘dangerous substances’ not identified in the Application Site Condition Report been used or produced as a result of the permitted activities? |  | N/A. |
| Checklist of supporting information | EPR-001_Site_Plan EPR-A01_Non-Technical_Summary_v1.0 Site Layout Drafts - Honeybourne (8.12.25) |  |

# Measures Taken to Protect Land

| Use records that you collected during the life of the permit to summarise whether pollution prevention measures worked. If you can’t, you need to collect land and/or groundwater data to assess whether the land has deteriorated. |  |
| --- | --- |
| Checklist of supporting information | - Inspection records and summary of findings of inspections for all pollution prevention measures - Records of maintenance, repair and replacement of pollution prevention measures |

# Pollution Incidents that may have had an impact on land, and their remediation

| Summarise any pollution incidents that may have damaged the land. Describe how you investigated and remedied each one. If you can’t, you need to collect land and /or groundwater reference data to assess whether the land has deteriorated while you’ve been there. |  |
| --- | --- |
| Checklist of supporting information | - Records of pollution incidents that may have impacted on land - Records of their investigation and remediation |

# Soil Gas and Water Quality Monitoring (where undertaken)

| Provide details of any soil gas and/or water monitoring you did. Include a summary of the findings. Say whether it shows that the land deteriorated as a result of the permitted activities. If it did, outline how you investigated and remedied this. |  |
| --- | --- |
| Checklist of supporting information | - Description of soil gas and/or water monitoring undertaken - Monitoring results (including graphs) |

# decommissioning and removal of pollution risk

| Describe how the site was decommissioned. Demonstrate that all sources of pollution risk have been removed. Describe whether the decommissioning had any impact on the land. Outline how you investigated and remedied this. |  |
| --- | --- |
| Checklist of supporting information | - Site closure plan - List of potential sources of pollution risk - Investigation and remediation reports (where relevant) |

# Reference Data and Remediation (where relevant)

| Say whether you had to collect land and/or groundwater data. Or say that you didn’t need to because the information from section 4 of the Surrender Site Condition Report shows that the land has not deteriorated. If you did collect land and/or groundwater reference data, summarise what this entailed, and what your data found. Say whether the data shows that the condition of the land has deteriorated, or whether the land at the site is in a “satisfactory state”. If it isn’t, summarise what you did to remedy this. Confirm that the land is now in a “satisfactory state” at surrender. |  |
| --- | --- |
| Checklist of supporting information | Land and/or groundwater data collected at application (if collected) Land and/or groundwater data collected at surrender (where needed) Assessment of satisfactory state Remediation and verification reports (where undertaken) |

# Statement of Site Condition

Using the information from sections 4 and 5, give a statement about the condition of the land at the site. This should confirm that: - The permitted activities have stopped. - Decommissioning is complete, and the pollution risk has been removed. - The land is in a satisfactory condition.