Site Condition Report

| 1.0 SITE DETAILS |  |
| --- | --- |
| Applicants Name | Foxholes Farm Ltd |
|  | Shackerley Poultry Farm Shackerley Lane Albrighton Shropshire WV7 3AB |
| National Grid Reference | 381685, 306086 |
| Document Reference for Site Condition Report at permit application and surrender | 10/07/2026 |
|  | Location Plan |
|  | Layout/Drainage Plan |

| 2.0 Condition of the Land at Permit issue |  |
| --- | --- |
| Environmental setting including: Geology Hydrogeology Surface Waters | The poultry unit lies on level ground to the north of Albrighton, Predominant land use is arable farming. Field pattern is semi large scale with a strong pattern of field boundaries. There are sensitive receptors within 400m of site. Topography and Drainage The poultry houses are 100 to 110m metres above sea level. Existing mature hedges and some woodland help to minimise the visual intrusion normally associated with poultry units. Site clean water drainage all goes to an unlined attenuation pond. Geology and Hydrogeology According to the BGS Extract Map shows the site geology to be of Triassic period Mercia Mudstone Group – Mudstone and Halite-stone. Searches indicate the site is underlain with a Bedrock Aquifer Secondary B. No superficial aquifer.. The Site is situated in a Groundwater Vulnerability Zone - Medium. The site is situated within a Surface water Nitrate Vulnerable Zone. The site lies in a Source Protection Zone – 3. The site is not situated in a Groundwater/Source or drinking water protection zone. Hydrology General drainage is in a Northerly direction. Average rainfall for this area is 1200 mm. |
| Pollution history including: Pollution incidents that may have affected land Historical land uses and associated contaminants Any visual/olfactory evidence of existing contamination Evidence of damage to pollution prevention measures Evidence of historic contamination, historical site investigation, assessments, remediation and verification reports. Baseline soil and groundwater reference data. | None noted Polluting substances – None noted Previous use prior to 2026 – Arable Farming. None noted None noted Not applicable No formal assessments have been made of the soils on site. |
| Evidence of historical contamination | N/A |
| 3.0 Permitted Activities | The site consists of four poultry houses for Broiler chicken. This will give a total of 180000 places. LPG heaters for heating the poultry houses. Houses will be fitted with heat exchangers. The working area where vehicles operate is laid to concrete. Feed is delivered in covered lorries and stored on site in vermin proof steel galvanised bins. Immediately following depopulation, litter is removed from sheds and sold to an AD plant. The houses are then washed and disinfected prior to the cycle beginning again. Underground storage tanks will have been installed to catch all wash waters. Dead birds are removed from the houses and stored in sealed containers awaiting collection from a licensed renderer in a temperature controlled store. Diesel fuel storage is in a bunded tank/storage area. |
| Non permitted activities undertaken | N/A |
| Document References | Location Plan Layout/Drainage Plan H1 Assessment |
| 4.0 Changes to Activity | N/A |
| 5.0 Measures taken to protect Land | Site will be operated in compliance with “how to comply” routine maintenance schedules are followed and recorded and with any abnormal operations recorded. |
| 6.0 Pollution Incidents | N/A |
| 7.0 Soil, gas and water quality monitoring | No monitoring will be undertaken at the installation. |

Version 1 July 2026