Site Condition Report

| 1.0 SITE DETAILS |  |
| --- | --- |
| Applicants Name | Catfoss Airfield Ltd |
| Activity Address | Bewholme Farm Poultry Unit Bewholme Farm , New Lord Mayors Development Brandesburton Driffield East Riding YO25 8ES |
| National Grid Reference | 513625 , 449236 |
| Document Reference for Site Condition Report at permit application and surrender | 15/07/2026 |
|  | Location Plan |
|  | Layout/Drainage Plan |

| 2.0 Condition of the Land at Permit issue |  |
| --- | --- |
| Environmental setting including: Geology Hydrogeology Surface Waters | The poultry unit lies on level ground to the northeast of Brandesburton. Predominant land use is arable farming. Field pattern is semi large scale with a strong pattern of field boundaries. There are sensitive receptors within 400m of site. Topography and Drainage The poultry houses are between 20 and 25 metres above sea level. Soil bunding and tree planting will help to minimise the visual intrusion normally associated with poultry units. Site clean water drainage all goes to French drains and soakaway trenches. Geology and Hydrogeology According to the BGS Extract Map shows the site geology to be of Rock Type Mercia mudstone group - Mudstone formation. Searches indicate the site is underlain with a Bedrock Aquifer classified Principal. Superficial drift aquifer classified – Secondary (Undifferentiated) The Site is situated in a Groundwater Vulnerability Zone, medium. The site is situated within a Surface Nitrate Vulnerable Zone. The site is not situated in, or within 250m of its boundary, to a Groundwater/Source water protection zone. The site is situated in a drinking water protection/safeguard zone (surface) Hydrology General drainage is in a northerly direction. Average rainfall for this area is 1100 mm. |
| Pollution history including: Pollution incidents that may have affected land Historical land uses and associated contaminants Any visual/olfactory evidence of existing contamination Evidence of damage to pollution prevention measures | None noted Polluting substances – None noted Previous use prior to 2026 – Poultry production. None noted None noted |
| Evidence of historical contamination | N/A |
| 3.0 Permitted Activities | Four poultry houses for Broiler chicken. This will give a total of 176,000 broiler places or 61,493 hen turkeys. Biomass heating with supplementary LPG heaters will provide heating the poultry houses. The working area where vehicles operate is laid to concrete. Dust deposited on grassed area by end gable fans will prevent run off to water course. Feed is delivered in covered lorries and stored on site in vermin proof steel galvanised bins. Immediately following depopulation, litter is removed off site and sold. The houses are then washed and disinfected prior to the cycle beginning again. Underground storage tanks will have been installed to catch all wash waters. Dead birds are removed from the houses and stored in sealed containers awaiting collection. Diesel fuel storage is in a bunded tank/storage area. |
| Non permitted activities undertaken | N/A |
| Document References | Location Plan Layout/Drainage Plan H1 Assessment |
| 4.0 Changes to Activity | Operator is converting the existing 4 poultry houses into two houses and adding an additional third poultry house. Broiler places increasing to 260,000 and removing the option of rearing hen turkeys. Additional ground to be included in the installation boundary. Heating will be from the permitted biomass boiler (1.022MWth, first commissioned in 2017) and supplementary LPG heaters. |
| 5.0 Measures taken to protect Land | Site will be operated in compliance with “how to comply” routine maintenance schedules are followed and recorded and with any abnormal operations recorded. |
| 6.0 Pollution Incidents | N/A |
| 7.0 Soil, gas and water quality monitoring | No monitoring will be undertaken at the installation. |