Noise Management Plan (NMP) – Manor Farm Eggs

This Noise Management Plan (NMP) has been prepared in accordance with Environment Agency guidance on noise and vibration management for environmental permits and Appendix 5 of the EPR Intensive Farming guidance on noise management at intensive livestock installations.

The purpose of this plan is to:

- identify potential sources of noise associated with the installation,

- assess the risk of noise impact on sensitive receptors,

- set out management and control measures to prevent or minimise noise emissions,

- describe the procedure for responding to noise complaints.

The installation is an enriched colony laying hen unit. Birds are housed within purpose-built poultry houses where noise levels are highest. Noise emissions reduce with distance from the houses due to attenuation from buildings, yard layout and surrounding land.

Sensitive Receptors

The installation is located within the rural village of Sinderby, North Yorkshire, and comprises purpose-built poultry housing and associated infrastructure. The surrounding area includes residential development associated with the village, together with isolated rural dwellings and agricultural holdings.

| Receptor Name | Description | Distance | Orientation |
| --- | --- | --- | --- |
| Manor Farm | Farm/staff property | Onsite |  |
| The Granary | Farm/staff property | Onsite |  |
| The Grange | Residential | 35m | SE |
| Sinderby Houe | Residential | 85m | E |
| Honeysuckle House | Residential | 81m | E |
| Anfield | Residential | 45m | E |
| Waverley House | Residential | 55m | NE |
| Waverley Cottage | Residential | 58m | NE |
| Fairfield Cottage | Residential | 60m | NE |
| Fairfields | Residential | 58m | N |
| 1-5 Sinderby Cottage | Residential | 93- 101 m | NE |
| Waldon House | Residential | 77m | NE |
| Aspen House | Residential | 88 m | NE |
| Northgate Lodge | Residential | 97m | NE |
| Keepers Cottage | Residential | 112m | NE |
| Church Farm | Residential | 122m | NE |
| Holmlea | Residential | 127m | NE |
| Masons Garth | Residential | 146m | NE |
| The Cottage | Residential | 170m | NE |
| Wayside | Residential | 187m | NE |
| Hawthorn House & Cottage | Residential | 208m | NE |
| Downholme, Sinderby Lodge, Lynville House & Marlow House | Residential | 222- 250m | NE |
| 1-2 Sinderby Farm | Residential | 113 – 130m | NE |
| The Cottages (Rosslyn, Garden, Spring, Middling, The Old Post Office) | Residential | 190-222m | NE |
| Devonshire House & Pear Tree House | Residential | 150-155m | NE |
| Village Green (1-4) | Residential | 200-350m | NE |
| East Lodge & Maythorn Cottage | Residential | 358-372m | NE |
| Kingsley House, Cherry Haven & East field | Residential | 328m-365m | NE |
| Village Farm (Maran, Orpington, Wyandotte, Sebright Cottage | Residential | 270-327m | NE |

Noise-generating activities primarily occur within enclosed poultry houses or during daytime operational activities. Noise levels reduce with distance due to attenuation from buildings, site layout and surrounding land use.

The identification and grouping of receptors has informed the development of noise control and management measures set out in this Noise Management Plan to ensure the continued protection of nearby noise-sensitive receptors.

Noise Risk Assessment Approach

Noise risks have been assessed by:

- identifying site activities that may generate noise,

- considering frequency, duration and timing of noise,

- applying the hierarchy of control set out in EA guidance:

  - Prevent noise at source through good design and maintenance

  - Minimise noise through good operational practice

  - Increase distance between source and receptor

  - Use buildings and site layout to screen noise

  - Control timing of unavoidable noisy activities

Potential Noise Sources

The main potential sources of noise associated with the installation are:

- Ventilation fans

- Feed deliveries

- Feeding systems

- Egg collection and handling

- Fuel deliveries

- Alarm systems

- Bird catching / depopulation

- Manure removal and clean-out operations

- Maintenance and repair activities

- Set-up and placement activities

- Standby generator testing and use

Noise Control & Management Measures

The table below identifies the potential sources of noise associated with the installation and the measures in place to prevent or minimise noise emissions. The controls reflect best available techniques (BAT) and good operational practice for an enriched colony laying hen unit.

| Noise Source / Activity | Potential Noise Issue | Noise Control and Management Measures | In Place |
| --- | --- | --- | --- |
| Ventilation fans | Continuous operational noise | Noise assessed during daily inspections at 7:00-8:30, 13:00-14:30 & 19:00-20:30 High-capacity fans used to minimise total fan numbers Fans operated and adjusted to flock age and weather conditions Routine maintenance is undertaken in line with the Routine Inspection and Maintenance Schedule All Staff trained and advised to report any excessive noise or faults immediately to site manager for immediate shutdown until repair | YES YES YES YES YES |
| Feed deliveries | Vehicle movements and unloading noise | Deliveries scheduled during normal working hours (7:00-17:00) Bulk deliveries used to reduce frequency Delivery vehicles use established access routes Unloading undertaken efficiently to minimise duration. | YES YES YES YES |
| Feeding systems | Noise from augers and feed distribution | Feed bins monitored daily to prevent augers running empty Feeding systems checked as part of daily inspections at 7:00-8:30, 13:00-14:30 & 19:00-20:30 Systems maintained and serviced to prevent abnormal noise | YES YES YES |
| Egg collection and handling | Vehicle and handling noise | Egg collection and handling undertaken during normal working hours 7:00 – 16:00 Careful handling of pallets and equipment Vehicle movements minimise and managed to reduce unnecessary noise. | YES YES YES |
| Fuel deliveries | Vehicle movements and unloading noise | Fuel deliveries restricted to normal working hours (7:00-17:00) Deliveries planned to minimise frequency. | YES YES |
| Alarm systems | Intermittent alarm noise | A single alarm system is in use. The alarm operates only during the early rearing stage of young birds and is time limited. The alarm is audible within the farmhouse and is not designed to operate continuously or externally. Alarm activations are investigated immediately and the system is reset once normal conditions are confirmed. | Yes |
| Bird catching and depopulation | Human activity, vehicle and crate handling noise | Bird catching and depopulation are carried out by the operator’s own trained staff. Staff are trained to handle birds calmly and quietly to minimise stress to the birds and associated noise. The installation operates an enriched colony cage system, which results in lower noise levels during depopulation compared to loose-housed systems. Catching and removal occur infrequently, typically once every approximately 20 weeks. Vehicle movements are managed to minimise engine noise and unnecessary idling. Activities may occasionally take place during unsociable hours to meet slaughterhouse scheduling and bird welfare requirements; however, noise duration is limited and activities are completed as efficiently as possible. | YES YES YES YES YES YES |
| Manure removal and clean-out | Loader and vehicle noise | Activities undertaken during normal working hours (7:00-17:00) Trailers positioned close to houses to reduce travel distances; larger trailers used to reduce vehicle movements Washing carried out during daytime hours. | YES YES YES |
| Maintenance and repair activities | Noise from tools and machinery | Routine maintenance undertaken between flocks where possible Non-essential works restricted to normal working hours (7:00-17:00) Emergency repairs only undertaken outside these hours where necessary. | YES YES YES |
| Set-up and placement activities | Vehicle and equipment noise | Activities planned and undertaken during normal working hours(7:00-17:00) duration minimized through planning. | YES |
| Standby generator testing and use | Engine noise | Generator testing restricted to normal working hours Generator located to benefit from screening provided by buildings; use limited to emergency situations. | YES YES |

Monitoring & Inspections

Noise is monitored through:

- daily site inspections,

- observation during routine operations,

- investigation of any reported abnormal noise,

- maintenance and servicing records.

Any abnormal or excessive noise is investigated immediately and corrective action implemented.

Noise Complaints Procedure

In the event of a noise complaint:

- the complaint is recorded using the Noise Complaint Record,

- the site is inspected as soon as practicable,

- the potential source of noise is identified,

- corrective and contingency measures are implemented where required,

- the complainant is contacted with the outcome of the investigation,

- the Environment Agency is notified where appropriate.

All complaints and actions taken are recorded and retained on site.

This plan will be reviewed every four years or following any substantiated Noise Complaint. Any changes will be communicated to the Environment Agency Area Officer for approval.

Noise Complaint Form _ Manor Farm Eggs

| Installation Name |  | Date Recorded |  | Reference Number |
| --- | --- | --- | --- | --- |
| Complainant Details |  |  |  |  |
| Name |  |  |  |  |
| Address |  |  |  |  |
| Tel. No. of |  |  |  |  |
| Location of caller in relation to installation |  |  |  |  |
| Time and Date of complaint |  |  |  |  |
| Date, Time and duration of offending noise |  |  |  |  |
| Has the caller any other comments about the noise? |  |  |  |  |
| Weather conditions/ Wind Speed & Direction |  |  |  |  |
| Any previous complaints relating to this Noise? |  |  |  |  |
| Any other relevant information |  |  |  |  |
| Potential Noise sources that could give rise to the complaint |  |  |  |  |
| Operating conditions at the time offending noise occurred |  |  |  |  |
| Follow up Date and time caller contacted |  |  |  |  |
| Action taken |  |  |  |  |
| Amendment requirement to Noise Management Plan |  |  |  |  |
| Form completed by |  | Signed |  |  |