Dust/Bioaerosol Management Plan (DBMP) – Manor Farm Eggs

This Dust and Bioaerosol Management Plan (DBMP) has been prepared to identify potential sources of dust and bioaerosols arising from the operation of the installation and to set out the measures in place to minimise emissions and protect nearby receptors.

The installation operates an enriched colony laying hen system. There is no littered housing, no bedding, no composting, no shredding or screening activities, and no long-term storage of manure or slurry on site. These factors result in a low potential for dust and bioaerosol emissions.

Sensitive Receptors

| Receptor Name | Description | Distance | Orientation |
| --- | --- | --- | --- |
| Manor Farm | Farm/staff property | Onsite |  |
| The Granary | Farm/staff property | Onsite |  |
| The Grange | Residential | 35m | SE |
| Sinderby Houe | Residential | 85m | E |
| Honeysuckle House | Residential | 81m | E |
| Anfield | Residential | 45m | E |
| Waverley House | Residential | 55m | NE |
| Waverley Cottage | Residential | 58m | NE |
| Fairfield Cottage | Residential | 60m | NE |
| Fairfields | Residential | 58m | N |
| 1-5 Sinderby Cottage | Residential | 93- 101 m | NE |
| Waldon House | Residential | 77m | NE |
| Aspen House | Residential | 88 m | NE |
| Northgate Lodge | Residential | 97m | NE |

Potential Sources of Dust & Bioaerosols

The main potential sources of dust and bioaerosols associated with the installation are:

- Feed deliveries

- Feed milling and mixing

- Feed distribution systems

- Bird activity

- Ventilation exhaust air

- Manure handling and removal

- Cleaning and washing operations

No bedding or litter management activities are undertaken.

Feed Milling and Mixing – Dust Control Measures

Feed milling and mixing is undertaken within a fully enclosed building with solid walls and roof. The building has no open sides or permanent openings, and access doors are kept closed while milling and mixing operations are underway to ensure dust is contained within the structure.

Dust generated during milling and mixing is controlled at source through the use of fabric dust socks fitted directly above the mill. These capture airborne particulate matter within the building and prevent the release of dusty air to the external environment. There are no external exhaust points, stacks or vents associated with the milling process. Although a fan is present within the building, it is not used during milling and mixing operations and does not act as an air discharge point.

The milling and mixing area is dry-operated. No process water is used, there are no internal drains, and rainwater cannot enter the building. As such, there is no pathway for dust or feed material to enter surface water or site drainage systems.

Good housekeeping is maintained within the milling building. Floors, ledges and equipment are kept clean to prevent dust accumulation. Dust collected via the dust socks and housekeeping activities is placed into the manure bucket and removed from site with manure for recovery via biomass.

In the event of excessive dust generation, milling activities are reviewed and corrective actions implemented. This may include cleaning or replacement of dust socks and additional housekeeping before normal operations resume.

Dust and Bioaerosol Control Measures

| Source | Method | Reduction Technique | Implementation |
| --- | --- | --- | --- |
| Feed deliveries | Delivery to silos | Feed delivered into sealed silos; delivery pipework enclosed to minimise dust release | In place |
| Feed milling and mixing | On-site processing | Milling and mixing undertaken using controlled equipment within an enclosed building; handling minimised; work areas kept clean and dry; doors kept closed while operational; dust captured at source using fabric dust socks | Continuous |
| Feed distribution | Internal feed systems | Enclosed feed transfer systems; controlled feeding rates to prevent overfilling and spillage | Continuous |
| Feed spillages | Housekeeping | Any feed spillages cleared immediately to prevent dust generation | Continuous |
| Bins and pipework | Equipment integrity | Daily visual inspection of bins and pipework; defective equipment isolated and repaired | In place |
| Bird activity | Housing system | Enriched colony system limits bird contact with manure; stocking densities within permit limits | In place |
| Ventilation | Exhaust air | Mechanical ventilation maintained and adjusted to bird age; exhaust air released at height to aid dispersion | Continuous |
| Manure handling | Belt removal | Manure removed frequently via belt systems; no long-term storage; manure transferred directly to covered transport and removed promptly. No storage of manure ono site | In place |
| Cleaning and washing | Wash-down operations | Houses pre-soaked prior to washing; washing undertaken in a controlled manner; wash water collected and removed off site | In place |

Bioaerosol Risk Assesment

The installation is considered to have a low bioaerosol risk profile due to:

- Enriched colony housing system (no litter)

- No composting, shredding or screening activities

- No on-site storage of manure or slurry

- Frequent manure removal and prompt removal of wash water

- Controlled ventilation and good housekeeping

Bioaerosol emissions are therefore controlled through standard good husbandry, hygiene and ventilation practices.

Monitoring

Routine instrumental monitoring of dust or bioaerosols is not considered necessary or proportionate given the low risk profile of the installation.

Monitoring is undertaken through:

- Operator awareness

- Routine site inspections

- Observation of abnormal conditions or activities

Further investigation would be undertaken in response to any substantiated complaint or abnormal operational conditions.

Complaints Procedure

In the event of a dust or bioaerosol complaint being received:

- The source of the complaint will be investigated immediately

- Appropriate mitigation measures will be implemented to cease or minimise emissions

- The Environment Agency Area Officer will be notified if required

- The complainant will be contacted following the investigation and informed of the findings and remedial actions taken

- A complaint record will be completed and retained on site

- The Dust and Bioaerosol Management Plan will be reviewed and updated if necessary, with any changes submitted to the Environment Agency for approval

This plan will be reviewed every four years or following any substantiated dust or bioaerosol complaint. Any changes will be communicated to the Environment Agency Area Officer for approval.

Dust/Bioaerosol Complaint Form _ Manor Farm Eggs

| Installation Name |  | Date Recorded |  | Reference Number |
| --- | --- | --- | --- | --- |
| Complainant Details |  |  |  |  |
| Name |  |  |  |  |
| Address |  |  |  |  |
| Tel. No. of |  |  |  |  |
| Location of caller in relation to installation |  |  |  |  |
| Time and Date of complaint |  |  |  |  |
| Date, Time and duration of offending Dust/Bioaerosol |  |  |  |  |
| Has the caller any other comments about the Dust/Bioaerosol? |  |  |  |  |
| Weather conditions/ Wind Speed & Direction |  |  |  |  |
| Any previous complaints relating to this Dust/Bioaerosol? |  |  |  |  |
| Any other relevant information |  |  |  |  |
| Potential Dust/Bioaerosol sources that could give rise to the complaint |  |  |  |  |
| Operating conditions at the time offending Dust/Bioaerosol occurred |  |  |  |  |
| Follow up Date and time caller contacted |  |  |  |  |
| Action taken |  |  |  |  |
| Amendment requirement to Dust/Bioaerosol Management Plan |  |  |  |  |
| Form completed by |  | Signed |  |  |