Odour Management Plan (OMP) – Manor Farm Eggs

Doc Ref Doc 9 OMP-Revised

This Odour Management Plan (OMP) has been prepared as part of the environmental permit application for Manor Farm Eggs, Sinderby.

The plan identifies potential sources of odour, assess associated risks and set out management and control measures to prevent odour emissions where practicable and minimise impacts where prevention is not possible in accordance with BAT ad EA Guidance Note EPR6.09

The OMP operates in conjunction with:

- Environmental Management System

- Routine inspection and Maintenance Schedule version1

- Emergency and Contingency Plan version 1

- Flock Health Plan

- Environmental Risk Assessment

Responsibility for implementation rests with the operator or nominated deputies.

Site Description

Manor Farm Eggs consists of eleven laying houses and three rearing house within a single installation boundary.  The installation comprises laying hen accommodation operating with manure belt systems, controlled mechanical ventilation and regular manure removal. The site is predominantly hardstanding/concrete, supporting good housekeeping and drainage control. There is no long-term storage of manure on site.

Prevailing wind direction for the region, which is typically south-westerly, has been considered alongside site layout and the location of sensitive receptors. Ventilation systems are operated to promote effective dispersion of exhaust air, and operational experience confirms no adverse odour impacts at site boundaries.

Sensitive Receptors

The installation is located within the rural village of Sinderby, North Yorkshire, which contains a number of residential properties.

Sensitive receptors within approximately 400 metres of the installation boundary

| Receptor Name | Description | Distance | Orientation |
| --- | --- | --- | --- |
| Manor Farm | Farm/staff property | Onsite |  |
| The Granary | Farm/staff property | Onsite |  |
| The Grange | Residential | 35m | SE |
| Sinderby Houe | Residential | 85m | E |
| Honeysuckle House | Residential | 81m | E |
| Anfield | Residential | 45m | E |
| Waverley House | Residential | 55m | NE |
| Waverley Cottage | Residential | 58m | NE |
| Fairfield Cottage | Residential | 60m | NE |
| Fairfields | Residential | 58m | N |
| 1-5 Sinderby Cottage | Residential | 93- 101 m | NE |
| Waldon House | Residential | 77m | NE |
| Aspen House | Residential | 88 m | NE |
| Northgate Lodge | Residential | 97m | NE |
| Keepers Cottage | Residential | 112m | NE |
| Church Farm | Residential | 122m | NE |
| Holmlea | Residential | 127m | NE |
| Masons Garth | Residential | 146m | NE |
| The Cottage | Residential | 170m | NE |
| Wayside | Residential | 187m | NE |
| Hawthorn House & Cottage | Residential | 208m | NE |
| Downholme, Sinderby Lodge, Lynville House & Marlow House | Residential | 222- 250m | NE |
| 1-2 Sinderby Farm | Residential | 113 – 130m | NE |
| The Cottages (Rosslyn, Garden, Spring, Middling, The Old Post Office) | Residential | 190-222m | NE |
| Devonshire House & Pear Tree House | Residential | 150-155m | NE |
| Village Green (1-4) | Residential | 200-350m | NE |
| East Lodge & Maythorn Cottage | Residential | 358-372m | NE |
| Kingsley House, Cherry Haven & East field | Residential | 328m-365m | NE |
| Village Farm (Maran, Orpington, Wyandotte, Sebright Cottage | Residential | 270-327m | NE |

The installation has no history of substantiated odour complaints, demonstrating that existing control measures are effective in protecting nearby receptors.

Odour Monitoring and Assessment

Odour monitoring is undertaken as part of routine site management to ensure that odour emissions from the installation do not cause nuisance at nearby sensitive receptors.

Site staff carry out daily inspections, including checks of poultry houses, manure handling systems, dirty water storage, mortality storage and other potentially odorous activities. These inspections include visual and nasal assessment to identify any abnormal odour conditions.

In addition, a daily boundary walk is undertaken, where practicable, to assess odour at and beyond the installation boundary, with particular attention given to the locations of the nearest residential receptors.

Odour checks are also undertaken periodically by persons who do not routinely work on the farm, such as visiting contractors, delivery drivers or family members, to provide an independent assessment of odour beyond the immediate operational environment.

Prevailing weather conditions, including wind direction and strength, are taken into account during monitoring to assess the potential for off-site odour impacts.

Where abnormal odour is identified during routine monitoring, or where an odour complaint is received, the Operator will immediately investigate the potential source and implement the appropriate measures set out in this Odour Management Plan and the Odour Contingency Table.

All odour monitoring findings, complaints and actions taken are recorded and retained on site and made available to the Environment Agency upon request.

Odour Risk Assessment and Control Measures

Table 1: Odour Sources, Risks and Control Measures

| Odour related Activity | Potential Risks & Problems | Actions Taken | Completion Date |
| --- | --- | --- | --- |
| Poultry housing | Odour from manure accumulation and bird activity | Manure belt systems operated every 2-3 days to prevent manure build-up; manure is maintained in a dry condition; stocking densities maintained in line with welfare and permit requirements; wet areas identified during daily inspections and addressed immediately | In Place – Monitored daily |
| Ventilation systems | Poor dispersion of exhaust air; increased humidity leading to odour | Ventilation systems operate continuously during bird occupation; settings are checked daily and adjusted to flock age and weather conditions; fans are cleaned and maintained | In place – Checked Daily |
| Manure handling and removal | Short-term odour during manure removal activities | Manure removed from site within 24 hours of belt removal; no long-term storage of manure on site; manure loaded directly to covered transport and removed off site to biomass plant; spillages avoided and cleaned immediately if they occur | In Place |
| Mortality storage and disposal | Odour from decomposition of mortalities | Mortalities removed from houses daily; stored in sealed, vermin-proof containers; containers located away from sensitive site boundaries; collections arranged to prevent container capacity being exceeded. Unused emergency storage is available should collection be delayed by supplier | In place |
| Feed storage, milling and mixing (on site) | Odour from feed ingredients during handling/milling/mixing; spills; spoiled feed; dust and odour release during transfer | Feed and ingredients stored in covered/contained areas and kept dry; good stock rotation to prevent spoilage; milling/mixing undertaken using enclosed/controlled equipment and kept maintained to minimise releases; handling kept to the minimum required; spillages cleaned up immediately; work areas kept clean and dry and tidies at the end of each day, operations avoided during adverse weather conditions where this can be achieved without compromising feed supply and doors kept closed during processing | In Place |
| Cleaning and disinfection activities | Odour from wash water or use of cleaning and disinfection products | Only approved and suitable cleaning products used; washing activities managed to ensure effective drainage; wash water collected and removed off site; good housekeeping maintained throughout cleaning operations Wash water levels monitored during washing and emptied as required to prevent overfill. | In Place |
| Washing operations including vehicles | Odour from wash water, organic residues or cleaning products during washing operations | Washing operations managed to minimise odour; designated wash areas used; approved cleaning products only; wash water directed to appropriate drainage and removed off site; good housekeeping maintained during and after washing; vehicles washed as required to maintain biosecurity and cleanliness. Dirty water tanks monitored during wash down to maintain freeboard | In Place |
| Fugitive emissions | Uncontrolled odour release from doors, openings, equipment or during routine activities | Buildings and equipment maintained in good condition; doors kept closed when not in use; Daily inspections undertaken to identify leaks or defects; prompt repair of any damaged structures or equipment; activities managed to minimise unnecessary opening of buildings | In Place |
| Dirty water management | Odour from standing or poorly managed dirty water | Dirty water managed to prevent stagnation; drainage systems kept clear and functional; dirty water collected and removed off site as required; tanks and sumps monitored to maintain capacity; no discharge to land or watercourses | In place |
| Abnormal operations | Increased odour risk from equipment failure, leaks or bird health issues | Abnormal events are investigated immediately; corrective action implemented on the same day; increased monitoring undertaken until normal conditions are restored; events recorded | In place |
| Waste production and storage | Odour from accumulation or poor storage of waste | Odorous waste is not stored on site; waste is removed off site by authorised contractors at least weekly | In Place |
| Egg collection and handling | Odour from broken eggs or egg residues | Eggs are collected daily as part of routine production activities; broken or damaged eggs are removed from the production area on the same day they are identified; broken eggs are placed directly into a sealed waste container; egg handling areas are kept clean and residues removed daily | In Place |

Odour Complaints procedure

Regular contact is maintained with neighbouring residents, and the site receives daily visits from customers and contractors, providing continual informal confirmation that odour impacts are not occurring.

In the event that an odour complaint is received, the Operator will notify the Environment Agency immediately, make a record of the complaint and implement the appropriate odour contingency measures as set out in this Odour Management Plan.

The following procedure will be implemented:

- All odour complaints are recorded on the day of receipt, including the date, time, location of the complainant and prevailing weather conditions.

- Site staff are alerted immediately and a site inspection is undertaken on the same day to identify any potential source of odour.

- Where a source of odour is identified, the actions set out in this Odour Management Plan and the Odour Contingency Table are implemented without delay to minimise or cease the release.

- Following implementation of corrective actions, further checks are carried out to confirm the effectiveness of the measures.

- The complainant is contacted and informed of the findings of the investigation and any remedial actions taken.

- Where an odour complaint is substantiated or an off-site odour impact is confirmed, the Environment Agency Area Officer is notified without delay.

- All substantiated odour complaints trigger a review of this Odour Management Plan at the earliest opportunity. Any amendments are documented, dated and implemented.

A completed odour complaint record is retained on site and made available to the Environment Agency upon request.

No substantiated odour complaints have been received to date.

Odour Contingencies Table

| Source | Potential Cause | Trigger Factor with Immediate Action | Mitigation Measures to be implemented & Implemented until cessation trigger verified | Additional Mitigation with timings | Cessation Trigger |
| --- | --- | --- | --- | --- | --- |
| Feed Delivery & Storage | Feed Bin or Pipework Failure causing Spillage | Identified during daily inspection or odour detection | Feed delivery stopped immediately; damaged pipework or bin isolated; spillages cleaned on the same day; alternative bin used where available | Damaged pipework or bin repaired or replaced within 48 hours | Visual confirmation of repair and no residual odour |
| Carcase storage and disposal | Damage to mortality container or delayed collection | Identified during daily inspection or odour detection | Mortalities transferred immediately to a secure, sealed container; damaged container repaired or replaced without delay | Spare sealed mortality storage available on site and used immediately if collection is delayed; collection by licensed contractor arranged within 24 hours | Secure storage confirmed and no odour detected |
| Egg production | Drop in egg production indicating possible bird health or environmental issue | Deviation from normal egg production levels identified during daily production checks | Bird health investigated immediately; veterinarian contacted where required; ventilation rates and manure belt operation reviewed and adjusted as necessary to minimise odour generation | Increased monitoring of bird condition and housing environment daily until egg production stabilises | Egg production returned to normal levels |
| Ventilation systems | Fan or control system failure | Alarm activation or fault identified | Alternative ventilation activated immediately; system operated using remaining installed fans designed to maintain adequate ventilation; | Mobile generator deployed within 1 hour if required electrician called out immediately | Ventilation system fully operational and corrective actions documented |
| Drinker systems | Leaking drinkers or pipe failure causing wet manure | Abnormal water consumption identified or leak observed during inspection | Leak isolated and repaired immediately; manure belts and ventilation adjusted to restore dry manure conditions and minimise odour generation | Increased monitoring of water consumption and manure condition at least twice daily until resolved | Water consumption returned to normal levels and manure dry |
| Bird depletion | Fugitive odour release following depletion | Raised odour levels identified during site inspection or odour detection | Minimum ventilation maintained; houses sealed following depletion | OMP reviewed within 24 hours if required and findings recorded | No abnormal odour detected during follow-up site inspection |
| Manure removal | Fugitive odour during manure removal activities | Raised odour levels identified during site inspection or odour detection | Manure removal activities adjusted immediately; ventilation settings optimised to aid dispersion and minimise odour during removal | Manure removal procedures reviewed and amended within 24 hours if required | No abnormal odour detected during follow-up site inspection |
| Dirty water storage | Odour from standing or stagnant dirty water | Odour detected during site inspection | Dirty water tanks inspected immediately; dirty water tanks filled with clean water and agitated prior to removal to prevent sediment build-up and stagnation | Dirty water removed off site using on-farm slurry tanker within 24 hours | No standing water present and no abnormal odour detected |
| Drainage system | Blocked drains causing standing dirty water | Standing water or odour detected during inspection | Drainage inspected immediately; blockage cleared without delay | Specialist drainage contractor engaged within 24 hours if clearance not achieved | Drainage flowing freely and no abnormal odour detected |
| Dirty water removal | Delay in removal of dirty water from site | Dirty water remaining on site longer than planned | Dirty water removed using on-farm slurry tanker | Standby slurry tanker deployed within 24 hours in the event of equipment failure | Dirty water removed from site and no abnormal odour detected |

OMP reviewed in line with the review requirements set out in this plan, following a substantiated complaint or any changes to operations.

If substantiated odour complaints are received over a one-month period without resolution to odour problem a formal action plan with measures and timescales for relevant odour issues shall be presented to the Environment Agency for approval

This plan will be reviewed every four years or following any substantiated odour complaints. Any changes will be communicated to the Environment Agency Area Officer for approval.

Odour Complaint Form _ Manor Farm Eggs

| Installation Name |  | Date Recorded |  | Reference Number |
| --- | --- | --- | --- | --- |
| Complainant Details |  |  |  |  |
| Name |  |  |  |  |
| Address |  |  |  |  |
| Tel. No. of |  |  |  |  |
| Location of caller in relation to installation |  |  |  |  |
| Time and Date of complaint |  |  |  |  |
| Date, Time and duration of offending odour |  |  |  |  |
| Has the caller any other comments about the odour? |  |  |  |  |
| Weather conditions/ Wind Speed & Direction |  |  |  |  |
| Any previous complaints relating to this odour? |  |  |  |  |
| Any other relevant information |  |  |  |  |
| Potential odour sources that could give rise to the complaint |  |  |  |  |
| Operating conditions at the time offending odour occurred |  |  |  |  |
| Follow up Date and time caller contacted |  |  |  |  |
| Action taken |  |  |  |  |
| Amendment requirement to Odour Management Plan |  |  |  |  |
| Form completed by |  | Signed |  |  |

In the event of odour complaints being received the Operator will notify the Environment Agency and make a record of the complaint. The Operator will undertake the necessary odour contingency as required.